3GPP TSG-RAN5 Meeting #91-e	R5-213646

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0209 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICs for SST handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The URLLC SST is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following PIC have been added to Table A.4.4-2A: UE APN/DNN Implementation details: pc_APN_ID_URLLC. This is a PIC to specify the APN/DNN for the URLLC SST. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only the eMBB SST will be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 2 revisions of R5-213158. |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.523-1: "5GS; UE conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 38.523-2: “5GS; User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases”.

[4]	3GPP TS 38.523-3: “5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites”.

[5]	3GPP TS 38.521-1: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”.

[6]	3GPP TS 38.521-2: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[7]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[8]	3GPP TS 38.521-4: “NR; User Equipment conformance specification; Radio transmission and reception; Part 4: Performance”.

[9]	3GPP TS 38.522: “NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases”.

[10]	3GPP TS 38.533: “NR; User Equipment (UE) conformance specification; Radio resource management”.

[11]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[12]	3GPP TS 38.509: "5GS; Special conformance testing functions for UE".

[13]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Common Test Environments for User Equipment (UE) Conformance Testing".

[14]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Special conformance testing functions for User Equipment (UE)".

[15]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP);User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) specification".

[16]	3GPP TS 36.523-2: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[17]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[18]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[19]	3GPP TS 38.307: “NR; User Equipments (UEs) supporting a release-independent frequency band”.

[20]	3GPP TS 37.340:"Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[21]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[22]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"

[23]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[24]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[25]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[26]	3GPP TS 23.003: “Numbering, addressing and identification”

## <<<< TEXT SKIPPED >>>>

## A.4.4	Additional information

Table A.4.4-1: Additional information

| Item |  | Additional information |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support of ICMP or ICMP IPv6 |  | RFC 792 OR RFC 4443, RFC 4884 |  | NA |  | pc_IP_Ping |  | UE supports ICMP or ICMPv6 protocol to enable IP Ping Operation |  |
| 2 |  | Support of IMS |  | 24.229, Annex U |  | Rel-15 |  | pc_IMS_5GS |  |  |  |
| 3 |  | Support of rachReport |  | 38.306, 4.2.17 |  | Rel-16 |  | pc_rachReport |  | UE supports delivery of rachReport upon request from the network. |  |
| 4 |  | Support of GNSS |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_GNSS_location |  | UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. |  |
| 5 |  | Support of UL PDCP Packet Average Delay |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_PDCP_Delay |  | UE supports UL PDCP Packet Average Delay measurement and reporting in RRC_CONNECTED state |  |
| 6 |  | Support logged MDT |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_logged_MDT |  | UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. |  |
| 7 |  | Support of uncompensated barometric pressure measurement reporting |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_barometer |  | UE supports uncompensated barometric pressure measurement reporting upon request from the network. |  |
| 8 |  | Support of orientation information reporting |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_orientation |  | UE supports orientation information reporting upon request from the network. |  |
| 9 |  | Support of speed information reporting |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_speed |  | UE supports speed information reporting upon request from the network. |  |
| 10 |  | Support of Bluetooth measurements in RRC_CONNECTED state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_immMeasBT |  | UE supports Bluetooth measurements in RRC_CONNECTED state. |  |
| 11 |  | Support of WLAN measurements in RRC_CONNECTED state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_immMeasWLAN |  | UE supports WLAN measurements in RRC_CONNECTED state. |  |
| 12 |  | Support of Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_loggedMeasBT |  | UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. |  |
| 13 |  | Support of WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  | 38.306, 4.2.18 |  | Rel-16 |  | pc_loggedMeasWLAN |  | UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. |  |

Table A.4.4-2: Definition of UE implementation capabilities

| Item | Definition of UE implementation capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |
| 2 | Void |  |  |  |  |
| 3 | Number of UE-requested PDU session establishments after REGISTRATION during the same signalling connection | 24.501 | Rel-15 | pc_noOf_PDUsSameConnection |  |
| 4 | Number of UE-requested PDU session establishments after REGISTRATION in a new signalling connection | 24.501 | Rel-15 | pc_noOf_PDUsNewConnection |  |
| 5 | Number of UE-requested PDN connection establishments after ATTACH during the same signalling connection | 24.301 | Rel-15 | pc_noOf_PDNsSameConnection |  |
| 6 | Number of UE-requested PDN connection establishments after ATTACH in a new signalling connection | 24.301 | Rel-15 | pc_noOf_PDNsNewConnection |  |
| 7 | Void | 24.501 | Rel-15 | pc_IMS_DNN_default |  |

Table A.4.4-2A: UE APN/DNN Implementation details

| Parameter Name | Parameter Type | Supported Value | Description |
| --- | --- | --- | --- |
| pc_APN_Default_Configuration | enumerated | none, internet, ims, urllc | The DNN/APN configuration specified in TS 38.508-1 [2], Table 4.8.4-1 which is to be used for the default DNN/APN. The value provided shall match one of the DNN/APN types if a Default DNN will be established, e.g. internet, ims, etc. or shall be set to none if the UE will not establish default DNN/APN. |
| pc_APN_ID_Internet | charstring |  | APN/DNN ID of type Internet (NOTE 1) The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [26], subclause 9.1 OR "none" if the UE will not establish PDN/PDU of type Internet If the provided value is different to "none" then for this APN/DNN the DNN/APN configuration of type "Internet" as specified in TS 38.508-1 [2], Table 4.8.4-1 applies. |
| pc_APN_ID_IMS | charstring |  | APN/DNN ID of type IMS (NOTE 1) The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [26], subclause 9.1 OR "none" if the UE will not establish PDN/PDU of type IMS If the provided value is different to "none" then for this APN/DNN the DNN/APN configuration of type "IMS" as specified in TS 38.508-1 [2], Table 4.8.4-1 applies. |
| pc_APN_ID_URLLC | charstring |  | APN/DNN ID of type URLLC (NOTE 1) The APN/DNN Network Identifier portion of the Access Point / Data Network Name, as defined in TS 23.003 [26], subclause 9.1 OR "none" if the UE will not establish PDN/PDU of type URLLC If the provided value is different to "none" then for this APN/DNN the DNN/APN configuration of type "URLLC" as specified in TS 38.508-1 [2], Table 4.8.4-1 applies. |
| NOTE 1: For each UE, the APN/DNN IDs which will be used during for PDN/PDU establishment shall be provided. These shall cover both: The APN/DNN IDs which the UE will provide itself in the PDN/PDU establishment request, and, An APN/DNN ID which the UE will prefer to be assigned by the SS in the case of Default APN/DNN, if the UE utilises Provided and/or Default APN/DNN. |  |  |  |