3GPP TSG-RAN5 Meeting #91-e						           Draft_R5-213803

Electronic Meeting, 17th May – 27th May 2021

Title:	Response to LS on band dependent parameters for the FR2 demodulation setup

Response to:

Release:	Release 17

Work Item:	FR2_enhTestMethods

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	Ashwin Mohan

Tel. Number:

E-mail Address	ashwin_mohan@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN5 thanks RAN4 for the LS on band dependent parameters for the FR2 demodulations setup.

RAN5 would like to provide the following responses to the questions raised in the RAN4 LS.

Question 1: Can RAN5 share any comments regarding the preliminary assessment of the demodulation test setup SNR calculation parameters in Table 1?

Please note that RAN5 has agreed to:

- Calculate achievable SNR for each frequency range: FR2a/FR2b/FR2c (endorsed in R5-206614)

- Include Fading Crest factor margin in computation of achievable SNR. (endorsed in R5-211947)

- Apply the highest Noc level among the bands of the respective frequency range for that frequency range for calculation of achievable SNR (endorsed in R5-211950)

- Use backoff from P1dB of 17.71 dB as working assumption (Annex D.3.2.2 in TS 38.903)

- For IFF and QZ=30cm, use 1 meter range length and 0.75 dB for MBR (endorsed in R5-206614)

Question 2: Can RAN5 share any updates related to FR2b, so that these can be applied to the SNR calculations for n262?

Please note that RAN5 has agreed to

- Assume maximum testable SNRBB of [7.3 dB] for FR2b (up to 40 GHz) under fading conditions as per the parameter values defined in TR 38.810

- In general, consider the faded signal (signal after channel emulation) crest factor for computing the achievable SNR in FR2 system

Any deviation from TR 38.810- values for backoff from P1dB and probe antenna gain, specific to FR2b, are pending discussion in RAN5

Following is the current working assumption for the max testable SNRbb updated in TR 38.903 for the FR2 bands defined in 38.101-4 with and without fading. RAN5 is still discussing on some of the key parameters such as “available DL power at CW 1 dB compression at QZ, dBm” and hence the max testable SNRbb for n260 band (also referred to as FR2b) is currently TBD in TR 38.903.

Table D.3.2.2-1: Predicted SNRBB upper bound values for Indirect far field (IFF) with 30cm QZ, PC3, 100MHz CHBW under fading conditions

|  | Operating Band | Maximum SNRBB (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| Multi-band UE (Note) | n257 | [24.0] | [20.8] | [17.8] |
|  | n258 | [24.0] | [20.8] | [17.8] |
|  | n259 | TBD | TBD | TBD |
|  | n260 | TBD | TBD | TBD |
|  | n261 | [24.0] | [20.8] | [17.8] |
| Note: For ∑MBp from TS 38.101-2 [16] Table 6.2.1.3-4 allow up to 0.75 dB in Rel-15. |  |  |  |  |

Table D.3.2.2-2: Predicted SNRBB upper bound values for Indirect far field (IFF) with 30cm QZ, PC3, 100MHz CHBW when no fading conditions apply

|  | Operating Band | Maximum SNRBB (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| Multi-band UE (Note) | n257 | [28.7] | [25.5] | [22.5] |
|  | n258 | [28.7] | [25.5] | [22.5] |
|  | n259 | TBD | TBD | TBD |
|  | n260 | TBD | TBD | TBD |
|  | n261 | [28.7] | [25.5] | [22.5] |
| Note: For ∑MBp from TS 38.101-2 [16] Table 6.2.1.3-4 allow up to 0.75 dB in Rel-15. |  |  |  |  |

2. Actions to RAN4 group.

None

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#92 	 16th – 27th Aug 2021	Electronic Meeting

TSG-RAN5 Meeting#93 	 8th – 19th Nov 2021	Electronic Meeting

4. Appendix

Table 1 included in the RAN4 LS, and referenced in the questions, is included below

Table 1: Proposed demodulation test setup SNR calculation parameters for band n262

| Parameter | Value | Comment |
| --- | --- | --- |
| REFSENS | -82.8 dBm/50 MHz | Using REFSENS agreed for band n262 |
| Multi-band relaxation | 1.0 dB | Defined as ceil(.); change from 2.0 dB |
| FS path loss | -63.2 dB | Change from -62.3 dB (scaling from 43.5 to 48.2 GHz) |
| Cable loss | -8.7 dB | Additional 0.33 dB/m in cable loss at 48.2 GHz |
| Probe antenna gain | [12.0] dB | Needs checking |
| Backoff from P1dB | [13.0] dB | Needs checking |