3GPP TSG-RAN5 Meeting # 91-e	R5-213815

# Electronic Meeting,  17th May - 28th May 2021

Title:	On FR2 General ON/OFF time mask testability, measurement uncertainties and                                                      test tolerances

Source:						Keysight Technologies

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

At RAN5#89e, a discussion paper [1] was presented reviewing current test procedure for FR2 General ON/OFF time mask taking into account implication of OFF power relaxations required due to testability issues.

In such document some changes were proposed to improve the test coverage so higher power values could be tested. However, the ON power increase could cause the degradation of OFF power measurement capability because of high TE noise floor. To discuss the feasibility of the new test method, TE vendors were requested to clarify some aspects:

This document provides Keysight views on this topic and makes some proposals to progress on this topic.

# 2.	Discussion

## 2.1.	Power window

According to [2], uplink power window for FR1 is defined as shown in Figure F.4.2.1-1: PW is obtained by the sum of UE power step size, UE power step tolerance, TE relative power MU, and double of TE absolute power MU, where UE power step size and UE power step tolerance are defined by Table 7.1.1-1 in TS 38.213 and Table 6.3.4.3-2 in TS 38.101-2, respectively. TE relative power MU will be the one to be agreed in TR 38.903 and TE absolute power MU is calculated by the budget sheet Table B.3.2-2 in TR 38.903.

In our opinion, current definition of the power window is not correct as it considers twice the test system absolute power measurement uncertainty: once for the initial power level and another one for the target power level after the power step, while for target power level the test system uncertainty is already accounted for through the test system absolute power measurement for the initial power level and the test system relative power measurement uncertainty already considered in the calculation.

Hence PW should be obtained by the sum of UE power step size, UE power step tolerance, TE relative power MU, and TE absolute power MU.

Proposal 1: Correct existing definition of Uplink Power Control window for FR1 in 38.521-3 [2] to remove one test system absolute power measurement uncertainty from total computation.

| Existing Uplink power window figure | Proposed Uplink power window figure |
| --- | --- |

Taking into account Proposal 1, uplink power window for FR2 could be calculated as shown in Table 1.

Table 1 Power window size calculation [dB]

|  | FR2a | FR2b | Assumption |
| --- | --- | --- | --- |
| UE power step size | 1.0 | 1.0 | From Table 7.1.1-1 in TS 38.213 - PUSCH - TPC Command Field 1 or 2 |
| UE power step tolerance | 1.0 | 1.0 | From Table 6.3.4.3-1 in TS 38.101-2 - PUSCH - Power step = 1 dB - Target power > minimum peak EIRP – 12.0 dB |
|  | TBD | TBD | From Table 6.3.4.3-2 in TS 38.101-2 - PUSCH - Power step = 1 dB - Target power ≤ minimum peak EIRP – 12.0 dB |
| TE relative power MU | 1.4 | 1.4 | EIRP, IFF As proposed in [4], [5] and [6] |
| TE absolute power MU | 4.89 | 5.09 | EIRP, IFF, MOP measurement |
| Power window size PW | 7.29 | 7.49 | Target power > minimum peak EIRP – 12.0 dB |
|  | 6.29 + TBD | 6.49 + TBD | Target power ≤ minimum peak EIRP – 12.0 dB |

Observation 1: Core requirements defines in Table 6.3.4.3-2 the relative power tolerance for PUSCH to PUSCH transitions for a power step P=1dB as ±1.0dB but only for the case of PUMAX ≥P>Pint. There is no test tolerance defined for the case Pint≥P≥Pmin.

Proposal 2: Send an LS to RAN4 to ask for clarification on relative power tolerance for PUSCH to PUSCH transitions for a power step P=1dB for the case Pint≥P≥Pmin. Meanwhile assume [1.0 dB] for RAN5 analysis.

As the answer to the Proposal 4 a) in [1], achievable overall power window size PW is 7.29 dB for FR2a and 7.49 dB for FR2b for the case of PUMAX ≥P>Pint and [7.29 dB] for FR2a and [7.49 dB] for FR2b for the case Pint≥P≥Pmin.

Proposal 3 : Apply the values 7.29 dB for FR2a and 7.49 dB for FR2b to FR2 power window size PW for PUSCH, power step = 1 dB, target power > minimum peak EIRP – 12.0 dB and [7.29 dB] for FR2a and [7.49 dB] for FR2b to FR2 power window size PW for PUSCH, power step = 1 dB, target power ≤ minimum peak EIRP – 12.0 dB

## 2.3.	On power setting for ON/OFF time mask test

## There are different alternatives to set the ON power in the ON/OFF times mask test case leading to different testability issues taking into account the relaxation required to measure OFF power defined in [1],[7], [9]:

- Option 1: Using absolute power control

- This option is the one implemented today for FR2 ON/OFF transmit time mask test case in [7].

- Assuming signaling parameters are selected such that UE should be transmitting close to min peak EIRP value, the remaining UE tolerance assuming Pmax ≥ P ≥ Pint will be ± (12 dB + MBR+MPR+ Tolerance(MPR)) according to core requirements in table 6.3.4.2-1 in [3]:

- Option 2: Relative + aggregate power control

- This is the baseline option proposed in [1].

- More detailed description for such proposal is suggested in this document as:

  - Target power level to be defined as Min peak EIRP as defined in table 6.2.1.3-1 in [3]:

## In such case, the remaining UE tolerance assuming Pmax ≥ P ≥ Pint will be:

## Total UE tolerance = ± (1 dB + 3.5 dB + MBR+MPR+ Tolerance(MPR)),  according to proposal 3 above and core requirements in table 6.3.4.4.-2 in [3].

  - Test tolerance to set the UL power as [0, -Total UE tolerance], considering that UE tolerance is bigger than UL power window as defined in Proposal 3.

- TE Noise floor in this case can be leveraged from ACLR test.

## These 2 options have been evaluated using a similar approach as the one used in [1], i.e. evaluating a theoretical UE time mask with linear transients in the power time mask in Watts. Differences from [1] are:

- Transients boundaries are defined outside the ON power slot as shown in Figure 6.3.3.2-1 in [3]:

| ON Power |  |
| --- | --- |

- Using OFF power equal to -35 dBm as defined in [3] Table 6.3.2-1.

- For 4 different values of transients: 5, 6, 10 and 20 µs

- Evaluating either a marginal UE as well as good UEs (in output power terms)

- With and without TE noise floor impact, using as TE noise floor values for FR2a and FR2b the ones defined for ACLR in [8] (-7.6 dBm for 400 MHz in FR2a and -5.5 dBm for 400 MHz in FR2b).

- Taking into account that for FR2 ON/OFF time mask initial conditions define a single test point for DFT-s-OFDM QPSK Outer Full (refer to [7] Table 6.3.3.2.4.1-1):

## hence applicable MPRs for a power class 3 device are:

  - For channel bandwidth lower than 400 MHz: 2 dB plus 1.5 dB tolerance according to configured output power requirements in [3] Table 6.2.4-1

  - For channel bandwidth equal to 400 MHz: 3 dB plus 2 dB tolerance according to configured output power requirements in [3] Table 6.2.4-1

- Considering different channel bandwidths and different frequency bands to evaluate the impact of different relaxations in OFF power measurements

## No other MU factor different from TE noise impact has been considered.

## Details results for those simulations can be found in the annex. The following table shows the performance of the different options per frequency band and channel bandwidth considering TE noise floor impact:

|  | ## n257, n258, n261 |  |  |  |  |  |  |  | ## n260 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## Opt1 |  |  |  | ## Opt 2 |  |  |  | ## Opt1 |  |  |  | ## Opt 2 |  |  |  |
|  | ## 5 | ## 6 | ## 10 | ## 20 | ## 5 | ## 6 | ## 10 | ## 20 | ## 5 | ## 6 | ## 10 | ## 20 | ## 5 | ## 6 | ## 10 | ## 20 |
| ## 50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## 100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## 200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## 400 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 2: ON power value impacts the OFF power outside the transient.

Proposal 4: Keep ON power measurement and limits in FR2 ON/OFF time mask test to ensure that the transient is checked properly.

Observation 3: Option 2 seems to have the best performance among the two options although showing some limitations to detect failing transients very close to the limits for:

- CBW=400 MHz in FR2 & FR2b

- CBW=200 MHz in FR2b

Proposal 5: Use Option 2 in FR2 ON/OFF time mask test case to improve test case value and coverage for all channel bandwidths.

Proposal 6: Further discuss whether any relaxation is required for transients not properly detected by option 3 or whether focusing only in OFF power in those cases as proposed in section 2.4 in [9].

## 2.4.	Measurement Uncertainties

## If option 1 is agreed, MPR MUs could be applied for ON power measurement and keep current EIRP OFF MUs for OFF power measurements in FR2 ON/OFF time mask.

## Proposal 7: Leverage MPR MUs for ON power measurement and keep current EIRP OFF MUs for OFF power measurements in FR2 ON/OFF time mask.

## 2.5.	Test Tolerances

## If option 1 is agreed, MPR TTs could be applied for ON power measurements and keep 0 dB TT for EIRP OFF power measurement in FR2 ON/OFF time mask.

## Proposal 8: Leverage MPR TTs for ON power and keep current EIRP OFF TTs (0 dB) for OFF power values in FR2 ON/OFF time mask.

# 3.	Conclusions

This document reviews testability discussions on FR2 ON/OFF time mask test cases and provides the following observations and proposals to progress on this topic:

Proposal 1: Correct existing definition of Uplink Power Control window for FR1 in 38.521-3 [2] to remove one test system absolute power measurement uncertainty from total computation.

Observation 1: Core requirements defines in Table 6.3.4.3-2 the relative power tolerance for PUSCH to PUSCH transitions for a power step P=1dB as ±1.0dB but only for the case of PUMAX ≥P>Pint. There is no test tolerance defined for the case Pint≥P≥Pmin.

Proposal 2: Send an LS to RAN4 to ask for clarification on relative power tolerance for PUSCH to PUSCH transitions for a power step P=1dB for the case Pint≥P≥Pmin. Meanwhile assume [1.0 dB] for RAN5 analysis.

Proposal 3 : Apply the values 7.29 dB for FR2a and 7.49 dB for FR2b to FR2 power window size PW for PUSCH, power step = 1 dB, target power > minimum peak EIRP – 12.0 dB and [7.29 dB] for FR2a and [7.49 dB] for FR2b to FR2 power window size PW for PUSCH, power step = 1 dB, target power ≤ minimum peak EIRP – 12.0 dB

Observation 2: ON power value impacts the OFF power outside the transient.

Proposal 4: Keep ON power measurement and limits in FR2 ON/OFF time mask test to ensure that the transient is checked properly.

Observation 3: Option 2 seems to have the best performance among the two options although showing some limitations to detect failing transients very close to the limits for:

- CBW=400 MHz in FR2 & FR2b

- CBW=200 MHz in FR2b

Proposal 5: Use Option 2 in FR2 ON/OFF time mask test case to improve test case value and coverage for all channel bandwidths.

## Proposal 6: Further discuss whether any relaxation is required for transients not properly detected by option 3 or whether focusing only in OFF power in those cases as proposed in section 2.4 in [9].

## Proposal 7: Leverage MPR MUs for ON power and keep current EIRP OFF MUs for OFF power values.

## Proposal 8: Leverage MPR TTs for ON power and keep current EIRP OFF TTs (0 dB) for OFF power values in FR2 ON/OFF time mask.

# 4.	References

[1] R5-206785, “Discussion on FR2 ON/OFF time mask”, Ericsson, RAN5#89e

[2] 38.521-3 v17.0.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[3] 38.101-2 v17.1.0 “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[4] R5-210845, “On relative power measurement uncertainty”, Anritsu, RAN5#90e

[5] R5-213280, “On FR2 relative and aggregate power tolerance Measurement Uncertainties and test tolerances”, Keysight Technologies, RAN5#91e

[6] R5-213278, “Measurement uncertainties and test tolerances for FR2 Relative and aggregate power tolerance”, Keysight Technologies, RAN5#91e

[7] 38.521-2 v16.7.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[8] R5-204741, “On FR2 EIRP OFF Power relaxation and measurement uncertainty”, Keysight Technologies, Samsung, RAN5#88e

[9] R5-211105, “The degradation of TE noise floor for ON OFF time mask and how to avoid it”, Anritsu, RAN5#90e

ANNEX A DETAILED RESULTS FOR FR2 ON/OFF TIME MASK OPTIONS

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -35 | ## 6.2 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -22.17 | ## 6.2 | ## -22.17 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -12.84 | ## 6.2 | ## -12.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -6.84 | ## 6.2 | ## -6.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -16.54 | ## 6.2 | ## -16.54 | ## -15.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -16.54 | ## 22.4 | ## -16.54 | ## -15.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -15.54 | ## 6.2 | ## -15.54 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -5.82 | ## 22.4 | ## -5.82 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -11.33 | ## 6.2 | ## -11.33 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## 3.38 | ## 22.4 | ## 3.38 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -6.42 | ## 6.2 | ## -6.42 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## 9.36 | ## 22.4 | ## 9.36 | ## -15.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 50 | ## No | ## N/A | ## -35 | ## 4.4 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 50 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 50 | ## No | ## N/A | ## -23.86 | ## 4.4 | ## -23.86 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 50 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 50 | ## No | ## N/A | ## -14.63 | ## 4.4 | ## -14.63 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 50 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 50 | ## No | ## N/A | ## -8.64 | ## 4.4 | ## -8.64 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 50 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -14.46 | ## 4.4 | ## -14.46 | ## -13.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -14.46 | ## 20.6 | ## -14.46 | ## -13.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -14.03 | ## 4.4 | ## -14.03 | ## -13.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -7.12 | ## 20.6 | ## -7.12 | ## -13.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -11.58 | ## 4.4 | ## -11.58 | ## -13.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## 1.64 | ## 20.6 | ## 1.64 | ## -13.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -7.6 | ## 4.4 | ## -7.6 | ## -13.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## 7.58 | ## 20.6 | ## 7.58 | ## -13.5 | ## 4.4 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -35 | ## 13.7 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -14.87 | ## 13.7 | ## -14.87 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## -5.36 | ## 13.7 | ## -5.36 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## 0.65 | ## 13.7 | ## 0.65 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 50 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -16.54 | ## 13.7 | ## -16.54 | ## -15.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -16.54 | ## 22.4 | ## -16.54 | ## -15.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -12.64 | ## 13.7 | ## -12.64 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -5.82 | ## 22.4 | ## -5.82 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## -5.05 | ## 13.7 | ## -5.05 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## 3.38 | ## 22.4 | ## 3.38 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## 0.73 | ## 13.7 | ## 0.73 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 50 | ## Yes | ## -16.6 | ## 9.36 | ## 22.4 | ## 9.36 | ## -15.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 50 | ## No | ## N/A | ## -35 | ## 11.9 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 50 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 50 | ## No | ## N/A | ## -16.64 | ## 11.9 | ## -16.64 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 50 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 50 | ## No | ## N/A | ## -7.16 | ## 11.9 | ## -7.16 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 50 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 50 | ## No | ## N/A | ## -1.14 | ## 11.9 | ## -1.14 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 50 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -14.46 | ## 11.9 | ## -14.46 | ## -13.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -14.46 | ## 20.6 | ## -14.46 | ## -13.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -12.43 | ## 11.9 | ## -12.43 | ## -13.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -7.12 | ## 20.6 | ## -7.12 | ## -13.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -6.43 | ## 11.9 | ## -6.43 | ## -13.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## 1.64 | ## 20.6 | ## 1.64 | ## -13.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## -0.96 | ## 11.9 | ## -0.96 | ## -13.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 50 | ## Yes | ## -14.5 | ## 7.58 | ## 20.6 | ## 7.58 | ## -13.5 | ## 11.9 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -35 | ## 6.2 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -22.18 | ## 6.2 | ## -22.18 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -12.84 | ## 6.2 | ## -12.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -6.84 | ## 6.2 | ## -6.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -13.57 | ## 6.2 | ## -13.57 | ## -12.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -13.57 | ## 22.4 | ## -13.57 | ## -12.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -13.04 | ## 6.2 | ## -13.04 | ## -12.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -5.48 | ## 22.4 | ## -5.48 | ## -12.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -10.22 | ## 6.2 | ## -10.22 | ## -12.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## 3.42 | ## 22.4 | ## 3.42 | ## -12.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -6.05 | ## 6.2 | ## -6.05 | ## -12.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## 9.38 | ## 22.4 | ## 9.38 | ## -12.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 100 | ## No | ## N/A | ## -35 | ## 4.4 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 100 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 100 | ## No | ## N/A | ## -23.86 | ## 4.4 | ## -23.86 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 100 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 100 | ## No | ## N/A | ## -14.63 | ## 4.4 | ## -14.63 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 100 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 100 | ## No | ## N/A | ## -8.64 | ## 4.4 | ## -8.64 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 100 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -11.48 | ## 4.4 | ## -11.48 | ## -10.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -11.48 | ## 20.6 | ## -11.48 | ## -10.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -11.26 | ## 4.4 | ## -11.26 | ## -10.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -6.40 | ## 20.6 | ## -6.40 | ## -10.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -9.81 | ## 4.4 | ## -9.81 | ## -10.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## 1.74 | ## 20.6 | ## 1.74 | ## -10.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -6.90 | ## 4.4 | ## -6.90 | ## -10.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## 7.60 | ## 20.6 | ## 7.60 | ## -10.5 | ## 4.4 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -35 | ## 13.7 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -14.87 | ## 13.7 | ## -14.87 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## -5.36 | ## 13.7 | ## -5.36 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## 0.65 | ## 13.7 | ## 0.65 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 100 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -13.57 | ## 13.7 | ## -13.57 | ## -12.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -13.57 | ## 22.4 | ## -13.57 | ## -12.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -11.18 | ## 13.7 | ## -11.18 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -5.48 | ## 22.4 | ## -5.48 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## -4.76 | ## 13.7 | ## -4.76 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## 3.42 | ## 22.4 | ## 3.42 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## 0.81 | ## 13.7 | ## 0.81 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 100 | ## Yes | ## -13.6 | ## 9.37 | ## 22.4 | ## 9.37 | ## -12.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 100 | ## No | ## N/A | ## -35 | ## 11.9 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 100 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 100 | ## No | ## N/A | ## -16.64 | ## 11.9 | ## -16.64 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 100 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 100 | ## No | ## N/A | ## -7.16 | ## 11.9 | ## -7.16 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 100 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 100 | ## No | ## N/A | ## -1.14 | ## 11.9 | ## -1.14 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 100 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -11.48 | ## 11.9 | ## -11.48 | ## -10.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -11.48 | ## 20.6 | ## -11.48 | ## -10.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -10.35 | ## 11.9 | ## -10.35 | ## -10.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -6.40 | ## 20.6 | ## -6.40 | ## -10.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -5.81 | ## 11.9 | ## -5.81 | ## -10.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## 1.74 | ## 20.6 | ## 1.74 | ## -10.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## -0.78 | ## 11.9 | ## -0.78 | ## -10.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 100 | ## Yes | ## -11.5 | ## 7.60 | ## 20.6 | ## 7.60 | ## -10.5 | ## 11.9 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -35 | ## 6.2 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -22.18 | ## 6.2 | ## -22.18 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -12.84 | ## 6.2 | ## -12.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -6.84 | ## 6.2 | ## -6.84 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -10.58 | ## 6.2 | ## -10.58 | ## -9.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -10.58 | ## 22.4 | ## -10.58 | ## -9.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -10.31 | ## 6.2 | ## -10.31 | ## -9.6 | ## 6.2 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -4.86 | ## 22.4 | ## -4.86 | ## -9.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -8.60 | ## 6.2 | ## -8.60 | ## -9.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## 3.50 | ## 22.4 | ## 3.50 | ## -9.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -5.38 | ## 6.2 | ## -5.38 | ## -9.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## 9.39 | ## 22.4 | ## 9.39 | ## -9.6 | ## 6.2 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 200 | ## No | ## N/A | ## -35 | ## 4.4 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 200 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 200 | ## No | ## N/A | ## -23.86 | ## 4.4 | ## -23.86 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 200 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 200 | ## No | ## N/A | ## -14.63 | ## 4.4 | ## -14.63 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 200 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 200 | ## No | ## N/A | ## -8.64 | ## 4.4 | ## -8.64 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 200 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -8.49 | ## 4.4 | ## -8.49 | ## -7.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -8.49 | ## 20.6 | ## -8.49 | ## -7.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -8.38 | ## 4.4 | ## -8.38 | ## -7.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -5.23 | ## 20.6 | ## -5.23 | ## -7.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -7.59 | ## 4.4 | ## -7. 59 | ## -7.5 | ## 4.4 | ## PASS |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## 1.94 | ## 20.6 | ## 1.94 | ## -7.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -5.67 | ## 4.4 | ## -5.67 | ## -7.5 | ## 4.4 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## 7.65 | ## 20.6 | ## 7.65 | ## -7.5 | ## 4.4 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -35 | ## 13.7 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -14.87 | ## 13.7 | ## -14.87 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## -5.36 | ## 13.7 | ## -5.36 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## 0.65 | ## 13.7 | ## 0.65 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 200 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -10.58 | ## 13.7 | ## -10.58 | ## -9.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -10.58 | ## 22.4 | ## -10.58 | ## -9.6 | ## 13.7 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -9.22 | ## 13.7 | ## -9.22 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -4.86 | ## 22.4 | ## -4.86 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## -4.24 | ## 13.7 | ## -4.24 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## 3.50 | ## 22.4 | ## 3.50 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## 0.95 | ## 13.7 | ## 0.95 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 200 | ## Yes | ## -10.6 | ## 9.39 | ## 22.4 | ## 9.39 | ## -9.6 | ## 13.7 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 200 | ## No | ## N/A | ## -35 | ## 11.9 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 200 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 200 | ## No | ## N/A | ## -16.65 | ## 11.9 | ## -16.65 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 200 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 200 | ## No | ## N/A | ## -7.16 | ## 11.9 | ## -7.16 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 200 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 200 | ## No | ## N/A | ## -1.14 | ## 11.9 | ## -1.14 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 200 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -8.49 | ## 11.9 | ## -8.49 | ## -7.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -8.49 | ## 20.6 | ## -8.49 | ## -7.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -7.89 | ## 11.9 | ## -7.89 | ## -7.5 | ## 11.9 | ## PASS |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -5.24 | ## 20.6 | ## -5.24 | ## -7.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -4.79 | ## 11.9 | ## -4.79 | ## -7.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## 1.94 | ## 20.6 | ## 1.94 | ## -7.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## -0.45 | ## 11.9 | ## -0.45 | ## -7.5 | ## 11.9 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 200 | ## Yes | ## -8.5 | ## 7.65 | ## 20.6 | ## 7.65 | ## -7.5 | ## 11.9 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -35 | ## 4.7 | ## -35 | ## -35 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -23.58 | ## 4.7 | ## -23.58 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -14.33 | ## 4.7 | ## -14.33 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -8.34 | ## 4.7 | ## -8.34 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.59 | ## 4.7 | ## -7.59 | ## -6.6 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.59 | ## 22.4 | ## -7.59 | ## -6.6 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.50 | ## 4.7 | ## -7.50 | ## -6.6 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -3.84 | ## 22.4 | ## -3.84 | ## -6.6 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -6.81 | ## 4.7 | ## -6.81 | ## -6.6 | ## 4.7 | ## PASS |
| ## Opt 1 | ## Good | ## 10 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## 3.67 | ## 22.4 | ## 3.67 | ## -6.6 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -5.06 | ## 4.7 | ## -5.06 | ## -6.6 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## 9.44 | ## 22.4 | ## 9.44 | ## -6.6 | ## 4.7 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 400 | ## No | ## N/A | ## -35 | ## 2.9 | ## -35 | ## -35 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 400 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 400 | ## No | ## N/A | ## -25.23 | ## 2.9 | ## -25.23 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 400 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 400 | ## No | ## N/A | ## -16.11 | ## 2.9 | ## -16.11 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 400 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 400 | ## No | ## N/A | ## -10.13 | ## 2.9 | ## -10.13 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 400 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 5 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.49 | ## 2.9 | ## -5.49 | ## -4.5 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Good | ## 5 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.49 | ## 20.6 | ## -5.49 | ## -4.5 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Marginal | ## 6 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.46 | ## 2.9 | ## -5.46 | ## -4.5 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Good | ## 6 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -3.57 | ## 20.6 | ## -3.57 | ## -4.5 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 10 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.19 | ## 2.9 | ## -5.19 | ## -4.5 | ## 2.9 | ## PASS |
| ## Opt 1 | ## Good | ## 10 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## 2.31 | ## 20.6 | ## 2.31 | ## -4.5 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Marginal | ## 20 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -4.38 | ## 2.9 | ## -4.38 | ## -4.5 | ## 2.9 | ## FAIL |
| ## Opt 1 | ## Good | ## 20 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## 7.75 | ## 20.6 | ## 7.75 | ## -4.5 | ## 2.9 | ## FAIL |

| ## Option | ## UE type | ## Transient [µs] | ## Frequency bands | ## Channel Bandwidth ## [MHz] | ## TE noise floor impact | ## TE Noise Floor /Channel bandwidth | ## OFF power before ## [dBm] | ## ON power ## [dBm] | ## OFF power after ## [dBm] | ## OFF power limit ## [dBm] | ## ON power limit ## [dBm] | ## Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -35 | ## 12.2 | ## -35 | ## -35 | ## 12.2 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -35 | ## 22.4 | ## -35 | ## -35 | ## 12.2 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -16.35 | ## 12.2 | ## -16.35 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -6.20 | ## 22.4 | ## -6.20 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -6.86 | ## 12.2 | ## -6.86 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## 3.33 | ## 22.4 | ## 3.33 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## -0.85 | ## 12.2 | ## -0.85 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 400 | ## No | ## N/A | ## 9.35 | ## 22.4 | ## 9.35 | ## -35 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.59 | ## 12.2 | ## -7.59 | ## -6.6 | ## 12.2 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.59 | ## 22.4 | ## -7.59 | ## -6.6 | ## 12.2 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -7.06 | ## 12.2 | ## -7.06 | ## -6.6 | ## 12.2 | ## PASS |
| ## Opt 2 | ## Good | ## 6 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -3.84 | ## 22.4 | ## -3.84 | ## -6.6 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -4.23 | ## 12.2 | ## -4.23 | ## -6.6 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## 3.67 | ## 22.4 | ## 3.67 | ## -6.6 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## -0.05 | ## 12.2 | ## -0.05 | ## -6.6 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n257, n58, n261 | ## 400 | ## Yes | ## -7.6 | ## 9.44 | ## 22.4 | ## 9.44 | ## -6.6 | ## 12.2 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 400 | ## No | ## N/A | ## -35 | ## 10.4 | ## -35 | ## -35 | ## 10.4 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 400 | ## No | ## N/A | ## -35 | ## 20.6 | ## -35 | ## -35 | ## 10.4 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 400 | ## No | ## N/A | ## -18.12 | ## 10.4 | ## -18.12 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 400 | ## No | ## N/A | ## -8.00 | ## 20.6 | ## -8.00 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 400 | ## No | ## N/A | ## -8.66 | ## 10.4 | ## -8.66 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 400 | ## No | ## N/A | ## 1.53 | ## 20.6 | ## 1.53 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 400 | ## No | ## N/A | ## -2.64 | ## 10.4 | ## -2.64 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 400 | ## No | ## N/A | ## 7.55 | ## 20.6 | ## 7.55 | ## -35 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 5 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.49 | ## 10.4 | ## -5.49 | ## -4.5 | ## 10.4 | ## PASS |
| ## Opt 2 | ## Good | ## 5 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.49 | ## 20.6 | ## -5.49 | ## -4.5 | ## 10.4 | ## PASS |
| ## Opt 2 | ## Marginal | ## 6 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -5.27 | ## 10.4 | ## -5.27 | ## -4.5 | ## 10.4 | ## PASS |
| ## Opt 2 | ## Good | ## 6 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -3.57 | ## 20.6 | ## -3.57 | ## -4.5 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 10 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -3.82 | ## 10.4 | ## -3.82 | ## -4.5 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Good | ## 10 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## 2.31 | ## 20.6 | ## 2.31 | ## -4.5 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Marginal | ## 20 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## -0.90 | ## 10.4 | ## -0.90 | ## -4.5 | ## 10.4 | ## FAIL |
| ## Opt 2 | ## Good | ## 20 | ## n260 | ## 400 | ## Yes | ## -5.5 | ## 7.75 | ## 20.6 | ## 7.75 | ## -4.5 | ## 10.4 | ## FAIL |