3GPP TSG-RAN5 Meeting #91-e	R5-213830

# Elctronic Meeting,  17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5289 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TDD sustained data rate performance for CA (5DL CA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following items are incomplete for TDD sustained data rate performance for CA (5DL CA) test case. - Editor’s note - Applicability spec - Test applicability - Test point in the RAN4 minimum requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following items are updated for TDD sustained data rate performance for CA (5DL CA) test case: - Remove editor’s note - Add test applicability - Update test point |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete and the WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.1_A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN4 dependency: R4-2108807 This is an update of R5-212399 and outcome of 1 revision |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

8.7.2.1_A.5	TDD Sustained data rate performance for CA (5DL CA)

8.7.2.1_A.5.1	Test purpose

Same test purpose as 8.7.2.1.

8.7.2.1_A.5.2	Test applicability

This test case applies to all types of E-UTRA TDD UE and forward of UE DL category 11, 12 and 15 that support 5DL with CA configurations in Table 7.1-2c.

Note: This test also applies to UE supporting 4Rx antenna ports.

8.7.2.1_A.5.3	Minimum conformance requirements

For UE not supporting 256QAM, the requirements are specified in Table 8.7.2.1_A.5.3-2, with the addition of the parameters in Table 8.7.2.1_A.5.3-1 and the downlink physical channel setup according to Table C.3.2-1 in Annex C. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified in Table 8.7.2.1_A.5.3-3. The TB success rate shall be sustained during at least 300 frames.

Table 8.7.2.1_A.5.3-1: Test parameters for sustained downlink data rate (TDD 64QAM)

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | ACK/NACK feedback mode | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 11 | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| Note 1: PUCCH format 1b with channel selection is used to feedback ACK/NACK. Note 2: PUCCH format 3 is used to feedback ACK/NACK. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.2.1_A.5.3-2: Minimum requirement (TDD 64QAM)

| Test | Number of bits of a DL-SCH transport block received within a TTI for normal/special sub-frame | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 10 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 11 | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 71112 bits for sub-frame 5. Note 3: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |  |

Table 8.7.2.1_A.5.3-3: Test points for sustained data rate (FRC 64QAM)

| CA config | Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9, 10 | Cat. 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DL Cat. 11, 12 |  |
| CA with 5 CCs | 5x20 |  |  |  |  |  |  | 10 | 10 |
|  | 15+4x20 |  |  |  |  |  |  | 11 | 11 |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. Note 2: For non-CA UE, test is selected for maximum supported bandwidth. Note 3: Void. Note 4: If the intra-band contiguous CA is the only CA configuration supported by category 3 or 4 UE, single carrier test is selected. Note 5: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.7.2.

8.7.2.1_A.5.4	Test description

8.7.2.1_A.5.4.1	Initial conditions

Table 8.7.2.1_A.5.4.1-1: Further test parameters per test

| Test | DL Measurement channel | UL Measurement channel | TBsize per Codeword per Component Carrier | Number of PDCP SDU per Codeword | PDCP SDU size [Octets] Calculation (Note 1) | PDCP SDU size [Octets] |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | R.31-4 TDD for five 20MHz CCs | R.1-4 TDD | 75376 (Note 3) | 7 for five 20MHz CCs | FLOOR((TBsize – 264)/56) | 1341 for four 20MHz CCs |
| 11 | R.31-5 TDD for 15MHz CC R.31-4 TDD for four 20MHz CCs | R.1-4 TDD | 55056 for 15MHz CC 75376 (Note 3) for 20MHz CC | 6 for 15MHz CC 7 for four 20 MHz CCs | FLOOR((TBsize – 240)/48) for 15 MHz CC FLOOR((TBsize – 264)/56) for 20 MHz CC | 1142 for 15MHz CC 1341 for four 20MHz CCs |
| Note 1: Transport block size under test according to applicable Fixed Reference Channel for sustained data-rate test in annex A.3.9. In case of varying TBS across SFs of the RMC, only the maximum TBS is used for PDCP SDU size calculation. In case of different resulting PDCP SDU sizes among component carriers, the smaller calculated PDCP SDU size is used across all the carriers. Note 2: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is: PDCP SDU size = (TBsize – N*PDCP header size - AMD PDU header size - MAC header size - Size of RLC STATUS PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is CEIL[(16+(N-1)*12)/8] bytes which includes 16 bit standard AM header and (N-1) Length indicators; and MAC header size = R/R/E/LCID/F/L MAC subheader (24 bits for MAC SDU for RLC STATUS PDU with 15 bit LI) + R/R/E/LCID MAC subheader (8 bits for MAC SDU for RLC Data PDU) = 32 bits. The size of RLC STATUS PDU including one ACK_SN field and one NACK_SN field is 32 bits (if no STATUS PDU is sent or if the size of the STATUS PDU is less than 32 bits then padding will be used to fill the 32 bits). This gives: PDCP SDU size = 8*FLOOR((TBsize – N*16- 8*CEIL((16+(N-1)*12)/8) – 64)/(8*N)) bits. The calculation of PDCP SDU sizes does not consider timing advance MAC CE as timing advance is not transmitted by SS for RF test cases, and the header sizes are informative and may vary during the test. Note 3: 71112 bits for sub-frame 5. |  |  |  |  |  |  |

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: according to Table 8.7.2.1_A.5.5-2 depending on the UE category according to Table 8.7.2.1_A.5.5-3.

1.	Connect the SS to the UE antenna connector(s) as shown in TS 36.508 [7] Annex A, Figure group A.60 (without using faders and AWGN generators) for UE supporting only 2Rx RF bands on all CC.  Annex A, Figure A.85 for UE supporting 4Rx RF band on any of the CC.

2.	The parameter settings for the cell are set up according to Table 8.7.1-1 and Table 8.7.2.1_A.5.5-1 depending on the UE category according to Table 8.7.2.1_A.5.5-3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 4A-RF according to TS 36.508 [7] clause 5.2A.3. Message contents are defined in clause 8.7.2.1_A.5.4.3.

8.7.2.1_A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4 with four SCCs configured. PhysicalConfigDedicated-DEFAULT is defined in Table 8.7.2.1_A.5.4.3-3, PhysicalConfigDedicatedSCell-r10-DEFAULT is defined in Table 8.7.2.1_A.5.4.3-3A.

3.	The SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13,6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	Propagation conditions are set according to Annex B.1 (No external noise sources are applied).

5.	The SS looks up TBsize in Table 8.7.2.1_A.5.4.1-1 for the tests to be performed depending on the UE category according to Table 8.7.2.1_A.5.5-3.

6.	The SS sets the counters, NDL_newtx ,NDL_retx, NUL_PDCP, and NDDL_PDCP to 0.

7.	For each new DL HARQ transmission the SS generates sufficient PDCP SDUs to fill up the TB in accordance with Table 8.7.2.1_A.5.4.1-1 (Note 1). The SS ciphers the PDCP SDUs, concatenates the resultant PDCP PDUs to form an RLC PDU and then a MAC PDU. The SS transmits the MAC PDU. The SS increments then NDL_newtx by one and NDL_PDCP by the number of new PDCP SDUs (Note 1) included in the sent MAC PDU.

8.	If PHY requests a DL HARQ retransmission, the SS performs a HARQ retransmission and increments NDL_retx by one.

9.	Steps 7 to 8 are repeated at every TTI for at least 300 frames and the SS waits for 300ms to let any HARQ retransmissions and RLC retransmissions to finish.

10.	For each PDCP SDU received at the SS, if the content of the data matches that of the truncated version of the original PDCP SDU generated at the SS, the SS increments NUL_PDCP by one

11.	The SS calculates the TB success rate as A = 100%*NDL_newtx/ (NDL_newtx + NDL_retx). Measurements are not based on UE feedback but on SS counters instead (NDL_newtx and NDL_retx) in order to validate HARQ feedback received from the UE.

12.	The SS calculates the PDCP SDU loss as B = NDL_PDCP - NUL_PDCP

13.	The UE passes the test if A ≥ "corresponding TB success rate according to Table 8.7.2.1_A.5.5-1" and B = 0.

NOTE 1:	In case of RLC PDU retransmission, the number of new required PDCP SDUs is as many as to fill the rest of TB.

8.7.2.1_A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 and 4.7A, with the following exceptions:

Table 8.7.2.1_A.5.4.3-1: CLOSE UE TEST LOOP (in the preamble)

| Derivation Path: 36.508 clause 4.7A Table 4.7A-3 with condition UE TEST LOOP MODE A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode A LB setup |  |  |  |  |
| Length of UE test loop mode A LB setup list in bytes |  | 0 0 0 0 0 0 1 1 | Length of one LB setup DRB (3 bytes) |  |
| LB setup DRB |  | 0 0 0 0 0 0 0 0, 0 0 1 0 1 0 0 0, 0 0 0 Q4 Q3 Q2 Q1 Q0 | UL PDCP SDU size = 40 bits (5 bytes) Q4..Q0 = Data Radio Bearer identity number for the default radio bearer. See 36.509 clause 6.1 |  |

Table 8.7.2.1_A.5.4.3-2: SecurityModeCommand (in the preamble)

| Derivation Path: TS 36.508 clause 4.6.1 Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfiguration SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | eea2 |  |  |
| nextHopChainingCount |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.7.2.1_A.5.4.3-3: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm3 |  | Transmission mode 3 |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm3 | 10 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.7.2.1_A.5.4.3-3A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 CHOICE { |  |  |  |
| transmissionMode-r10 | tm3 |  | Transmission mode 3 |
| codebookSubsetRestriction-r10 | 10 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.7.2.1_A.5.5	Test requirement

The requirements are specified in Table 8.7.2.1_A.5.5-1 depending on the UE category according to Table 8.7.2.1_A.5.5-3. The PDCP SDU success rate shall be sustained during at least 300 frames.

Table 8.7.2.1_A.5.5-1: Test requirements per test

| Test | Number of bits of a DL-SCH transport block received within a TTI for normal/special sub-frame | Measurement channel | Reference value |
| --- | --- | --- | --- |
|  |  |  | TB success rate [%] |
| 10 | 75376/0 (Note 2) | R.31-4 TDD | 85 |
| 11 | 55056/0 for 15MHz CC 75376/0 for 20MHz CC (Note 2) | R.31-5 TDD for 15MHz CC R.31-4 TDD for 20MHz CC | 85 |
| Note 1: For 2 layer transmissions, 2 transport blocks are received within a TTI. Note 2: 71112 bits for sub-frame 5. Note 3: The TB success rate is defined as TB success rate = 100%*NDL_correct_rx/ (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks. |  |  |  |

Table 8.7.2.1_A.5.5-2: Test parameters per test

| Test | Bandwidth (MHz) | Transmission mode | Antenna configuration | Codebook subset restriction | Downlink power allocation (dB) |  |  | at antenna port (dBm/15kHz) | ACK/NACK feedback mode | Symbols for unused PRBs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 5x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| 11 | 15+4x20 | 3 | 2 x 2 | 10 | -3 | -3 | 0 | -85 | (Note 2) | OP.1 TDD |
| Note 1: PUCCH format 1b with channel selection is used to feedback ACK/NACK. Note 2: PUCCH format 3 is used to feedback ACK/NACK. |  |  |  |  |  |  |  |  |  |  |

Table 8.7.2.1_A.5.5-3: Test points per UE category

| CA config | Bandwidth/ Bandwidth combination (MHz) | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6,7 | Cat. 9, 10 | Cat. 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | DL Cat. 11, 12 |  |
| CA with 5 CCs | 5x20 | - | - | - | - | - |  | 10 | 10 |
|  | 15+4x20 | - | - | - | - | - |  | 11 | 11 |
| Note 1: Void. Note 2: For non-CA UE, test is selected for maximum supported bandwidth. Note 3: Void. Note 4: Void. Note 5: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3. Note 6: Only one test point is tested. Select the largest aggregated CA bandwidth combination supported by the UE among the UE supported CA capabilities. Note 7: Test 7 is tested in 8.7.2.1_A.2. |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition HST | ± 0.6 dB | Overall system uncertainty for HST condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 8.2.1.1.1 Single PRB - Propagation Condition ETU70 | ± 0.8 dB | Overall system uncertainty for fading condition comprises three quantities: 1. Average Signal-to-noise ratio uncertainty 2. Signal-to noise ratio variation for single PRB 3. Fading profile power uncertainty Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx |
| <<Too many rows skipped>> |  |  |
| 8.7.1.1 | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 8.7.1.1_1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.1.1_2 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.1.1_A.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.2 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.4 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.5 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.6 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_A.7 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.1.1_H.2 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.3 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.4 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.5 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.6 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.1.1_H.7 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 Calculation applies for each CC |
| 8.7.2.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.2.1_2 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.2.1_A.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.2 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.3 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.5 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.6 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_A.7 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_H.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.4 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.5 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.6 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.2.1_H.7 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.3.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.4.1 | Same as 8.7.3.1 | Same as 8.7.3.1 |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1.5 TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.1_H.5 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (6DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2.5 TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.2_H.5 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (6DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |
| <<Too many rows skipped>> |  |  |
| 14.2 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| 14.3 | ± 0.8 dB | Same as 14.2 |
| 14.4 | ± 0.6 dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 14.6 | ± 0.8 dB | Same as 14.2 |
| 14.7 | ± 0.6 dB | Same as 14.4 |
| 14.8 | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |
| 14.9 | E-UTRA: Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% Sidelink: ± 0.6 dB | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.4 |
| [Other tests FFS] |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  | ±3 dB |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  | ±0.5 dB |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  | ±3 dB |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  | ±0.3 dB |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |
| Fading profile power uncertainty |  | Test-specific |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |
| CA, DC and LAA performance requirements only: Relative frequency error between carriers |  | 30Hz, measured over a 1ms period, and maximum carrier spacing 80MHz |
| enhanced performance requirement Type B only: Frequency error on the frequency offset to cell 1 |  | 30Hz, measured over a 1ms period |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN or N ocx parameters apply to all test cases except 8.7.1 and 8.7.2. The fading parameters apply to test cases using fading Note 3: In CA, DC and LAA test cases using multiple component carriers (CCs), the uncertainties and related constraints apply for each CC. Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |

<< Unchanged sections omitted >>

F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test | Minimum Requirement in TS 36.101 | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 | SNRs as specified | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition HST | SNR as specified | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| <<Too many rows skipped>> |  |  |  |  |  |
| 8.7.1.1 FDD sustained data rate performance | Downlink power -85dBm/15kHz | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_1 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_2 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.2 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.4 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.5 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_A.7 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.2 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.3 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.4 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.5 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.1.1_H.7 | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  |  | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1 TDD sustained data rate performance | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.2.1_1 | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_2 | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.1 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.2 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.3 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.5 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.7 | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.2 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.3 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.4 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.5 | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_H.7 | Same as 8.7.2.1 |  | Same as 8.7.2.1 for each CC |  | Formula: Downlink power + TT T-put limit unchanged |
| Haule13098.7.3.1 FDD sustained data rate performance for EPDCCH scheduling | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC |  | Same as 8.7.1.1 |  |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC |  | Same as 8.7.2.1 |  |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.2.1 | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| <<Too many rows skipped>> |  |  |  |  |  |
| 10.1 | SNR as specified | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 10.2 | SNR as specified | 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.2 | SNRs as specified | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.3 | SNRs as specified | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.4 | SNRs as specified | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.6 | SNRs as specified | 0.8dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.7 | SNRs as specified | 0.6dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 14.8 | Downlink power -85dBm/15kHz | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 14.9 | SNRs as specified | E-UTRA: 0dB Sidelink: 0.6dB |  | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.7 |  |
| [Other tests FFS] |  |  |  |  |  |