3GPP TSG-RAN5 Meeting #91-e	R5-213850

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1193 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PRACH reference configuration in Annex H |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The configuration of RACH-ConfigGeneric is inconsistent with PRACH.4 FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the value of prach-ConfigurationIndex and ra-ResponseWindow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration will be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-212908r1. |  |  |  |  |  |  |  |  |

<Start of modified section>

BeamFailureRecoveryConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-10: BeamFailureRecoveryConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric-DEFAULT |  | SSB |
|  | RACH-ConfigGeneric-DEFAULT with condition CSI-RS |  | CSI-RS |
| rsrp-ThresholdSSB | 58 | -98dBm |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF CHOICE { | 1 entry |  |  |
| ssb[1] SEQUENCE { |  |  | SSB |
| ssb | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| csi-RS[1] SEQUENCE { |  |  | CSI-RS |
| csi-RS | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  |  |
| ra-OccasionList | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| ra-ssb-OccasionMaskIndex | Not present |  |  |
| recoverySearchSpaceId | SearchSpaceId of the search space used for BFD RAR in DL active BWP |  |  |
| ra-Prioritization | Not present |  |  |
| beamFailureRecoveryTimer | Not present |  |  |
| msg1-SubcarrierSpacing-v1530 | kHz120 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CBD |
| CSI-RS | Configuration for CSI-RS based CBD |

RACH-ConfigGeneric-DEFAULT

Default generic configuration for contention-free RACH in link recovery.

Table H.3.1-11: RACH-ConfigGeneric-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| ra-ResponseWindow | sl40 |  | FR2 AND CSI-RS |
|  | sl1 |  | FR1 AND CSI-RS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSI-RS | Configuration for CSI-RS based CBD |

<End of modified section>