3GPP TSG-RAN5 Meeting #91-e	R5-213853

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1864 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for CA_n260(A-I) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current test frequencies for NR CA intra-band non-contiguous configuration CA_n260(A-I) does not take UE capability of maximum supported frequency separation between lower edge of lowest component carrier and higher edge of highest component carrier into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for CA_n260(A-I) are corrected to take UE capabilities for maximum supported frequency separation into account based on the principles introduced in CR#1862 to TS 38.508-1 Annex C. Separate tables for the case without UL CA and with UL CA introduced. For the case without UL CA the n260A sub-block/CC is located at the band's low and high edge. For the case with UL CA the CA_n260I sub-block/CCs are located at the band's low and high edge. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test frequencies remains violating UE capability of maximum supported frequency separation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.2.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR#1862 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The present CR is an update of R5-212687 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.4.4.2	CA_n260(A-I)

Editor’s note:	CBW=400 MHz for NR band n260 is only supported by for SCS 120kHz. Test frequencies for CA_n260(A-I) are currently limited to SCS 120kHz for all CCs. Test frequencies for mixed numerologies between CCs is FFS.

Editor's note: The number of test points for intra-band non-contiguous CA configurations is under investigation, e.g. “Low” and “High”, or “Mid”.

Table 4.3.1.2.4.4.2-1: NR Intra-Band non-contiguous CA configuration CA_n260(A-I) without UL CA, SCS=120 kHz, Max Wgap

| CBW combination | Sub-block Note 2 | CC within Sub-Block Note 2 | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50, | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50+100+100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=50 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | 37275.6 | 2233759 | 37252.56 | 2233375 | 0 | 120 | 23010 | 2233819 | N/A | 4 | 1 (4) | 16 |
|  |  |  |  |  |  |  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 37350 | 2234999 | 37302.48 | 2234207 | 0 | 120 | 23013 | 2234683 | 10 | 5 | 1 (4) | 18 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 37449.96 | 2236665 | 37402.44 | 2235873 | 0 | 120 | 23018 | 2236123 | 5 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 37549.92 | 2238331 | 37502.4 | 2237539 | 0 | 120 | 23024 | 2237851 | 0 | 3 | 0 (0) | 6 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=50 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | 39425.04 | 2269583 | 38676.24 | 2257103 | 504 | 120 | 23134 | 2269531 | N/A | 3 | 0 (0) | 1014 |
|  |  |  |  |  |  |  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 39499.44 | 2270823 | 38726.16 | 2257935 | 504 | 120 | 23137 | 2270395 | N/A | 1 | 1 (4) | 1018 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 39599.4 | 2272489 | 38826.12 | 2259601 | 504 | 120 | 23143 | 2272123 | 9 | 3 | 1 (4) | 1022 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 39699.36 | 2274155 | 38926.08 | 2261267 | 504 | 120 | 23149 | 2273851 | 4 | 6 | 1 (4) | 1028 |
| 50, | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+100+100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=50 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 37250.04 | 2233333 | 37202.52 | 2232541 | 0 | 120 | 23007 | 2232955 | N/A | 3 | 1 (4) | 14 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 37350 | 2234999 | 37302.48 | 2234207 | 0 | 120 | 23013 | 2234683 | 10 | 5 | 1 (4) | 18 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 37449.96 | 2236665 | 37402.44 | 2235873 | 0 | 120 | 23018 | 2236123 | 5 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 37549.92 | 2238331 | 37502.4 | 2237539 | 0 | 120 | 23024 | 2237851 | 0 | 3 | 0 (0) | 6 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=50 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 39450 | 2269999 | 38676.72 | 2257111 | 504 | 120 | 23134 | 2269531 | N/A | 3 | 0 (0) | 1014 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 39549.96 | 2271665 | 38776.68 | 2258777 | 504 | 120 | 23140 | 2271259 | N/A | 2 | 1 (4) | 1020 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 39649.92 | 2273331 | 38876.64 | 2260443 | 504 | 120 | 23146 | 2272987 | 8 | 4 | 1 (4) | 1024 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 39749.88 | 2274997 | 38976.6 | 2262109 | 504 | 120 | 23152 | 2274715 | 3 | 7 | 1 (4) | 1030 |
| 100, | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50+100+100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=100 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | Same test frequencies as for CBW combination 50, 50+100+100+100 for Low range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB1 | CC1 | 100 | 66 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=100 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | Same test frequencies as for CBW combination 50, 50+100+100+100 for High range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100, | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+100+100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=100 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | Same test frequencies as for CBW combination 50, 100+100+100+100 for Low range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB1 | CC1 | 100 | 66 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=100 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | Same test frequencies as for CBW combination 50, 100+100+100+100 for High range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 200, | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50+100+100+100 | SB1 | CC1 | 200 | 132 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=200 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | Same test frequencies as for CBW combination 50, 50+100+100+100 for Low range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB1 | CC1 | 200 | 32 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=200 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | Same test frequencies as for CBW combination 50, 50+100+100+100 for High range. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 200, | CA_n260(A-I): Maximum frequency separation = 800 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+100+100+100 | SB1 | CC1 | 200 | 132 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=200 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 37450.08 | 2236667 | 37402.56 | 2235875 | 0 |  | 23018 | 2236123 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 37550.04 | 2238333 | 37502.52 | 2237541 | 0 | 120 | 23024 | 2237851 | 11 | 2 | 0 (0) | 4 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 37650 | 2239999 | 37602.48 | 2239207 | 0 | 120 | 23030 | 2239579 | 6 | 1 | 1 (4) | 10 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 37749.96 | 2241665 | 37702.44 | 2240873 | 0 | 120 | 23036 | 2241307 | 1 | 4 | 1 (4) | 16 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=50 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 39250.08 | 2266667 | 38476.8 | 2253779 | 504 | 120 | 23123 | 2266363 | 4 | 6 | 1 (4) | 1028 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 39350.04 | 2268333 | 38576.76 | 2255445 | 504 | 120 | 23128 | 2267803 | 11 | 0 | 0 (0) | 1008 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 39450 | 2269999 | 38676.72 | 2257111 | 504 | 120 | 23134 | 2269531 | 6 | 3 | 0 (0) | 1014 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 39549.96 | 2271665 | 38776.68 | 2258777 | 504 | 120 | 23140 | 2271259 | 1 | 2 | 1 (4) | 1020 |
| 400, | CA_n260(A-I): Maximum frequency separation = 800 MHz (Note 5), Wgap SB1-SB2=50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50+100+100+100 | SB1 | CC1 | 400 | 264 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth=400 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | 37475.64 | 2237093 | 37452.6 | 2236709 | 0 | 120 | 23021 | 2236987 | 7 | 1 | 0 (0) | 2 |
|  |  |  |  |  |  |  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 37550.04 | 2238333 | 37502.52 | 2237541 | 0 | 120 | 23024 | 2237851 | 11 | 2 | 0 (0) | 4 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 37650 | 2239999 | 37602.48 | 2239207 | 0 | 120 | 23030 | 2239579 | 6 | 1 | 1 (4) | 10 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 37749.96 | 2241665 | 37702.44 | 2240873 | 0 | 120 | 23036 | 2241307 | 1 | 4 | 1 (4) | 16 |
|  | SB1 | CC1 | 400 | 264 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=400 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink | 39225 | 2266249 | 38476.2 | 2253769 | 504 | 120 | 23123 | 2266363 | 9 | 6 | 1 (4) | 1028 |
|  |  |  |  |  |  |  | Channel spacing CC1-CC2=74.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 39299.4 | 2267489 | 38526.12 | 2254601 | 504 | 120 | 23125 | 2266939 | 1 | 0 | 0 (0) | 1008 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 39399.36 | 2269155 | 38626.08 | 2256267 | 504 | 120 | 23131 | 2268667 | 8 | 2 | 0 (0) | 1012 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 39499.32 | 2270821 | 38726.04 | 2257933 | 504 | 120 | 23137 | 2270395 | 3 | 1 | 1 (4) | 1018 |
| 400, | CA_n260(A-I): Maximum frequency separation = 1000 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100+100+100+100 | SB1 | CC1 | 400 | 264 | Low | Downlink & Uplink | Same test frequencies as n260 for Low range and channel bandwidth= 400 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 37650.12 | 2240001 | 37602.6 | 2239209 | 0 | 120 | 23030 | 2239579 | 5 | 1 | 1 (4) | 10 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 37750.08 | 2241667 | 37702.56 | 2240875 | 0 |  | 23036 | 2241307 | 0 | 4 | 1 (4) | 16 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 37850.04 | 2243333 | 37802.52 | 2242541 | 0 |  | 23042 | 2243035 | 7 | 6 | 1 (4) | 20 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 37950 | 2244999 | 37902.48 | 2244207 | 0 |  | 23047 | 2244475 | 2 | 1 | 0 (0) | 2 |
|  | SB1 | CC1 | 400 | 264 | High | Downlink & Uplink | Same test frequencies as n260 for High range and channel bandwidth=400 MHz and SCS=120kHz in Table 4.3.1.2.1.4-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink | 39050.04 | 2263333 | 38276.76 | 2250445 | 504 | 120 | 23111 | 2262907 | 3 | 1 | 1 (4) | 1018 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  |  | 39150 | 2264999 | 38376.72 | 2252111 | 504 | 120 | 23117 | 2264635 | 10 | 3 | 1 (4) | 1022 |
|  |  |  |  |  |  |  | Channel spacing CC2-CC3=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  |  | 39249.96 | 2266665 | 38476.68 | 2253777 | 504 | 120 | 23123 | 2266363 | 5 | 6 | 1 (4) | 1028 |
|  |  |  |  |  |  |  | Channel spacing CC3-CC4=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  | 39349.92 | 2268331 | 38576.64 | 2255443 | 504 | 120 | 23128 | 2267803 | 0 | 1 | 0 (0) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC channel bandwidth combination. Note 2: CCs within the sub-block are specified in increasing frequency order. CC1 in SB1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: The maximum frequency separation has been selected as the lowest DL maximum frequency separation in TS 38.101-2 [8],Table 5.3A.4-2 that fits the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.4.4.2-2: NR Intra-Band non-contiguous CA configuration CA_n260(A-I) with UL CA, SCS=120 kHz, Max Wgap

| CBW combination | Sub-block Note 2 | CC within Sub-Block Note 2 | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+100 | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 50 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink & Uplink | 37575 | 2238749 | 37551.96 | 2238365 | 0 | 120 | 23027 | 2238715 | 7 | 0 | 1 (4) | 8 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink & Uplink | 39425.04 | 2269583 | 38676.24 | 2257103 | 504 | 120 | 23134 | 2269531 | 10 | 3 | 0 (0) | 1014 |
| 100+100 | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 50 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink & Uplink | 37575 | 2238749 | 37551.96 | 2238365 | 0 | 120 | 23027 | 2238715 | 7 | 0 | 1 (4) | 8 |
|  | SB1 | CC1 | 100 | 66 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 50 | 32 |  | Downlink & Uplink | 39425.04 | 2269583 | 38676.24 | 2257103 | 504 | 120 | 23134 | 2269531 | 10 | 3 | 0 (0) | 1014 |
| 50+100 | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 100 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37549.92 | 2238331 | 37502.4 | 2237539 | 0 | 120 | 23024 | 2237851 | 0 | 3 | 0 (0) | 6 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 39450 | 2269999 | 38676.72 | 2257111 | 504 | 120 | 23134 | 2269531 | 6 | 3 | 0 (0) | 1014 |
| 100+100 | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=100 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 100 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37549.92 | 2238331 | 37502.4 | 2237539 | 0 | 120 | 23024 | 2237851 | 0 | 3 | 0 (0) | 6 |
|  | SB1 | CC1 | 100 | 66 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 39450 | 2269999 | 38676.72 | 2257111 | 504 | 120 | 23134 | 2269531 | 6 | 3 | 0 (0) | 1014 |
| 50+100 | CA_n260(A-I): Maximum frequency separation = 600 MHz (Note 5), Wgap SB1-SB2=50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 200 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37500 | 2237499 | 37404.96 | 2235915 | 0 | 120 | 23019 | 2236411 | 8 | 6 | 1 (4) | 20 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 200 | 132 |  | Downlink & Uplink | 39500.04 | 2270833 | 38679.24 | 2257153 | 504 | 120 | 23134 | 2269531 | 9 | 1 | 0 (0) | 1010 |
| 100+100 | CA_n260(A-I): Maximum frequency separation = 800 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 200 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37699.92 | 2240831 | 37604.88 | 2239247 | 0 | 120 | 23030 | 2239579 | 10 | 3 | 0 (0) | 6 |
|  | SB1 | CC1 | 100 | 66 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 200 | 132 |  | Downlink & Uplink | 39300 | 2267499 | 38479.2 | 2253819 | 504 | 120 | 23123 | 2266363 | 8 | 4 | 1 (4) | 1024 |
| 50+100 | CA_n260(A-I): Maximum frequency separation = 800 MHz (Note 5), Wgap SB1-SB2=50 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 50 | 32 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 400 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37599.96 | 2239165 | 37409.88 | 2235997 | 0 | 120 | 23019 | 2236411 | 3 | 3 | 1 (4) | 14 |
|  | SB1 | CC1 | 50 | 32 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 50+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 400 | 264 |  | Downlink & Uplink | 39400.08 | 2269167 | 38484.24 | 2253903 | 504 | 120 | 23123 | 2266363 | 2 | 1 | 1 (4) | 1018 |
| 100+100 | CA_n260(A-I): Maximum frequency separation = 1000 MHz (Note 5), Wgap SB1-SB2=200 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| +100+100 | SB1 | CC1 | 100 | 66 | Low | Downlink | Same test frequencies as for CA_n260I for Low range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
| , 400 |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 100 | 66 |  | Downlink & Uplink | 37800 | 2242499 | 37609.92 | 2239331 | 0 | 120 | 23030 | 2239579 | 4 | 0 | 0 (0) | 0 |
|  | SB1 | CC1 | 100 | 66 | High | Downlink | Same test frequencies as for CA_n260I for High range, CBW combination 100+100+100+100 and SCS=120kHz in Table 4.3.1.2.3.4.8-2. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 100 | 66 |  | & |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC3 | 100 | 66 |  | Uplink |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC4 | 100 | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB2 | CC1 | 400 | 264 |  | Downlink & Uplink | 39200.04 | 2265833 | 38284.2 | 2250569 | 504 | 120 | 23111 | 2262907 | 1 | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC channel bandwidth combination. Note 2: CCs within the sub-block are specified in increasing frequency order. CC1 in SB1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: The maximum frequency separation has been selected as the lowest DL maximum frequency separation in TS 38.101-2 [8],Table 5.3A.4-2 that fits the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |