3GPP TSG-RAN5 Meeting # 91-e	R5-213863

# Electronic Meeting,  17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1930 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Testing frequencies update for band n66 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Current testing frequencies for band n66 include wrong Offset Carrier CORESET#0 [RBs] for low frequency, SCS= 15 kHz and channel bandwidth combination 20/40 (It should be 1 instead of 6). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For SCS=15 kHz, Offset Carrier CORESET#0 [RBs] for low frequency and channel bandwidth combination 20/40 corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Affected test frequencies will remain untestable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-213170 and the outcome of 2 revisions |  |  |  |  |  |  |  |  |

<< Start of change >>

###### 4.3.1.1.1.66	Reference test frequencies for NR operating band n66

Table 4.3.1.1.1.66-1: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2124.6 | 424920 | 102 |  | 5364 | 429150 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2084.74 | 416948 | 504 |  | 5446 | 435650 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1737.5 | 347500 | 1644.53 | 328906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1762.5 | 352500 | 1759.17 | 351834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2150 | 430000 | 2122.1 | 424420 | 102 |  | 5357 | 428650 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2180 | 436000 | 2079.74 | 415948 | 504 |  | 5432 | 434650 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1644.6 | 328920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1759.24 | 351848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  | 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2114.67 | 422934 | 102 |  | 5337 | 426990 | 8 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2167.5 | 433500 | 2064.81 | 412962 | 504 |  | 5394 | 431550 | 4 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2112.24 | 422448 | 102 |  | 5331 | 426510 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2165 | 433000 | 2059.88 | 411976 | 504 |  | 5381 | 430570 | 6 | 0 | 1 (2) | 506 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  | 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-1A: Test frequencies for NR operating band n66 and asymmetric channel bandwidth combination set 1, uplink and downlink channel bandwidth combination and SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/20 | 20 | 106 | Downlink | Low | Same as for UL/DLBandwidth combination 5/20 in Table 4.3.1.1.1.66-1. |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 25 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 5/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2124.67 | 424934 | 102 |  | 5365 | 429170 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2187.5 | 437500 | 2084.81 | 416962 | 504 |  | 5444 | 435610 | 0 | 0 | 2 (4) | 508 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1742.5 | 348500 | 1649.53 | 329906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1769.17 | 353834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | Same as for UL/DLBandwidth combination 5/40 in Table 4.3.1.1.1.66-1. |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 25 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/20 | 20 | 106 | Downlink | Low | Same as for UL/DLBandwidth combination 10/20 in Table 4.3.1.1.1.66-1. |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2122.17 | 424434 | 102 |  | 5358 | 428670 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2182.5 | 436500 | 2079.81 | 415962 | 504 |  | 5433 | 434670 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | Same as for UL/DLBandwidth combination 10/40 in Table 4.3.1.1.1.66-1. |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/40 | 40 | 216 | Downlink | Low | Same as for UL/DLBandwidth combination 20/40 in Table 4.3.1.1.1.66-1. |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 | 106 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 25/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2114.7 | 422940 | 102 |  | 5337 | 426990 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2064.84 | 412968 | 504 |  | 5394 | 431550 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2150 | 430000 | 2112.2 | 422440 | 102 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2059.84 | 411968 | 504 |  | 5383 | 430610 | 10 | 1 | 1 (2) | 507 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-2: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  | 5361 | 428910 | 18 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  | 5436 | 434910 | 18 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2150 | 430000 | 2104.1 | 420820 | 102 |  | 5360 | 428890 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1989.38 | 397876 | 504 |  | 5435 | 434890 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1554.24 | 310848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1758.52 | 351704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  | 5355 | 428430 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  | 5423 | 433930 | 2 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  | 5349 | 427950 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  | 5413 | 433010 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2096.58 | 419316 | 102 |  | 5343 | 427470 | 6 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2167.5 | 433500 | 1974.36 | 394872 | 504 |  | 5400 | 432030 | 2 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2094.24 | 418848 | 102 |  | 5337 | 426990 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2165 | 433000 | 1969.52 | 393904 | 504 |  | 5387 | 431050 | 22 | 0 | 1 (6) | 1020 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  | 5325 | 426030 | 18 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  | 5361 | 428910 | 2 | 0 | 0 (5) | 1018 |
|  | 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-2A: Test frequencies for NR operating band n66 asymmetric channel bandwidth combination set 1, uplink and downlink channel bandwidth combinationand SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/20 | 20 | 106 | Downlink | Low | Same as for UL/DLBandwidth combination 10/20 in Table 4.3.1.1.1.66-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2104.08 | 420816 | 102 |  | 5361 | 428910 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2182.5 | 436500 | 1989.36 | 397872 | 504 |  | 5436 | 434910 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2155 | 431000 | 2104.24 | 420848 | 102 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2185 | 437000 | 1989.52 | 397904 | 504 |  | 5437 | 434930 | 6 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | Same as for UL/DLBandwidth combination 10/40 in Table 4.3.1.1.1.66-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 20/40 | 40 | 216 | Downlink | Low | Same as for UL/DLBandwidth combination 20/40 in Table 4.3.1.1.1.66-2. |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 | 106 | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| 25/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2096.7 | 419340 | 102 |  | 5343 | 427470 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1974.48 | 394896 | 504 |  | 5400 | 432030 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2150 | 430000 | 2094.2 | 418840 | 102 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1969.48 | 393896 | 504 |  | 5389 | 431090 | 14 | 0 | 2 (7) | 1022 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-3: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  |  | Mid | 2145 | 429000 | 2067.6 | 413520 | 102 |  | - | 428568 |
|  |  |  |  | High | 2175 | 435000 | 1808.16 | 361632 | 504 |  | - | 434568 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2150 | 430000 | 2067.92 | 413584 | 102 |  | - | 428632 |
|  |  |  |  | High | 2180 | 436000 | 1808.48 | 361696 | 504 |  | - | 434632 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1373.16 | 274632 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1756.72 | 351344 | 6 |  | - | - |
| 15/15 | 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  |  | Mid | 2145 | 429000 | 2065.08 | 413016 | 102 |  | - | 428064 |
|  |  |  |  | High | 2172.5 | 434500 | 1803.14 | 360628 | 504 |  | - | 433564 |
|  | 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |  | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |  | - | - |
| 20/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2145 | 429000 | 2062.92 | 412584 | 102 |  | - | 427632 |
|  |  |  |  | High | 2170 | 434000 | 1798.48 | 359696 | 504 |  | - | 432632 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 20/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 25/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  |  | Mid | 2145 | 429000 | 2060.4 | 412080 | 102 |  | - | 427128 |
|  |  |  |  | High | 2167.5 | 433500 | 1793.46 | 358692 | 504 |  | - | 431628 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 30/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  |  | Mid | 2145 | 429000 | 2057.88 | 411576 | 102 |  | - | 426624 |
|  |  |  |  | High | 2165 | 433000 | 1788.44 | 357688 | 504 |  | - | 430624 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 40/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2145 | 429000 | 2053.2 | 410640 | 102 |  | - | 425688 |
|  |  |  |  | High | 2160 | 432000 | 1778.76 | 355752 | 504 |  | - | 428688 |
|  | 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |  | - | - |
|  |  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-3A: Test frequencies for NR operating band n66 and asymmetric channel bandwidth combination set 1, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/20 | 20 | 24 | Downlink | Low | Same as for UL/DLBandwidth combination 10/20 in Table 4.3.1.1.1.66-3. |  |  |  |  |  |  |  |
|  | 10 | 11 |  | Mid |  |  |  |  |  |  |  |  |
| 10/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2067.9 | 413580 | 102 |  | - | 428628 |
|  |  |  |  | High | 2182.5 | 436500 | 1808.46 | 361692 | 504 |  | - | 434628 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  |  | Mid | 2155 | 431000 | 2067.88 | 413576 | 102 |  | - | 428624 |
|  |  |  |  | High | 2185 | 437000 | 1808.44 | 361688 | 504 |  | - | 434624 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 20/40 | 40 | 51 | Downlink | Low | Same as for UL/DLBandwidth combination 20/40 in Table 4.3.1.1.1.66-3. |  |  |  |  |  |  |  |
|  | 20 | 24 |  | Mid |  |  |  |  |  |  |  |  |
| 25/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2152.5 | 430500 | 2060.7 | 412140 | 102 |  | - | 427188 |
|  |  |  |  | High | 2175 | 435000 | 1793.76 | 358752 | 504 |  | - | 431688 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 30/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2150 | 430000 | 2058.2 | 411640 | 102 |  | - | 426688 |
|  |  |  |  | High | 2170 | 434000 | 1788.76 | 357752 | 504 |  | - | 430688 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.1.67 – 4.3.1.1.1.69	FFS

<< End of change>>