3GPP TSG-RAN5 Meeting #91-e	R5-213908

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0975 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | References to voided clause 5.2B.2 corrected |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 5.3B.2 is voided . Some test cases has references to that clause and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References to 5.2B.2 corrected to 5.5B.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect refrences will remain in 38.521-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.4.1.4.1, 7.3B.2.3_1.2.4.1, 7.4B.1.4.1, 7.5B.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-212645 and he result of 1 revision |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

6.5B.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.5B.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Table 6.5B.4.1.4.1-1 for E-UTRA and Table 6.5B.4.1.4.1-2 for NR. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex TS 36.521-1 [10] Annex C and in TS 38.521-1 [8] Annex C2 for LTE link and NR link respectively.

Table 6.5B.4.1.4.1-1: EN-DC test configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |
|  |  |  | E-UTRA Cell |  | NR Cell |  | Common |
|  |  |  | Modulation | RB allocation (Note 5) | Modulation | RB allocation (Note 1) | Power config (Note 6) |
| 1 | Default |  | 16QAM | Outer_Full | DFT-s-OFDM QPSK | Outer_Full | B or C |
| 2 (Note 3) | Default |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | Edge_1RB_Right | B or D |
| 3 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | DFT-s-OFDM QPSK | N/A | A |
| 4 (Note 3) | High |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Right | A |
| 5 (Note 4) | Default |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | Edge_1RB_Left | B or D |
| 6 (Note 4) | Low |  | 16QAM | N/A | DFT-s-OFDM QPSK | Edge_1RB_Left | A |
| 7 (Note 4) | High |  | 16QAM | Outer_1RB_Right | DFT-s-OFDM QPSK | N/A | A |
| 8 | Default |  | 16QAM | Outer_Full | CP-OFDM 256QAM | Outer_Full | B or C |
| 9 (Note 3) | Low |  | 16QAM | Outer_1RB_Left | CP-OFDM 256QAM | Edge_1RB_Right | B or D |
| 10 (Note 4) | High |  | 16QAM | Outer_1RB_Right | CP-OFDM 256QAM | Edge_1RB_Left | B or D |
| 11 (Note 4) | Default |  | 16QAM | Edge_Full_Right | CP-OFDM 256QAM | Edge_Full_Left | B or E |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. NOTE 3: Applicable when E-UTRA cell carrier frequency is lower than NR cell carrier. NOTE 4: Applicable when NR cell carrier frequency is lower than E-UTRA cell carrier. NOTE 5: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. NOTE 6: Power config as specified in Table 6.2B.3.1.4.3-2. NOTE 7: Test IDs with simultaneous E-UTRA and NR UL transmission only apply for UEs indicating dualPA-Architecture. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for SS diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	E-UTRA downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

5.	NR downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 of TS 38.521-1 [8].

6.	The UL Reference Measurement channels are set according to Table 6.5B.4.1.4.1-1.

7.	NR propagation conditions are set according to B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.5B.4.1.4.3.

<< Clauses skipped here>>

7.3B.2.3_1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.5B.2-1. The 4CC EN-DC configurations listed in table 7.3B.2.3_1.2.4.1-0 shall not be tested according to TR 38.905 test point analysis. The other 4CC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

Table 7.3B.2.3_1.2.4.1-0: 4CC EN-DC configurations that shall not be tested

| EN-DC type |  | E-UTRA CA | NR CA | Notation |
| --- | --- | --- | --- | --- |
| Intra-band non-contiguous EN-DC (1 band) |  | Yes (cont) | No | DC_XD_nXA |
|  |  | Yes (non-cont) | No | DC_XA-XC_nXA |
| Inter-band EN-DC | Yes (all types) | No | DC_XD-nYA, DC_XA-YC_nZA, DC_XA-XA-YA-nZA, DC_XA-YA-ZA_nRA |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC |
|  |  | Yes (cont) | DC_XC_nY(2A), DC_XC_nY(2A), DC_XA-YA_nY(2A) |
|  |  | Yes (inter) | DC_XC-nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA |
| Intra-band contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA |
|  | Yes | Yes | DC_(n)XCA-nYA |
| Intra-band non contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA-ZA_nZA |
|  | Yes | Yes | DC_XA-YA_nYA-nZA |
| NOTE 1: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 2: The table apply to all band numbers, i.e. all values of X, Y and Z defined in TS38.101-3 NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. |  |  |  |

Table 7.3B.2.3_1.2.4.1-1: Test Configuration Table for EN-DC configurations not affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_(n)XDA: Low range, High Range DC_XA-nYA-nZC: Mid range for X Low range, High Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 (Note 6) | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 (Note 7) | PCC(M) | X |  | Highest NRB | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration– Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: LTE anchor agnostic configuration NOTE 5: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases NOTE 6: Test point for UE supporting dual UL NOTE 7: Test point for UE supporting single UL NOTE 8: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-2: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_XA-nYA-nZC: Mid range for X Mid Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N/A |  |  |  |  |  |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: No refsens exceptions with this EN-DC configuration exist in TS38.101-3 NOTE 5: This test setting is valid for EN-DC configurations affected by UL harmonics, harmonic mixing or cross band isolation exceptions when the exception is avoided by selecting specific test frequency. Test only one of the test points. UL to be active on the cell that form the aggressor of the exception NOTE 6: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. NOTE 7: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). NOTE 8: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-3: Void

<< Clauses skipped here>>

7.4B.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in table 5.5B.2-1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 7.4B.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A.2 for E-UTRA RMC for FDD , and TS 38.521-1 [8] Annex A.2 for NR RMC Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.4B.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 5) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  | Lowest |  |  |  |
| Test Parameters for Intra-band Contiguous EN-DC Configuration |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |  |
| Test ID | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation |  | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |  |
| 1 | CP-OFDM 64QAM | Full RB (NOTE 1) | CP-OFDM 64QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |  |
| 2 | CP-OFDM 256QAM | Full RB (NOTE 1) | CP-OFDM 256QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Same RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 3: Same RB allocation shall be used per the E-UTRA band and channel BW as specified in Table 7.3.3-2 of TS 36.521 [10]. NOTE 4: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE5: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1,C.2,C3.1 and TS 38.521-1 [8] Annex C.0,C.1,C.2,C3.1for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H.0,H.1,H.2,H.3.1 and TS 38.521-1 [8] Annex G.0,G.1,G.2,G.3.1 for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.4B.1.4.3.

<< Clauses skipped here>>

7.5B.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in table 5.5B.2-1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.5B.2, and are shown in table 7.3B.2.1.4.1-1 for intra-band DC_(n)71AA.

In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements.

The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A.2 for E-UTRA RMC for FDD , TS 38.521-1 [8] Annex A.2 for NR UL RMC and TS 38.521-1 [8] Annex A.3 for NR DL RMC. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

1.	Connect the SS to the UE antenna connectors as shown in A.3.1.1 for SS diagram and A.3.2 for UE diagram in TS 38.508-1 [6].

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.5B.1.4.3.

<< End of changes>>