3GPP TSG-RAN5 Meeting #91-e	R5-213917

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1029 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC Rx test case and format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas, Keysight, Anritsu, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the 3GU style and format of DC configuration are incorrectly indicated; The test cases has inconsistence and need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated 3GU style; - Deleted redundant row; - Update DC configuration between LTE and NR to use “_”; - Added missing bandwidth class in the test configuration table; - Removed redundant column in Table 7.8B.2.3_1.1.4.2-1 and information indicated in NOTE 3; - Corrected initial condition reference clause and test repetition and measurement configuration to TC7.8B.2.3_1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The format will stay incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3, 7.3B.2.3_1.1, 7.8B.2.3_1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Re-issue tdoc from R5-212751 with following changes: undo change to 7.8B.2.3.1.1 editor’s note to avoid conflicted with R5-212629 and R5-212347; Correction format to Table 7.8B.2.3_1.1.4.2-1. Revised from R5-213386 with following changes: r1: Update CR Title and source company to align with TdocList. r2: Corrected initial condition reference clause of 7.8B.2.3_1.1; deleted redundant EN-DC configuration as 1CC fallback tested from Table 7.8B.2.3_1.1.4.2-1 Test repetition and measurement configuration. |  |  |  |  |  |  |  |  |

## {Start of changes}

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

<Unchanged Sections Skipped>

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |
|  | DC Configuration |  |  |  |  |  | E-UTRA Ch BW |  | NR Ch BW |  | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR |  | E-UTRA & NR allocations (LCRB @ RBstart) |  |  |  |
|  | E-UTRA |  |  |  | NR |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |
|  | Band |  | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  | Mid |  | Y | Mid | 5 |  | Mid |  | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK |  | REFSENS |  |  | REFSENS |
| Test Settings for DC_1A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 5 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 7 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 20 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 77/ 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 5 | High | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 66 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 77 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 1 | High | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 3 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 41 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A_n77A/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| 2 | 8 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 79 | Note 5 | 5 |  | 40 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 216@0 |
| Test Settings for DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | All RBs |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Note 5 |  | 79 |  | Note 5 | 5 | 40 |  | Note 7 |  | All RBs |  |  | Note 7 |  | 25@0 | 216@0 |  |
| Test Settings for DC_26_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 41 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_26A_n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 1 | 26 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_48A- n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 |  | Note 5 |  | 66 | Note 5 | 20 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 100@0 |  |  | 25@0 |
| Test Settings for DC_28A- n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 41 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A_n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 50 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A_n51A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 51 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_28A_n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 28 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 2 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 71 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for CA_7A_20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 20 | Note 5 | 10 |  | 5 |  | QPSK | All RBs |  | QPSK |  | 50@0 |  |  | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  | Band |  |  |  | Range |  | NRB |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc / UL alloc | UL MOD |  |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 | Mid |  |  |  |  | n3 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | QPSK |  | 40 MHz | All RBs / 0 |  |
| 24,10 | 1 | UL 1950 / DL 2140 |  |  |  |  | n3 |  |  |  | UL 1760 / DL 1855 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 36 | 1 | Low |  |  |  |  | n3 |  |  |  | High |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / 0 |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 1 | Mid |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | N/A | N/A |  | 5MHz |  | N/A | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100MHz | All RBs / REFSENS_NR |  |
| 24 | 1 | UL 1950 / DL 2140 |  |  |  |  | n78 |  |  |  | 3710 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 26 | 3 | High |  |  |  |  | n1 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_3 |  |
| 34 | 3 | UL 1760 / DL 1855 |  |  |  |  | n1 |  |  |  | UL 1950 / DL 2140 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 47 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | N/A |  | 5MHz |  | N/A | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | Low |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 26 | 3 | Mid |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 36 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 46 | 3 | High |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 54, 10 | 3 | DL 1835 |  |  |  |  | n41 |  |  |  | 2657.5 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3495 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3525 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3685 |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 3a9 | 3 | Low |  |  |  |  | n78 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 |  | All RBs/0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | UL 1740 / DL 1835 |  |  |  |  | n78 |  |  |  | 3575 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | UL 1765 / DL 1860 |  |  |  |  | n78 |  |  |  | 3435 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 7 | DL 2655 |  |  |  |  | n3 |  |  |  | DL 1825 |  |  |  |  |
|  | QPSK | N/A |  | 10 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 8 | DL 932.5 |  |  |  |  | n1 |  |  |  | DL 2155 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 8 | DL 945 |  |  |  |  | n3 |  |  |  | DL 1850 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 27 | 8 | DL 942.5 |  |  |  |  | n3 |  |  |  | DL 1842.5 |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | N/A |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 | Mid |  |  |  |  | n77 |  |  |  | 3590 |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 22,8 | 8 | Mid |  |  |  |  | n77 |  |  |  | 3520 |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 8 | UL 897.5 |  |  |  |  | n77 |  |  |  | 3635 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 | Mid |  |  |  |  | n79 |  |  |  | 4457.7 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 22,9 | 11 | Mid |  |  |  |  | n79 |  |  |  | 4512.7 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 | DL 884 |  |  |  |  | n79 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| 22,9 | 19 | DL 884 |  |  |  |  | n79 |  |  |  | 4445.02 |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 20 | Mid |  |  |  |  | n3 |  |  |  | Mid |  |  |  |  |
|  | QPSK | N/A |  | 5 MHz |  | N/A | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 40 MHz | All RBs / REFSENS_NR |  |
| 24 | 20 | UL 840 / DL 799 |  |  |  |  | n3 |  |  |  | UL 1775 / DL 1870 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 34 | 20 | UL 847 / DL 806 |  |  |  |  | n3 |  |  |  | UL 1735 / DL 1830 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,8 | 20 | Mid |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 20 | Mid |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | UL 850 / DL 809 MHz |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  | n41 |  | 2524.5 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 22,8 | 26 | High |  |  |  |  |  | n41 |  | 2572 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 50 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 839 |  |  |  |  |  | n41 |  | 2562 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 | High |  |  |  |  |  | n77/n78 |  | 3366 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 22,8 | 26 | High |  |  |  |  |  | n77/n78 |  | 3446 |  |  |  |  |  |
|  | QPSK | QPSK | 15 MHz |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  | QPSK |  | 100 MHz |  |  | All RBs / 0 |
| 34 | 26 | UL 836.5 |  |  |  |  |  | n77/n78 |  | 3391 |  |  |  |  |  |
|  | QPSK | QPSK | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 10 MHz |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 | High |  |  |  |  |  | n79 |  | 4432.5 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 | High |  |  |  |  |  | n79 |  | 4470.5 |  |  |  |  |  |
|  | N/A | QPSK | 15 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 60 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_40_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  | n78 |  | 3525 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 40 | Low |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |
|  | QPSKN/A | QPSK | 20 MHz |  | All RBs / REFSENS_ENDC_20 |  |  | DFT-s-OFDM QPSKN/A |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_20 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Mid |  |  |  |  |  | n77/n78 |  | 3889.5 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 | Mid |  |  |  |  |  | n77/n78 |  | 3974.5 |  |  |  |  |  |
|  | N/A | QPSK | 20 MHz |  | All RBs / 0 |  |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  | 100 MHz |  |  | All RBs / REFSENS_ENDC_3 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation according to table 7.3B.2.0.3.5.1-1. NOTE 2: Test ID with no exception NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Test ID with LTE anchor agnostic, which applies to NSA only capable UEs NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84.. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

Reference sensitivity test requirements for EN-DC configurations without any reference sensitivity exception are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC with uplink configuration specified in Table 7.3B.2.3.4.2.1-0.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

<Unchanged Sections Skipped>

#### 7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)

##### 7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

7.3B.2.3_1.1.1	Test purpose

To verify the ability of UE that support EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply whenever possible. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.3_1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3CC EN-DC.

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for EN-DC configurations not affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA_nZA and DC_XA-YA_nZA: Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range For non-contiguous carriers: TBD |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  | CG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  |  | nX | default |  |  |
|  | QPSK | REFSENS | Highest | All RBs / REFSENS | N/A | QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2) |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2) |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA_nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| 2 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | CP-OFDM QPSK | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / REFSENS | N/A | QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |
|  | Band |  |  | Range | NRB |  |  |  |  | Band | Range |  |  |  | NRB |  |  |  |  | Band |  | Range | NRB |  |  |  |
|  | UL MOD |  |  | DL MOD | CH BW |  | DLalloc/UL alloc |  |  | UL MOD | DL MOD |  |  |  | UL/DL Ch BW |  |  | DLalloc/ ULalloc |  | UL MOD |  | DL MOD | UL/DL Ch BW |  | DLalloc/UL alloc |  |
| Default Test Settings for a DC_(n)XAC Configuration (Intra-band contiguous EN-DC with NR CA) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-XA_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 2, 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  | default |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nY |  | default |  |  |  |  |
|  | QPSK |  |  | REFSENS | Highest |  | All RBs / REFSENS |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | N/A |  | CP-OFDM QPSK | Highest |  | All RBs / 0 |  |
| 2 | X |  |  | default |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nY |  | default |  |  |  |  |
|  | N/A |  |  | REFSENS | Highest |  | All RBs / 0 |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK | Highest |  | All RBs / REFSENS |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA_nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 2, 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  | Mid |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nZ |  | default |  |  |  |  |
|  | N/A |  |  | N/A | 5 MHz |  | 0/0 |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK | Highest |  | All RBs / REFSENS |  |
| 2 | X |  |  | Mid |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nZ |  | default |  |  |  |  |
|  | N/A |  |  | N/A | 5 MHz |  | 0/0 |  |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / REFSENS |  | N/A |  | CP-OFDM QPSK | Highest |  | All RBs / 0 |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2140 MHz |  |  |  |  |  | 3 |  |  | DL 1807.5 MHz |  |  |  |  |  |  | n78 |  | 3757.5 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2125 MHz |  |  |  |  |  | 3 |  |  | DL 1870 MHz |  |  |  |  |  |  | n78 |  | 3725.5 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2122 MHz |  |  |  |  |  | 5 |  |  | DL 874 MHz |  |  |  |  |  |  | n78 |  | 3780 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2165 MHz |  |  |  |  |  | 5 |  |  | DL 885 MHz |  |  |  |  |  |  | n78 |  | 3405 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  |  | 7 |  |  | DL 2627.5 MHz |  |  |  |  |  |  | n78 |  | 3305 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2140 MHz |  |  |  |  |  | 7 |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  | n78 |  | 3580 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 10MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | UL 1930 / DL 2120 MHz |  |  |  |  |  | 20 |  |  | DL 794 MHz |  |  |  |  |  |  | n78 |  | 3790 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2140 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3330 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 1875 MHz |  |  |  |  |  | 28 |  |  | DL 765.5 MHz |  |  |  |  |  |  | n3 |  | 1875 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1820 MHz |  |  |  |  |  | 7 |  |  | DL 2685 MHz |  |  |  |  |  |  | n78 |  | 3310 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 3 | DL 1820 MHz |  |  |  |  |  | 7 |  |  | DL 2685 MHz |  |  |  |  |  |  | n78 |  | 3475 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1820 MHz |  |  |  |  |  | 8 |  |  | DL 955 MHz |  |  |  |  |  |  | n78 |  | 3640 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1820 MHz |  |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  | n78 |  | 3510 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | UL 1735 / DL 1830 MHz |  |  |  |  |  | 40 |  |  | DL 2380 MHz |  |  |  |  |  |  | n1A |  | UL 1950 / 2140 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5 | DL 889 MHz |  |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  | n78 |  | 3489 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 5 | DL 879 MHz |  |  |  |  |  | 7 |  |  | DL 2670 MHz |  |  |  |  |  |  | n78 |  | 3429 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 5 | DL 875 MHz |  |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  | n78 |  | 3350 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | 2630 MHz |  |  |  |  |  | 20 |  |  | 800 MHz |  |  |  |  |  |  | n1 |  | 2130 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 10 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | DL 2663 MHz |  |  |  |  |  | 20 |  |  | DL 806 MHz |  |  |  |  |  |  | n3 |  | DL 1832 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 10 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| 2 |  | 7 | DL 2630 MHz |  |  |  |  |  | 20 |  |  | DL 896 MHz |  |  |  |  |  |  | n3 |  | DL 1870 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | UL 2560 / DL 2680 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3370 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 7 | UL 2560 / DL 2680 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3435 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 7 | DL 2675 MHz |  |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  | n78 |  | 3520 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | DL 2663 MHz |  |  |  |  |  | 28 |  |  | DL 769 MHz |  |  |  |  |  |  | n3 |  | 1842 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| 2 |  | 7 | DL 2685 MHz |  |  |  |  |  | 28 |  |  | DL 800 MHz |  |  |  |  |  |  | n3 |  | 1810 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | DL 2687.5 MHz |  |  |  |  |  | 20 |  |  | DL 782.5 MHz |  |  |  |  |  |  | n78 |  | 3350 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 7 | DL 2687.5 MHz |  |  |  |  |  | 20 |  |  | DL 782.5 MHz |  |  |  |  |  |  | n78 |  | 3460 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 7 | DL 2650 MHz |  |  |  |  |  | 20 |  |  | DL 795 MHz |  |  |  |  |  |  | n78 |  | 3390 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 2 | DL 1954 MHz |  |  |  |  |  | 14 |  |  | DL 763 MHz |  |  |  |  |  |  | n66 |  | DL 2170 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 14 | DL 763 MHz |  |  |  |  |  | 66 |  |  | DL 2162 MHz |  |  |  |  |  |  | n2 |  | DL 1954 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 | Mid |  |  |  |  |  | n28 |  |  | High |  |  |  |  |  |  | n75 |  | DL 1476MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 20MHz | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |
| 28 |  | 20 | Mid |  |  |  |  |  | n28 |  |  | Low |  |  |  |  |  |  | n75 |  | High |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 20MHz | All RBs / REFSENS_ENDC_1 |  | N/A |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |
| Note 1: No refsens exceptions with this EN-DC configuration exist in TS38.101-3 Note 2: This test setting is valid for EN-DC configurations affected by UL harmonics, harmonic mixing exceptions when the exception is avoided by selecting specific test frequency. Test only one of the test points. UL to be active on the cell that form the aggressor of the exception Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for 2UL UE. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.1.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 2UL intermodulation interference in NR FR1, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for EN-DC configurations not affected by 2UL intermodulation interference in NR FR1, are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

<Unchanged Sections Skipped>

##### 7.8B.2.3_1.1	Wideband Intermodulation for EN-DC within FR1 (3 CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	Test repetition and measurement configuration for DC_XA-nYA-nZA, DC_XA-nYC and DC_XA-nXA-nYA still TBD.

7.8B.2.3_1.1.1	Test Purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8B.2.3_1.1.2	Test Applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC within FR1 (3 CCs).

7.8B.2.3_1.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.8B.2.0.

7.8B.2.3_1.1.4	Test Description

7.8B.2.3_1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.2, 5.5B.3, and 5.5B.4, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.5B.2, 5.5B.3, and 5.5B.4 and are shown in table 7.8B.2.3_1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.8B.2.3_1.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | Mid range |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Highest NRB_agg |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Highest |
| Network signalling value |  |  |  |  |  | NS_01 by default |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |
| EN-DC Configurations | Environment | Frequency | Bandwidth Combination | SCS | Other Parameter Settings |  |
| DC_(n)XCA | Default | Default | Default | Default | As per DC_(n)XCA in Table 7.3B.2.3_1.1.4.1-0 |  |
| DC_XC_nXA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.2) |  |
| DC_XA-XA_nXA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.2) |  |
| DC_XA_nYC | Default | Default | Default | Default | As per DC_XA_nYC in Table 7.3B.2.3_1.1.4.1-0 |  |
| DC_XA_nY(2A) | Default | Default | Default | Default | As per DC_XA_nY(2A) in Table 7.3B.2.3_1.1.4.1-0 |  |
| DC_XA_nYA-nZA | Default | Default | Default | Default | As per DC_XA_nYA-nZA in Table 7.3B.2.3_1.1.4.1-0 |  |
| DC_XC_nYA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |  |
| DC_XA-XA_nYA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |  |
| DC_XA-YA_nZA | - | - | - | - | No test required (LTE 1CC fallback is tested in 7.8B.2.3) |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 5: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA NOTE 6: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (TS 38.521-1 [8] Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.1.1.4.3.

7.	For the E-UTRA CC(s) where anchor agnostic apply according to NOTE 2 in Table 7.8B.2.3_1.1.4.1-1, downlink signal level and uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B, clause B.0 of TS 36.521-1 [10]. Disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.8B.2.3_1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to table 7.8B.2.3_1.1.4.2-1 on E-UTRA CC and NR CC respectively. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the Downlink signal level to the value as defined in Table 7.8B.2.3_1.1.4.2-1. For E-UTRA CC and NR CC where uplink is allocated according to Table 7.8B.2.3_1.1.4.1-1, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to - (MU + Uplink power control window size) dB of the 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2B.4 for at least the duration of the Throughput measurement.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	For NR CC, Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

-	For E-UTRA CC, Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101 [5], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1 of TS 36.521-1 [10].

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than, or no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.

3.	Set the Interfering signal levels to the values as defined in Table 7.8B.2.3_1.1.4.2-1 and frequency below the wanted signal.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G, clause G.2.

5.	Repeat steps from 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Repeat steps 2 to 5 for component carriers listed in Table 7.8B.2.3_1.1.4.2-1.

Table 7.8B.2.3_1.1.4.2-1: Test repetition and measurement configuration

| DC configuration | Test parameters to select (NOTE 2) |
| --- | --- |
| DC_(n)XCA | 7.8B.2.1.5 |
| DC_XA_nYC | 7.8B.2.3.5 |
| DC_XA_nY(2A) |  |
| DC_XA_nYA-nZA |  |
| NOTE 1: Void NOTE 2: The reference for the placement of the interferer signals is the centre frequency of the carrier closest to the interferer among the carriers throughput is measured on. NOTE 3: Measure throughtput on DL allocated CC. Where there are multiple rows for a single Test point ID, the test is repeated for each row. |  |

7.8B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config.

7.8B.2.3_1.1.5	Test Requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A, clause A.3.2 with parameters specified in Table 7.8B.2.3_1.1.4.2-1 for the specified wanted signal mean power in the presence of two interfering signals.

## {End of changes}