3GPP TSG-RAN5 Meeting #91-e	R5-213997

# Electronic, 17th – 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1903 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of V2X test frequencies of band n47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | V2X operation is specified on band n47. The test frequencies are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding V2X test frequencies on band n47. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X testing on band n47 can’t be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-212923r2 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.7	Test frequencies for Non-3GPP Access

4.3.1.7.1	WLAN Test frequencies

The same WLAN test frequencies as in TS 36.508 [2] clause 4.3.1.6 applies.

4.3.1.8	Test frequencies for NR Sidelink operating bands

4.3.1.8.1	Test frequencies for NR Sidelink operating bands in FR1

4.3.1.8.1.1	Reference test frequencies for NR Sidelink operating band n38

FFS

4.3.1.8.1.2	Reference test frequencies for NR Sidelink operating band n47

Table 4.3.1.8.1.2-1: Test frequencies for NR operating band n47 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 5860.005 | 790667 | 5855.325 | 790355 | 0 | 790421 | 790673 | 790913 |
|  |  | Mid | 5889.99 | 792666 | 5794.59 | 786306 | 504 | 792420 | 792672 | 792912 |
|  |  | High | 5919.99 | 794666 | 5914.23 | 794282 | 6 | 794420 | 794672 | 794912 |
| 20 | 106 | Low | 5865 | 791000 | 5855.46 | 790364 | 0 | 790430 | 791006 | 791570 |
|  |  | Mid | 5889.99 | 792666 | 5789.73 | 785982 | 504 | 792096 | 792672 | 793236 |
|  |  | High | 5914.995 | 794333 | 5904.375 | 793625 | 6 | 793763 | 794339 | 794903 |
| 30 | 160 | Low | 5870.01 | 791334 | 5855.61 | 790374 | 0 | 790440 | 791340 | 792228 |
|  |  | Mid | 5889.99 | 792666 | 5784.87 | 785658 | 504 | 791772 | 792672 | 793560 |
|  |  | High | 5910 | 794000 | 5894.52 | 792968 | 6 | 793106 | 794006 | 794894 |
| 40 | 216 | Low | 5875.005 | 791667 | 5855.565 | 790371 | 0 | 790437 | 791673 | 792897 |
|  |  | Mid | 5889.99 | 792666 | 5779.83 | 785322 | 504 | 791436 | 792672 | 793896 |
|  |  | High | 5904.99 | 793666 | 5884.47 | 792298 | 6 | 792436 | 793672 | 794896 |
| sNOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-2: Test frequencies for NR operating band n47 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 24 | Low | 5860.005 | 790667 | 5855.685 | 790379 | 0 | 790511 | 790679 | 790823 |
|  |  | Mid | 5889.99 | 792666 | 5704.23 | 780282 | 504 | 792510 | 792678 | 792822 |
|  |  | High | 5919.99 | 794666 | 5913.51 | 794234 | 6 | 794510 | 794678 | 794822 |
| 20 | 51 | Low | 5865 | 791000 | 5855.82 | 790388 | 0 | 790520 | 791000 | 791480 |
|  |  | Mid | 5889.99 | 792666 | 5699.37 | 779958 | 504 | 792186 | 792666 | 793146 |
|  |  | High | 5914.995 | 794333 | 5903.655 | 793577 | 6 | 793853 | 794333 | 794813 |
| 30 | 78 | Low | 5870.01 | 791334 | 5855.97 | 790398 | 0 | 790530 | 791346 | 792138 |
|  |  | Mid | 5889.9 | 792666 | 5694.51 | 779634 | 504 | 791862 | 792678 | 793470 |
|  |  | High | 5910 | 794000 | 5893.8 | 792920 | 6 | 793196 | 794012 | 794804 |
| 40 | 106 | Low | 5875.005 | 791667 | 5855.925 | 790395 | 0 | 790527 | 791679 | 792807 |
|  |  | Mid | 5889.99 | 792666 | 5689.47 | 779298 | 504 | 791526 | 792678 | 793806 |
|  |  | High | 5904.99 | 793666 | 5883.75 | 792250 | 6 | 792526 | 793678 | 794806 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-3: Test frequencies for NR operating band n47 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 | Low | 5860.005 | 790667 | 5856.045 | 790403 | 0 | 790667 | 790667 | 790667 |
|  |  | Mid | 5889.99 | 792666 | 5523.15 | 768210 | 504 | 792666 | 792666 | 792666 |
|  |  | High | 5919.99 | 794666 | 5911.71 | 794114 | 6 | 794666 | 794666 | 794666 |
| 20 | 24 | Low | 5865 | 791000 | 5856.36 | 790424 | 0 | 790688 | 791024 | 791312 |
|  |  | Mid | 5889.99 | 792666 | 5518.47 | 767898 | 504 | 792354 | 792690 | 792978 |
|  |  | High | 5914.995 | 794333 | 5902.035 | 793469 | 6 | 794021 | 794357 | 794645 |
| 30 | 38 | Low | 5870.01 | 791334 | 5856.33 | 790422 | 0 | 790686 | 791358 | 791982 |
|  |  | Mid | 5889.99 | 792666 | 5513.43 | 767562 | 504 | 792018 | 792690 | 793314 |
|  |  | High | 5910 | 794000 | 5892 | 792800 | 6 | 793352 | 794024 | 794648 |
| 40 | 51 | Low | 5875.005 | 791667 | 5856.645 | 790443 | 0 | 790707 | 791667 | 792627 |
|  |  | Mid | 5889.99 | 792666 | 5508.75 | 767250 | 504 | 791706 | 792666 | 793626 |
|  |  | High | 5904.99 | 793666 | 5882.31 | 792154 | 6 | 792706 | 793666 | 794626 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

4.3.2	Radio conditions

4.3.2.1	FR1, normal propagation condition for connected

<Unchanged Text Skipped>

6.2.3.6	Test frequencies for NR DC configurations for signalling testing

The default channel bandwidths for NR DC signalling test are specified per NR band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For NR DC (2 bands, 2CC) the NR DC configurations are specified in clause 4.3.1.3.2.1 for 1CC FR1 and 1CC FR2. NR test frequencies are specified in clause 6.2.3.1 for the NR band used as PCell (NRf1, NRf2, NRf3, NRf4) and for the NR band used as PSCell (NRf5, NRf6, NRf7).

For NR DC with NR intra-band contiguous CA (2 bands, 3CC) the NR DC configurations are specified in clause 4.3.1.3.2.1 for 1CC FR1 and 2CC FR2. NR test frequencies are specified in clause 6.2.3.1 for the NR FR1 band used as PCell (NRf1) and in Table 6.2.3.4-2 for NR intra-band contiguous CA as PSCell (CC1, NRf5) and SCell (CC2, NRf6).

6.2.3.7	Test frequencies for NR sidelink configurations for signalling testing

The default channel bandwidths for NR sidelink signalling test are specified per NR sidelink operation band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR sidelink test frequencies are as follows:

-	for band with only one test frequency: Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

The test frequencies, subcarrier spacing, default channel bandwidth and SL-SS/PBCH block parameters for signalling is specified in Table 6.2.3.7-1.

Table 6.2.3.7-1: Test frequencies for NR Sidelink operating bands using FFS MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n47 | 15 | FFS | FFS | Low, High | Same values as for Low and High range in clause 4.3.1.8.1.2 for bandwidth=FFS MHz and SCS=15 kHz. For sl-absoluteFrequencySSB, same value as for S-SSB Low in clause 4.3.1.8.1.2 for bandwidth=FFS MHz and SCS=15 kHz is used. |  |  |  |  |  |

6.3	Reference system configurations

<End of Changes >