3GPP TSG-RAN5 Meeting # 91-e	R5-214039

# Electronic Meeting,  17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0533 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 TX test cases ETC applicability update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Extreme testing conditions is only affecting Min Peak EIRP measurements and should be extended to other Tx test cases according to R5-213210. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Initial conditions reviewed for following test cases: - 6.2.1.1 UE maximum output power - EIRP and TRP - 6.2.2 UE maximum output power reduction - 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) - 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) - 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) - 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) - 6.3.1 Minimum output power - 6.3.3.2 General ON/OFF time mask - 6.3.3.4 PRACH time mask - 6.3A.1.1 Minimum output power for CA (2UL CA) - 6.3A.1.2 Minimum output power for CA (3UL CA) - 6.3A.1.3 Minimum output power for CA (4UL CA) - 6.3A.1.4 Minimum output power for CA (5UL CA) - 6.3A.1.5 Minimum output power for CA (6UL CA) - 6.3A.1.6 Minimum output power for CA (7UL CA) - 6.3A.1.7 Minimum output power for CA (8UL CA) - 6.3A.3.1 Transmit ON/OFF time mask for CA (2UL CA) - 6.3D.1 Minimum output power for UL MIMO - 6.3D.3.1 General ON/OFF time mask for UL MIMO - 6.4.1 Frequency error - 6.4A.1.1 Frequency error for CA (2UL CA) - 6.4A.1.2 Frequency error for CA (3UL CA) - 6.4A.1.3 Frequency error for CA (4UL CA) - 6.5.2.3 Adjacent channel leakage ratio - 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO - Editor’s note reviewed for following test cases: - 6.2.1.1 UE maximum output power - EIRP and TRP - 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) - 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) - 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) - 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) - 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) - 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) - 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) - 6.3.1 Minimum output power - 6.4.1 Frequency error |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage under extreme environmental testing conditions will remain insufficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.2.4.1, 6.2A.1.1.1, 6.2A.1.1.2, 6.2A.1.1.3, 6.2A.1.1.4, 6.2A.1.1.5, 6.2A.1.1.6, 6.2A.1.1.7, 6.2A.2.1.4.1, 6.3.1, 6.3.3.2.4.1, 6.3.3.4.4.1, 6.3A.1.1.4.1, 6.3A.1.2.4, 6.3A.1.3.4, 6.3A.1.4.4, 6.3A.1.5.4, 6.3A.1.6.4, 6.3A.1.7.4, 6.3A.3.1.4.1, 6.3D.1.4.1, 6.3D.3.1.4.1, 6.4A.1.1.4.1, 6.4A.1.2.4, 6.4A.1.3.4, 6.5.2.3.4.1, 6.5D.2.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.903 CR 0234 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR approval will require endorsement of Proposals 2 & 3 in R5-213210 - Measurement uncertainties updates are covered in R5-213216 and R5-213217. - Test Procedures for EIRP beam peak Extreme Conditions are covered in R5-213251. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-213212 and the outcome of 1 revision |  |  |  |  |  |  |  |  |

<< Start of change 1>>

#### 6.2.1.1	UE maximum output power - EIRP and TRP

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- 	The test case is incomplete for band n259.

6.2.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.2.1.1.3	Minimum conformance requirements

###### 6.2.1.1.3.1	UE maximum output power for power class 1

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.1-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.1-1: UE minimum peak EIRP for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.1-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.1-2: UE maximum output power limits for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.1-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.1-3: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| NOTE 1: Minimum EIRP at 85%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

###### 6.2.1.1.3.2	UE maximum output power for power class 2

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.2-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.2-1: UE minimum peak EIRP for power class 2

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 29 |
| n258 | 29 |
| n261 | 29 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.2-2: UE maximum output power limits for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 60th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.2-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.2-3: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0 |
| n258 | 18.0 |
| n261 | 18.0 |
| NOTE 1: Minimum EIRP at 60%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

###### 6.2.1.1.3.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.2-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.1.3.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.1.3.3-1 and Table 6.2.1.1.3.3-4.

Table 6.2.1.1.3.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.1.3.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.3-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.3-3 below. The requirement is verified with the test metric of the total component of EIRP, as defined in [5] (Link=Spherical coverage grid, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.1.3.3-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.1.3.3-3 and Table 6.2.1.1.3.3-4.

Table 6.2.1.1.3.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.1.3.3-1 and 6.2.1.1.3.3-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter DMBP,n and EIRP spherical coverage relaxation parameter DMBS,n, as indicated in Table 6.2.1.1.3.3-4 to 6.2.1.1.3.3-5.

Table 6.2.1.1.3.3-4: UE multi-band relaxation factors for power class 3 (Rel-15)

| Supported bands | ∑MBP (dB) | ∑MBS (dB) |
| --- | --- | --- |
| n257, n258 | ≤ 1.3 | ≤ 1.25 |
| n257, n260 | ≤ 1.0 | ≤ 0.753 |
| n258, n260 | ≤ 1.0 | ≤ 0.753 |
| n258, n261 | ≤ 1.0 | ≤ 1.25 |
| n260, n261 | 0.0 | ≤ 0.752 |
| n257, n258, n260 | ≤ 1.7 | ≤ 1.753 |
| n257, n258, n261 | ≤ 1.7 | ≤ 1.75 |
| n257, n260, n261 | ≤ 0.5 | ≤ 1.253 |
| n258, n260, n261 | ≤ 1.5 | ≤ 1.253 |
| n257, n258, n260, n261 | ≤ 1.7 | ≤ 1.753 |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands NOTE 2: For supported bands n260 + n261, ΔMBS,n is not applied for band n260 NOTE 3: For n260, maximum applicable DMBS,n is 0.4 dB and DMBP,n is 0.75 dB NOTE 4: For all bands except n260, the maximum applicable DMBP,n and DMBS,n is 0.75 dB. |  |  |

Table 6.2.1.1.3.3-5: UE multi-band relaxation factors for power class 3 (Rel-16 and forward)

| Band | DMBP,n (dB) | DMBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

###### 6.2.1.1.3.4	UE maximum output power for power class 4

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.4-2. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.4-1: UE minimum peak EIRP for power class 4

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 34 |
| n258 | 34 |
| n260 | 31 |
| n261 | 34 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.4-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.4-2: UE maximum output power limits for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 20th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.4-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.4-3: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |
| NOTE 1: Minimum EIRP at 20%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.1.

6.2.1.1.4	Test description

6.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.2.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | N/A | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |
| 2 | 100 |  |  |  |  | and PC4 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Void |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.1.4.3

6.2.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 6.2.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.1.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.1.1.5-1 to 6.2.1.1.5-4. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi.

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2.1.1.5	Test requirement

The EIRP derived in step 4 and TRP derived in step 5 shall not exceed the values specified in Table 6.2.1.1.5-1 to Table 6.2.1.1.5-4.

Table 6.2.1.1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 35+TT | 55 | 40.0-TT |
| n258 | 35+TT | 55 | 40.0-TT |
| n260 | 35+TT | 55 | 38.0-TT |
| n261 | 35+TT | 55 | 40.0-TT |

Table 6.2.1.1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 29-TT |
| n258 | 23+TT | 43 | 29-TT |
| n260 |  |  |  |
| n261 | 23+TT | 43 | 29-TT |

Table 6.2.1.1.5-3: UE maximum output test requirements for power class 3 for single band UE or multiband UE declaring MBp = 0 in all FR2 bands

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 22.4-TT |
| n258 | 23+TT | 43 | 22.4-TT |
| n260 | 23+TT | 43 | 20.6-TT |
| n261 | 23+TT | 43 | 22.4-TT |

Table 6.2.1.1.5-3a: UE maximum output test requirements for power class 3 for multi band UE declaring MBp>0 in any FR2 band (Rel-15)

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 22.4-TT-MBp | 22.4-TT-MBp |  |  | 1.3 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 22.4-TT-MBp |  | 20.6-TT-MBp |  | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 3 | n258, n260 |  | 22.4-TT-MBp | 20.6-TT-MBp |  | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 4 | n258, n261 |  | 22.4-TT-MBp |  | 22.4-TT-MBp | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 |  |  |  |  | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 | 22.4-TT-MBp | 22.4-TT-MBp | 20.6-TT-MBp |  | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 7 | n257, n258, n261 | 22.4-TT-MBp | 22.4-TT-MBp |  | 22.4-TT-MBp | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 22.4-TT-MBp |  | 20.6-TT-MBp | 22.4-TT-MBp | 0.5 | Maximum 0.75 dB relaxation allowed for each band |
| 9 | n258, n260, n261 |  | 22.4-TT-MBp | 20.6-TT-MBp | 22.4-TT-MBp | 1.5 | Maximum 0.75 dB relaxation allowed for each band |
| 10 | n257, n258, n260, n261 | 22.4-TT-MBp | 22.4-TT-MBp | 20.6-TT-MBp | 22.4-TT-MBp | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| Note 1: MBp is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in Table 6.2.1.1.3.3-4. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant Note 3: Max allowed sum of MBp over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 6.2.1.1.3.3-4, Table 6.2.1.1.5-3d applies. |  |  |  |  |  |  |  |

Table 6.2.1.1.5-3b: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.65 dB | 2.77 dB |

Table 6.2.1.1.5-3c: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.87 dB | 2.87 dB |

Table 6.2.1.1.5-3d: UE maximum output test requirements for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 22.4-TT-DMBP,n |  |  |  |  |  |
| 2 | n258 |  | 22.4-TT-DMBP,n |  |  |  |  |
| 3 | n259 |  |  | 18.7-TT-DMBP,n |  |  |  |
| 4 | n260 |  |  |  | 20.6-TT-DMBP,n |  |  |
| 5 | n261 |  |  |  |  | 22.4-TT-DMBP,n |  |
| 6 | n257, n261 | 22.4-TT-DMBP,n |  |  |  | 22.4-TT-DMBP,n | DMBP,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 20.6-TT-DMBP,n | 22.4-TT-DMBP,n | DMBP,n relaxation is 0 dB |
| Note 1: DMBP,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.1.1.5-4: UE maximum output power test requirements for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 34-TT |
| n258 | 23+TT | 43 | 34-TT |
| n260 | 23+TT | 43 | 31-TT |
| n261 | 23+TT | 43 | 34-TT |

<< End of change 1>>

<< Start of change 2>>

<< End of change 2>>

<< Start of change 2>>

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 2 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 3 | Low |  |  |  | CP-OFDM 64 QAM | 2@0 |
| 4 | High |  |  |  | CP-OFDM 64 QAM | 2@NRB-2 |
| 5 | Low |  |  |  | CP-OFDM 64 QAM | 7@0 |
| 6 | High |  |  |  | CP-OFDM 64 QAM | 7@NRB-7 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-2: Test Configuration Table (Power Class 1, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range,High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 5 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 6 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 7 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 8 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 9 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 10 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 11 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 12 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 13 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 14 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full_Region2 |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table (Power Class 1, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region2 |
| 5 | Mid |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region2 |
| 6 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 7 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 8 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full_Region2 |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 15 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full_Region2 |
| 17 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 18 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 19 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 20 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 21 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 22 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 23 | Mid |  |  |  | CP-OFDM 16 QAM | Inner_Full_Region2 |
| 24 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 25 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 26 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in clause 6.1-2. |  |  |  |  |  |  |

Table 6.2.2.4.1-4: Void

Table 6.2.2.4.1-5: Void

Table 6.2.2.4.1-6: Void

Table 6.2.2.4.1-7: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highes supported channel bandwidth that ≤ 200 MHz t |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-8: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2.4.1-9: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2.2.4.1-1 to Table 6.2.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

<< End of change 2>>

<< Start of change 3>>

##### 6.2A.1.1.1	UE maximum output power - EIRP and TRP for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.2A.1.1.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.2A.1.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.2A.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause6.2A.1.0.

6.2A.1.1.1.4	Test description

6.2A.1.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.2A.1.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.1.1.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated BW of the CA configuration (≤ 400 MHz aggregated channel bandwidth) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping ( NOTE 4） | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 100 | N/A for this test |  | DFT-s-OFDM QPSK | Inner Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
|  | SCC/CC2 | 100 |  |  | - | - |
| 2 | PCC/CC1 | 200 |  |  | DFT-s-OFDM QPSK | Inner Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
|  | SCC/CC2 | 200 |  |  | - | - |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Void NOTE 3: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 4: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2A.1.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.1.1.1.4.3

6.2A.1.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1. Message contents are defined in clause 6.2A.1.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [x], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2A.1.1.1.4.3.

5.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

6.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (Note 1) for the UE Tx beam selection to complete.

7.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

8.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2A.1.1.1.5-1. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

9.	Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.2A.1.1.1.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi.

10.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2A.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.1.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.1.5-1 to Table 6.2A.1.1.1.5-4.

Table 6.2A.1.1.1.5-1: Intra-band Contiguous CA UE maximum output test requirements for power class 1

| UL CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257D | 35+TT | 55 | 40-TT |
| CA_n257G | 35+TT | 55 | 40-TT |
| CA_n260D | 35+TT | 55 | 38-TT |
| CA_n260G | 35+TT | 55 | 38-TT |
| CA_n260O | 35+TT | 55 | 38-TT |
| CA_n261D | 35+TT | 55 | 40-TT |
| CA_n261G | 35+TT | 55 | 40-TT |
| CA_n261O | 35+TT | 55 | 40-TT |

Table 6.2A.1.1.1.5-2: Intra-band Contiguous CA UE maximum output test requirements for power class 2

| UL CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257D | 23+TT | 43 | 29-TT |
| CA_n257G | 23+TT | 43 | 29-TT |
| CA_n261D | 23+TT | 43 | 29-TT |
| CA_n261G | 23+TT | 43 | 29-TT |
| CA_n261O | 23+TT | 43 | 29-TT |

Table 6.2A.1.1.1.5-3: Intra-band Contiguous CA UE maximum output test requirements for power class 3

| UL CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257D | 23+TT | 43 | 22.4-TT |
| CA_n257G | 23+TT | 43 | 22.4-TT |
| CA_n260D | 23+TT | 43 | 20.6-TT |
| CA_n260G | 23+TT | 43 | 20.6-TT |
| CA_n260O | 23+TT | 43 | 20.6-TT |
| CA_n261D | 23+TT | 43 | 22.4-TT |
| CA_n261G | 23+TT | 43 | 22.4-TT |
| CA_n261O | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.1.5-3a: UE maximum output test requirements for power class 3 for multi band UE declaring MBp>0 in any FR2 band

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CA_n257D/G | CA_n258 | CA_n260D/G/O | CA_n261D/G/O |  |  |
| 1 | n257, n258 | 22.4-TT-MBp |  |  |  | 1.3 |  |
| 2 | n257, n260 | 22.4-TT-MBp |  | 20.6-TT-MBp |  | 1.0 |  |
| 3 | n258, n260 |  |  | 20.6-TT-MBp |  | 1.0 |  |
| 4 | n258, n261 |  |  |  | 22.4-TT-MBp | 1.0 |  |
| 5 | n260, n261 |  |  |  |  | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 | 22.4-TT-MBp |  | 20.6-TT-MBp |  | 1.7 |  |
| 7 | n257, n258, n261 | 22.4-TT-MBp |  |  | 22.4-TT-MBp | 1.7 |  |
| 8 | n257, n260, n261 | 22.4-TT-MBp |  | 20.6-TT-MBp | 22.4-TT-MBp | 0.5 |  |
| 9 | n258, n260, n261 |  |  | 20.6-TT-MBp | 22.4-TT-MBp | 1.5 |  |
| 10 | n257, n258, n260, n261 | 22.4-TT-MBp |  | 20.6-TT-MBp | 22.4-TT-MBp | 1.7 |  |
| Note 1: MBp is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant Note 3: Max allowed sum of MBp over all supported FR2 bands as defined in clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |

Table 6.2A.1.1.1.5-3b: Test Tolerance (Max TRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.65 dB | 2.77 dB |

Table 6.2A.1.1.1.5-3c: Test Tolerance (Min peak EIRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30cm | 2.87 dB | 2.87 dB |

Table 6.2A.1.1.1.5-4: Intra-band Contiguous CA UE maximum output test requirements for power class 4

| UL CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257D | 23+TT | 43 | 34-TT |
| CA_n257G | 23+TT | 43 | 34-TT |
| CA_n260B | 23+TT | 43 | 31-TT |
| CA_n260D | 23+TT | 43 | 31-TT |
| CA_n260G | 23+TT | 43 | 31-TT |
| CA_n260O | 23+TT | 43 | 31-TT |
| CA_n261B | 23+TT | 43 | 34-TT |
| CA_n261D | 23+TT | 43 | 34-TT |
| CA_n261G | 23+TT | 43 | 34-TT |
| CA_n261O | 23+TT | 43 | 34-TT |

<< End of change 3>>

<< Start of change 4>>

##### 6.2A.1.1.2	UE maximum output power - EIRP and TRP for CA (3UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.2A.1.1.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.2A.1.1.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA.

6.2A.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.2.4	Test description

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.2.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.2.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.2.5-1.

Table 6.2A.1.1.2.4.1-1: Test Configuration Tabl

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated BW (≤ 400 MHz aggregated channel bandwidth) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 100 | N/A for this test |  | DFT-s-OFDM QPSK | Inner Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
|  | SCC/CC2 | 100 |  |  | - | - |
|  | SCC/CC3 | 100 |  |  | - | - |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Void. NOTE 3: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

6.2A.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.2.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.2.5-1.

Table 6.2A.1.1.2.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257H | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.2.5-1a: Test Tolerance (Max TRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.65 dB | 2.77 dB |

Table 6.2A.1.1.2.5-1b: Test Tolerance (Min peak EIRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.87 dB | 2.87 dB |

<< End of change 4>>

<< Start of change 5>>

##### 6.2A.1.1.3	UE maximum output power - EIRP and TRP for CA (4UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.2A.1.1.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.2A.1.1.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.2A.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.3.4	Test description

6.2A.1.1.3.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.3.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.3.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.3.5-1.

Table 6.2A.1.1.3.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes, and PCC and SCC are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (cumulative aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated BW (≤ 400 MHz aggregated channel bandwidth) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | ChBw | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 100 | N/A for this test |  | DFT-s-OFDM QPSK | Inner Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
|  | SCC/CC2 | 100 |  |  | - | - |
|  | SCC/CC3 | 100 |  |  | - | - |
|  | SCC/CC4 | 100 |  |  | - | - |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Void. NOTE 3: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

6.2A.1.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.3.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.3.5-1.

Table 6.2A.1.1.3.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257I | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.3.5-1a: Test Tolerance (Max TRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.65 dB | 2.77 dB |

Table 6.2A.1.1.3.5-1b: Test Tolerance (Min peak EIRP for Power class 3) (Aggregated BW ≤ 400MHz)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | 2.87 dB | 2.87 dB |

<< End of change 5>>

<< Start of change 6>>

##### 6.2A.1.1.4	UE maximum output power - EIRP and TRP for CA (5UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 3 and 4.

- Test Tolerances in the Test Requirement are FFS.

6.2A.1.1.4.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 5UL CA.

NOTE: This test case can’t be performed due to lack of appropriate test points.

6.2A.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.4.4	Test description

6.2A.1.1.4.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.4.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.4.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.4.5-1.

Table 6.2A.1.1.4.4.1-1: Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

6.2A.1.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.4.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.4.5-1.

Table 6.2A.1.1.4.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257J | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.4.5-1a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

Table 6.2A.1.1.4.5-1b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

<< End of change 6>>

<< Start of change 7>>

##### 6.2A.1.1.5	UE maximum output power - EIRP and TRP for CA (6UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 3 and 4.

- Test Tolerances in the Test Requirement are FFS.

6.2A.1.1.5.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

NOTE:	This test case can’t be performed due to lack of appropriate test points.

6.2A.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.5.4	Test description

6.2A.1.1.5.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.5.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.5.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.5.5-1.

Table 6.2A.1.1.5.4.1-1: Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

6.2A.1.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.5.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.5.5-1.

Table 6.2A.1.1.5.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257K | 23+TT | 43 | 22.4-TT |
| CA_n260K | 23+TT | 43 | 20.6-TT |
| CA_n261K | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.5.5-1a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

Table 6.2A.1.1.5.5-1b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

<< End of change 7>>

<< Start of change 8>>

##### 6.2A.1.1.6	UE maximum output power - EIRP and TRP for CA (7UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 3 and 4.

- Test Tolerances in the Test Requirement are FFS.

6.2A.1.1.6.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7UL CA.

NOTE:	This test case can’t be performed due to lack of appropriate test points.

6.2A.1.1.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.6.4	Test description

6.2A.1.1.6.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.6.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.6.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.6.5-1.

Table 6.2A.1.1.6.4.1-1: Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

6.2A.1.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.6.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.6.5-1.

Table 6.2A.1.1.6.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257L | 23+TT | 43 | 22.4-TT |
| CA_n260L | 23+TT | 43 | 20.6-TT |
| CA_n261L | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.6.5-1a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

Table 6.2A.1.1.6.5-1b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

<< End of change 8>>

<< Start of change 9>>

##### 6.2A.1.1.7	UE maximum output power - EIRP and TRP for CA (8UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, 3 and 4.

- Test Tolerances in the Test Requirement are FFS.

6.2A.1.1.7.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.1.7.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 8UL CA.

NOTE:	This test case can’t be performed due to lack of appropriate test points.

6.2A.1.1.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2A.1.1.0.

6.2A.1.1.7.4	Test description

6.2A.1.1.7.4.1	Initial condition

Same as in clause 6.2A.1.1.1.4 with following exceptions:

-	Instead of Table 6.2A.1.1.1.4.1-1 use Table 6.2A.1.1.7.4.1-1.

-	Instead of clause 6.2A.1.1.1.4.3 use clause 6.2A.1.1.7.4.3.

-	Instead of Table 6.2A.1.1.1.5-1 use Table 6.2A.1.1.7.5-1.

Table 6.2A.1.1.7.4.1-1: Test Configuration Table

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

6.2A.1.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2A.1.1.7.5	Test Requirements

The EIRP derived in step 8 and TRP derived in step 9 shall not exceed the values specified in Table 6.2A.1.1.7.5-1.

Table 6.2A.1.1.7.5-1: UE maximum output test requirements for power class 3

| CA configuration | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| CA_n257M | 23+TT | 43 | 22.4-TT |
| CA_n260M | 23+TT | 43 | 20.6-TT |
| CA_n261M | 23+TT | 43 | 22.4-TT |

Table 6.2A.1.1.7.5-1a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

Table 6.2A.1.1.7.5-1b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Quiet Zone size ≤ 30 cm | TBD | TBD |

<< End of change 9>>

<< Start of change 10>>

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRnarrow)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Refer to “Test frequency” column |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation (NOTE 1) |
| Default Test Settings for a CA_nXB, CA_nXD, CA_nXG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | N/A for this | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  | Low | test | - | - |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC/CC2 |  | High |  | - | - |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC/CC2 |  | Low |  | - | - |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC/CC2 |  | High |  | - | - |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | N/A for this | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low | test | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | N/A for this | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low | test | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Low | N/A for this | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC1/CC2 |  | Low | test | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 2 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | Outer_1RB_Right |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| 3 | PCC/CC1 |  | Low |  | CP-OFDM 64QAM | 7@0 |
|  | SCC1/CC2 |  | Low |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | Low |  | N/A | N/A |
|  | SCC3/CC4 |  | Low |  | N/A | N/A |
|  | SCC4/CC5 |  | Low |  | N/A | N/A |
|  | SCC5/CC6 |  | Low |  | N/A | N/A |
| 4 | PCC/CC1 |  | High |  | CP-OFDM 64QAM | 7@NRB-7 |
|  | SCC1/CC2 |  | High |  | - | - |
|  | Wgap |  | Max Wgap |  | N/A | N/A |
|  | SCC2/CC3 |  | High |  | N/A | N/A |
|  | SCC3/CC4 |  | High |  | N/A | N/A |
|  | SCC4/CC5 |  | High |  | N/A | N/A |
|  | SCC5/CC6 |  | High |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-2: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, single CC MPR requirement)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Lowest range, Highest range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  | test | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM PI/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  | test | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Inner_Full_Region1 |
|  | SCC/CC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, MPRC_CA)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: Lowest range with Max Wgap, Highest range with Max Wgap. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC& Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 400MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 400MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC/CC2 | 200MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 |  |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 4 | PCC/CC1 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| 5 | PCC/CC1 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 |  |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
|  | SCC4/CC5 |  |  |  | N/A | N/A |
|  | SCC5/CC6 |  |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 200MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(O-E)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 200MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 200MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 90MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 100MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 100MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 190MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 100MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 100MHz |  |  | N/A | N/A |
|  | SCC4/CC5 | 100MHz |  |  | N/A | N/A |
|  | SCC5/CC6 | 100MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXD Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | N/A for this | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC1/CC2 | 200MHz |  | test | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 2 | PCC/CC1 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| 3 | PCC/CC1 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | N/A | N/A |
|  | SCC3/CC4 | 50MHz |  |  | N/A | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default | N/A for this | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  | test | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 2 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 16QAM | Outer_Full |
| 3 | PCC/CC1 | 50MHz |  |  | N/A | N/A |
|  | SCC1/CC2 | 200MHz |  |  | N/A | N/A |
|  | Wgap | 40MHz |  |  | N/A | N/A |
|  | SCC2/CC3 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  | CP-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-4: Intra-band Contiguous UL CA Test Configuration Table (Power Class 1, Non-contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: Lowest range with Max Wgap, Highest range with Max Wgap. |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXB, CA_nXD, CA_XG, CA_nXO Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | CP-OFDM 64QAM | Outer_1RB_Left |
|  | SCC/CC2 |  |  | test | CP-OFDM 64QAM | Outer_1RB_Right |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
| Default Test Settings for a CA_nX(D-G), CA_nX(D-O) Configuration |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM QPSK | Outer_1RB_Left |
|  | SCC1/CC2 |  |  | test | DFT-s-OFDM QPSK | Outer_1RB_Right |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC1/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
|  | Wgap |  |  |  | N/A | N/A |
|  | SCC2/CC3 |  |  |  | N/A | N/A |
|  | SCC3/CC4 |  |  |  | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-5: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, single CC MPR requirement)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Lowest range, Highest range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel <= 200MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC/CC2 |  |  | test | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel <= 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM QPSK | Inner_Full |
|  | SCC/CC2 |  |  | test | - | - |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC/CC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-6: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, MPRC_CA)

| Default Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  |  |  | Normal, TL, TH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  |  |  |  | For intra-band contiguous CA: Mid range. For intra-band non-contiguous CA: Lowest range with Max Wgap, Highest range with Max Wgap. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 120 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency |  |  |  | DL RB allocation |  |  |  |  |  | UL Modulation |  |  |  |  | UL RB allocation |  |  |  |
| Default Test Settings for a CA_nXB, nXC_UL_nXB Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | N/A for this |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  | test |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | N/A for this |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  | test |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXB Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz | Default |  |  |  | N/A for this |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  | test |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 400MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | N/A for this |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  | test |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nXD Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  | N/A for this |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  | test |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
| Default Test Settings for a CA_nX(D-G) )_UL_nXD, CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXD, CA_nX(D-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default |  |  |  | N/A for this |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  | test |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, CA_nX(E-O)_UL_nXO Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default |  |  |  | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  | Outer_Full |  |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  | Outer_Full |  |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  | N/A |  |  |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD, CA_nX(G-I)_UL_nXG Configuration (800MHz <= Cumulative aggregated BWchannel <= 1400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default |  |  | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 2 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DFT-s-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 3 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 4 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 5 | PCC/CC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | Wgap |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC4/CC5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC5/CC6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-G)_UL_nXD, CA_nX(D-O)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz |  |  | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-G)_UL_nXG, CA_nX(D-O)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel <800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz |  | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | c |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-H)_UL_nXD, CA_nX(D-P)_UL_nXD, Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 200MHz |  | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(O-E)_UL_nXO Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-I)_UL_nXD, CA_nX(D-Q)_UL_nXD Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 90MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(G-I)_UL_nXG Configuration (400MHz <= Cumulative aggregated BWchannel < 800MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 100MHz | Default |  |  |  |  |  | N/A for this |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  | test |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 2 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 3 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| 4 | PCC/CC1 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | SCC1/CC2 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  | Outer_Full |  |
|  | Wgap | 190MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC2/CC3 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC3/CC4 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC4/CC5 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
|  | SCC5/CC6 | 100MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  |
| Default Test Settings for a CA_nX(D-O)_UL_nXD Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default |  |  |  |  | N/A for this |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  | test |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| 2 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| 3 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
| Default Test Settings for a CA_nX(D-O)_UL_nXO Configuration (Cumulative aggregated BWchannel <400MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC/CC1 | 50MHz | Default |  |  |  |  | N/A for this |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  | test |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | Outer_Full |
| 2 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 16QAM |  |  |  |  |  |  | Outer_Full |
| 3 | PCC/CC1 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC1/CC2 | 200MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | Wgap | 40MHz |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | N/A |
|  | SCC2/CC3 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
|  | SCC3/CC4 | 50MHz |  |  |  |  |  |  |  |  |  |  |  | CP-OFDM 64QAM |  |  |  |  |  |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-7: Intra-band Contiguous UL CA Test Configuration Table (Power Class 2, 3 and 4, Non-contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Highest aggregated channel bandwidth of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC & Mapping (NOTE 2) | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_XG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |
| 1 | PCC/CC1 | Default | Default | N/A for this | DFT-s-OFDM QPSK | Outer_1RB_Left |
|  | SCC/CC2 |  |  | test | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 2 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 BPSK | [Outer_0.9_Right] |
| 3 | PCC/CC1 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Left] |
|  | SCC/CC2 |  |  |  | DFT-s-OFDM Pi/2 QPSK | [Outer_0.9_Right] |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2A.2.1.4.1-1 to Table 6.2A.2.1.4.1-7.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

<< End of change 10>>

<< Start of change 11>>

### 6.3.1	Minimum output power

Editor’s Note: The following aspects of the clause are for future consideration:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

6.3.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.1.3	Minimum conformance requirements

The minimum controlled output power of the UE is defined as the EIRP in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one subframe (1ms).

6.3.1.3.1	Minimum output power for power class 1

For power class 1 UE, the minimum output power shall not exceed the values specified in Table 6.3.1.3.1-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.3.1-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4 | 47.58 |
|  | 100 | 4 | 95.16 |
|  | 200 | 4 | 190.20 |
|  | 400 | 4 | 380.28 |

6.3.1.3.2	Minimum output power for power class 2, 3, and 4

The minimum output power shall not exceed the values specified in Table 6.3.1.3.2-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.3.2-1: Minimum output power for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.16 |
|  | 200 | -13 | 190.20 |
|  | 400 | -13 | 380.28 |
| NOTE 1: n260 is not applied for power class 2. |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.1.

6.3.1.4	Test description

6.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for minimum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.1.4.3.

6.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "down" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 for the specific channel bandwidth under test. EIRP test procedure is defined in Annex K.1.3. The measuring duration is at least one active subrame (1ms). EIRP is calculated considering both polarizations, theta and phi. For TDD, only slots consisting of only UL symbols are under test.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.3.1.5	Test requirement

The maximum EIRP, derived in step 5 shall not exceed the values specified in Table 6.3.1.5-1 and Table 6.3.1.5-2.

Table 6.3.1.5-1: Minimum output power for power class 1

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n260, n261 | 50 | 4+TT | 47.58 |
|  | 100 | 4+TT | 95.16 |
|  | 200 | 4+TT | 190.20 |
|  | 400 | 4+TT | 380.28 |

Table 6.3.1.5-2: Minimum output power for power class 2, 3, and 4

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n261 | 50 | -13+TT | 47.58 |
|  | 100 | -13+2.4+TT | 95.16 |
|  | 200 | -13+5.4+TT | 190.20 |
|  | 400 | -13+8.4+TT | 380.28 |
| n260 | 50 | -13+4.5+TT | 47.58 |
|  | 100 | -13+7.5+TT | 95.16 |
|  | 200 | -13+10.5+TT | 190.20 |
|  | 400 | -13+13.5+TT | 380.28 |
| NOTE 1: Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). |  |  |  |

Table 6.3.1.5-3: Minimum output power Test tolerance for power class 1, 2, 4

FFS

Table 6.3.1.5-4: Minimum output power Test tolerance for power class 3

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | 3.35 dB | 3.35 dB |

<< End of change 11>>

<< Start of change 12>>

6.3.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for maximum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channels are set according to Table 6.3.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.3.2.4.3.

<< End of change 12>>

<< Start of change 13>>

6.3.3.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.4.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 | Normal, TL, TH |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 | Mid range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 | Lowest, Mid, Highest |
| Test SCS as specified in Table 5.3.5-1 | SCS defined in TS 38.211 [8] subclause 6.3.3.2 |
| PRACH preamble format |  |
| PRACH Configuration Index | [0] |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0 , and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.3.4.4.3.

<< End of change 13>>

<< Start of change 14>>

6.3A.1.1.4	Test description

6.3A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A.1-1. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.1.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3A.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3A.1.1.4.3.

<< End of change 14>>

<< Start of change 15>>

6.3A.1.2.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.2.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.2.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.2.5-1 and 6.3A.1.2.5-2.

Table 6.3A.1.2.4.1-1: Test Configuration Table for 3UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 15>>

<< Start of change 16>>

6.3A.1.3.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.3.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.3.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.3.5-1 and 6.3A.1.3.5-2.

Table 6.3A.1.3.4.1-1: Test Configuration Table for 3UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 16>>

<< Start of change 17>>

6.3A.1.4.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.4.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.4.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.4.5-1 and 6.3A.1.4.5-2.

Table 6.3A.1.4.4.1-1: Test Configuration Table for 5UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC4 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 17>>

<< Start of change 18>>

6.3A.1.5.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.5.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.5.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.5.5-1 and 6.3A.1.5.5-2.

Table 6.3A.1.5.4.1-1: Test Configuration Table for 6UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC4 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC5 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 18>>

<< Start of change 19>>

6.3A.1.6.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.6.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.6.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.6.5-1 and 6.3A.1.6.5-2.

Table 6.3A.1.6.4.1-1: Test Configuration Table for 7UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC4 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC5 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC6 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 19>>

<< Start of change 20>>

6.3A.1.7.4	Test description

Same as in clause 6.3A.1.1.4 with following exceptions:

-	Instead of Table 6.3A.1.1.4.1-1  use Table 6.3A.1.7.4.1-1.

-	Instead of clause 6.3A.1.1.4.3  use clause 6.3A.1.7.4.3.

-	Instead of Table 6.3A.1.1.5-1 and 6.3A.1.1.5-2  use Table 6.3A.1.7.5-1 and 6.3A.1.7.5-2.

Table 6.3A.1.7.4.1-1: Test Configuration Table for 8UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | CC | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| 1 | PCC | Default | Default | N/A | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC1 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC2 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC3 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC4 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC5 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC6 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCC7 |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

<< End of change 20>>

<< Start of change 21>>

6.3A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.1.4.1-1: Intra-band Contiguous UL CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes |  |  |  |  | FFS |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | FFS |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | FFS |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | CC | Band | ChBw(MHz) | Test frequency | DL RB allocation | UL Modulation | UL RB allocation |
| Default Test Settings for a CA_XG, CA_nXO Configuration (Cumulative aggregated BWchannel < 400MHz) |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.3A.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

<< End of change 21>>

<< Start of change 22>>

6.3D.1.4	Test description

6.3D.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3D.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for minimum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3D.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3D.1.4.3.

<< End of change 22>>

<< Start of change 23>>

6.3D.3.1.4	Test description

6.3D.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for maximum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channels are set according to Table 6.3D.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3D.3.1.4.3.

<< End of change 23>>

<< Start of change 24>>

### 6.4.1	Frequency error

6.4.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.4.1.3	Minimum conformance requirements

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of 1 msec of cumulated measurement intervals compared to the carrier frequency received from the NR gNB.

The frequency error is defined as a directional requirement. The requirement is verified in beam locked mode with the test metric of Frequency (Link=TX beam peak direction, Meas=Link angle).

The normative reference for this requirement is TS 38.101-2 [3] clause 6.4.1

6.4.1.4	Test description

6.4.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

1. 	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and section A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The DL and UL Reference Measurement channels are set according to Table 6.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4.1.4.3

6.4.1.4.2	Test procedure

1.	Retrieve the LO position from the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE.

2.	SS transmits PDSCH via PDCCH DCI format 1_0 for C_RNTI to transmit the DL RMC according to Table 6.4.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Set the UE in the Inband Tx beam peak direction and apply the associated polarization for the DL, both found with a 3D EIRP scan as performed in Annex K.1.1. Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolLink to form the TX beam towards the TX beam peak direction and respective polarization. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test. Allow at least 200ms starting from the first TPC Command for the UE to reach PUMAX level. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

6.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition TxRx.

7.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E) for the θ- and φ-polarization of the UL. For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with DFT-s-OFDM condition in Table 4.6.3-118 PUSCH-Config and with the exceptions in subclause 7.3.2.4.3 and Table 7.3.2.5-3.

6.4.1.5	Test requirement

The 10 frequency error Δf results for the θ-polarization or the n frequency error Δf results for the φ-polarization must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 0.005 PPM)

<< End of change 24>>

<< Start of change 25>>

6.4A.1.1.4	Test description

6.4A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.4A.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.1.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | CBW (MHz) | Modulation | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC2 |  | - | - |  | - | - |
| 2 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4A.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4A.1.1.4.3

<< End of change 25>>

<< Start of change 26>>

6.4A.1.2.4	Test description

Same as in clause 6.4A.1.1.4 with following exceptions:

-	Instead of Table 6.4A.1.1.4.1-1 use Table 6.4A.1.2.4.1-1.

-	Instead of clause 6.4A.1.1.4.3 use clause 6.4A.1.2.4.3.

-	Instead of Table 6.4A.1.1.5-1 use Table 6.4A.1.2.5-1.

Table 6.4A.1.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | CBW (MHz) | Modulation | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | default | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
| 2 | PCC/CC1 |  | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC3 |  | - | - |  | - | - |
| 3 | PCC/CC1 |  | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |  |

<< End of change 26>>

<< Start of change 27>>

6.4A.1.3.4	Test description

Same as in clause 6.4A.1.1.4 with following exceptions:

-	Instead of Table 6.4A.1.1.4.1-1 use Table 6.4A.1.3.4.1-1.

-	Instead of clause 6.4A.1.1.4.3 use clause 6.4A.1.3.4.3.

-	Instead of Table 6.4A.1.1.5-1 use Table 6.4A.1.3.5-1.

Table 6.4A.1.3.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes |  |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | CBW (MHz) | Modulation | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
| 2 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
| 3 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC4 |  | - | - |  | - | - |
| 4 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |  |

<< End of change 27>>

<< Start of change 28>>

6.5.2.3.4	Test description

6.5.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.5.2.3.4.1-1 and table 6.5.2.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3.4.1-1: Test Configuration Table (Power Class 1)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@0 |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | 8@0 |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | 8@NRB-8 |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | 8@0 |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | 8@NRB-8 |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | 8@0 |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | 8@NRB-8 |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 14 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 10-13 for FR2b 200MHz Channel Bandwidth - Test ID 10 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-1, table 6.2.2.4.1-2, table 6.2.2.4.1-3 (MPR). |  |  |  |  |  |  |

Table 6.5.2.3.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 10-13 for FR2b 200MHz Channel Bandwidth - Test ID 10 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-7, table 6.2.2.4.1-8, table 6.2.2.4.1-9 (MPR). |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and section A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.3.4.1-1 and Table 6.5.2.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.2.3.4.3

<< End of change 28>>

<< Start of change 29>>

6.5D.2.2.4	Test description

6.5D.2.2.4.1	Initial condition

Same initial condition in clause 6.5.2.3.4.1 with following exceptions:

-	Instead of Table 6.5.2.3.4.1-1 use Table 6.5D.2.2.4.1-1.

-	Instead of Table 6.5.2.3.4.1-2  use Table 6.5D.2.2.4.1-2.

Table 6.5D.2.2.4.1-1: Test Configuration Table (Power Class 1)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 2 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 5 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 8 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | 8@0 |
| 11 | High |  |  |  | CP-OFDM 256 QAM | 8@NRB-8 |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 10-13 for FR2b 200MHz Channel Bandwidth - Test ID 10 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-1, table 6.2.2.4.1-2, table 6.2.2.4.1-3 (MPR). |  |  |  |  |  |  |

Table 6.5D.2.2.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 9-12 for FR2b 200MHz Channel Bandwidth - Test ID 9 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-7, table 6.2.2.4.1-8, table 6.2.2.4.1-9 (MPR). |  |  |  |  |  |  |

<< End of change 29>>