3GPP TSG-RAN5 Meeting #91-e	R5-214052

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0993 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of for further study notes about FR2 ETC testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT, Keysight Technologies, Intertek, Huawei, Bureau Veritas |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - ETC testing for 6.2B.1.4 and 6.4B.1.4 has been determined during RAN5#90e meeting. - ETC testing for 6.2B.1.4_1.2.1, 6.2B.1.4_1.3.1 and 7.3B.2.4_1 is still FFS. - 6.3B.1.4 does not need ETC testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - FFS notes about ETC testing for 6.2B.1.4, 6.3B.1.4 and 6.4B.1.4 were removed. - FFS notes about ETC testing for 6.2B.1.4_1.2.1, 6.2B.1.4_1.3.1 and 7.3B.2.4_1 were added. Other Editor’s notes were also added to 6.3B.1.4 and 7.3B.2.4_1 as per 38.521-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.4, 6.2B.1.4_1.1, 6.2B.1.4_1.1.2, 6.2B.1.4_1.2, 6.2B.1.4_1.3.1, 6.2B.1.4_1.3.2, 6.3B.1.4, 6.4B.1.4, 7.3B.2.4_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Merge the revisions in R5-213080 and add Intertek, Huawei, Bureau Veritas as co-source. - Undo the revision for ETC in 6.3B.1.4. - Add Keysight Technologies as co-source. r2: Correct company name for Bureau Veritas. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.1.4	UE Maximum Output Power for Inter-Band EN-DC including FR2

6.2B.1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 - EIRP and TR

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2.1.1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

# 6.2B.1.4.1.1	Test purpose

Same test purpose as in clause 6.2.1.1.1 in TS 38.521-2 [9] for the NR carrier.

<< Unchanged sections omitted >>

6.2B.1.4_1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (>2 CCs)

6.2B.1.4_1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 CCs)

6.2B.1.4_1.1.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.1 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.1 in TS 38.521-2 [9] is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

6.2B.1.4_1.1.1.1	Test purpose

Same test purpose as in clause 6.2.1.1.1 in TS 38.521-2 [9] for the NR carrier.

<< Unchanged sections omitted >>

6.2B.1.4_1.1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (3 CCs) - Spherical Coverage

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.2.1 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.2.1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.1.2.1	Test purpose

Same test purpose as in clause 6.2.1.2.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.1.4_1.1.2.2	Test applicability

<< Unchanged sections omitted >>

6.2B.1.4_1.1.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.1.5 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 CCs)

6.2B.1.4_1.2.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.2 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.2 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

# 6.2B.1.4_1.2.1.1	Test purpose

Same test purpose as in clause 6.2.1.1.1 in TS 38.521-2 [9] for the NR carriers.

<< Unchanged sections omitted >>

6.2B.1.4_1.2.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (4 CCs) - Spherical Coverage

Editor’s note: The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.2.2 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.2.2 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.2.2.1	Test purpose

Same test purpose as in clause 6.2.1.2.1 in TS 38.521-2 [9] for the NR carriers.

<< Unchanged sections omitted >>

6.2B.1.4_1.2.2.5	Test Requirements

Same test requirement as in clause 6.2A.1.2.2.5 in TS 38.521-2 [9] for the NR carriers.

6.2B.1.4_1.3	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 CCs)

6.2B.1.4_1.3.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 CCs) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.1.3 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.1.3 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

# 6.2B.1.4_1.3.1.1	Test purpose

Same test purpose as in clause 6.2.1.1.1 in TS 38.521-2 [9] for the NR carriers.

<< Unchanged sections omitted >>

6.2B.1.4_1.3.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (5 CCs) – Spherical Coverage

Editor’s note: The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.1.2.3 in TS 38.521-2 is incomplete for aggregated BW > 400MHz.

-	The referred test case 6.2A.1.2.3 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.2B.1.4_1.3.2.1	Test purpose

Same test purpose as in clause 6.2.1.2.1 in TS 38.521-2 [9] for the NR carriers.

<< Unchanged sections omitted >>

6.3B.1.4	Minimum Output Power for EN-DC Interband including FR2

Editor's note:	The following aspects of the clause are for future consideration:

-	Testing of extreme conditions for FR2 is FFS.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

# 6.3B.1.4.1	Test purpose

Same test purpose as in clause 6.3.1.1 in TS 38.521-2 [9] for the NR carrier.

<< Unchanged sections omitted >>

6.4B.1.4	Frequency Error for inter-band EN-DC including FR2

# 6.4B.1.4.1	Test purpose

Same test purpose as in clause 6.4.1.1 in TS 38.521-2 [9] for the NR carrier.

<< Unchanged sections omitted >>

7.3B.2.4_1	Reference sensitivity for Inter-band EN-DC including FR2 (>2 CCs)

7.3B.2.4_1.1	Reference sensitivity for Inter-band EN-DC including FR2 (3 CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.1.1	Test purpose

Same test purpose as in clause 7.3B.2.4.1.

# 7.3B.2.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 DL CCs (2NR DL CCs).

# 7.3B.2.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.0.4.

# 7.3B.2.4_1.1.4	Test description

The FR2 reference sensitivity test description for all 3CC EN-DC CA combinations (1LTE+2FR2, 2LTE+1FR2), is the same as in corresponding clause of clause 7.5A or clause 7.5 (if only 1 FR2 carrier) in TS 38.521-2 [9] with the exceptions described below.

# 7.3B.2.4_1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.2B.1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-2 [9] clause 5.3 or clause 5.3A and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2 or clause 5.4.2A. All valid configurations shall be tested with applicable test parameters for inter-band EN-DC including FR2 configuration specified in clause 5.5B.5, and the configuration for NR carrier are shown in TS 38.521-2 [9] Table 7.3.2.4.1-1 or Table 7.3A.2.1.4.1-1.

For Initial conditions as in clause 7.3.2.4.1 or clause 7.3A.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are configured as per clause 4.7 with parameters set according to Table 4.7-1 and propagation conditions set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.2.4.1 or clause 7.3A.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.3B.2.4_1.1.4.2	Test Procedure

Same test procedure as in clause 7.3.2.4.2 or clause 7.3A.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.2.4_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

# 7.3B.2.4_1.1.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3.2.5 or clause 7.3A.2.1.5 in TS 38.521-2 [9].

7.3B.2.4_1.2	Reference sensitivity for Inter-band EN-DC including FR2 (4 CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.2.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

# 7.3B.2.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 4 DL CCs (3NR DL CCs).

# 7.3B.2.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

# 7.3B.2.4_1.2.4	Test description

The FR2 reference sensitivity test description for all 4CC EN-DC CA combinations (1LTE+3FR2, 2LTE+2FR2, 3LTE+1FR2), is the same as in corresponding clause of clause 7.5A or clause 7.5 (if only 1 FR2 carrier) in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

# 7.3B.2.4_1.2.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3.2.5 or clause 7.3A.2.1.5 in TS 38.521-2 [9].

7.3B.2.4_1.3	Reference sensitivity for Inter-band EN-DC including FR2 (5 CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.3.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

# 7.3B.2.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 5 DL CCs (4NR DL CCs).

# 7.3B.2.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

# 7.3B.2.4_1.3.4	Test description

The FR2 reference sensitivity test description for all 5CC EN-DC CA combinations (1LTE+4FR2, 2LTE+3FR2, 3LTE+2FR2, 4LTE+1FR2), is the same as in corresponding clause of clause 7.5A or clause 7.5 (if only 1 FR2 carrier) in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

# 7.3B.2.4_1.3.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3.2.5 or clause 7.3A.2.1.5 in TS 38.521-2 [9].

7.3B.2.4_1.4	Reference sensitivity for Inter-band EN-DC including FR2 (6 CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.4.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

# 7.3B.2.4_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 6 DL CCs (5NR DL CCs).

# 7.3B.2.4_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

# 7.3B.2.4_1.4.4	Test description

The FR2 reference sensitivity test description for all 6CC EN-DC CA combinations (1LTE+5FR2, 2LTE+4FR2, 3LTE+3FR2, 4LTE+2FR2, 5LTE+1FR2), is the same as in corresponding clause of clause 7.5A or clause 7.5 (if only 1 FR2 carrier) in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

# 7.3B.2.4_1.4.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3.2.5 or clause 7.3A.2.1.5 in TS 38.521-2 [9].

<< End of changes >>