Discussion on test point selection for Absolute Power Tolerance in FR2

Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR2 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Absolute Power Tolerance test case in FR2.

Discussion

Absolute Power Tolerance for FR2 is defined in Power Control section in [2] where its scope is described as follows:

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

We also use the test point choice from the relevant FR1 testcase from 38.521-1 (6.3.4.2) as inspiration:

Table 6.3.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Absolute power tolerance test case |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with High range test frequencies. |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

According to general comment in [2] core spec, the power control testcases shall be tested “under normal conditions” only:

### 6.3.4	Power control

#### 6.3.4.1	General

The requirements on power control accuracy apply under normal conditions and are defined as a directional requirement. The requirements are verified in beam locked mode on beam peak direction.

Proposal 1: Select Test Environment as Normal Conditions (NC) for Absolute Power Tolerance measurement in FR2.

Test Subcarrier Spacing

Currently, some of the actual UEs do not support SCS = 60 kHz. Therefore, SCS = 120 kHz is required.

Proposal 2: Select Highest SCS supported for UE for Absolute Power Tolerance measurement in FR2.

  - Test Frequencies

In FR1, this testcase is tested for mid frequency. We believe this is also reasonable for NR.

Proposal 3: Select Mid Frequency for Absolute Power Tolerance measurement in FR2.

Test Channel Bandwidth

In FR1, this testcase is tested for Lowest, Mid, and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivity with different channel bandwidths. Note that Lowest, Mid, and Highest channel bandwidths actually mean 50 MHz, 200 MHz, 400 MHz in FR2. However, there are no requirement for FR2b, 200 MHz and 400 MHz due to the relationship between expected measured power and requirement range. We propose to add 100 MHz to maximize test coverage.

Proposal 4: Test 50 MHz, 100 MHz, 200 MHz, 400 MHz for Absolute Power Tolerance measurement in FR2.

Modulations

In FR1, this test case is tested for CP-OFDM QPSK due to its highest PAPR. However, in FR2, we propose to select DFT-s-OFDM QPSK for 0 dB MPR.

Proposal 5: Select DFT-s-OFDM QPSK in the Absolute Power Tolerance measurement in FR2.

Uplink Configuration

In FR1, this test case is tested for Outer_Full. We propose to use Inner_Full allocation for 0 dB MPR in FR2.

Proposal 6: Use Inner_Full for Absolute Power Tolerance measurement in FR2.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR2 Absolute Power Tolerance test case definition:

Proposal 1: Select Test Environment as Normal Conditions (NC) for Absolute Power Tolerance measurement in FR2.

Proposal 2: Select Highest SCS supported for UE for Absolute Power Tolerance measurement in FR2.

Proposal 3: Select Mid Frequency for Absolute Power Tolerance measurement in FR2.

Proposal 4: Test 50 MHz, 100 MHz, 200 MHz, 400 MHz for Absolute Power Tolerance measurement in FR2.

Proposal 5: Select DFT-s-OFDM QPSK in the Absolute Power Tolerance measurement in FR2.

Proposal 6: Use Inner_Full for Absolute Power Tolerance measurement in FR2.

Number of test points

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | 2 | 1 | 8 |

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-2 v17.1.0 User Equipment (UE) radio transmission and reception; Part 1: Range 2 Standalone (Release 17)