3GPP TSG-RAN5 Meeting #91-e	R5-214097

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.901-5 | CR | 0013 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to 37.901-5 Annex A for Downlink Throughput tests with Fading and FRC scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UE_5GNR_App_Data_Perf |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Tests in A.7.1.1.1 and A.9.1.1.1 needs update in TR 37.901-5. - Annex D needs update for FR2 test point |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updates to Test case contents of Tests A.7.1.1.1 and A.9.1.1.1. - Updates to FR2 test point in Annex D |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - A.7.1.1.1, A.9.1.1.1 will remain incomplete. - Test point for FR2 2Rx will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.1.1.1, A.9.1.1.1, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Updated table header for A.7.1.1.1.3-1, A.9.1.1.1.3-1, Table D.2-1 Added Editor notes concerning untestable test points pending optimization in 38.521-4 |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# A.7	5G NR /TCP Downlink Throughput /Radiated for Fixed Reference Channel Scenarios (FRC) with Fading

## A.7.1	5G NR /TCP Downlink Throughput /Radiated/Fading/FRC

### A.7.1.1	5G NR /TCP Downlink Throughput /Radiated/Fading/FRC/2Rx

#### A.7.1.1.1	5G NR /TCP Downlink Throughput /Radiated/Fading/2Rx TDD/FR2 PDSCH mapping Type A performance - for SA and NSA

Editor’s note: Test points 2-2, 2-6 are currently not testable for n260 and n259 pending further optimization of  maximum testable SNR in TS 38.521-4

##### A.7.1.1.1.1	Definition

The UE application layer downlink performance for TCP under different fading environment is determined by the UE application layer TCP throughput.

##### A.7.1.1.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment under 2 receive antenna conditions for FR2.

##### A.7.1.1.1.3	Test Parameters

The common test parameters are defined in 38.101-4 [4] Table 7.5A.1-1. CORESET details are in TS 38.101-4 [4] Table 7.5A.1-2 and MCS indices for indicated UE capabilities are in TS 38.101-4 [4] Table 7.5A.1-3. SNR required to achieve lower layer throughput requirements is specified in TS 38.101-4 [4] Table 7.5A.1-4. The test parameter selection procedure is defined in TS 38.101-4 [4] clause 7.5A.1. In addition, the following test statements from TS 38.521-4 [3] clause 5.5.1.3 apply:

For NSA FR2 case, the E-UTRA anchor is functional link and is setup via the parameters defined in Annex E.

Test point is detailed in Annex D.2-1.

Table A.7.1.1.1.3-1: FR2 Test Point 2Rx

| 38.521-4 Reference | Test num. | Reference channel | Bandwidth (MHz)/Subcarrier spacing (kHz) | Modulation format | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 7.2.2.2.1_1 | 1-2 | R.PDSCH.5-2.1 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 | Exercises HARQ Combining |
| 7.2.2.2.1_1 | 2-2 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 | High doppler |
| 7.2.2.2.1_1 | 2-6 | R.PDSCH.5-6.1 TDD | 100/120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 | Large TBS, Low Doppler |

##### A.7.1.1.1.4	Test Description

A.7.1.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 7.2.2.2.1_1.3.1 with the following additional steps and/or exceptions:

1.1	Connect an application server to the IP output of the SS.

1.2	For an embedded configuration, ensure that the UE has an client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2. 	In Step 2 skip reference to TS 38.521-4 [3] 7.2.2.2.1.0-2 since test parameters are already defined for this l test.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State (NR for SA case and E-UTRA for NSA with NR Pscell added).

A.7.1.1.1.4.2	Procedure

1.	Set the UE in a direction found using one of the test procedures defined in Annex H of TS 38.521-4 [3]

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC over PUSCH according to parameters set during initial conditions. The purpose of this scheduling is to accommodate for TCP UL ACK/NACK feedback transmissions.

3.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result. (This is iteration 1) Continue data transfer for the test duration outlined in Table A.1-1.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval. Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

6.	Using the values in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.7.1.1.1.3-1 to obtain reference Application Layer Throughput value.

# < Unchanged sections omitted >

# A.9	5G NR /UDP Downlink Throughput /Radiated for Fixed Reference Channel Scenarios (FRC) with Fading

## A.9.1	5G NR /UDP Downlink Throughput /Radiated/Fading/FRC

### A.9.1.1	5G NR /UDP Downlink Throughput /Radiated/Fading/FRC/2Rx

#### A.9.1.1.1	5G NR /UDP Downlink Throughput /Radiated/Fading/2Rx TDD/FR2 PDSCH mapping Type A performance - for SA and NSA

Editor’s note: Test points 2-2, 2-6 are currently not testable for n260 and n259 pending further optimization of  maximum testable SNR in TS 38.521-4

##### A.9.1.1.1.1	Definition

The UE application layer downlink performance for UDP under different fading environment is determined by the UE application layer UDP throughput.

##### A.9.1.1.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading UDP based data in a fading channel environment under 2 receive antenna conditions for FR2.

##### A.9.1.1.1.3	Test Parameters

The common test parameters are defined in 38.101-4 [4] Table 7.5A.1-1. CORESET details are in TS 38.101-4 [4] Table 7.5A.1-2 and MCS indices for indicated UE capabilities are in TS 38.101-4 [4] Table 7.5A.1-3. SNR required to achieve lower layer throughput requirements is specified in TS 38.101-4 [4] Table 7.5A.1-4. The test parameter selection procedure is defined in TS 38.101-4 [4] clause 7.5A.1. In addition, the following test statements from TS 38.521-4 [3] clause 5.5.1.3 apply.

For NSA FR2 case, the E-UTRA anchor is functional link and is setup via the parameters defined in Annex E.

Test point is detailed in Annex D.2-1.

Table A.9.1.1.1.3-1: FR2 Test Point 2Rx

| 38.521-4 Reference | Test num. | Reference channel | Bandwidth (MHz)/Subcarrier spacing (kHz) | Modulation format | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 7.2.2.2.1_1 | 1-2 | R.PDSCH.5-2.1 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 | Exercises HARQ Combining |
| 7.2.2.2.1_1 | 2-2 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 | High doppler |
| 7.2.2.2.1_1 | 2-6 | R.PDSCH.5-6.1 TDD | 100/120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 | Large TBS, Low Doppler |

##### A.9.1.1.1.4	Test Description

A.9.1.1.1.4.1	Initial Conditions

UDPUDPInitial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 7.2.2.2.1_1.3.1 with the following additional steps and/or exceptions

1.1	Connect an application server to the IP output of the SS.

1.2	For an embedded configuration, ensure that the UE has an client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2.	In Step 2 skip reference to TS 38.521-4 [3] 7.2.2.2.1.0-2 since test parameters are already defined for this l test.

3.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State (NR for SA case and E-UTRA for NSA with NR Pscell added).

A.9.1.1.1.4.2	Procedure

1.	Set the UE in a direction found using one of the test procedures defined in Annex H of TS 38.521-4 [3].

2.	SS sets the parameters of the bandwidth, MCS and DL reference channel (RMC) according to the test parameter selection procedure defined in TS 38.101-4 [4] clause 7.5A.1. The SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC.

3.	Using the data client, begin UDP downlink data transfer from the application server. Wait for 15 seconds and then start recording the UDP throughput result. (This is iteration 1) Continue data transfer for the test duration outlined in Table A.1-1.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval. Record the IP address type (IPv4 or IPv6) used during the UDP data transfers.

6.	Using the values in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.9.1.1.1.3-1  to obtain reference Application Layer Throughput value.

# < Unchanged sections omitted >

Annex D: Reference Test Points

# < Unchanged sections omitted >

# D.1	FR2 Reference Test Points

Table D.2-1: FR2 Test Point 2Rx

| 38.521-4 Reference | Test num. | Reference channel | Bandwidth (MHz)/Subcarrier spacing (kHz) | Modulation format | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 7.2.2.2.1_1 | 1-2 | R.PDSCH.5-2.1 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 | Exercises HARQ Combining |
| 7.2.2.2.1_1 | 2-2 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 | High doppler |
| 7.2.2.2.1_1 | 2-6 | R.PDSCH.5-6.1 TDD | 100/120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 | Large TBS, Low Doppler |

< End of changes >