3GPP TSG-RAN5 Meeting #91-e	R5-212224r2

# Electronic meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1809 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC signalling for UL power boosting via suspended IBE requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 has defined a feature to boost FR2 UL Tx power by allowing UEs to suspend in-band emission requirements when configured to do so. RAN2 has specified corresponding RRC signalling by adding condition to ServingCellConfig information element mpr-PowerBoost-FR2-r16. This is captured in clause 4.6.3 (Table 4.6.3-167). UplinkConfig allow network to indicate to UE that it’s allowed to avoid IBE requirements to boost its UL transmission power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE capability for indicating to network that UE supports the UL tx power boosting by IBE requirement suspension in Table 4.6.4-38. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capability for uplink power boosting via IBE requirement relaxation not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision (R5-212224r1): Corrected the coversheet to latest release and version. Rev2: Move IE mpr-PowerBoost-FR2-r16 to correct location in table. Include also other missing fields into RF-parameters IE, which are added in the Rel-16 version of the ASN.1, none of which are currently included in the default message contents in 38.508-1 table 4.6.4-38. Additional fields are included for completeness to aid TTCN implementation. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 4.6.3	Radio resource control information elements

#### –	AdditionalSpectrumEmission

Table 4.6.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 |  |  |

-------- Skipped unchanged clauses --------

#### ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |

-------- Skipped unchanged clauses --------

### 4.6.4	UE capability information elements

#### –	AccessStratumRelease

Table 4.6.4-1: AccessStratumRelease

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AccessStratumRelease | Same as indicated in TC applicability in TS 38.523-2 [19] |  |  |

-------- Skipped unchanged clauses --------

#### RF-Parameters

Table 4.6.4-38: RF-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RF-Parameters ::= SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { | At least 1 entry |  |  |
| BandNR[1] SEQUENCE { |  | entry 1 |  |
| bandNR | FreqBandIndicatorNR |  |  |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Not checked |  |  |
| extendedCP | Not checked |  |  |
| multipleTCI | Not checked |  |  |
| bwp-WithoutRestriction | Not checked |  |  |
| bwp-SameNumerology | Not checked |  |  |
| bwp-DiffNumerology | Not checked |  |  |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Not checked |  |  |
| pusch-256QAM | Not checked |  |  |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxUplinkDutyCycle-PC2-FR1 | Not checked |  |  |
| pucch-SpatialRelInfoMAC-CE | Not checked |  |  |
| powerBoosting-pi2BPSK | Not checked |  |  |
| maxUplinkDutyCycle-FR2 | Not checked |  |  |
| channelBWs-DL-v1590 | Not checked |  |  |
| channelBWs-UL-v1590 | Not checked |  |  |
| asymmetricBandwidthCombinationSet | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 | Not checked |  |  |
| cancelOverlappingPUSCH-r16 | Not checked |  |  |
| multipleRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| pdsch-MappingTypeB-Alt-r16 | Not checked |  |  |
| oneSlotPeriodicTRS-r16 | Not checked |  |  |
| olpc-SRS-Pos-r16 | Not checked |  |  |
| spatialRelationsSRS-Pos-r16 | Not checked |  |  |
| simulSRS-MIMO-TransWithinBand-r16 | Not checked |  |  |
| channelBW-DL-IAB-r16 | Not checked |  |  |
| channelBW-UL-IAB-r16 | Not checked |  |  |
| rasterShift7dot5-IAB-r16 | Not checked |  |  |
| ue-PowerClass-v1610 | Not checked |  |  |
| condHandover-r16 | Not checked |  |  |
| condHandoverFailure-r16 | Not checked |  |  |
| condHandoverTwoTriggerEvents-r16 | Not checked |  |  |
| condPSCellChange-r16 | Not checked |  |  |
| condPSCellChangeTwoTriggerEvents-r16 | Not checked |  |  |
| mpr-PowerBoost-FR2-r16 | Not checked |  |  |
| activeConfiguredGrant-r16 | Not checked |  |  |
| maxNumberConfigsPerBWP-r16 | Not checked |  |  |
| maxNumberConfigsAllCC-r16 | Not checked |  |  |
| jointReleaseConfiguredGrantType2-r16 | Not checked |  |  |
| sps-r16 | Not checked |  |  |
| maxNumberConfigsPerBWP-r16 | Not checked |  |  |
| maxNumberConfigsAllCC-r16 | Not checked |  |  |
| jointReleaseSPS-r16 | Not checked |  |  |
| simulSRS-TransWithinBand-r16 | Not checked |  |  |
| trs-AdditionalBandwidth-r16 | Not checked |  |  |
| handoverIntraF-IAB-r16 | Not checked |  |  |
| simulTX-SRS-AntSwitchingIntraBandUL-CA-r16 | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1630 | Not checked |  |  |
| handoverUTRA-FDD-r16 | Not checked |  |  |
| enhancedUL-TransientPeriod-r16 | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1640 | Not checked |  |  |
| supportedBandCombinationList | Not checked |  |  |
| appliedFreqBandListFilter | Not present |  |  |
|  | FreqBandList |  | FILTER_REQUESTED |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FILTER_REQUESTED | This condition shall be set to true when UE is requested to filter the information via ‘capabilityRequestFilter’ IE in the NR5GC UECapabilityEnquiry message or via ‘requestedFreqBandsNR-MRDC’ IE in the EN-DC UECapabilityEnquiry message |

-------- End of changes --------