3GPP TSG-RAN5 Meeting #91-e	R5-212282r1

# Electronic Meeting, 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4611 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Misunderstandings have emerged on the SS handling of MIB repetitions for BR System information. 2. Execution guidelines need to be updated for the latest list of test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. It has been clarified that TTCN may configure the SS to repeat the MIB. In that case the SS shall repeat the MIB as per TS 36.331 clause 5.2.1.2. 2. Added test case 6.1.1.6b to clause 11.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The misunderstandings will remain. 2. Execution guidelines will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.2a, 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision r1: Change1 updated to modify cl.7.7.2a instead of cl.7.3.3.9. |  |  |  |  |  |  |  |  |

<Start of Next Modified Section>

### 7.7.2a	Scheduling information for BR System information

The scheduling of BR System Information is based upon the following conditions (TS 36.211 [35] clause 6.4.1):

-	 =4 i.e. the schedulingInfoSIB1-BR-r13 values are selected from {1,4,7,10,13,16} which as per TS 36.213 [30] table 7.1.6-1 would result in 4 repetitions.

-	si-WindowLength-BR-r13 =20 ms (same value as si-WindowLength)

-	si-RepetitionPattern-r13 = every2ndRF (hence results eventually in only transmission in first frame of window)

-	Except for SIB1-BR (mandated for SIB1-BR by TS 36.211 [35]) hopping is assumed to be not enabled for anything else.

-	The current test plan excludes inter frequency and inter rat test cases due to limitations in RAN4. Inter frequency, inter RAT system information combinations are not considered.

-	The typical size of SIB1/2/3/4 is less than 700 bits, the Max TB size for SIx-BR is 936 bits, TS 36.213 [30] table 7.1.7.2.7-1, hence no segmentation is assumed. For SIB1-BR schedulingInfoSIB1-BR-r13 =10, and for SIB 2,3,4 the si-TBS-r13 is set as b712.

-	TTCN may configure the SS to repeat the MIB. In that case the SS shall repeat the MIB as per TS 36.331[19] clause 5.2.1.2 in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD, the same can be achieved in the below schedule.

-	System information broadcasting rules defined in clause 7.7.1 apply with the exception that SIB1 is replaced with SIB1 & SIB1-BR and SI is replaced with SI & SI-BR.

The subframe offset values used for SI messages are according to table 7.7.2a-1.

<End of Modified Section>

<Start of Next Modified Section>

### 11.1.1	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.1.1-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.1.1-1: Replacement of test cases

| Original test case | Replacing test case |
| --- | --- |
| 6.1.1.1 | 6.1.1.1b |
| 6.1.1.2 | 6.1.1.2a |
| 6.1.1.3 | 6.1.1.3b |
| 6.1.1.6 | 6.1.1.6a or 6.1.1.6b |
| 6.1.1.7 | 6.1.1.7a |
| 6.1.2.7 | 6.1.2.7a |
| 6.1.2.8 | 6.1.2.8a |
| 6.1.2.9 | 6.1.2.9a |
| 8.1.3.12 | 8.1.3.12b |
| 8.3.1.9 | 8.3.1.9a |
| 8.3.1.11 | 8.3.1.11a |
| 9.2.1.1.1a | 9.2.1.1.1b |
| 9.2.1.1.7 | 9.2.1.1.7a |
| 9.2.1.1.13 | 9.2.1.1.13a |
| 9.2.1.1.15 | 9.2.1.1.15a |
| 9.2.1.1.16 | 9.2.1.1.16a |
| 9.2.3.1.15 | 9.2.3.1.15a |
| 9.2.3.1.18 | 9.2.3.1.18a |
| 17.4.1 | 17.4.1a |

<End of Modified Section>