3GPP TSG-RAN WG5 Meeting #91-e	R5-212306r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0147 | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCPTT Test Case 5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - After the preamble the user belongs to only default GROUP A - Message Flow has incorrect message order and missing messages SIP NOTIFY for User Profile change and request missing (addition as well as removal from groups change user profile – see TS 33.180 clause 5.7.3) Retrieval of documents missing According to TS 24.481 clause 6.3.14.2: If the GMC receives a HTTP 409 (conflict) when creating a temporary group, the GMC may repeat the procedure. - Incorrect message structure and specific message content (e.g. HTTP PUT in Step 9 creates the same document as in step 3 – with same group-id) - Conformance requirements of TS 33.179 obsolete as from Rel-13 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - GROUP A selection in Step 1 removed - Adaptation to generic procedure for group creation - Specific message content corrections - Conformance requirements of TS 33.179 replaced by Conformance requirements of TS 33.180 – Rel-14 - Editorial corrections - User Information and referred NOTE 2 removed - Table header and derivation path corrections - Missing specific message content added resp. removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and message contents may result in incorrect MCPTT test case implementation, test case cannot be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0126, 0134, 0137 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision r1: UserInformation,and NOTE 2 removed, table header and derivation path corrections, specific message content added resp. removed |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.2	Configuration / Group Creation / Group Regroup Creation / Group Regroup Teardown

5.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) attached to EPS services }

ensure that {

when { the UE (MCPTT Client) requests formation of a new MCPTT group }

then { on successful group creation the UE (MCPTT Client) has access to the new group }

}

(2)

with { UE (MCPTT Client) having access to at least two MCPTT groups }

ensure that {

when { the UE (MCPTT Client) requests the groups to be combined }

then { on successful group regrouping the UE (MCPTT Client) has access to the temporary group }

}

(3)

with { UE (MCPTT Client) having access to a temporary group }

ensure that {

when { the UE (MCPTT Client) requests temporary group tear down }

then { on successful group tear down the UE (MCPTT Client) removes the temporary group }

}

5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.481 clauses 6.3.2.2.1, 6.3.2.2.2, 6.3.14.1, 6.3.14.2, 6.3.15.1 and 6.3.15.2; TS 33.180 clauses 7.3.2. Unless otherwise stated these are Rel-13 requirements.

[TS 24.481 clause 6.3.2.2.1]

In order to create a group document, a GC shall create an XML document of the application usage specified in subclause 7.2.1 and shall send the XML document to the network according to procedures specified in IETF RFC 4825 [22] "Create or Replace a Document". The GC shall set the Request-URI of the HTTP PUT request to an XCAP URI in users' tree where the XUI is set to a group creation XUI configuration parameter.

[TS 24.481 clause 6.3.2.2.2]

In order to create a group document, a GMC shall perform the procedures in subclause 6.3.2.2.1 specified for GC.

[TS 24.481 clause 6.3.14.1]

This procedure enables a GMC to initiate creation of a temporary MCPTT group by combining MCPTT groups.

[TS 24.481 clause 6.3.14.2]

In order to form a temporary MCPTT group, a GMC shall send a HTTP POST request according to procedures specified in IETF RFC 2616 [21] and subclause 6.2.3. In the HTTP POST request, the GMC:

a)	shall set the Request-URI to an XCAP URI:

1)	in users tree where the XUI is set to a group creation XUI configuration parameter; and

2)	with the document selector identifying the temporary MCPTT group to be created; and

b)	shall include an application/vnd.3gpp.GMOP+xml MIME body containing a GMOP document requesting group regroup creation specified in subclause 7.3.4.3, with a <group> element containing a group document for an MCPTT group. In the group document, the GMC shall include the <on-network-temporary> element according to subclause 7.2. In the <on-network-temporary> element, the GMC shall include <constituent-MCPTT-group-IDs> element according to subclause 7.2. In the <constituent-MCPTT-group-IDs> element, the GMC shall include one <constituent-MCPTT-group-ID> element according to subclause 7.2 for each MCPTT group to be combined.

Upon reception of an HTTP 2xx response to the sent HTTP POST request, the GMC shall consider the temporary MCPTT group formation as successful.

Upon reception of an HTTP 409 (Conflict) response with at least one <alt-value> element in the <uniqueness-failure> error element, the GMC may repeat procedures of the present subclause and identify the temporary MCPTT group being formed with an MCPTT Group ID indicated in an <alt-value> element.

[TS 24.481 clause 6.3.15.1] This procedure enables a GMC to initiate tear down of a temporary MCPTT group.

[TS 24.481 clause 6.3.15.2]

In order to tear down a temporary MCPTT group, the GMC shall send an HTTP DELETE request with Request-URI with an XCAP URI identifying a group document of the temporary MCPTT group according to procedures specified in IETF RFC 4825 [22] "Delete an Element".

### [TS 33.180 clause 7.3.2]

The group creation procedure is described in clause 10.2.3 of 3GPP TS 23.280 [36] and applies to the MCPTT scenario of normal group creation by an MC administrator and user regrouping operations by an authorized user/dispatcher.  To establish the security context for the group, the GMS follows the procedures in clause 5.7 to create a new GMK and GMK-ID.

The encapsulated GMK and GUK-ID is sent to group members by the GMS within a notification message (step 4 in clause 10.2.3 of 3GPP TS 23.280 [36]). The procedure is equivalent to that described in clause 5.7 of this specification.

#### [TS 33.180 clause 7.3.3.2]

Group Regroup procedures for the MC system are described in clause 10.2.4.1 of 3GPP TS 23.280 [36]. To create the security context for the temporary group, the GMS follows the procedures in clause 5.7, creating a new GMK and GMK-ID for the temporary group.

An encapsulated GMK and GUK-ID is sent to the temporary group members by the GMS within a notification message (step 5 in clause 10.2.4.1 of 3GPP TS 23.280 [36]). The procedure is equivalent to that described in clause 5.7.

5.2.3	Test description

5.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4.

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10, is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.2.3.2	Test procedure sequence

Table 5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC related actions are described in TS 36.579-1 [2], subclause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages being exchanged. | - | - | - | - |
| 2 | Make the MCPTT User request the creation of a new group, MCPTT Group B, for communication with the user and two other users, MCPTT Client B and MCPTT Client C. (NOTE 1) | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) correctly perform Generic Test Procedure for MCPTT CO Group Creation as described in TS 36.579-1 [2] Table 5.3.26.3-1 to create GROUP B and retrieve group configuration? | - | - | 1 | P |
| 4-7 | Void | - | - | - | - |
| 8 | Make the MCPTT User request the creation of a new group, MCPTT Group C, for communication with the user and two other users, MCPTT Client B and MCPTT Client C. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCPTT client) correctly perform Generic Test Procedure for MCPTT CO Group Creation as described in TS 36.579-1 [2] Table 5.3.26.3-1 to create GROUP C and retrieve group configuration? | - | - | 1 | P |
| 10-12 | Void | - | - | - | - |
| 13 | Make the MCPTT User request the creation of a temporary group MCPPT Group T formed from MCPTT Group B and MCPTT Group C to communicate with itself and MCPTT Client B and MCPTT Client C. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCPTT Client) correctly perform Generic Test Procedure for MCPTT CO Group Creation as described in TS 36.579-1 [2] Table 5.3.26.3-1 to create temporary GROUP T and retrieve group configuration?? | - | - | 1 | P |
| 15-20 | Void | - | - | - | - |
| 21 | Make the MCPTT User request tear down of the temporary MCPTT Group T (NOTE 1) | - | - | - | - |
| 22 | Check: Does the UE (MCPTT Client) send a HTTP DELETE to the SS to request tear down of the temporary group? | --> | HTTP DELETE | 3 | P |
| 22A | The SS (MCPTT Server) sends a HTTP 200 (OK) indicating group tear down completed. | <-- | HTTP 200 (OK) | - | - |
| 23 | Step 2 to Step 5 of Generic Test Procedure for MCPTT CO Group Creation as described in TS 36.579-1 [2] Table 5.3.26.3-1 are performed (update the user profile after temporary group deletion) | - | - | - | - |
| 24-26 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent mechanism and may be manually or automatically initiated. |  |  |  |  |  |

5.2.3.3	Specific message contents

Table 5.2.3.3-1: HTTP PUT (Step 3, Table 5.2.3.2-1; Step 1b1, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 5.2.3.3-1A: HTTP 201 (Created) (Step 3, Table 5.2.3.2-1; Step 1b2, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, conditions GROUPCONFIG. GROUP_B

Table 5.2.3.3-2: Void

Table 5.2.3.3-3: Void

Table 5.2.3.3-3A: Void

Table 5.2.3.3-3B: HTTP 200 (OK) (Step 3, Table 5.2.3.2-1 Step 5, TS 36.579-1 [2] Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| mcptt-user-profile | User Profile Document described in Table 5.2.3.3-3C | User Profile Document returned |  |  |

Table 5.2.3.3-3C: User Profile Document in HTTP 200 (OK) (Table 5.2.3.3-3B)

Derivation Path: Table 5.5.8.3-1 condition GROUP_B

Table 5.2.3.3-3D: SIP SUBSCRIBE (Step 3, Table 5.2.3.2-1; Step 1, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2],Table 5.3.26.4-13, condition GROUPCONFIG, GROUP_B

Table 5.2.3.3-4: SIP NOTIFY (Step 3, Table 5.2.3.2-1; Step 3, TS 36.579-1 [2] Table 5.3.2.3-2B)

| Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-14, conditions GROUPCONFIG, GROUP_B |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 5.2.3.3-4A: Void

Table 5.2.3.3-4B: SIP SUBSCRIBE (Step 3, Table 5.2.3.2-1; Step 2a1, TS 36.579-1 [2] Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-19, condition GROUPCONFIG, GROUP_B

Table 5.2.3.3-4C: SIP NOTIFY (Step 3, Table 5.2.3.2-1; S Step 7a1, Table 5.3.2.3-2B and Step 2a3, Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-20, conditions GROUPCONFIG, GROUP_B

Table 5.2.3.3-5: Void)

Table 5.2.3.3-6: Void

Table 5.2.3.3-7: Void

Table 55.2.3.3-7A: HTTP 200 (OK) (Step 3, Table 5.2.3.2-1; Step 5, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG, GROUP_B

Table 5.2.3.3-7B: HTTP PUT (Step 9, Table 5.2.3.2-1; Step 1b1, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.4-1, conditions GROUPCREATE, GROUP_C

Table 5.2.3.3-7C: HTTP 201 (Created) (Step 9, Table 5.2.3.2-1; Step 1b2, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 conditions GROUPCONFIG. GROUP_C

Table 5.2.3.3-7D: HTTP 200 (OK) (Step 9, Table 5.2.3.2-1 Step 5, TS 36.579-1 [2] Table 5.3.26.3-1)

| Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| mcptt-user-profile | User Profile Document described in Table 5.2.3.3-7E | User Profile Document returned |  |  |

Table 5.2.3.3-7E: User Profile Document in HTTP 200 (OK) (Table 5.2.3.3-7D)

Derivation Path: Table 5.5.8.3-1 conditions GROUP_B, GROUP_C

Table 5.2.3.3-8: Void

Table 5.2.3.3-8A: SIP SUBSCRIBE (Step 9, Table 5.2.3.2-1; Step 1, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2],Table 5.3.26.4-13, condition GROUPCONFIG, GROUP_C

Table 5.2.3.3-8B: SIP NOTIFY (Step 9, Table 5.2.3.2-1; Step 3, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-14, conditions GROUPCONFIG, GROUP_C

Table 5.2.3.3-8C: HTTP GET (Step 9, Table 5.2.3.2-1; Step 5, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-16, condition GROUPCONFIG, GROUP_C

Table 5.2.3.3-8D: HTTP 200 (OK) (Step 9, Table 5.2.3.2-1; Step 6, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-17 condition GROUPCONFIG, GROUP_C

Table 5.2.3.3-8E: SIP SUBSCRIBE (Step 9, Table 5.2.3.2-1; Step 2a1, TS 36.579-1 [2] Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-19, condition GROUPCONFIG, GROUP_C

Table 5.2.3.3-8F: SIP NOTIFY (Step 9, Table 5.2.3.2-1; S Step 7a1, Table 5.3.2.3-2B and Step 2a3, Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-20, conditions GROUPCONFIG, GROUP_C

Table 5.2.3.3-9: HTTP POST (Step 14, Table 5.2.3.2-1; Step 1a1, TS 36.579-1 [2] Table 5.3.26.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1 condition TEMPGROUP |  |  |  |  |
| --- | --- | --- | --- | --- |

Table 5.2.3.3-10: Void

Table 5.2.3.3-11: Void

Table 5.2.3.3-11A:Void

Table 5.2.3.3-12: Void

Table 5.2.3.3-13: HTTP 200 (OK) (Step 14, Table 5.2.3.2-1; Step 5, TS 36.579-1 [2] Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, conditions UEUSERPROF , GROUP_T

Table 5.2.3.3-13A: SIP SUBSCRIBE (Step 14, Table 5.2.3.2-1; Step 1, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2],Table 5.3.26.4-13, condition GROUPCONFIG, GROUP_T

Table 5.2.3.3-13B: SIP NOTIFY (Step 14, Table 5.2.3.2-1; Step 3, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-14 conditions GROUPCONFIG, GROUP_T

Table 5.2.3.3-13C: HTTP GET (Step 14, Table 5.2.3.2-1; Step 5, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-16, condition GROUPCONFIG, GROUP_T

Table 5.2.3.3-13D: HTTP 200 (OK) (Step 14, Table 5.2.3.2-1; Step 6, TS 36.579-1 [2] Table 5.3.2.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-17, condition GROUPCONFIG, GROUP_T

Table 5.2.3.3-13E: SIP SUBSCRIBE (Step 14, Table 5.2.3.2-1; Step 2a1, TS 36.579-1 [2] Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-19, condition GROUPCONFIG, GROUP_T

Table 5.2.3.3-13F: SIP NOTIFY (Step 14, Table 5.2.3.2-1; Step 7a1, Table 5.3.2.3-2B and Step 2a3, Table 5.3.2.3-2C)

Derivation Path: TS 36.579-1 [2], Table 5.3.26.4-20, conditions GROUPCONFIG, GROUP_T

Table 5.2.3.3-14: HTTP DELETE (Step 22, Table 5.2.3.2-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1

Table 5.2.3.3-15: Void

Table 5.2.3.3-16: HTTP 200 (OK) (Step 23, Table 5.2.3.3-1; (Step 5 TS 36.579-1 [2], Table 5.3.26.3-1)

Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF

Editor's Note:	Additionally to the conditions in default message contents, the conditions GROUP_B, GROUP_C and GROUP_T refer to tables in generic test procedures, how to note this in a better way is for FFS.