3GPP TSG-RAN WG5 Meeting #91-e	R5-212337r2

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0017 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.579-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - XML documents contained in SIP or HTTP message may be partly encrypted. Encryption/decryption is done in TTCN by using external functions so that in general traditional logging does not provide readable messages. - Most of the PIXITs in clause 9.2 are not MCPTT specific - Several PIXITs defined in clause 9.2 are not used or not used anymore - When using the IPCAN model the E-UTRA/EPC black-box the values of EPS default/dedicated bearers are implementation dependent - There has been no possibility for the system simulator to indicate errors for the IPCAN model - Test case 5.4 needs further XML schema urn:3gpp:ns:mcpttGMOP:1.0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New external function to allow explicit logging of XML documents (presented at TTCN Workshop #53 in R5w210110) - "MCPTT" replaced by "MCX" in the headers of clause 9.2 - Unused PIXITs removed - New PIXITs for EPS default/dedicated bearers (presented at TTCN Workshop #53 in R5w210110) - New branch for an error indication in MCX_IPCAN_SystemConfirmation_Type (presented at TTCN Workshop #53 in R5w210110) - XML schema added - Pixit for functional alias added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification, verification over IPCAN model not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 8.2, 9, annex D, annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision r1: missing pixit added Revision r2: Filename and linenumber added as parameter for fx_XML_Log |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.2	MCX test model with SS based E-UTRA/EPC implementation (MCX IPCAN test model)

### 5.2.1	MCX Client on-network test model

In contrast to the MCX EUTRA test model the MCX IPCAN test model shown in figure 5.2.1-1 uses an external E-UTRA/EPC (black-box) implementation which is integrated in the system simulator. In general this E-UTRA/EPC implementation shall be conformant in terms of TS 36.508 [61] but this is out of the scope for the MCX IPCAN test model as long as the E-UTRA/EPC implementation obeys the commands at the system interface as specified in annex E.

NOTE:	Whether or how the MCX IPCAN test model may use other IP-connectivity access networks is out of scope of this specification.

Figure 5.2.1-1: MCX Client on-network test model with SS based E-UTRA/EPC implementation

As described in clause 5.4.1A of TS 36.579-1 [2] after switch on the UE may register to an internet PDN, an IMS PDN and the MCX PDN. Nevertheless in TTCN the MCX IPCAN test model supports signalling for the MCX PDN only: The IPCAN test model does not handle any signalling for the other PDNs and especially not the SIP signalling for an IMS registration to the IMS PDN. 
 It is up to SS implementation to handle the potential registrations to an internet PDN and/or an IMS PDN.

To avoid conflicts due to use of one and the same value at the SS and in TTCN, the SS gets configured at the IPCAN configuration port (IPCANSYS) with the information shown in table 5.2.1-1 and table 5.2.1-2.

Table 5.2.1-1: Default EPS bearer(s)

| Parameter | PDN | Value/Comment (NOTE 1) |
| --- | --- | --- |
| Bearer ID (NOTE 2) | MCX | px_MCX_IPCAN_EpsBearerId_MCX |
|  | IMS | px_MCX_IPCAN_EpsBearerId_IMS |
|  | Internet | px_MCX_IPCAN_EpsBearerId_INTERNET |
| NW IP address | MCX | Not configured at IPCAN interface but used by TTCN: px_IPv4_Address1_NW, px_IPv6_Address1_NW |
|  | IMS | px_IPv4_Address2_NW, px_IPv6_Address2_NW |
|  | Internet | px_IPv4_Address3_NW, px_IPv6_Address3_NW |
| UE IP address | MCX | Not configured at IPCAN interface but used by TTCN: px_IPv4_Address1_UE, px_IPv6_Address1_UE |
|  | IMS | px_IPv4_Address2_UE, px_IPv6_Address2_UE |
|  | Internet | px_IPv4_Address3_UE, px_IPv6_Address3_UE |
| NOTE 1: Values may change depending on TTCN implementation NOTE 2: The DRB associated with the EPS bearer shall have the DRB ID := EPS Bearer ID – 4. |  |  |

Table 5.2.1-2: Dedicated EPS bearer(s)

| Parameter | Service | Value/Comment (NOTE 1) |  |  |
| --- | --- | --- | --- | --- |
| Bearer ID (NOTE 2) | MCPTT | px_MCX_IPCAN_EpsBearerId_MCX_Audio |  |  |
|  | MCVideo | px_MCX_IPCAN_EpsBearerId_MCX_Audio, px_MCX_IPCAN_EpsBearerId_MCX_Video |  |  |
|  | MCData | FFS |  |  |
| Packet filters | MCPTT | Audio | Remote port range | tsc_MCX_MediaPort_Audio .. tsc_MCX_MediaPort_Audio + 1 |
|  |  |  | Protocol | UDP |
|  |  | Media Control | Single remote port | tsc_MCX_MediaControlPortNW |
|  |  |  | Protocol | UDP |
|  | MCVideo | FFS |  |  |
|  | MCData | FFS |  |  |
| NOTE 1, 2: see Table 5.2.1-1. |  |  |  |  |

<End of modification>

<Start of modification>

## 8.2	External function definitions

The external functions specified in TS 36.523-3 [27] clause 8.2 apply to the present document.

In addition there are the following MCX specific external functions:

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_GeneratePublicKey |  |
| Description | Generate KMS public key (Z_T) for SAKKE (RFC 6508 [45] clause 2.2): Z_T := [z_T]P |  |
| Parameters | p_MasterSecret | master secret z_T (octetstring) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_GenerateRSK |  |
| Description | Generate receiver secret key (RSK) for SAKKE (RFC 6508 [45] clause 2.2): RSK := [(a + z_T)^-1]P with 'a' being the identifier (UID) corresponding to the receiver's URI |  |
| Parameters | p_MasterSecret | master secret z_T (octetstring) |
|  | p_Identifier | UID for a given URI (octetstring) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_EncapsulateKey |  |
| Description | Generate encapsulated data for SAKKE exchange according to RFC 6508 [45] |  |
| Parameters | p_SSV | Shared secret value (octetstring): Key to be exchanged; according to 33.180 [43] E.1.1: The GMK, PCK, CSK and MuSiK shall be 16 octets in length |
|  | p_SakkePublicKey | SAKKE public key (octetstring) generated with fx_SAKKE_GeneratePublicKey |
|  | p_UID | UID (octetstring) generated for the receiving entity's URI (in general the same URI as in IDRr payload of the MIKEY message carrying the encapsulated data) |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SAKKE_ExtractKey |  |
| Description | Extract and validate 16 octet key from the encapsulated date for SAKKE exchange according to RFC 6508 [45] clause 6.2.2: if validation fails (TEST does not equal R_(b,S)) then omit shall be returned |  |
| Parameters | p_EncapsulatedData | encapsulated data (octetstring) as received in the SAKKE payload of a MIKEY message |
|  | p_SakkeRSK | receiver secret key (RSK) for SAKKE (octetstring) |
|  | p_SakkePublicKey | SAKKE public key (octetstring) generated with fx_SAKKE_GeneratePublicKey |
|  | p_UID | UID (octetstring) generated for the receiving entity's URI (should be the same URI as in IDRr payload of the MIKEY message carrying the encapsulated data |
|  | p_ParameterSet | parameter set to be used; 1 per default, indicating use of parameter set 1 as defined in appendix A of RFC 6509 [46] |
| Return Value | template (omit) octetstring (16 octets if the key valid, omit otherwise) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_GenerateKPAK |  |
| Description | Generate KMS Public Authentication Key (KPAK) for ECCSI (RFC 6507 [44] clause 4.2): KPAK := [KSAK]G The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_KSAK | KMS Secret Authentication Key (KSAK): random secret non-zero integer modulo q (octetstring) |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_GenerateSskPvtPair |  |
| Description | Generate (SSK,PVT) Pair according to clause 5.1.1 of RFC 6507 [44]; if either the SSK or HS is zero modulo q (step 5 of the algorithm), the function shall return omit The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_UID | User ID (octetstring) |
|  | p_KSAK | KMS Secret Authentication Key (KSAK) (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_EphemeralValue | random (ephemeral) non-zero integer value v according to RFC 6507 [44] clause 5.1.1 |
| Return Value | template (omit) type record ECCSI_SskPvtPair_Type { octetstring SSK, octetstring PVT } |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_SignMessage |  |
| Description | Sign a message according to RFC 6507 [44] clause 5.2.1: return signature of the message or omit if HE + r * SSK is non-zero modulo q (step 4 of the algorithm) The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_Message | Message to be signed (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_UID | Signer's User ID (octetstring) |
|  | p_SSK | Secret Signing Key (octetstring) |
|  | p_PVT | Public Validation Token (octetstring) |
|  | p_EphemeralValue | random (ephemeral) non-zero integer value j according to RFC 6507 [44] clause 5.2.1 |
| Return Value | template (omit) octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ECCSI_VerifySignature |  |
| Description | Verify a signature according to RFC 6507 [44] clause 5.2.2: return true, when the signature is valid, false otherwise The P-256 elliptic curve, base point and SHA-256 function as according to RFC 6509 [46] clause 2.1.1 shall be used. |  |
| Parameters | p_Message | Message (octetstring) |
|  | p_Signature | Message's signature (octetstring) |
|  | p_KPAK | KMS Public Authentication Key (KPAK) (octetstring) |
|  | p_UID | Signer's User ID (octetstring) |
| Return Value | boolean |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Encrypt |  |
| Description | Encrypt data NOTE: the function is defined similar to openssl_encrypt and in principle it is independent from XML; nevertheless it is used for XML encrytion in context of MCX |  |
| Parameters | p_Data | (octetstring) |
|  | p_Method | type enumerated XML_EncryptionMethod_Type { AES_128_GCM, // AES-GCM according to clause 5.2.4 of // https://www.w3.org/TR/xmlenc-core1/ [41] // with 96 bit Initialization Vector and //128 bit Authentication Tag AES_256_KEY_WRAP // according to RFC 3394 [47] } |
|  | p_Key | key for encryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring | In case of an authenticated encryption mechanism (e.g. AES_128_GCM) the Authentication Tag is added at the end of the encrypted data. |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Decrypt |  |
| Description | Decrypt data |  |
| Parameters | p_Data | In case of an authenticated encryption mechanism (e.g. AES_128_GCM) the input data is the concatination of the encrypted data and Authentication Tag (octetstring) |
|  | p_Method | type enumerated XML_EncryptionMethod_Type { AES_128_GCM, // AES-GCM according to clause 5.2.4 of // https://www.w3.org/TR/xmlenc-core1/ [41] // with 96 bit Initialization Vector and //128 bit Authentication Tag AES_256_KEY_WRAP // according to RFC 3394 [47] } |
|  | p_Key | key for encryption (bitstring) |
|  | p_IV | initial vector (octetstring) |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_MCX_XML_AddEnvelopedSignature |  |
| Description | Add XML signature to the given XML document and return resulting XML document; according to W3C: "XML Signature Syntax and Processing (Second Edition)" [48]: 1. The given document has a Signature element with the name of the key to be used to sign the Signature's SignedInfo 2. Elements(s) to be signed: For elements which are addressed by a reference URI in the Signature's SignedInfo the DigestValue shall be generated and added to the corresponding Reference element of the Signature's SignedInfo 3. The SignedInfo shall be signed by generating the hash for the Signature's SignedInfo using the given key; this hash value shall be added to the Signature's SignatureValue. |  |
| Parameters | p_XmlDocument | XML document (charstring) to be signed; the document contains all information to get signed with the given key: - id(s) for the object(s) to be signed (e.g. KMS response) - SignedInfo with reference(s) to objects to be signed within the document ("same-document" reference(s) according to RFC 3986 [53] clause 4.4), DigestAlgorithm, empty DigestValue |
|  | p_Key | Key corresponding to the KeyName in the Signature's KeyInfo element (bitstring) |
| Return Value | universal charstring containing the document with all DigestValues and the signature of the SignedInfo |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_MCX_XML_CreateDetachedSignature |  |
| Description | Create XML signature for the given XML document and return resulting (encoded) XML signature (detached signature); according to TS 33.180 [43] clause 9.3.5, TS 24.379 [9] clause 6.6.3.1 and W3C: "XML Signature Syntax and Processing (Second Edition)" [48]: p_SignatureInfo contains the parameters needed to create the signature and to sign the document; the whole document gets signed irrespective of the Reference URI |  |
| Parameters | p_XmlDocument | XML document to be signed |
|  | p_XmldsigDocument | XML xmldsig document with signature element containing all information to sign the document (p_XmlDocument) |
|  | p_Key | Key corresponding to the KeyName in the Signature's KeyInfo element |
| Return Value | universal charstring | XML xmldsig document with DigestValue and SignatureValue added to p_XmldsigDocument |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_GetElementContent |  |
| Description | Return content of the node in an XML tree or XML branch as pointed at by the xpath expression |  |
| Parameters | p_XmlBranch | XML tree (including the root element) or branch ot an XML tree |
|  | p_XpathExpression | XCAP expression according to https://www.w3.org/TR/1999/REC-xpath-19991116/ |
| Return Value | universal charstring | string containing the content of the element or empty string if p_XpathExpression does not address an existing element of the given XML branch NOTE: if the element may be empty, it may be necessary to check existence of the element with fx_XML_GetElement first |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_SetElementContent |  |
| Description | Return XML tree or XML branch with replaced content of the node pointed at by the xpath expression |  |
| Parameters | p_XmlBranch | XML tree (including the root element) or branch ot an XML tree |
|  | p_XpathExpression | XCAP expression according to https://www.w3.org/TR/1999/REC-xpath-19991116/ |
|  | p_NewContent | new content for the given node |
| Return Value | universal charstring | modified XML tree or XML branch |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_SHA_2 |  |
| Description | Generic SHA-2 function |  |
| Parameters | p_Function | type enumerated HASH_Function_Type { SHA_256 // may be extended e.g SHA_224 etc. } |
|  | p_Data | data (octetstring) |
| Return Value | octetstring (representing 256 bits for SHA-256, 224 bits for SHA-224, ...) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_JWK_Signature |  |
| Description | Generate JWK signature according to RFC 7515 [49] |  |
| Parameters | p_String | string for which the signature shall be generated (charstring) |
|  | p_Algorithm | algorithm to generate the hash: type enumerated JWK_HashAlgorithm_Type { // RFC 7515 [49] HS256, // HMAC SHA-256 RS256 // RSASSA-PKCS1-v1_5 SHA-256 } |
|  | p_Key | key (octetstring) |
| Return Value | charstring (base64url encoded signature according to RFC 7515 [49]) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_XML_Log |  |
| Description | Log function for (unencrypted) XML documents |  |
| Parameters | p_FileName | TTCN module from where the function is called |
|  | p_LineNo | Line number from where the function is called |
|  | p_UplinkDownlink | XML document has been received (UPLINK) or will be sent (DOWNLINK) |
|  | p_XmlDocument | XML document |

<End of modification>

<Start of modification>

# 9	IXIT Proforma

## 9.1	General

This partial IXIT proforma contained in the present document is provided for completion, when the related Abstract Test Suite is to be used against the Implementation Under Test (IUT).

Text in italics is a comment for guidance for the production of an IXIT, and is not to be included in the actual IXIT.

The completed partial IXIT will normally be used in conjunction with the completed ICS, as it adds precision to the information provided by the ICS.

## 9.2	MCX

### 9.2.1	MCX Client PIXIT

Table 9.2.1-1: MCX Client Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| Client relevant IXIT |  |  |  |  |
| px_MCPTT_Client_B_ID | charstring | "urn:uuid:cdcd34fb-c2f1-4488-a03a-6b5a5d360c47" |  | MCPTT client ID for a remote user: As defined in TS 24.379 [9] clause 4.10 it is a UUID URN according to RFC 4122 [62], i.e. has the format "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" |
| Users relevant IXIT |  |  |  |  |
| px_MCPTT_ID_User_A | charstring | "mcptt-user-A-id@ mcptt-op.gov" | NOTE 1 | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCVideo_ID_User_A | charstring |  | NOTE 1 | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |
| px_MCData_ID_User_A | charstring |  | NOTE 1 | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_A_Profile_Name | charstring | "mcptt-user-A-Profile-Name" |  | Profile name for the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_A_Alias | charstring | "mcptt-user-A-alias" |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_ID_FA_A | charstring | "mcptt-FA-A-id@ mcptt-op.gov" |  | The value is a "uri" attribute. TS 24.483 [13] clause 5.2.48W6 |
| px_MCPTT_User_A_ParticipantType | charstring | "first responder" |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCX_DomainName_Organization_A | charstring | "mcptt-op.gov" |  | Domain name of the organization the user belongs to. |
| px_MCX_User_A_username | charstring | "MCPTT#U01" |  | UE's User username used for user authentication |
| px_MCX_User_A_password | charstring | "psw@MCPTT&7" |  | UE's User password used for user authentication |
| px_MCPTT_TokenRsaPrivateKey | octetstring |  |  | Private key being used to create signature for ID and Access Token |
| px_MCPTT_UserDecryptKey_name | charstring | “tk.12.userA_decrypt@mcptt-op.gov” |  | Name of the key being used to cipher the SAKKE "Receiver Secret Key". in the KMS Key Set. |
| px_MCPTT_UserDecryptKey_value | bitstring |  |  | Value of the key being used to cipher the SAKKE "Receiver Secret Key" in the KMS Key Set |
| px_MCPTT_UserSigningKeySSK_name | charstring | “tk.12.userA_sign@mcptt-op.gov” |  | Name of the key being used to cipher the ECCSI private Key, "SSK". in the KMS Key Set |
| px_MCPTT_UserSigningKeySSK_value | bitstring |  |  | Value of the key being used to cipher the ECCSI private Key, "SSK", in the KMS Key Set |
| px_MCPTT_UserPubTokenPVT_name | charstring | “tk.12.userA_pub@mcptt-op.gov” |  | Name of the key being used to cipher the ECCSI public validation token, "PVT". in the KMS Key Set |
| px_MCPTT_UserPubTokenPVT_value | bitstring |  |  | Value of the key being used to cipher the ECCSI public validation token, "PVT", in the KMS Key Set |
| px_MCPTT_SigningKey_name | charstring | “tk.12.userA_signing@mcptt-op.gov” |  | Name of the key being used to sign the kmsResponse carrying the key set |
| px_MCPTT_SigningKey_value | bitstring |  |  | Value of the key being used to sign the kmsResponse carrying the key set |
| px_MCX_TrK | bitstring |  |  | Transport key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_TrK_ID | B32_Type |  |  | Transport key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_InK | bitstring |  |  | Integrity protection key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCX_InK_ID | B32_Type |  |  | Integrity protection key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |
| px_MCPTT_ID_User_B | charstring | "mcptt-user-B-id@ mcptt-op.gov" |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_B_Profile_Name | charstring | "mcptt-user-B-Profile-Name" |  | Profile name for the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_B_Alias | charstring | "mcptt-user-B-alias" |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_B_ParticipantType | charstring | "first responder" |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_ID_User_C | charstring | "mcptt-user-C-id@ mcptt-op.gov" |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_C_Profile_Name | charstring | "mcptt-user-C-Profile-Name" |  | Profile name for the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_C_Alias | charstring | "mcptt-user-C-alias" |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |
| px_MCPTT_User_C_ParticipantType | charstring | "first responder" |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |
| px_MCX_SIP_PublicUserId_A_1 | charstring |  |  | (First) public user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM; the parameter shall be a SIP URI |
| px_MCX_SIP_PrivateUserId_A | charstring |  |  | Private user id of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |
| px_MCX_SIP_HomeDomain_A | charstring |  |  | Home domain name of the SIP subscriber (user A) as stored in the HSS and on the ISIM, if the UE has an ISIM |
| px_MCX_SIP_RegistrationWithTemporaryIdentities | boolean |  |  | If true the UE shall derive the public user id, private user id and home domain name from the IMSI for SIP registration |
| Groups relevant IXIT |  |  |  |  |
| px_MCPTT_Group_A_ID | charstring | "mcptt-group-A@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_A_Name | charstring | "mcptt-group-A-name" |  | A human readable Group name for the group |
| px_MCPTT_Group_A_Owner_Organization | charstring | "mcptt-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_A_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| px_MCPTT_Group_B_ID | charstring | "mcptt-group-B@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_B_Name | charstring | "mcptt-group-B-name" |  | A human readable Group name for the group |
| px_MCPTT_Group_B_Owner_Organization | charstring | "mcptt-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_B_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| px_MCPTT_Group_C_ID | charstring | "mcptt-group-C@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_C_Name | charstring | "mcptt-group-C-name" |  | A human readable Group name for the group |
| px_MCPTT_Group_C_Owner_Organization | charstring | "mcptt-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_C_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| px_MCPTT_Group_D_ID | charstring | "mcptt-group-D@mcptt-op.gov" |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_D_Name | charstring | "mcptt-group-D-name" |  | A human readable Group name for the group |
| px_MCPTT_Group_D_Owner_Organization | charstring | "mcptt-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_D_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| px_MCPTT_Group_T_ID | charstring | "mcptt-group-T@mcptt-op.gov" |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_T_Name | charstring | "mcptt-group-T-name" |  | A human readable Group name for the group |
| px_MCPTT_Group_T_Owner_Organization | charstring | "mcptt-op.gov" |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |
| px_MCPTT_Group_T_preferred_VCodec | charstring | "AMR-WB" |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |
| Miscellaneous IXIT |  |  |  |  |
| px_MCPTT_CertUri | charstring | “cert1.mcptt-op.gov” |  | The URI of the Certificate (this object). Ref. TS 33.179 [15] |
| px_MCPTT_IP_ConnectionAddressAll | charstring | "0.0.0.0" |  | The unicast IP address |
| px_MCPTT_IP_ConnectionAddressAudio | charstring | "0.0.0.0" |  | The media=audio plane control channel IP address. NOTE: Can be the same as the unicast IP address. |
| px_MCPTT_IP_ConnectionAddressApp | charstring | "0.0.0.0" |  | The media=application plane control channel IP address. NOTE: Can be the same as the unicast IP address. |
| px_MCPTT_ALL_APN | charstring | "mcptt-apn" |  | A single APN which the UE shall use to access each and all MCPTT relevant services including the MCPTT SIP-1 reference point, the MC common core services for the HTTP-1 reference point and the MC identity management service for the CSC-1 reference point. The APN is provided in the initial UE configuration as specified in TS 36.579-1 [2] Table 5.5.8.1-1. |
| px_MCX_InitialRegistration_TypeOfPDN1 | MCX_Registration_PDN_Type | mcx | ims, internet, mcx | First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’; ‘none’ is not applicable as first PDN) |
| px_MCX_InitialRegistration_TypeOfPDN2 | MCX_Registration_PDN_Type | none | ims, internet, mcx, none | Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration |
| px_MCX_InitialRegistration_TypeOfPDN3 | MCX_Registration_PDN_Type | none | ims, internet, mcx, none | Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration |
| NOTE 1: According to TS 23.280 [54] clause 8.1.2 a MC service ID shall be a URI; nevertheless in context of this specification only URIs consisting of userinfo, host and path elements (see RFC 3986 [53]) are supported. Editor's note: A similar note may need to be added to other PIXITs too |  |  |  |  |

### 9.2.2	MCX Server PIXIT

Table 9.2.2-1: MCX Server Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCPTT_PCSCF_A_URI | charstring | "mcptt-p-cscf-A@mcptt-op.gov" |  | The URI of the P-CSCF simulated by the SS. |
| px_MCPTT_GroupCreationXUI | charstring | "mcptt-gms@mcptt-op.gov" |  | Indicates the group creation XUI information for creation of groups. Ref. TS 23.003 [21]. |
| px_MCPTT_GroupConfigDoc_URI | charstring | “xcap.mcptt-op.gov/group_config.xml” |  | Points to the group configuration document. Ref. TS 24.481 [11]. |
| px_MCPTT_IDMSAuthEndpoint | charstring | "IDMSAuthEndpoint.mcptt-op.gov" |  | Identity management server authorisation endpoint identity information. Ref. TS 23.003 [21]. Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_IDMSTokenEndpoint | charstring | "IDMSTokenEndpoint.mcptt-op.gov" |  | Identity management server token endpoint identity information. Ref. TS 23.003 [21]. Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_GMS | charstring | "mcptt-gms.mcptt-op.gov" |  | Indicates the group management server identity information. Ref. TS 23.003 [21]. Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_CMS | charstring | "mcptt-cms.mcptt-op.gov" |  | Indicates the configuration management server identity information. Ref. TS 23.003 [21]. Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_KMS | charstring | "kms.mcptt-op.gov" |  | Indicates the key management server identity information. Ref. TS 23.003 [21]. Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_IdM_Server_URI | charstring | "IdM.server.com:9031" |  | Request-URI (AUID) for HTTP GET (IdM server) Editor's note: To be removed when not used in 36.579-1 anymore |
| px_MCPTT_XCAP_UE_Config_URI | charstring | “xcap.org.3gpp.mcptt.ue-config” |  | Request-URI (AUID) for HTTP GET (UE configuration) |
| px_MCPTT_XCAP_User_Profile_URI | charstring | “xcap.org.3gpp.mcptt.user-profile” |  | Request-URI (AUID) for HTTP GET (User Profile) |
| px_MCPTT_XCAP_Service_Config_URI | charstring | “xcap.org.3gpp.mcptt.service-config” |  | Request-URI (AUID) for HTTP GET (Service Configuration) |
| px_MCPTT_XCAP_Group_Config_URI | charstring | “xcap.org.3gpp.mcptt.group-config” |  | Request-URI (AUID) for HTTP GET (Group Configuration) |
| px_MCPTT_User_XUI_URI | charstring |  |  | "XUI-URI" attribute of the user profile document |
| px_MCX_IdMS_auth_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Port | integer |  |  | Port number of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS authorization endpoint |
| px_MCX_IdMS_token_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS token endpoint |
| px_MCX_IdMS_token_Port | integer |  |  | Port number of the IdMS token endpoint |
| px_MCX_IdMS_token_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS token endpoint |
| px_MCX_HTTP_Proxy_IPAddress | charstring |  |  | IPv4/IPv6 address of the HTTP proxy |
| px_MCX_HTTP_Proxy_Port | integer |  |  | Port number of the HTTP proxy |
| px_MCX_HTTP_Proxy_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the HTTP proxy |
| px_MCX_InitialConfigServer_IPAddress | charstring |  |  | IPv4/IPv6 address of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_Port | integer |  |  | Port number of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_UriPath | charstring | "/cms/initial-ue-config" |  | URI Path component: Absolute path used for HTTP requests addressing a server to download the UE initial configuration document |
| px_MCX_TLS_CipherSuite | TLS_CIPHER_Type | TLS_RSA_WITH_NULL_SHA |  | Cipher suite to be used for TLS connections |
| px_MCX_OAuth_ClientId_A | charstring |  |  | Client ID of the UE's MCX application as used in OAuth signalling with the IdMS |
| px_MCX_OAuth_RedirectURI_A | charstring |  |  | Redirect URI used by the UE implementation (user agent, MCX client and OS) to redirect the OAuth authentication response to the MCX client application |

### 9.2.3	MCX Constant PIXIT Definitions

Several parameters for MCX conformance testing can be defined as constants as they are neither preconfigured at the UE nor at the SS. Table 9.2.3-1 lists these constants.

Table 9.2.3-1: MCX Constant PIXIT Definitions

| Constant Name | Constant Type | Value | Description |
| --- | --- | --- | --- |
| tsc_MCX_KMS_Hostname | charstring | "kms." & px_MCX_DomainName_Organization_A | FQDN of the KMS; used in initial UE configuration as domain name for the 'kms' URI in the App-Server-Info. |
| tsc_MCX_CMS_Hostname | charstring | "cms." & px_MCX_DomainName_Organization_A | FQDN of the CMS; used in initial UE configuration as domain name for the 'cms' URI in the App-Server-Info. |
| tsc_MCX_GMS_Hostname | charstring | "gms." & px_MCX_DomainName_Organization_A | FQDN of the GMS; used in initial UE configuration as domain name for the 'gms' URI in the App-Server-Info. |
| tsc_MCX_KMS_CertUri | charstring | "certificate1." & tsc_MCX_KMS_Hostname | Name of the KMS certificate sent to the UE during MCX user authentication |
| tsc_MCX_IdMS_auth_UriPath | charstring | "/idms/auth" | URI path to address the IdMS authorization endpoint |
| tsc_MCX_IdMS_token_UriPath | charstring | "/idms/token" | URI path to address the IdMS token endpoint |
| tsc_MCX_IdMS_userauth_UriPath | charstring | "/idms/userauth" | relative URI at IdMS used for user authentication |
| tsc_MCX_KMS_init_UriPath | charstring | "/keymanagement/identity/v1/init" | Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 |
| tsc_MCX_KMS_keyprov_UriPath | charstring | "/keymanagement/identity/v1/keyprov" | Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 |
| tsc_MCX_KMS_ClientReqUrl_init | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath | used as <ClientReqUrl> entry in the KMS Key Set |
| tsc_MCX_KMS_ClientReqUrl | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath | used as <ClientReqUrl> entry in the KMS Key Set |
| tsc_MCX_GMSURI | charstring | "subscription-proxy." & tsc_MCX_GMS_Hostname | used for <GMS-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9; sub-domain of the GMS's hostname |
| tsc_MCX_CMSXCAPRootURI | charstring | "xcap." & tsc_MCX_CMS_Hostname | used for <CMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9C |
| tsc_MCX_GMSXCAPRootURI | charstring | "xcap." & tsc_MCX_GMS_Hostname | used for <GMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9B |
| tsc_MCX_MC_ID_User_A | charstring | px_MCX_User_A_username | According to TS 24.482 [12] clause 6.3.1 the MC ID set to the user name |
| tsc_MCPTT_PublicServiceId_A | charstring | "sip:" & "mcptt-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCPTT Server which is simulated by the SS |
| tsc_MCPTT_PublicServiceId_B | charstring | "sip:" & "mcptt-server-B@" & px_MCX_DomainName_Organization_A | The URI of a second MCPTT Server which is implemented in the DUT used in MCPTT Server testing. |
| tsc_MCX_SessionID_A | charstring | "sip:" & "sessionA@cf." & px_MCX_DomainName_Organization_A" | The URI of the MCPTT session A identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCX_SessionID_B | charstring | "sip:" & "sessionB@cf." & px_MCX_DomainName_Organization_A | The URI of the MCPTT session B identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCPTT_Group_A_ProSeLayer2GroupID | octetstring | '00000A'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCPTT_Group_D_ProSeLayer2GroupID | octetstring | '00000D'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |

## 9.3	MCX IPCAN test model

Table 9.3-1: MCX IPCAN test model specific PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCX_IPCAN_EpsBearerId_IMS | MCX_IPCAN_EpsBearerId_Type | 5 | 5..13 | Default EPS bearer for IMS PDN (if any) |
| px_MCX_IPCAN_EpsBearerId_INTERNET | MCX_IPCAN_EpsBearerId_Type | 6 | 5..13 | Default EPS bearer for Internet PDN (if any) |
| px_MCX_IPCAN_EpsBearerId_MCX | MCX_IPCAN_EpsBearerId_Type | 7 | 5..13 | Default EPS bearer for MCX PDN |
| px_MCX_IPCAN_EpsBearerId_MCX_Audio | MCX_IPCAN_EpsBearerId_Type | 8 | 5..13 | Dedicated EPS bearer for MCX audio |
| px_MCX_IPCAN_EpsBearerId_MCX_Video | MCX_IPCAN_EpsBearerId_Type | 9 | 5..13 | Dedicated EPS bearer for MCX video |

<End of modification>

<Start of modification>

Annex D (Normative):
SIP Type Definitions and XSD References

# D.1	XML Schema Definitions (XSD)

Common XML schema definitions according to TS 34.229-3 [28] Table G.0.1-1 are used. In addition there are the MCX specific XML schema definitions as according to table D.1-1.

Table D.1-1: MCX specific definitions

| XML Schema (XSD) | Source | Name space |
| --- | --- | --- |
| RFC4354-poc-settings | RFC 4354 [52] clause 6.1 | urn:oma:params:xml:ns:poc:poc-settings |
| RFC4661-SimpleFilter | RFC 4661 [34] clause 7 | urn:ietf:params:xml:ns:simple-filter |
| RFC5261-patch-ops (NOTE 2) | RFC 5261 [50] clause 8 | (NOTE 1) |
| RFC5874-xcap-diff | RFC 5874 [51] clause 4 | urn:ietf:params:xml:ns:xcap-diff |
| IANA-resource-lists | RFC 4826 [34] clause 3.2 https://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd | urn:ietf:params:xml:ns:resource-lists |
| poc_listService-v1_0 | OMA [37] | urn:oma:xml:poc:list-service |
| xdm_extensions-v1_0 | OMA [39] | urn:oma:xml:xdm:extensions |
| xdm_rsrclst_uriusage-v1_0 | OMA [40] | urn:oma:xml:xdm:resource-list:oma-uriusage |
| xenc-schema | W3C [41] | http://www.w3.org/2001/04/xmlenc#' |
| xmldsig-core-schema | W3C [42] | http://www.w3.org/2000/09/xmldsig# |
| TS24379_mcpttaff | TS 24.379 [9] Annex F.4.2 | urn:3gpp:ns:affiliationCommand:1.0 |
| TS24379_mcpttinfo | TS 24.379 [9] Annex F.1.2 | urn:3gpp:ns:mcpttInfo:1.0 |
| TS24379_mcpttloc | TS 24.379 [9] Annex F.3.2 | urn:3gpp:ns:mcpttLocationInfo:1.0 |
| TS24379_mcpttsigneddoc | TS 24.379 [9] Annex F.6.2 | urn:3gpp:ns:mcpttSignedDoc:1.0 |
| TS24379_pidf+xml-ext | TS 24.379 [9] Table 9.3.1.2-1 | urn:3gpp:ns:mcpttPresInfo:1.0 |
| TS24379_poc-settings+xml-ext | TS 24.379 [9] Table 7.4.1.2.2-2 | urn:3gpp:mcsSettings:1.0 |
| TS24481_mcptt-group | TS 24.481 [11] clause 7.2.4.2 | urn:3gpp:ns:mcpttGroupInfo:1.0 |
| TS24481-mcpttGMOP | TS 24.481 [11] clause 7.3.3 | urn:3gpp:ns:mcpttGMOP:1.0 |
| TS24481_mcpttGKTP | TS 24.481 [11] clause 7.7.4.2 | urn:3gpp:ns:mcpttGKTP:1.0 |
| TS24484_ue-init-config | TS 24.484 [14] clause 7.2.2.3 | urn:3gpp:mcptt:mcpttUEinitConfig:1.0 |
| TS24484_ue-config | TS 24.484 [14] clause 8.2.2.3 | urn:3gpp:mcptt:mcpttUEConfig:1.0 |
| TS24484_mcptt-user-profile | TS 24.484 [14] clause 8.3.2.3 | urn:3gpp:mcptt:user-profile:1.0 |
| TS24484_Servconf | TS 24.484 [14] clause 8.4.2.3 | urn:3gpp:ns:mcpttServiceConfig:1.0 |
| TS33180 mcsecKMSInterface | TS 33.180 [43] Annex D.3.5.1 | urn:3gpp:ns:mcsecKMSInterface:1.0 |
| TS33180 mcsecKMSKRR | TS 33.180 [43] Annex D.4.4 | urn:3gpp:ns:mcsecKMSKRR:1.0 |
| NOTE 1: The schema does not define any target namespace but is referred to as "urn:ietf:params:xml:schema:patch-ops" e.g. in the original include statemement in urn:ietf:params:xml:ns:xcap-diff. NOTE 2: The XML schema in RFC 5261 [50] clause 8 is specified as DTD with ENTITY declarations being referred by the pattern of the schema. These references are resolved in RFC5261-patch-ops in order to get schema definitions which can be processed by TTCN-3 compilers. |  |  |

In order to avoid ambiguities and unexpected side effects due to tool specific behaviour the schemaLocation in xs:import statements of XSD files are modified to use local references rather than references to internet locations.

The schemaLocations of imported namespaces are shown in table D.1-2. The schemaLocations of included namespaces are shown in table D.1-3.

Table D.1-2: schemaLocation of imported namespaces

| XML Schema | Imported name space | Original schemaLocation | Modified schemaLocation |
| --- | --- | --- | --- |
| RFC4354-poc-settings | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| RFC4661-SimpleFilter | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| IANA-resource-lists | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| poc_listService-v1_0 | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| poc_listService-v1_0 | urn:ietf:params:xml:ns:resource-lists | http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd | IANA-resource-lists.xsd (NOTE 2) |
| xdm_extensions-v1_0 | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/03/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| xenc-schema | http://www.w3.org/2000/09/xmldsig# | xmldsig-core-schema.xsd | (no change needed) |
| TS24379_mcpttsigneddoc | http://www.w3.org/2000/09/xmldsig# | http://www.w3.org/TR/xmldsig-core/xmldsig-core-schema.xsd | xmldsig-core-schema.xsd |
| TS24484-ue-init-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-ue-config | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcptt-user-profile | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| TS24484-mcptt-user-profile | urn:ietf:params:xml:ns:common-policy | http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd | ../../Common/IMS_XSD/RFC4745-common-policy.xsd (NOTE 1) |
| TS24484-Servconf | http://www.w3.org/XML/1998/namespace | http://www.w3.org/2001/xml.xsd | ../../Common/IMS_XSD/xml.xsd |
| NOTE 1: The namespace urn:ietf:params:xml:ns:common-policy is specified in RFC 4745 and at http://www.iana.org/assignments/xml-registry/schema/common-policy.xsd with difference in the minOccurs for identityType and validityType. Nevertheless for use in TTCN there shall be only one schema as part 9 of the TTCN-3 language [31] does not support different variants of one and the same namespace. In addition according to DIFF1 for RFC 4745 it seems that the RFC's variant is newer than the IANA's variant. NOTE 2: The namespace urn:ietf:params:xml:ns:resource-lists is specified in RFC 4826 and at http://www.iana.org/assignments/xml-registry/schema/resource-lists.xsd with the difference of attribute anchor not being mandatory in the RFC. |  |  |  |

Table D.1-3: schemaLocation of included namespaces

| XML Schema | Included schema | Original schemaLocation | Modified schemaLocation |
| --- | --- | --- | --- |
| RFC5874-xcap-diff | urn:ietf:params:xml:schema:patch-ops | urn:ietf:params:xml:schema:patch-ops | RFC5261-patch-ops.xsd |

# D.2	Common TTCN-3 Libraries

The same LibSip modules are used as according to TS 34.229-3 [28] annex G.0.2 and the same additional LibSip_MessageBodyTypes as according to TS 34.229-3 [28] annex G.1 and G.2

<End of modification>

<Start of modification>

Annex E (normative): 
TTCN-3 Definitions for the MCX IPCAN test model

# E.0	Introduction

The present Annex E specifies the TTCN-3 type definitions used at the system interface to configure and control the SS. The type definitions are specific for the MCX IPCAN test model.

In case of discrepancy between the content of the present Annex E and the equivalent TTCN-3 definitions / semantic requirements found in the TTCN modules provided as attachments to the present specification, the latter shall take precedence.

NOTE:	This annex is automatically generated from the TTCN-3 modules provided as attachment to the present specification and containing the listed TTCN-3 type definitions.

# E.1	MCX_IPCAN_ASP_TypeDefs

MCX_IPCAN_ASP_TypeDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MCX_IPCAN_AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm); same as AbsoluteCellPower_Type |

MCX_IPCAN_EpsBearerIdList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_EpsBearerIdList_Type |
| Comment |  |
| record of MCX_IPCAN_EpsBearerId_Type |  |

MCX_IPCAN_PacketFilterDirection_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterDirection_Type |
| Comment | packet filter direction according to TS 24.008 Table 10.5.162 |
| downlinkOnly |  |
| uplinkOnly |  |
| bidirectional |  |

MCX_IPCAN_PacketFilterComponentPortRange_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PacketFilterComponentPortRange_Type |  |  |
| Comment |  |  |  |
| Low | integer |  |  |
| High | integer |  |  |

MCX_IPCAN_PacketFilterComponent_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_PacketFilterComponent_Type |  |
| Comment | TS 24.008 Table 10.5.162 |  |
| Protocol | integer | Protocol identifier/Next header type: 17 for UDP |
| SingleRemotePort | integer |  |
| RemotePortRange | MCX_IPCAN_PacketFilterComponentPortRange_Type |  |

MCX_IPCAN_PacketFilterComponentList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterComponentList_Type |
| Comment |  |
| record of MCX_IPCAN_PacketFilterComponent_Type |  |

MCX_IPCAN_PacketFilter_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PacketFilter_Type |  |  |
| Comment |  |  |  |
| Direction | MCX_IPCAN_PacketFilterDirection_Type |  |  |
| ComponentList | MCX_IPCAN_PacketFilterComponentList_Type |  |  |

MCX_IPCAN_PacketFilterList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PacketFilterList_Type |
| Comment |  |
| record of MCX_IPCAN_PacketFilter_Type |  |

MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type |  |  |
| Comment |  |  |  |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  | EPS bearer id of the dedicated EPS bearer |
| PacketFilterList | MCX_IPCAN_PacketFilterList_Type |  |  |

MCX_IPCAN_IPv4IPv6Address_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_IPv4IPv6Address_Type |  |  |
| Comment |  |  |  |
| IPv4 | charstring |  |  |
| IPv6 | charstring |  |  |

MCX_IPCAN_PDN_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_PDN_Info_Type |  |  |
| Comment |  |  |  |
| PdnType | MCX_Registration_PDN_Type |  | MCX, IMS or INTERNET |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  | EPS bearer id of the PDN's default EPS bearer |
| IPAddressNW | MCX_IPCAN_IPv4IPv6Address_Type | opt | not present for MCX PDN as the IP signalling is handled by TTCN |
| IPAddressUE | MCX_IPCAN_IPv4IPv6Address_Type | opt | not present for MCX PDN as the IP signalling is handled by TTCN |

MCX_IPCAN_PDN_InfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MCX_IPCAN_PDN_InfoList_Type |
| Comment | list of PDNs to be connected to during Idle Update procedure |
| record length (1..3) of MCX_IPCAN_PDN_Info_Type |  |

MCX_IPCAN_ASP_CommonReqPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_ASP_CommonReqPart_Type |  |  |
| Comment |  |  |  |
| CellId | EUTRA_CellId_Type |  |  |
| CnfFlag | boolean |  |  |

MCX_IPCAN_Init_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_Init_Type |
| Comment | Initialisation of the system simulator. CellId : eutra_Cell_NonSpecific Reference implementation: f_EUTRA_Init with p_SysinfoCombination:=c1, p_HandleULDataUM:=HANDLE_UM_DATA, p_UseBigGrants:=USE_BIG_GRANTS |

MCX_IPCAN_CellConfig_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_CellConfig_Type |
| Comment | Create a cell and send out system information. CellId : cell id of the cell to be created Reference implementation: f_EUTRA_CellConfig_Def |

MCX_IPCAN_CellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_CellPower_Type |  |  |
| Comment | Set the cell power of a cell. CellId : cell id of the cell Reference implementation: f_EUTRA_SetCellPower |  |  |
| AbsoluteCellPower | MCX_IPCAN_AbsoluteCellPower_Type |  |  |

MCX_IPCAN_InitialRegistration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_InitialRegistration_Type |  |  |
| Comment | Perform initial registration procedure according to TS 36.579-1 clause 5.4.1A and Table 5.4.2.3-1 steps 2 - 16A to bring the UE into state 3 according to Figure 4.5.1-1 in TS 36.508. As described in clause 5.4.1A of TS 36.579-1 after switch on the UE may register to an internet PDN, an IMS PDN and the MCX PDN. NOTE: The UE does not need to register to MCX during initial registration after switch-on. CellId : cell id of the cell |  |  |
| PdnList | MCX_IPCAN_PDN_InfoList_Type |  | list of PDNs the UE is expected to connect to during Idle Update procedure; the list may or may not contain MCX |
| DefaultAPN | octetstring |  | APN to be used for the PDN (if any) for which the UE does not provide an APN; APN encoded according to 23.003 clause 9.1 and RFC 1035 clause 4.1.2 |

MCX_IPCAN_AdditionalRegistrationMCX_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_AdditionalRegistrationMCX_Type |  |  |
| Comment | Procedure used, if the UE is configured not to connect automatically to the MCX PDN after switch-on: In this case after initial registration the UE gets triggered from TTCN by an MMI command to start MCX registration and this procedure is started to perform steps 19 - 26 of TS 36.579-1 Table 5.4.2.3-1. CellId : cell id of the cell |  |  |
| EpsBearerId | MCX_IPCAN_EpsBearerId_Type |  |  |

MCX_IPCAN_RRCConnectionSetupMO_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_RRCConnectionSetupMO_Type |  |  |
| Comment | MO RRC connection establishment (TS 36.508 cl. 4.5.3.3 steps 3-9) for DedicatedEpsBearers == {}; for DedicatedEpsBearers != {} steps 2 - 8 of the Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session according to 36.579-1 clause 5.4.13 apply. CellId : cell id of the cell Reference implementation: f_EUTRA_ServiceRequestAndActivate_SRB2_DRB (for DedicatedEpsBearers == {}) |  |  |
| DedicatedEpsBearers | MCX_IPCAN_EpsBearerIdList_Type |  | list of dedicated EPS bearers for which the DRBs need to be established for a pre-established session |

MCX_IPCAN_RRCConnectionSetupMT_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_RRCConnectionSetupMT_Type |  |  |
| Comment | MT RRC connection establishment (TS 36.508 cl. 4.5.3.3 steps 1-9) for DedicatedEpsBearers == {}; for DedicatedEpsBearers != {} the Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session according to 36.579-1 clause 5.4.13 applies. CellId : cell id of the cell Reference implementation: f_EUTRA_RbEst_Def (for DedicatedEpsBearers == {}) |  |  |
| DedicatedEpsBearers | MCX_IPCAN_EpsBearerIdList_Type |  | list of dedicated EPS bearers for which the DRBs need to be established for a pre-established session |

MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type |  |  |
| Comment | Establish dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_EUTRA_ActivateDedicatedEpsBearer_Speech, f_EUTRA_ActivateDedicatedEpsBearer_SpeechVideo |  |  |
| EpsBearerSpeech | MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type |  |  |
| EpsBearerVideo | MCX_IPCAN_ASP_DedicatedEpsBearerInfo_Type | opt | present in case of video call |

MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type |  |  |
| Comment | Deactivate dedicated EPS bearer(s) for the MCX PDN CellId : cell id of the cell Reference implementation: f_EUTRA_DeactivateEPS_BearerContext |  |  |
| EpsBearerIdSpeech | MCX_IPCAN_EpsBearerId_Type | opt | may be not present if only the video bearer shall be released |
| EpsBearerIdVideo | MCX_IPCAN_EpsBearerId_Type | opt | present in case of video call |

MCX_IPCAN_RRCConnectionRelease_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_RRCConnectionRelease_Type |
| Comment | Relase RRC connection CellId : cell id of the cell Reference implementation: f_EUTRA_RRC_ConnectionRelease |

MCX_IPCAN_Release_Type

| TTCN-3 Record Type |  |
| --- | --- |
| Name | MCX_IPCAN_Release_Type |
| Comment | Release all configuration at the system simulator; CellId : eutra_Cell_NonSpecific |

MCX_IPCAN_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SystemRequest_Type |  |
| Comment |  |  |
| Init | MCX_IPCAN_Init_Type |  |
| CellConfig | MCX_IPCAN_CellConfig_Type |  |
| CellPower | MCX_IPCAN_CellPower_Type |  |
| InitialRegistration | MCX_IPCAN_InitialRegistration_Type |  |
| AdditionalRegistrationMCX | MCX_IPCAN_AdditionalRegistrationMCX_Type |  |
| RRCConnectionSetupMO | MCX_IPCAN_RRCConnectionSetupMO_Type |  |
| RRCConnectionSetupMT | MCX_IPCAN_RRCConnectionSetupMT_Type |  |
| ActivateDedicatedEpsBearerMCX | MCX_IPCAN_ActivateDedicatedEpsBearerMCX_Type |  |
| DeactivateDedicatedEpsBearerMCX | MCX_IPCAN_DeactivateDedicatedEpsBearerMCX_Type |  |
| RRCConnectionRelease | MCX_IPCAN_RRCConnectionRelease_Type |  |
| Release | MCX_IPCAN_Release_Type |  |

MCX_IPCAN_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | MCX_IPCAN_ASP_CommonReqPart_Type |  |  |
| Request | MCX_IPCAN_SystemRequest_Type |  |  |

MCX_IPCAN_InitConfirmation_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_InitConfirmation_Type |  |  |
| Comment |  |  |  |
| RanType | IPCAN_RAN_Type |  |  |

MCX_IPCAN_SystemConfirmation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SystemConfirmation_Type |  |
| Comment |  |  |
| Init | MCX_IPCAN_InitConfirmation_Type | Confirmation for Init request: carries the RAN Type |
| Other | Null_Type | Confirmation for any other request |
| Error | charstring | Error indication when the corresponding request causes error; the charstring may be used to provide information about the error |

MCX_IPCAN_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Confirmation | MCX_IPCAN_SystemConfirmation_Type |  |  |

MCX_IPCAN_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | MCX_IPCAN_SYSTEM_PORT |  |
| Comment |  |  |
| out | MCX_IPCAN_SYSTEM_CTRL_REQ |  |
| in | MCX_IPCAN_SYSTEM_CTRL_CNF |  |

# E.2	MCX_CommonDefs

MCX_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MCX_IPCAN_EpsBearerId_Type | integer (5..13) | EPS bearer id: the associated DRB Id shall always be EpsBearerId - 4 |

MCX_Registration_PDN_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | MCX_Registration_PDN_Type |
| Comment | Type definition for PIXITs px_MCX_InitialRegistration_TypeOfPDN1, px_MCX_InitialRegistration_TypeOfPDN2, px_MCX_InitialRegistration_TypeOfPDN3; (see TS 36.579-1 clause 5.4.1A) NOTE: 'ims', 'internet', 'mcx' and 'none' cannot be used as 'none' is a keyword; 'IMS', 'INTERNET', 'MCX' and 'NONE' cannot be used as 'IMS' is a port name of various PTCs |
| imsPDN |  |
| internetPDN |  |
| mcxPDN |  |
| noPDN |  |

# E.3	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |

EUTRA_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CellId_Type |
| Comment |  |
| eutra_Cell_NonSpecific |  |
| eutra_Cell1 |  |
| eutra_Cell2 |  |
| eutra_Cell3 |  |
| eutra_Cell4 |  |
| eutra_Cell6 |  |
| eutra_Cell10 |  |
| eutra_Cell11 |  |
| eutra_Cell12 |  |
| eutra_Cell13 |  |
| eutra_Cell14 |  |
| eutra_Cell23 |  |
| eutra_Cell28 |  |
| eutra_Cell29 |  |
| eutra_Cell30 |  |
| eutra_Cell31 |  |
| eutra_CellA |  |
| eutra_CellB |  |
| eutra_CellC |  |
| eutra_CellD |  |
| eutra_CellE |  |
| eutra_CellG |  |
| eutra_CellH |  |
| eutra_CellI |  |
| eutra_CellJ |  |
| eutra_CellK |  |
| eutra_CellL |  |
| eutra_CellM |  |

IPCAN_RAN_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IPCAN_RAN_Type |
| Comment | radio access network technology used by the IPCAN PTC |
| GERAN |  |
| UTRAN_FDD |  |
| UTRAN_TDD |  |
| EUTRA_FDD |  |
| EUTRA_TDD |  |
| WLAN |  |
| NR |  |

# E.4	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| MCX_IPCAN_ASP_TypeDefs | IpcanModel_IPCAN/MCX_IPCAN_ASP_TypeDefs.ttcn | Rev 30470 |
| MCX_CommonDefs | Common/MCX_CommonDefs.ttcn | Rev 30470 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 30456 |

<End of modification>