3GPP TSG-RAN5 Meeting #91-e	R5-212418r1

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2153 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 8.1.3.1.13-SS/PBCH block and CSI-RS based intra-frequency measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For T2 in Table 8.1.3.1.13.3.2-1, Mn = -90 – 12 = -102dbm, Mp = -106dbm, Hys = 4 * 0.5 = 2dbm, Off = 2 * 0.5 = 1dbm, -102 + 2 > -106 + 1. It is not aligned with Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off. Therefore, it is proposed to change the value of “SS/PBCH SSS EPRE” to -94. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the value of “SS/PBCH SSS EPRE” of T2 in Table 8.2.3.9.1.3.2-1 to -94. Typos corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test config will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: 3GU issue on coversheet is solved. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.3.1.13	Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR cell

8.1.3.1.13.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and measurement configured for SS/PBCH measurement reporting of intra-frequency on specified frequency}

ensure that {

when { SS/PBCH block sorting quantity is above absThreshSS-BlocksConsolidation for each beam of Neighbour Cell}

then { UE sends MeasurementReport message containing rsIndexResults with resultsSSB-Indexes}

}

(2)

with { UE in NR RRC_CONNECTED state and measurement configured for SS/PBCH measurement reporting of intra-frequency on specified frequency }

ensure that {

when { SS/PBCH block sorting quantity is below absThreshSS-BlocksConsolidation for one beam of Neighbour Cell and another beam(s) is above absThreshSS-BlocksConsolidation}

then { UE sends MeasurementReport message containing rsIndexResults with resultsSSB-Indexes includes RsIndex above absThreshSS-BlocksConsolidation and excludes RsIndex below absThreshSS-BlocksConsolidation }

}

(3)

with { UE in NR RRC_CONNECTED state and measurement configured for CSI-RS measurement reporting of intra frequency on specified frequency }

ensure that {

when { CSI-RS sorting quantity is above absThreshCSI-RS-Consolidation for each beam of Neighbour Cell}

then { UE sends MeasurementReport message containing rsIndexResults with results CSI-RS-Indexes}

}

(4)

with { UE in NR RRC_CONNECTED state and measurement configured for CSI-RS measurement reporting of intra frequency on specified frequency }

ensure that {

when { CSI-RS sorting quantity is below absThreshCSI-RS-Consolidation for one beam of Neighbour Cell and another beam(s) is above absThreshCSI-RS-Consolidation }

then { UE sends MeasurementReport message containing rsIndexResults with results CSI-RS-Indexes includes RsIndex above absThreshCSI-RS-Consolidation and excludes RsIndex below absThreshCSI-RS-Consolidation }

}

8.1.3.1.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.331, clauses 5.5.5.1 and 5.5.5.2]. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.5.5.1]

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the ServingCellId within measResultServingMOList to include for each NR serving cell that is configured with servingCellMO, if any;

1>	if there is at least one applicable neighbouring cell to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

4>	if the reportType is set to eventTriggered:

5>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

4>	for each cell that is included in the measResultNeighCells, include the physCellId;

4>	if the reportType is set to eventTriggered or periodical:

5>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	if the measObject associated with this measId concerns NR:

7>	if rsType in the associated reportConfig is set to ssb:

8>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	else if rsType in the associated reportConfig is set to csi-rs:

8>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

[TS 38.331, clause 5.5.5.1]

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the UE is configured with EN-DC:

2>	if SRB3 is configured:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>else:

3>	submit the MeasurementReport message via the EUTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else:

2>submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.5.5.2]

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity if available , otherwise RSRP as sorting quantity if available, otherwise RSRQ as sorting quantity if available, otherwise SINR as sorting quantity;

1>	if reportType is set to periodical:

2>	if a single reporting quantity is set to TRUE in reportQuantityRs-Indexes;

3>	consider the configured single quantity as the sorting quantity;

2>	else:

3>	if rsrp is set to TRUE;

4>	consider RSRP as the sorting quantity;

3>	else:

4>	consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRs-IndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurements is configured, include the SS/PBCH based measurement results for the quantities in reportQuantity-RsIndexes set to TRUE for each SS/PBCH blockindex;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurementsis configured, include the CSI-RS based measurement results for the quantities in reportQuantity-RsIndexes set to TRUE for each CSI-RS index.

8.1.3.1.13.3	Test description

8.1.3.1.13.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving Cell.

-	NR Cell 2 is the intra-frequency neighbour cell of NR Cell 1. NR Cell2 has two beams: SSB/CSI-RS index#0 and SSB/CSI-RS index#1

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1

8.1.3.1.13.3.2	Test procedure sequence

Table 8.1.3.1.13.3.2-1 and Table 8.1.3.1.13.3.2-1A illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.1.13.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 2 Beam 0 (NOTE) | NR Cell 2 Beam 1 (NOTE) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -94 | - | - | - |  |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -4 | -4 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -106 | - | - | - | Power level is such that SS/PBCH quality and CSI-RS quality for both beams of NR Cell 2 are above absThreshSS-BlocksConsolidation. |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -4 | -4 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | - | - | - | Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off Power level is such that SS/PBCH quality and CSI-RS quality for both beams of NR Cell 2 are below absThreshSS-BlocksConsolidation |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -12 | -12 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -106 | - | - | - | Power level is such that SS/PBCH quality and CSI-RS quality for beam 0 of NR Cell 2 is above absThreshSS-BlocksConsolidation and for beam 1 of NR Cell 2 is below absThreshSS-BlocksConsolidation. |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -4 | -12 |  |

Table 8.1.3.1.13.3.2-1A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 2 Beam 0 (NOTE) | NR Cell 2 Beam 1 (NOTE) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | - | - | - |  |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -1 | -1 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -100 | - | - | - | Power level is such that SS/PBCH quality and CSI-RS quality for both beams of NR Cell 2 are above absThreshSS-BlocksConsolidation. |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -1 | -1 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -100 | - | - | - | Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off Power level is such that SS/PBCH quality and CSI-RS quality for both beams of NR Cell 2 are below absThreshSS-BlocksConsolidation |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -10 | -10 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -100 | - | - | - | Power level is such that SS/PBCH quality and CSI-RS quality for beam 0 of NR Cell 2 is above absThreshSS-BlocksConsolidation and for beam 1 of NR Cell 2 is below absThreshSS-BlocksConsolidation. |
|  | Reference Power | dBm/SCS | - | -90 | - | - |  |
|  | SS/PBCH SSS EPRE, CSI-RS EPRE offset | dB | - | - | -1 | -10 |  |

Table 8.1.3.1.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an NR RRCReconfiguration message including measConfig to setup SS/PBCH block based intra- frequency NR measurement for NR Cell 1 and reporting for event A3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A3 with the measured results for NR Cell 2 with beam information containing RsIndex[0] and RsIndex[1]? | --> | NR RRC: MeasurementReport | 1 | P |
| 4A | The SS re-adjusts the cell-specific reference signal level according to the row "T2". | - | - | - | - |
| 4B | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 to report the event A3 during the next 10s? | - | - | 1 | F |
| 5 | The SS re-adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message to report event A3 with the measured results for NR Cell 2 with beam information containing RsIndex[0] and excludes RsIndex[1]? | --> | NR RRC: MeasurementReport | 2 | P |
| 7-8 | Void | - | - | - | - |
| 9 | The SS transmits an NR RRCReconfiguration including measConfig to setup CSI-RS based intra-frequency NR measurement for NR Cell 1 and reporting for event A3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCConfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 12 | Check: Does the UE transmit a MeasurementReport message to report event A3 with the measured results for NR Cell 2 with beam information containing RsIndex[0] and RsIndex[1]? | --> | NR RRC: MeasurementReport | 3 | P |
| 12A | The SS re-adjusts the cell-specific reference signal level according to the row "T2". | - | - | - | - |
| 12B | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 to report the event A3 during the next 10s? | - | - | 1 | F |
| 13 | The SS re-adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 14 | Check: Does the UE transmit a MeasurementReport message to report event A3 with the measured results for NR Cell 2 with beam information containing RsIndex[0] and excludes RsIndex[1]? | --> | NR RRC: MeasurementReport | 4 | P |

8.1.3.1.13.3.3	Specific message contents

Table 8.1.3.1.13.3.3-1: RRCReconfiguration (steps 1, 9)

Derivation Path: 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.1.3.1.13.3.3-2: MeasConfig (Table 8.1.3.1.13.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| measObjectToAddModList | IdMeasObjectToAdd |  | Step 1 |
|  | Not present |  | Step 9 |
| reportConfigToAddModList | IdReportConfigToAdd |  | Step 1 |
|  | Not present |  | Step 9 |
| } |  |  |  |

Table 8.1.3.1.13.3.3-3: IdMeasObjectToAdd (Table 8.1.3.1.13.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | Id-MeasObjectNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.13.3.3-4: Id-MeasObjectNR (Table 8.1.3.1.13.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | Downlink ARFCN of NR Cell1 |  |  |
| refFreqCSI-RS | Same as “Point A” defined for the downlink of NR Cell1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| shortBitmap | 1100 |  | (FREQ<=3GHz AND (FR1_FDD OR NOT CASE_C)) OR (FREQ<=2.4GHz AND FR1_TDD) |
| mediumBitmap | 11000000 |  | (FREQ>3GHz AND FR1) OR (FREQ>2.4GHz AND FR1_TDD AND CASE_C) |
| longBitmap | 11000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-rs-ResourceConfigMobility CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| csi-RS-CellList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility { | 2 entries |  |  |
| CSI-RS-CellMobility[1] SEQUENCE { |  | entry 1 |  |
| cellId | Physical CellID of the NR Cell 1 |  |  |
| csi-rs-MeasurementBW SEQUENCE { |  |  |  |
| nrofPRBs | size24 |  |  |
| startPRB | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| } |  |  |  |
| density | d3 |  |  |
| csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility { | 1 entry |  |  |
| RS-Resource-Mobility[1] SEQUENCE { |  | entry 1 |  |
| csi-RS-Index | 0 |  |  |
| slotConfig CHOICE { |  |  |  |
| ms20 | 0 | To avoid collision with CSI-RS of NR Cell 2 |  |
| } |  |  |  |
| associatedSSB | Not present |  |  |
| associatedSSB SEQUENCE { |  |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| ssb-Index | 0 |  |  |
| isQuasiColocated | false |  |  |
| } |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 |  |  |
| sequenceGenerationConfig | 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-RS-CellMobility[2] SEQUENCE { |  | entry 2 |  |
| cellId | Physical CellID of the NR Cell 2 |  |  |
| csi-rs-MeasurementBW SEQUENCE { |  |  |  |
| nrofPRBs | size24 |  |  |
| startPRB | same value as ‘offsetToCarrier’ as defined for the DL frequency of the cell |  |  |
| } |  |  |  |
| density | d3 |  |  |
| csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility { | 2 entries |  |  |
| RS-Resource-Mobility[1] SEQUENCE { |  | entry 1 |  |
| csi-RS-Index | 0 | CSI-RS with Index 0 |  |
| slotConfig CHOICE { |  |  |  |
| ms20 | 10 * 2µ | µ is numerology index (SCS=15kHz: 0, SCS=30kHz: 1, etc.) |  |
| } |  |  |  |
| associatedSSB | Not present |  |  |
| associatedSSB SEQUENCE { |  |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| ssb-Index | 0 |  |  |
| isQuasiColocated | false |  |  |
| } |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 1000 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 4 |  |  |
| sequenceGenerationConfig | 24 |  |  |
| } |  |  |  |
| RS-Resource-Mobility[2] SEQUENCE { |  | entry 2 |  |
| csi-RS-Index | 1 | CSI-RS with Index 1 |  |
| slotConfig CHOICE { |  |  |  |
| ms20 | 10 * 2µ | µ: numerology index (SCS=15kHz: 0, SCS=30kHz: 1, etc.) |  |
| } |  |  |  |
| associatedSSB | Not present |  |  |
| associatedSSB SEQUENCE { |  |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| ssb-Index | 1 |  |  |
| isQuasiColocated | false |  |  |
| } |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 0 |  |  |
| sequenceGenerationConfig | 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | 57 | corresponding to -100dBm | FR1 |
|  | 66 +Delta(NRf1) | corresponding to -91dBm | FR2 |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: delta(NRf1) is derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 on NR Cell1 |  |  |  |

Table 8.1.3.1.13.3.3-5: IdReportConfigToAdd (Table 8.1.3.1.13.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigToAddModList::= SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | IdReportConfig-A3-SSB |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | IdReportConfig-A3-CSI-RS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.13.3.3-6: IdReportConfig-A3-SSB (Table 8.1.3.1.13.3.3-5)

| Derivation Path: 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 |  |  |
|  | 2+ Delta(NRf2) - Delta(NRf1) | a3-Offset value set to (2+ Delta(NRf2) - Delta(NRf1)). (Note 1) | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 2 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Void Note 2: Delta(NRf2) and Delta(NRf1) are derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 where Delta(NRf1) is derived for the carrier of NR PCell and Delta(NRf2) is derived for the carrier of neighbour cell |  |  |  |

Table 8.1.3.1.13.3.3-6A: IdReportConfig-A3-CSI-RS (Table 8.1.3.1.13.3.3-5)

| Derivation Path: 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 |  |  |
|  | 2+ Delta(NRf2) - Delta(NRf1) | a3-Offset value set to (2+ Delta(NRf2) - Delta(NRf1)). (Note 1) | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | csi-rs |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| sinr | false |  |  |
| } |  |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 2 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Delta(NRf2) and Delta(NRf1) are derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 where Delta(NRf1) is derived for the carrier of NR PCell and Delta(NRf2) is derived for the carrier of neighbour cell |  |  |  |

Table 8.1.3.1.13.3.3-7: IdMeasIdToAdd (Table 8.1.3.1.13.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod |  |  |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  | Step 1 |
|  | 2 |  | Step 9 |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.13.3.3-8: MeasurementReport (step 4, Table 8.1.3.1.13.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-9: MeasResults (Table 8.1.3.1.13.3.3-8)

| Derivation Path: 38.508-1 [4], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |  |
| measId |  | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { |  | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  |  | entry 1 |  |
| servCellId |  | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |  |
| physCellId |  | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE{ |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | pc_ss_SINR_Meas |
|  |  | Not present |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |
| ResultsPerSSB-Index[1] EQUENCE { |  |  | entry 1 |  |
| ssb-Index |  | Not checked |  |  |
| ssb-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Indexes |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { |  | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  |  | entry 1 |  |
| physCellId |  | Physical CellID of the NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | pc_ss_SINR_Meas |
|  |  | Not present |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Cell |  | Not present |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 2 entries that may appear in an arbitrary order |  |  |
| ResultsPerSSB-Index [1] SEQUENCE { |  |  | entry 1 |  |
| ssb-Index |  | SSB index 0 |  |  |
| ssb-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| ResultsPerSSB-Index [2] SEQUENCE { |  |  | entry 2 |  |
| ssb-Index |  | SSB index 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Indexes |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-10: MeasurementReport (step 6, Table 8.1.3.1.13.3.2-2)

| Derivation Path: 38.508-1 [4], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-11: MeasResults (Table 8.1.3.1.13.3.3-10)

| Derivation Path: 38.508-1 [4], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |  |
| measId |  | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { |  | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  |  | entry 1 |  |
| servCellId |  | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |  |
| physCellId |  | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE{ |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | pc_ss_SINR_Meas |
|  |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  |  | entry 1 |  |
| ssb-Index |  | Not checked |  |  |
| ssb-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Indexes |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { |  | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  |  | entry 1 |  |
| physCellId |  | Physical CellID of the NR Cell2 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | pc_ss_SINR_Meas |
|  |  | Not present |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Cell |  | Not present |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  |  | entry 1 |  |
| ssb-Index[1] |  | SSB index 0 |  |  |
| ssb-Results[1] SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| resultsCSI-RS-Indexes |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-12: Void

Table 8.1.3.1.13.3.3-13: Void

Table 8.1.3.1.13.3.3-14: Void

Table 8.1.3.1.13.3.3-15: Void

Table 8.1.3.1.13.3.3-16: Void

Table 8.1.3.1.13.3.3-17: Void

Table 8.1.3.1.13.3.3-18: MeasurementReport (step 12, Table 8.1.3.1.13.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-19: MeasResults (Table 8.1.3.1.13.3.3-18)

| Derivation Path: 38.508-1 [4], Table 4.6.3-79 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |  |  |
| measId |  | 1 |  |  |  |
| measResultServingMOList SEQUENCE {(SIZE (1..maxNrofServingCells)) OF MeasResultServMO { |  | 1 entry |  |  |  |
| MeasResultServMO[1] SEQUENCE { |  |  |  | entry 1 |  |
| servCellId |  | ServCellIndex of NR Cell 1 |  |  |  |
| measResultServingCell SEQUENCE { |  |  |  |  |  |
| physCellId |  | Physical CellID of the NR Cell 1 |  |  |  |
| measResult SEQUENCE { |  |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |  |
| resultsSSB-Cell |  | Not present |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |  |  |
| rsrp |  | (0..127) |  |  |  |
| rsrq |  | (0..127) |  |  |  |
| sinr |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |  |
| resultsSSB-Indexes |  | Not present |  |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  |  | entry 1 |  |  |
| csi-RS-Index |  | Not checked |  |  |  |
| csi-RS-Results SEQUENCE { |  |  |  |  |  |
| rsrp |  | (0..127) |  |  |  |
| rsrq |  | (0..127) |  |  |  |
| sinr |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |  |
| measResultListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { |  | 1 entry |  |  |  |
| MeasResultNR[1] SEQUENCE { |  |  |  | entry 1 |  |
| physCellId |  | Physical CellID of the NR Cell 2 |  |  |  |
| measResult SEQUENCE { |  |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |  |
| resultsSSB-Cell |  | Not present |  |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |  |  |
| rsrp |  | (0..127) |  |  |  |
| rsrq |  | (0..127) |  |  |  |
| sinr |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |  |
| resultsSSB-Indexes |  | Not Present |  |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index { |  | 2 entries that may appear in an arbitrary order |  |  |  |
| ResultsPerCSI-RS-Index[1] SEQUENCE { |  |  |  | entry 1 |  |
| csi-RS-Index |  | CSI-RS-index 0 |  |  |  |
| csi-RS-Results SEQUENCE { |  |  |  |  |  |
| rsrp |  | Not checked |  |  |  |
| rsrq |  | Not present |  |  |  |
| sinr |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| ResultsPerCSI-RS-Index[2] SEQUENCE { |  |  |  | entry 2 |  |
| csi-RS-Index |  | CSI-RS-index 1 |  |  |  |
| csi-RS-Results SEQUENCE { |  |  |  |  |  |
| rsrp |  | Not checked |  |  |  |
| rsrq |  | Not present |  |  |  |
| sinr |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| cgi-Info |  | Not present |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 8.1.3.1.13.3.3-20: MeasurementReport (step 14, Table 8.1.3.1.13.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.13.3.3-21: MeasResults (Table 8.1.3.1.13.3.3-20)

| Derivation Path: 38.508-1 [4], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |  |
| measId |  | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { |  | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  |  | entry 1 |  |
| servCellId |  | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |  |
| physCellId |  | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE{ |  |  |  |  |
| resultsSSB-Cell |  | Not present |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes |  | Not present |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  |  | entry 1 |  |
| csi-RS-Index |  | Not checked |  |  |
| csi-RS-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { |  | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  |  | entry 1 |  |
| physCellId |  | Physical CellID of the NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell |  | Not present |  |  |
| resultsCSI-RS-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |  |
| resultsSSB-Indexes |  | Not Present |  |  |
| resultsCSI-RS-Indexes SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { |  | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  |  | entry 1 |  |
| csi-RS-Index |  | CSI-RS-Index 0 |  |  |
| csi-RS-Results SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | Not present |  |  |
| sinr |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 1>