3GPP TSG-RAN5 Meeting #91-e	R5-212451r1

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0036 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ProSe Packet Counter value reporting procedure for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. To add NR Sidelink Packet Counter value reporting procedure for NR sidelink 2. To add message definitions of UE TEST LOOP NR SIDELINK PACKET COUNTER REQUEST and UE TEST LOOP NR SIDELINK PACKET COUNTER RESPONSE. 3. To correct typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. NR Sidelink Packet Counter value reporting procedure for NR SL tests are added. 2. Message definitions of UE TEST LOOP NR SIDELINK PACKET COUNTER REQUEST and UE TEST LOOP NR SIDELINK PACKET COUNTER RESPONSE are added. 3. Typos are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3, 5.9(new), 6, 6.9(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: CR impact tick box for ME is set Wrong clause number is corrected. Names of the newly added TC messages are changed. Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

5.8	Set UAI test function

5.8.1	General

The SS uses the Set UAI test procedure to trigger a change of preference in the UE that will cause the indication of UE Assistance Information (UAI) by the UE, see figure 5.8.1-1. The different types of information that can be sent in a SET UAI REQUEST is preferred RRC state, see clause 6.8.

Figure 5.8.1-1: Set UAI test procedure

5.8.2	Initiation

The SS requests the UE to set the preferred RRC state of UE Assistance Information by transmitting a SET UAI REQUEST message and the UE confirms by responding with a SET UAI RESPONSE message.

### 5.8.3	Reception of SET UAI REQUEST message by UE

When UE receives SET UAI REQUEST message then the UE shall:

- if the UE is operating in RRC_CONNECTED state:

2>	set its preferred RRC state within the release preference, to the equivalent value as received in the Preferred RRC State of the SET UAI REQUEST message.

2>	transmit SET UAI RESPONSE message.

1>	else:

2>	the UE behaviour is unspecified.

5.9	NR Sidelink Packet Counter reporting procedure

5.9.1	Request NR Sidelink Packet Counter value

Figure 5.9.1-1: NR Sidelink Packet Counter reporting procedure based on TC protocol messages

Figure 5.9.1-2: NR Sidelink Packet Counter reporting procedure based on AT Commands

5.9.1.1	General

The SS uses the NR Sidelink Packet Counter reporting procedure to request reporting of current NR sidelink related state variables. For mode E the state variables STCH_PACKET_COUNTER, PSCCH_PACKET_COUNTER and PSSCH_PACKET_COUNTER of NR sidelink are reported.

5.9.1.2	Initiation

The SS requests the UE to report its current value of STCH_PACKET_COUNTER, PSCCH_PACKET_COUNTER and PSSCH_PACKET_COUNTER of NR sidelink for test loop mode E by transmitting an UE TEST LOOP NR SIDELINK PACKET COUNTER REQUEST message or by transmitting an AT Command +CUSPCREQ.

5.9.1.3	Reception of UE TEST LOOP NR SIDELINK COUNTER REQUEST by the UE

Upon receiving the UE TEST LOOP NR SIDELINK PACKET COUNTER REQUEST message the UE shall:

1>	if TEST_LOOP_MODE_E_ACTIVE is TRUE:

2>	set the IE NR Sidelink PSCCH Packet Counter(s) Value in the UE TEST LOOP PROSE COUNTER RESPONSE message to the value of the state variable(s) PSCCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	set the IE NR Sidelink STCH Packet Counter(s) Value in the UE TEST LOOP PROSE COUNTER RESPONSE message to the value of the state variable STCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	set the IE NR Sidelink PSSCH Packet Counter(s) Value in the UE TEST LOOP PROSE COUNTER RESPONSE message to the value of the state variable(s) PSSCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	send the UE TEST LOOP NR SIDELINK COUNTER RESPONSE message.

- else:

2>	the UE behaviour is unspecified.

5.8.1.4	Reception of AT Command +CUSPCREQ by the UE

Upon receiving the AT Command +CUSPCREQ the UE shall:

1>	if TEST_LOOP_MODE_E_ACTIVE is TRUE:

2>	in the response message of command +CUSPCREQ, set the parameter <type1> to 2; set the parameter <format> to 1; set the parameter <length1> to 4*(PROSE_COMMUNICATION_MONITOR_N+1); set the parameter <counter1> to the value of the state variable(s) PSCCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	in the response message of command +CUSPCREQ, set the parameter <type2> to 2; set the parameter <format> to 1; set the parameter <length2> to 4*(PROSE_COMMUNICATION_MONITOR_N+1); set the parameter <counter2> to the value of the state variable STCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	in the response message of command +CUSPCREQ, set the parameter <type3> to 2; set the parameter <format> to 1; set the parameter <length3> to 4*PROSE_COMMUNICATION_MONITOR_N+1; set the parameter <counter3> to the value of the state variable(s) PSSCH_PACKET_COUNTER(SL_ID) with SL_ID = 0…PROSE_COMMUNICATION_MONITOR_N;

2>	send the response message +CUSPCREQ: [<type1>,<format>,<length1>,<counter1>], [<type2>,<format>,<length2>,<counter2>], [<type3>,<format>,<length3>,<counter3>]..

1>	else:

2>	the UE behaviour is unspecified.

6	Test Mode Control message definitions

6.1	General

<End of modified section 1>

<Start of modified section 2>

6.8	SET UAI REQUEST

6.8.1	SET UAI REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| Preferred RRC State |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | octet 1 |

where Preferred RRC State is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  | E1 | E0 | octet 2 |

E1=0 and E0=0; idle.

E1=0 and E0=1; inactive.

E1=1 and E0=0: connected.

E1=1 and E0=1: outOfConnected.

6.8.2	SET UAI RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | octet 1 |

6.9	NR SIDELINK PACKET COUNTER reporting messages

6.9.1	UE TEST LOOP NR SIDELINK PACKET COUNTER REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | octet 1 |

6.9.2	UE TEST LOOP NR SIDELINK PACKET COUNTER RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| NR Sidelink PSCCH Packet Counter(s) Value |  | CV-ModeE | TLV | 4*(NC+1)+2 |
| NR Sidelink STCH Packet Counter(s) Value |  | CV-ModeE | TLV | 4*(NC+1)+2 |
| NR Sidelink PSSCH Packet Counter(s) Value |  | CV-ModeE | TLV | 4*(NC+1)+2 |

| Condition | Explanation |
| --- | --- |
| CV-ModeE | This IE is mandatory present if TEST_LOOP_MODE_E_ACTIVE is TRUE else it shall be absent. NC = PROSE_COMMUNICATION_MONITOR_N. |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | octet 1 |

And where NR Sidelink PSCCH Packet Counter(s) Value is:

| 8 7 6 5 4 3 2 1 |  |
| --- | --- |
| NR Sidelink PSCCH Packet Counter(s) Value type | octet 1 |
| Length of NR Sidelink PSCCH Packet Counter(s) Value contents in bytes | octet 2 |
| NR Sidelink PSCCH Packet Counter IE #0 | octet 3 octet 4 octet 5 octet 6 |
| … |  |
| NR Sidelink PSCCH Packet Counter IE #NC | octet 3+4 * NC octet 4+4 * NC octet 5+4 * NC octet 6+4 * NC |

And where NR Sidelink PSCCH Packet Counter(s) Value type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | octet 1 |

And where NR Sidelink PSCCH Packet Counter IE #n is:

| D31 | D30 | D29 | D28 | D27 | D26 | D25 | D24 | octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D23 | D22 | D21 | D20 | D19 | D18 | D17 | D16 | octet 2 |
| D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | octet 3 |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | octet 4 |

D31..D0 = PSCCH packet counter 0.. 4294967295 (binary coded, D31 is most significant bit and D0 least significant bit) corresponding to PSCCH_PACKET_COUNTER(SL_ID = n).

And where NR Sidelink STCH Packet Counter(s) Value is:

| 8 7 6 5 4 3 2 1 |  |
| --- | --- |
| NR Sidelink STCH Packet Counter(s) Value type | octet 1 |
| Length of NR Sidelink STCH Packet Counter(s) Value in bytes | octet 2 |
| NR Sidelink STCH Packet Counter IE #0 | octet 3 octet 4 octet 5 octet 6 |
| … |  |
| NR Sidelink STCH Packet Counter IE #NC | octet 3+4 * NC octet 4+4 * NC octet 5+4 * NC octet 6+4 * NC |

where NR Sidelink STCH Packet Counter(s) Value type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | octet 1 |

And where NR Sidelink STCH Packet Counter IE #n is:

| E31 | E30 | E29 | E28 | E27 | E26 | E25 | E24 | octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E23 | E22 | E21 | E20 | E19 | E18 | E17 | E16 | octet 2 |
| E15 | E14 | E13 | E12 | E11 | E10 | E9 | E8 | octet 3 |
| E7 | E6 | E5 | E4 | E3 | E2 | E1 | E0 | octet 4 |

E31..E0 = STCH Packet Counter value 0.. 4294967295 (binary coded, E31 is most significant bit and E0 least significant bit) corresponding to STCH_PACKET_COUNTER(SL_ID = n).

And where NR Sidelink PSSCH Packet Counter(s) Value is:

| 8 7 6 5 4 3 2 1 |  |
| --- | --- |
| NR Sidelink PSSCH Packet Counter(s) Value type | octet 1 |
| Length of NR Sidelink PSSCH Packet Counter(s) Value contents in bytes | octet 2 |
| NR Sidelink PSSCH Packet Counter IE #0 | octet 3 octet 4 octet 5 octet 6 |
| … |  |
| NR Sidelink PSSCH Packet Counter IE #NC | octet 3+4 * NC octet 4+4 * NC octet 5+4 * N octet 6+4 * NC |

And where NR Sidelink PSSCH Packet Counter(s) Value type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | octet 1 |

And where NR Sidelink PSSCH Packet Counter IE #n is:

| F31 | F30 | F29 | F28 | F27 | F26 | F25 | F24 | octet 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F23 | F22 | F21 | F20 | F19 | F18 | F17 | F16 | octet 2 |
| F15 | F14 | F13 | F12 | F11 | F10 | F9 | F8 | octet 3 |
| F7 | F6 | F5 | F4 | F3 | F2 | F1 | F0 | octet 4 |

F31..F0 = PSSCH packet counter 0.. 4294967295 (binary coded, F31 is most significant bit and F0 least significant bit) corresponding to PSSCH_PACKET_COUNTER(SL_ID = n).

<End of modified section 2>