3GPP TSG-RAN5 Meeting #91-e	R5-212610r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1858 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IE EAP message in NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message is still set to FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Add the content of EAP message IE in Table 4.7.1-33 NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message 2.Add the corresponding Conditions 3.Remove FFS in Table 4.7.1-33 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message content for NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: correct spec version and release on coversheet |  |  |  |  |  |  |  |  |

4.7.1	Contents of 5GMM messages

<Unchanged Sections Skipped >

<Start of modified section>

–	Network slice-specific authentication result

Table 4.7.1-33: NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.2.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message identity |  | ‘0101 0010’B |  |  |
| S-NSSAI |  | Set according to specific message content |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-3 | NSSAA Success |
|  |  | EAP-Failure | See Table 4.7.3.2-6 | NSSAA Failure |

| Condition | Explanation |
| --- | --- |
| NSSAA Success | Network slice-specific authentication succeeds |
| NSSAA Failure | Network slice-specific authentication fails |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

<End of modified section>