3GPP TSG-RAN5 Meeting #91-e	R5-212638r2

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0232 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update and add Test Tolerance analysis for FR2 Intra-freq Event-trig Test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Test tolerance analysis for FR2 Spherical Coverage Intra-frequency event triggered reporting test cases 5.6.1.1, 5.6.1.3, 7.6.1.1 and 7.6.1.3 are missing. The existing draft Test tolerance analysis for FR2 Rx Beam Peak Intra-frequency event triggered reporting test cases 5.6.1.2, 5.6.1.4, 7.6.1.2 and 7.6.1.4 use a tentative Downlink absolute signal level uncertainty of ±6dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new TT analysis for FR2 Spherical Coverage Intra-frequency event triggered reporting test cases 5.6.1.1, 5.6.1.3, 7.6.1.1 and 7.6.1.3. Update the TT analysis for the Rx Beam Peak Intra-frequency event triggered reporting test cases 5.6.1.2, 5.6.1.4, 7.6.1.2 and 7.6.1.4, based on the now-accepted downlink absolute signal level uncertainty value of ±5.65dB, and update the TT offset values to align. Add the attached zip files to TR 38.903: - ‘’38.533 5.6.1.1+5.6.1.3+7.6.1.1+7.6.1.3 TT’’ - ‘’38.533 5.6.1.2+5.6.1.4+7.6.1.2+7.6.1.4 TT’’ Remove “38.533 5.6.1.2+5.6.1.4+7.6.1.2+7.6.1.4 TT draft” from TR 38.903. Remove text relating to 38.533 5.6.1.2+5.6.1.4+7.6.1.2+7.6.1.4 TT in A.3. Update the TT analysis zip files to ‘’38.533 5.7.1.2+7.7.1.2 TT” in TR 38.903. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analyses to derive the Test Tolerance for TS 38.533 Intra-frequency event triggered reporting test cases would be missing or incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, add and replace zip files in TR 38.903 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | There is a dependency on R4-2108886 which has been included in the 38.133 extract, updating parameters for the Spherical Coverage Intra-freq event triggered reporting test cases 5.6.1.1, 5.6.1.3, 7.6.1.1 and 7.6.1.3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in r1: - Revert to original 2AoA for Test cases 5.6.1.1, 5.6.1.3, 7.6.1.1 and 7.6.1.3 - Use OP.5 on AoA1 only, to avoid unpredictable interference to AoA2 - Change A3 offset to -27dB to allow spherical coverage variations - Change SSB configuration on AoA2, and specify time-multiplexing - Specify Es only (no Noc) to avoid unpredicable interference Changes in r2: - Resolve parameters for Test cases 5.6.1.1, 5.6.1.3, 7.6.1.1 and 7.6.1.3: - Change A3 offset to -11dB to be within allowed TS 38.331 range - Add OffsetMO value 16dB to allow spherical coverage variations - Remove redundant coversheet text relating to other test cases |  |  |  |  |  |  |  |  |

## << Start of changes >>

# A.3	Test Tolerance analysis templates for radiated test cases awaiting completion

Test Tolerance analyses for Radiated testing listed below are not yet complete, but contain the main features for the test cases covered and can be used as templates. For each analysis, the missing aspects are listed.

The analysis documents (and spreadsheets where applicable) are included in the present document as zip files with “draft” at the end of the filename. When the test case analyses are complete, the draft versions and listing in this clause should be removed.

38.533 5.3.2.2.1+5.3.2.2.2+7.3.2.2.1+7.3.2.2.2 TT draft

Editor’s note: This test tolerance analysis is incomplete. The following aspects are missing:

- Settable window for first preamble uplink power and the uplink calibration process

- Derivation of test requirement for absolute uplink power after uplink calibration process

- Derivation of test requirement for relative uplink power

- The uncertainty value and test requirement for PRACH timing are in [ ] and not yet finalised

- The results of the TT analysis are provisional until the corresponding MU values are agreed

38.533 5.7.1.2+7.7.1.2 TT draft

Editor’s note: This test tolerance analysis is incomplete. The following aspects are missing:

- The test system measurement uncertainties for Noc absolute levels are not yet agreed

- The Test Tolerance final values depend on the Noc absolute level uncertainty

### << Unchanged sections skipped >>

## Add the following attached .zip files to TR 36.903:

## “38.533 5.6.1.1+5.6.1.3+7.6.1.1+7.6.1.3 TT”

“38.533 5.6.1.2+5.6.1.4+7.6.1.2+7.6.1.4 TT”

## Please remove the existing .zip file:

## “38.533 5.6.1.2+5.6.1.4+7.6.1.2+7.6.1.4 TT draft”

## << End of changes >>