Title:	Discussion on test point selection for NS_06 A-MPR in FR1

Document for:	Discussion and approval

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

This discussion paper is very similar to the TP analysis of NS_35 version 2 [6], except for MIMO, which is FFS for NS_06. Only operating bands and channel bandwidths differentiate between NS_35 and NS_06. Which means that the proposals approved for NS_35 should be applicable to NS_06.

Both NS_06 and NS_35 should be used for A-MPR and A-SEM testing, where A-MPR is defined as N/A, which means that the MPR requirement should be used instead of A-MPR.

# Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] and for FR1. In general for A-MPR the emissions minimum requirements are SEM and/or Spurious. Specifically, for NS_06 only SEM requirements apply and no A-MPR is allowed.

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

## Test Frequencies

The selection of test frequencies can be based on the agreed proposals for SEM [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

## Test Channel Bandwidth

Minimum requirements for NS_06 applies for all channel bandwidths. Select channel bandwidth to test as for SEM [2]

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1.

## Uplink RB allocation

Since there is no additional MPR for NS_06, the selection of UL RB allocation can be based on the agreed proposals for SEM [2].

PROPOSAL 6: For NS_06 A-MPR base the uplink RB allocation on the agreed proposal for SEM.

2.7.	Number of Test Points

The number of test points for NS_06 AMPR is equal to the number of test points (144) in the agreed proposals for SEM [4].

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_06 A-MPR in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1

PROPOSAL 6: For NS_06 A-MPR base the uplink RB allocation on the agreed proposal for SEM.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- R5-186256, “Update test points analysis for multiple FR1 test cases”, (Huawei, HiSilicon, Keysight, CAICT)

- TS 38.101-1 v15.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel-15)

- R5-194169, “Discussion on test point selection for NS_35 A-MPR in FR1, Ericsson, Huawei, Hisilicon, RAN5#83, Reno, Athens, May.2019