3GPP TSG-RAN5 Meeting #91-e	R5-212700r2

# Electronic Meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1877 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequencies for CA_n66(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently CA_n66(2A) is only specified in TS 38.101-1 without CA UL but the test frequencies and parameters for CC1 and CC2 are specified to enable use of either CC1 or CC2 as PCC in RF test cases. Thus need UL test frequencies and parameters be specified for both CC1 and CC2 in test frequency tables for CA_n66(2A). There are incorrect reference to UL/DL CBW combinations in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. The current reference to list of assymetric UL/DL CBW combinations is only valid for DL CBW 20 and 40 MHz as per TS 38.101, Table 5.3.6-1. For NR intra-band non-contiguous configurations it is more appropriate to refer to Sub-Blocks (SBs) and Carrier Components (CCs) within the sub-block for the combination of carrier components the Wgap refer to than referring to CCs only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed from "Downlink" to "Downlink & Uplink" in column "Range" in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. Added Note 1 explaining why Uplink tets frequencies specified for both SB1 and SB2 in spite of CA_n66(2A) does not support UL CA. Corrected references to CBW combinations in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. Removed asymmetric UL/DL CBW combinations for CBW combinations wher DL CBW is 5 MHz, 10 MHz and 15 MHz. Changed from "CC" tot "SB/CC" in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to use both SB1/CC1 and SB2/CC1 as PCC in RF test cases. Incorrect references remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-212700r1: Corrected references to CBW combinations in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. Changed from "CC" tot "Sub-Block"/"SB" in Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2. R5-212700r2: Table 4.3.1.1.5.66-1 and Table 4.3.1.1.5.66-2: Removed asymmetric UL/DL CBW combinations for CBW combinations where DL CBW is 5 MHz, 10 MHz and 15 MHz. Aligned references to UL/DL CBW combinations. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.5.66	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A), SCS=15 kHz, Max Wgap

| CBW combination | Sub-Block/CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (5MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 5-5 | SB1/CC1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2/CC1 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 5-10 | SB1/CC1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 5-15 | SB1/CC1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 5-20 | SB1/CC1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 5-40 | SB1/CC1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (10MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 10-5 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 10-10 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 10-15 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10-20 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 10-40 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 15-5 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 15-10 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15-15 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15-20 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 15-40 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(5-40MHz) |  |  |  |  |  |  |
| 20-5 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 20-10 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20-15 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20-20 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 20-40 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(5-20MHz) |  |  |  |  |  |  |
| 40-5 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 40-10 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40-15 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40-20 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1-CC1 and SB2-CC1 have been specified for both downlink and uplink to enable that either SB1-CC1 as SB2-CC1 are used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.66-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A), SCS=30 kHz, Max Wgap

| CBW combination | Sub-Block/CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); A (10MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 10-10 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 10-15 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10-20 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 10-40 | SB1/CC1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (15MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 15-10 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15-15 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15-20 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 20/20 |
| 15-40 | SB1/CC1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (20MHz) - A(10-40MHz) |  |  |  |  |  |  |
| 20-10 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20-15 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20-20 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 20-40 | SB1/CC1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2/CC1 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); A (40MHz) - A(10-20MHz) |  |  |  |  |  |  |
| 40-10 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40-15 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/40 or 20/40 depending on required UL bandwidth |
|  | SB2/CC1 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40-20 | SB1/CC1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2/CC1 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1-CC1 and SB2-CC1 have been specified for both downlink and uplink to enable that either SB1-CC1 as SB2-CC1 are used as PCC. |  |  |  |  |  |  |

###### 4.3.1.1.5.67 – 4.3.1.1.5.76	FFS