3GPP TSG RAN WG5 Meeting #91-e	R5-212798r1

# Electronic Meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0184 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for Test function for RACS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test function SET UL MESSAGE for using a pre-defined UE capability container is optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PICS for test function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability of the test cases cannot be correctly specified and conformant UEs may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.509 CR 0039 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212798R5-212798r1 : PICS name is changed to pc_Set_UE_Cap_Info_NR since the test function is also being defined for EUTRA. |  |  |  |  |  |  |  |  |

### A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item | UE General Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitSRB_WithOneUL_Path | No |  |  |
| 2 | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitDRB_withUL_Both_MCG_SCG | Yes |  |  |
| 3 | Support direct SRB between the SN and the UE as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_srb3 | Yes |  |  |
| 4 | Support of reflective QoS | 38.306, 4.2.2 | Rel-15 | pc_as_ReflectiveQoS | No |  |  |
| 5 | Support of NAS reflective QoS | 24.501, 6.2.5.1.4.1, 9.11.4.1 | Rel-15 | pc_nas_ReflectiveQoS | No |  |  |
| 6 | Support of SMS over NAS | 24.501, 5.5.1.2 | Rel-15 | pc_sms_over_NAS | No |  |  |
| 7 | Support of CMAS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_CMAS_NR | No |  |  |
| 8 | Support of ETWS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_ETWS_NR | No |  |  |
| 9 | The UE supports additional UE-requested PDU establishment | 24.501, 6.4.1.5 | Rel-15 | pc_Additional_PDU_establishment | No |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |
| 10 | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message | 24.501, 6.4.1.2 | Rel-15 | pc_SM_PDU_DN_RequestContainer | No |  |  |
| 11 | Support of emergency services fallback in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyService_fallback | No |  |  |
| 12 | Support of EPS fallback | 24.501, | Rel-15 | pc_EPS_fallback | No |  |  |
| 13 | Support of UE requested PDU session modification | 24.501, 6.4.2.2 | Rel-15 | pc_MO_PDU_Session_Modification | Yes |  |  |
| 14 | Support of emergency services in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyServices | No |  |  |
| 15 | Support of voiceFallbackIndication | 38.306, 4.2.13 | Rel-16 | pc_voiceFallbackIndication | No |  |  |
| 16 | Support provision of referenceTimeInfo | 38.306, 4.2.2 | Rel-16 | pc_referenceTimeProvision_r16 | No |  | specifically for TSC (time sensitive communication) services |
| 17 | Support of RACS | 24.501, 9.11.3.1 | Rel-16 | pc_5GC_RACS | No |  |  |
| 18 | Support of RRC message Segmentation in the UL | 38.306, 5.4 | Rel-16 | pc_NR_UL_Segmentation | No |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |
| 19 | Support of RRC_INACTIVE as specified in TS 38.331 [9]. | 38.306, 4.2.2 | Rel-15 | pc_inactiveState | Yes |  |  |
| 20 | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested | 23.122 clause C.2 | Rel-15 | pc_SOR_ACKNotReqLocalRel | No |  |  |
| xx | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR | 38.509, 5.9 | Rel-16 | pc_Set_UE_Cap_Info_NR | No |  |  |