3GPP TSG-RAN WG5 Meeting #91-e	R5-212822r1

# Electronic meeting, 17th - 28th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1894 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 4.3.1.1.3.66.1 for test frequencies for CA n66B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The calculation of nominal channel spacing is not aligned with the definition in TS 38.101-1 sub-clause 5.4A.1. The nominal channel spacing for CA_n66B with channel bandwidths 5MHz/15kHz _20MHz/15kHz and 5MHz/15kHz _40MHz/15kHz should be calculated based on μ0 = 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the nominal channel spacing and the corresponding test frequencies for CA_n66B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The nominal channel spacing and the test frequencies for CA_n66B will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.66.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct the issue of title inconsistency between 3GU and CR coversheet. |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

4.3.1.1.3.66	NR Intra-band contiguous configurations CA_n66

4.3.1.1.3.66.1	CA_n66B

Editor's note: Test frequencies for CA_n66B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.66.1-1: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 15 kHz

| CBW combination | CA Aggregated CBW [MHz] Note 5 | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5+20 | 24.98 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 |  | 5279 | 422410 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2135 | 427000 | 2114.39 | 422878 | 102 |  |  | 5336 | 426970 |  | 8 |  | 1 | 0 (0) | 103 |
|  |  |  |  |  |  |  | High | 2157.7 | 431540 | 2064.73 | 412946 | 504 |  |  | 5395 | 431570 |  | 4 |  | 1 | 1 (2) | 507 |
|  |  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1735 | 347000 | 1642.03 | 328406 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1757.7 | 351540 | 1754.37 | 350874 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=12.29 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 20 | 15 | 106 | Downlink | Low | 2124.8 | 424960 | 2115.26 | 423052 | 0 | 15 |  | 5294 | 423610 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2147.3 | 429460 | 2119.4 | 423880 | 102 |  |  | 5348 | 427930 |  | 6 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  |  | 5407 | 432530 |  | 2 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1724.8 | 344960 | 1715.26 | 343052 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1747.3 | 349460 | 1647.04 | 329408 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  |  | - | - |  | - |  | - | - | - |
| 5+40 | 44.98 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 |  | 5279 | 422410 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2125 | 425000 | 2104.39 | 420878 | 102 |  |  | 5314 | 425090 |  | 0 |  | 1 | 2 (4) | 107 |
|  |  |  |  |  |  |  | High | 2137.8 | 427560 | 2044.83 | 408966 | 504 |  |  | 5343 | 427470 |  | 0 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1725 | 345000 | 1632.03 | 326406 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1737.8 | 347560 | 1734.47 | 346894 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=22.19 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 40 | 15 | 216 | Downlink | Low | 2134.7 | 426940 | 2115.26 | 423052 | 0 | 15 |  | 5294 | 423610 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2147.2 | 429440 | 2109.4 | 421880 | 102 |  |  | 5326 | 426050 |  | 10 |  | 1 | 1 (2) | 105 |
|  |  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  |  | 5358 | 428670 |  | 6 |  | 1 | 2 (4) | 509 |
|  |  |  |  |  |  | Uplink | Low | 1734.7 | 346940 | 1715.26 | 343052 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1747.2 | 349440 | 1637.04 | 327408 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  |  | - | - |  | - |  | - | - | - |
| 10+15 | 24.94 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 |  | 5280 | 422430 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2114.46 | 422892 | 102 |  |  | 5337 | 426990 |  | 10 |  | 1 | 0 (0) | 103 |
|  |  |  |  |  |  |  | High | 2160.2 | 432040 | 2064.8 | 412960 | 504 |  |  | 5393 | 431530 |  | 10 |  | 1 | 0 (0) | 505 |
|  |  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1642.1 | 328420 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1760.2 | 352040 | 1754.44 | 350888 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 15 | 15 | 79 | Downlink | Low | 2127.3 | 425460 | 2120.19 | 424038 | 0 | 15 |  | 5307 | 424590 |  | 4 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2149.8 | 429960 | 2124.33 | 424866 | 102 |  |  | 5361 | 428910 |  | 4 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  |  | 5417 | 433450 |  | 4 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1727.3 | 345460 | 1720.19 | 344038 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1749.8 | 349960 | 1651.97 | 330394 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  |  | - | - |  | - |  | - | - | - |
| 10+20 | 29.9 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 |  | 5280 | 422430 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2135 | 427000 | 2111.96 | 422392 | 102 |  |  | 5330 | 426490 |  | 10 |  | 1 | 0 (0) | 103 |
|  |  |  |  |  |  |  | High | 2155.6 | 431120 | 2060.2 | 412040 | 504 |  |  | 5383 | 430610 |  | 10 |  | 1 | 0 (0) | 505 |
|  |  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1735 | 347000 | 1639.6 | 327920 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1755.6 | 351120 | 1749.84 | 349968 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 20 | 15 | 106 | Downlink | Low | 2129.4 | 425880 | 2119.86 | 423972 | 0 | 15 |  | 5304 | 424350 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2149.4 | 429880 | 2121.5 | 424300 | 102 |  |  | 5354 | 428410 |  | 2 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  |  | 5407 | 432530 |  | 2 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1729.4 | 345880 | 1719.86 | 343972 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1749.4 | 349880 | 1649.14 | 329828 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  |  | - | - |  | - |  | - | - | - |
| 10+40 | 49.9 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 |  | 5280 | 422430 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2125 | 425000 | 2101.96 | 420392 | 102 |  |  | 5308 | 424610 |  | 2 |  | 1 | 2 (4) | 107 |
|  |  |  |  |  |  |  | High | 2135.7 | 427140 | 2040.3 | 408060 | 504 |  |  | 5334 | 426750 |  | 2 |  | 1 | 2 (4) | 509 |
|  |  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1725 | 345000 | 1629.6 | 325920 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1735.7 | 347140 | 1729.94 | 345988 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 40 | 15 | 216 | Downlink | Low | 2139.3 | 427860 | 2119.86 | 423972 | 0 | 15 |  | 5304 | 424350 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2149.3 | 429860 | 2111.5 | 422300 | 102 |  |  | 5332 | 426530 |  | 6 |  | 1 | 2 (4) | 107 |
|  |  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  |  | 5358 | 428670 |  | 6 |  | 1 | 2 (4) | 509 |
|  |  |  |  |  |  | Uplink | Low | 1739.3 | 347860 | 1719.86 | 343972 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1749.3 | 349860 | 1639.14 | 327828 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+10 | 24.94 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 |  | 5281 | 422450 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2140 | 428000 | 2114.53 | 422906 | 102 |  |  | 5338 | 427010 |  | 0 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2162.7 | 432540 | 2064.87 | 412974 | 504 |  |  | 5394 | 431550 |  | 0 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1740 | 348000 | 1642.17 | 328434 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1762.7 | 352540 | 1754.51 | 350902 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 10 | 15 | 52 | Downlink | Low | 2129.8 | 425960 | 2125.12 | 425024 | 0 | 15 |  | 5320 | 425570 |  | 2 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2152.3 | 430460 | 2129.26 | 425852 | 102 |  |  | 5374 | 429890 |  | 2 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  |  | 5430 | 434430 |  | 2 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1729.8 | 345960 | 1725.12 | 345024 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1752.3 | 350460 | 1656.9 | 331380 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+15 | 29.94 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 |  | 5281 | 422450 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2112.03 | 422406 | 102 |  |  | 5331 | 426510 |  | 0 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2157.5 | 431500 | 2059.67 | 411934 | 504 |  |  | 5381 | 430570 |  | 8 |  | 1 | 1 (2) | 507 |
|  |  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1639.67 | 327934 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1757.5 | 351500 | 1749.31 | 349862 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 15 | 15 | 79 | Downlink | Low | 2132.5 | 426500 | 2125.39 | 425078 | 0 | 15 |  | 5320 | 425570 |  | 8 |  | 1 | 1 (2) | 3 |
|  |  |  |  |  |  |  | Mid | 2152.5 | 430500 | 2127.03 | 425406 | 102 |  |  | 5370 | 429630 |  | 4 |  | 1 | 2 (4) | 107 |
|  |  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  |  | 5417 | 433450 |  | 4 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1732.5 | 346500 | 1725.39 | 345078 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1752.5 | 350500 | 1654.67 | 330934 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+20 | 34.94 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 |  | 5281 | 422450 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2135 | 427000 | 2109.53 | 421906 | 102 |  |  | 5324 | 426010 |  | 0 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2152.9 | 430580 | 2055.07 | 411014 | 504 |  |  | 5371 | 429650 |  | 8 |  | 1 | 1 (2) | 507 |
|  |  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1735 | 347000 | 1637.17 | 327434 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1752.9 | 350580 | 1744.71 | 348942 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 20 | 15 | 106 | Downlink | Low | 2134.6 | 426920 | 2125.06 | 425012 | 0 | 15 |  | 5320 | 425570 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2152.1 | 430420 | 2124.2 | 424840 | 102 |  |  | 5360 | 428890 |  | 6 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  |  | 5407 | 432530 |  | 2 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1734.6 | 346920 | 1725.06 | 345012 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1752.1 | 350420 | 1651.84 | 330368 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  |  | - | - |  | - |  | - | - | - |
| 20+5 | 24.98 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 |  | 5282 | 422650 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2142.5 | 428500 | 2114.6 | 422920 | 102 |  |  | 5336 | 426970 |  | 6 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2165.2 | 433040 | 2064.94 | 412988 | 504 |  |  | 5395 | 431570 |  | 2 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1742.5 | 348500 | 1642.24 | 328448 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1765.2 | 353040 | 1754.58 | 350916 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=12.29 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 5 | 15 | 25 | Downlink | Low | 2132.3 | 426460 | 2130.05 | 426010 | 0 | 15 |  | 5330 | 426490 |  | 4 |  | 1 | 1 (2) | 3 |
|  |  |  |  |  |  |  | Mid | 2154.8 | 430960 | 2134.19 | 426838 | 102 |  |  | 5387 | 431050 |  | 0 |  | 1 | 2 (4) | 107 |
|  |  |  |  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  |  | 5443 | 435410 |  | 0 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1732.3 | 346460 | 1730.05 | 346010 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1754.8 | 350960 | 1661.83 | 332366 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  |  | - | - |  | - |  | - | - | - |
| 20+10 | 29.9 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 |  | 5282 | 422650 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2140 | 428000 | 2112.1 | 422420 | 102 |  |  | 5332 | 426530 |  | 2 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2160.6 | 432120 | 2060.34 | 412068 | 504 |  |  | 5382 | 430590 |  | 6 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1740 | 348000 | 1639.74 | 327948 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1760.6 | 352120 | 1749.98 | 349996 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 10 | 15 | 52 | Downlink | Low | 2134.4 | 426880 | 2129.72 | 425944 | 0 | 15 |  | 5330 | 426490 |  | 2 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2154.4 | 430880 | 2131.36 | 426272 | 102 |  |  | 5380 | 430370 |  | 10 |  | 1 | 0 (0) | 103 |
|  |  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  |  | 5430 | 434430 |  | 2 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1734.4 | 346880 | 1729.72 | 345944 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1754.4 | 350880 | 1659 | 331800 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  |  | - | - |  | - |  | - | - | - |
| 20+15 | 34.94 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 |  | 5282 | 422650 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2109.6 | 421920 | 102 |  |  | 5325 | 426030 |  | 2 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2155.4 | 431080 | 2055.14 | 411028 | 504 |  |  | 5369 | 429610 |  | 2 |  | 0 | 1 (2) | 506 |
|  |  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1637.24 | 327448 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1755.4 | 351080 | 1744.78 | 348956 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 15 | 15 | 79 | Downlink | Low | 2137.1 | 427420 | 2129.99 | 425998 | 0 | 15 |  | 5330 | 426490 |  | 8 |  | 1 | 1 (2) | 3 |
|  |  |  |  |  |  |  | Mid | 2154.6 | 430920 | 2129.13 | 425826 | 102 |  |  | 5373 | 429870 |  | 4 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  |  | 5417 | 433450 |  | 4 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1737.1 | 347420 | 1729.99 | 345998 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1754.6 | 350920 | 1656.77 | 331354 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  |  | - | - |  | - |  | - | - | - |
| 40+5 | 44.98 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 |  | 5283 | 422670 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2142.5 | 428500 | 2104.7 | 420940 | 102 |  |  | 5312 | 425050 |  | 2 |  | 0 | 1 (2) | 104 |
|  |  |  |  |  |  |  | High | 2155.3 | 431060 | 2045.14 | 409028 | 504 |  |  | 5347 | 427730 |  | 6 |  | 1 | 2 (4) | 509 |
|  |  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1742.5 | 348500 | 1632.34 | 326468 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1755.3 | 351060 | 1734.78 | 346956 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=22.19 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 5 | 15 | 25 | Downlink | Low | 2152.2 | 430440 | 2149.95 | 429990 | 0 | 15 |  | 5379 | 430350 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  |  |  | Mid | 2164.7 | 432940 | 2144.09 | 428818 | 102 |  |  | 5411 | 432970 |  | 4 |  | 1 | 1 (2) | 105 |
|  |  |  |  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  |  | 5443 | 435410 |  | 0 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1752.2 | 350440 | 1749.95 | 349990 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1764.7 | 352940 | 1671.73 | 334346 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  |  | - | - |  | - |  | - | - | - |
| 40+10 | 49.9 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 |  | 5283 | 422670 |  | 6 |  | 1 | 2 (4) | 5 |
|  |  |  |  |  |  |  | Mid | 2140 | 428000 | 2102.2 | 420440 | 102 |  |  | 5308 | 424610 |  | 10 |  | 1 | 1 (2) | 105 |
|  |  |  |  |  |  |  | High | 2150.7 | 430140 | 2040.54 | 408108 | 504 |  |  | 5334 | 426750 |  | 10 |  | 1 | 1 (2) | 507 |
|  |  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1740 | 348000 | 1629.84 | 325968 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1750.7 | 350140 | 1730.18 | 346036 | 6 |  |  | - | - |  | - |  | - | - | - |
|  |  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CC2 | 10 | 15 | 52 | Downlink | Low | 2154.3 | 430860 | 2149.62 | 429924 | 0 | 15 |  | 5379 | 430350 |  | 10 |  | 1 | 0 (0) | 1 |
|  |  |  |  |  |  |  | Mid | 2164.3 | 432860 | 2141.26 | 428252 | 102 |  |  | 5404 | 432290 |  | 2 |  | 0 | 0 (0) | 102 |
|  |  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  |  | 5430 | 434430 |  | 2 |  | 0 | 0 (0) | 504 |
|  |  |  |  |  |  | Uplink | Low | 1754.3 | 350860 | 1749.62 | 349924 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | Mid | 1764.3 | 352860 | 1668.9 | 333780 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  |  | - | - |  | - |  | - | - | - |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: Corresponds to CA aggregated channel bandwidth BWChannel_CA in accordance with TS 38.101-1 [7] clause 5.3A.3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.66.1-2: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+15 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2096.46 | 419292 | 102 |  |  | 5343 | 427470 |  | 14 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2160.2 | 432040 | 1974.44 | 394888 | 504 |  |  | 5399 | 432010 |  | 14 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1551.74 | 310348 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1760.2 | 352040 | 1753.72 | 350744 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2127.3 | 425460 | 2120.46 | 424092 | 0 | 15 |  | 5310 | 424830 |  | 6 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2149.8 | 429960 | 2106.24 | 421248 | 102 |  |  | 5367 | 429390 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1727.3 | 345460 | 1720.46 | 344092 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1749.8 | 349960 | 1561.52 | 312304 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
| 10+20 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2135 | 427000 | 2093.96 | 418792 | 102 |  |  | 5336 | 426970 |  | 14 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2155.6 | 431120 | 1969.84 | 393968 | 504 |  |  | 5389 | 431090 |  | 14 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1735 | 347000 | 1549.24 | 309848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1755.6 | 351120 | 1749.12 | 349824 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2129.4 | 425880 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2149.4 | 429880 | 2103.5 | 420700 | 102 |  |  | 5360 | 428890 |  | 18 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1729.4 | 345880 | 1720.22 | 344044 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1749.4 | 349880 | 1558.78 | 311756 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
| 10+40 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2125 | 425000 | 2083.96 | 416792 | 102 |  |  | 5314 | 425090 |  | 6 |  | 0 | 3 (8) | 220 |
|  |  |  |  |  |  | High | 2135.7 | 427140 | 1949.94 | 389988 | 504 |  |  | 5340 | 427230 |  | 6 |  | 0 | 3 (8) | 1024 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1725 | 345000 | 1539.24 | 307848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1735.7 | 347140 | 1729.22 | 345844 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2139.3 | 427860 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2149.3 | 429860 | 2093.5 | 418700 | 102 |  |  | 5335 | 426770 |  | 2 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  |  | 5361 | 428910 |  | 2 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1739.3 | 347860 | 1720.22 | 344044 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1749.3 | 349860 | 1548.78 | 309756 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+10 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2140 | 428000 | 2096.44 | 419288 | 102 |  |  | 5344 | 427490 |  | 22 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2162.7 | 432540 | 1974.42 | 394884 | 504 |  |  | 5400 | 432030 |  | 22 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1740 | 348000 | 1551.72 | 310344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1762.7 | 352540 | 1753.7 | 350740 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2129.8 | 425960 | 2125.48 | 425096 | 0 | 15 |  | 5326 | 426050 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2152.3 | 430460 | 2111.26 | 422252 | 102 |  |  | 5380 | 430370 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1729.8 | 345960 | 1725.48 | 345096 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1752.3 | 350460 | 1566.54 | 313308 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+15 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2093.94 | 418788 | 102 |  |  | 5337 | 426990 |  | 22 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2157.5 | 431500 | 1969.22 | 393844 | 504 |  |  | 5387 | 431050 |  | 18 |  | 0 | 2 (7) | 1022 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1549.22 | 309844 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1757.5 | 351500 | 1748.5 | 349700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2132.5 | 426500 | 2125.66 | 425132 | 0 | 15 |  | 5326 | 426050 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2152.5 | 430500 | 2108.94 | 421788 | 102 |  |  | 5373 | 429870 |  | 6 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1732.5 | 346500 | 1725.66 | 345132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1752.5 | 350500 | 1564.22 | 312844 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
| 15+20 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2135 | 427000 | 2091.44 | 418288 | 102 |  |  | 5330 | 426490 |  | 22 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2152.9 | 430580 | 1964.62 | 392924 | 504 |  |  | 5377 | 430130 |  | 18 |  | 0 | 2 (7) | 1022 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1735 | 347000 | 1546.72 | 309344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1752.9 | 350580 | 1743.9 | 348780 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2134.6 | 426920 | 2125.42 | 425084 | 0 | 15 |  | 5323 | 425810 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2152.1 | 430420 | 2106.2 | 421240 | 102 |  |  | 5366 | 429370 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1734.6 | 346920 | 1725.42 | 345084 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1752.1 | 350420 | 1561.48 | 312296 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
| 20+10 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2140 | 428000 | 2094.1 | 418820 | 102 |  |  | 5338 | 427010 |  | 18 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2160.6 | 432120 | 1969.98 | 393996 | 504 |  |  | 5388 | 431070 |  | 22 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1740 | 348000 | 1549.38 | 309876 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1760.6 | 352120 | 1749.26 | 349852 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2134.4 | 426880 | 2130.08 | 426016 | 0 | 15 |  | 5336 | 426970 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2154.4 | 430880 | 2113.36 | 422672 | 102 |  |  | 5386 | 430850 |  | 14 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1734.4 | 346880 | 1730.08 | 346016 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1754.4 | 350880 | 1568.64 | 313728 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
| 20+15 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2137.5 | 427500 | 2091.6 | 418320 | 102 |  |  | 5331 | 426510 |  | 18 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2155.4 | 431080 | 1964.78 | 392956 | 504 |  |  | 5375 | 430090 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1737.5 | 347500 | 1546.88 | 309376 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1755.4 | 351080 | 1744.06 | 348812 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2137.1 | 427420 | 2130.26 | 426052 | 0 | 15 |  | 5336 | 426970 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2154.6 | 430920 | 2111.04 | 422208 | 102 |  |  | 5379 | 430350 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1737.1 | 347420 | 1730.26 | 346052 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1754.6 | 350920 | 1566.32 | 313264 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
| 40+10 | CC1 | 40 | 30 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 |  | 5286 | 422910 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2140 | 428000 | 2084.2 | 416840 | 102 |  |  | 5314 | 425090 |  | 14 |  | 0 | 2 (7) | 218 |
|  |  |  |  |  |  | High | 2150.7 | 430140 | 1950.18 | 390036 | 504 |  |  | 5340 | 427230 |  | 14 |  | 0 | 2 (7) | 1022 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1740 | 348000 | 1539.48 | 307896 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1750.7 | 350140 | 1729.46 | 345892 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2154.3 | 430860 | 2149.98 | 429996 | 0 | 15 |  | 5385 | 430830 |  | 14 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2164.3 | 432860 | 2123.26 | 424652 | 102 |  |  | 5410 | 432770 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1754.3 | 350860 | 1749.98 | 349996 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1764.3 | 352860 | 1578.54 | 315708 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>