3GPP TSG-RAN5 Meeting #91-e	R5-212873r1

# Electronic Meeting, 17th May – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2575 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Test Case 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage has been introduced in RAN4 [4] core-spec, but not added in RAN5 TS 36.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Case 7.3.91 and TT reused from HD-FDD Test Cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 7.3.91 will be still missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.91 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - 3GU inconsistency R2: - Editor’s note is needed to be added to clarify the inconsistence between antenna configuration & RMC in core-specification. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	As per core spec. requirement, the antenna configuration to be used is 2x1. But in RAN4 spec. Table A.7.3.91.1-2, it is asking to follow R.14 NB-TDD which is defined for 1x1.

7.3.91.1	Test purpose

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in TS 36.133 clause 7.23.

7.3.91.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 15 and forward of UE Category NB1 and NB2.   Applicability requires support for FGI bit 5.

7.3.91.3	Minimum conformance requirements

Same minimum conformance requirements as defined in 7.3.62.3.

7.3.91.4	Test description

Same test description as defined in 7.3.62.4

7.3.91.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.3.1 and 8.1.4.2.

Channel Bandwidth to be tested: E-UTRA Bandwidth is 10 MHz, as defined in TS 36.508 [7] clause 4.3.1.1. Ncell bandwidth is as specified in Table 7.3.91.5-1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.10 using only main Tx/Rx antenna and without using the faders.

2.	The general test parameter settings are set up according to Table 7.3.91.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 7.3.91.4.3

5.	There are two cells specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test. Cell 1 is a EUTRAN cell.

Table 7.3.91.4.1-1: General test parameters for TDD in-sync test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in TS 36.133[4] clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in TS 36.133[4] clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX cycle |  | ms | 256 | See Table 7.3.91.5-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

7.3.91.4.2	Test procedure

Same test procedure as in clause 7.3.62.4.2.

7.3.91.4.3	Message contents

Same message contents as in clause 7.3.62.4.3.

7.3.91.5	Test requirement

Table 7.3.91.5-1: nCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.14 NB-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.91.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in TS 36.133[4] figure A.7.3.91.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table 7.3.91.5-2: eCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table 7.3.91.5-3: DRX-Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table 7.3.91.5-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

During the period from time point D until T3 expiration, the UE behaviours in each test shall be as follows:

UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. Thereafter UE switches back to downlink. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

## << End of changes >>