Title: 	Discussion on test point selection for TX general spurious emission for inter-band EN-DC within FR1

Introduction

Generic initial test conditions, such as environmental conditions, test frequencies, channel bandwidths, subcarrier spacing (SCS) were discussed in previous RAN5 presentation [1] [2]. The general agreement is that FR1 testing mostly follows similar requirement as for LTE. A recent paper [3] also referenced to an agreed common uplink configuration table for all TX test cases.

In another RAN5 paper (R5-185892), it is proposed that NR test point selection shall follow what is specified in SA scenarios.

The purpose of this paper is to discuss test point selection of E-UTRA and NR carriers for TX general spurious emission measurement for inter-band EN-DC within FR1. Considering there are so many EN-DC combos expected to be supported, it is reasonable to carefully investigate the possibility of reducing test points. Since the objective of 38.521-3 specification is provide conformance testing requirement for NR in the case of EN-DC scenarios, the number of LTE test points can be reduced.

Discussion

From TS 38.521-1, the initial conditions used for NR general/UE co-ex spurious emission is given below:

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

As a reference, the initial test condition used for LTE CA (inter-band DL CA and UL CA) transmitter spurious emission is listed below:

Table 6.6.3.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

The initial test condition used for NR CA (inter-band 2UL CA) general spurious emission is listed below:

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

For spurious emissions, it is reasonable to make the test configuration for EN-DC follow what is specified in LTE CA scenarios or NR CA scenarios. Here, NR inter-band CA is used for a reference to derive the test configuration for inter-band EN-DC.

Test Environment

Considering EN-DC testing scenario is very similar to NR CA test case, similar Test Environment should be used for EN-DC testing.

Proposal 1: Propose Test Environment as Normal for spurious emission test cases in EN-DC mode.

Test Frequencies selection

In NR CA scenario, both Low range and High range are selected for general spurious emission test cases.

For inter-band EN-DC, it is reasonable to follow similar test frequencies selection as for NR inter-band CA to test general spurious emissions.

Proposal 2: For inter-band EN-DC, both Low range and High range shall be selected for general spurious emission test cases.

Test Channel Bandwidth selection

Tx spurious emission testing is to verify that the unwanted spurious emissions out of band meet the minimum requirements defined by RAN4. In NR CA scenario, two extreme bandwidth combinations corresponding to Lowest bandwidth and Highest bandwidth are selected for general spurious emission test case.

Similarly, lowest and highest channel bandwidth shall be selected for general spurious emission test cases in EN-DC mode.

Proposal 3: Lowest and highest channel bandwidth shall be selected for E-UTRA carrier and NR carrier in inter-band EN-DC general spurious emission test cases.

RB allocation and RB location selection

For inter-band EN-DC, it is reasonable to follow similar RB allocation and RB location selection as for NR inter-band CA in general spurious emission test cases. Therefore, it is proposed to use the RB configuration described in Table 2.4-1 for general spurious emission for inter-band EN-DC.

Table 2.4-1: Test configuration table for inter-band EN-DC

| Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |
|  |  | E-UTRA Cell |  | NR Cell |  |
|  |  | Modulation | RB allocation (NOTE 2) | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Spurious emission. | QPSK | Outer_1RB_Left | CP-OFDM QPSK | Edge_1RB_Left |
| 2 |  | QPSK | Outer_1RB _Right | CP-OFDM QPSK | Edge_1RB_Right |
| 3 |  | QPSK | Outer_Full | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: Outer_Full defined as the transmission bandwidth configuration NRB per channel bandwidth for the E-UTRA component as indicated in TS 36.521-1 [10] Table 5.4.2-1. Outer_1RB_Left defined as 1 RB allocated at the left edge of the E-UTRA component. Outer_1RB_Right defined as 1 RB allocated at the right edge of the E-UTRA component. |  |  |  |  |  |

Proposal 4: Propose the same RB configuration as for NR CA test cases to test general spurious emissions for inter-band EN-DC as described in Table 2.4-1.

Modulation selections

For LTE and NR SA spurious emissions measurement, the modulation QPSK and CP-OFDM are used for the uplink modulation, respectively.

For inter-band EN-DC, there is no particular reason to deviate from the selected modulation in LTE and NR SA scenarios.

Proposal 5: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

Number of Test Points for each EN-DC combo

Table 2.6-1 Maximum Number of Test Points for general spurious emission for each EN-DC combo

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of RB allocations | Maximum Number of Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 1 | 3 | 12 |

Conclusion

In conclusion, the following test points selection are proposed for inter-band EN-DC in general spurious emission measurement:

Proposal 1: Propose Test Environment as Normal for spurious emission test cases in EN-DC mode.

Proposal 2: For inter-band EN-DC, both Low range and High range shall be selected for general spurious emission test cases.

Proposal 3: Lowest and highest channel bandwidth shall be selected for E-UTRA carrier and NR carrier in inter-band EN-DC general spurious emission test cases.

Proposal 4: Propose the same RB configuration as for NR CA test cases to test general spurious emissions for inter-band EN-DC as described in Table 2.4-1.

Proposal 5: Propose the same modulation scheme as for LTE and NR SA testing for E-UTRA carrier and NR carrier for EN-DC spurious emission test cases.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-1 v0.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TR 38.817-01 v0.3.0 General aspects for UE RF for NR (Release 15)

[5] R5-181895, “TP for adding Mid channel BW definition in TS 38.508-1”

[6] R5-180886, “Discussion_NR_FR1_TestPoints_For_SEM”

[7] R5-191451r1, “Resolving ambiguity in LTE anchor agnostic approach”