3GPP TSG-RAN5 Meeting #91-e	R5-213063r1

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2237 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SA TC 8.1.1.2.1-T300 expiry |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In RAN5 #90e it was agreed that TP3 of 7.1.1.1.3 shall be moved to TC 8.1.1.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Originial TP3 of TC 7.1.1.1.3 is added as TP4 of TC 8.1.1.2.1. Correspondingly, - steps to verify TP4 are added to test procedure. - nessesary message contents are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Changes to TC title is undone to avoid clash with CR R5-212765. Coversheet is updated accordingly. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.1.2	RRC connection establishment

8.1.1.2.1	RRC connection establishment / Return to idle state after T300 expiry

8.1.1.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state having sent an RRCSetupRequest message }

ensure that {

when { the SS does not answer to the UE during T300 }

then { UE goes to RRC_IDLE state }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1 for a period as indicated by connEstFailOffsetValidity }

then { UE uses connEstFailOffset for the parameter Qoffsettemp when performing cell reselection }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { connEstFailOffsetValidity has expired }

then { UE stops using the parameter Qoffsettemp when performing cell reselection }

}

(4)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE receives a short message transmitted on PDCCH using P-RNTI indicating a systemInfoModification }

then {UE re-acquires and applies the new system information }

}

8.1.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.3.3.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	if the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

8.1.1.2.1.3	Test description

8.1.1.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 3.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A as defined in TS 38.508-1 [4], clause 4.4A on NR Cell 1 and 5G-GUTI is allocated in REGISTRATION ACCEPT.

8.1.1.2.1.3.2	Test procedure sequence

Table 8.1.1.2.1.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 8.1.1.2.1.3.2-3.

Table 8.1.1.2.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -88 |  |
| Note: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.1.2.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off |  |
|  | Qrxlevmin | dBm | -91+Delta(NRf1) | - |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
|  | Qrxlevmin | dBm | -91+Delta(NRf1) | -100+Delta(NRf2) |  |

Table 8.1.1.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS waits for 2s (T300 expire). | - | - | - | - |
| 4 | Check: Does UE send RRCSetupRequest in 5 second? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 5 | Check: Does the test result of generic test procedure step 1-8 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE? | - | - | 1 | - |
| 6 | SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE | <-- | NR RRC: RRCRelease | - | - |
| 7 | The SS remove connEstFailOffset in SIB1. | - | - | - | - |
| 8 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 8A | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 11 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 12 | The SS waits for 2s to let T300 expire. | - | - | - | - |
| 12A | The SS re-adjusts the SS levels according to row "T1" in table 8.1.1.2.1.3.2-1/2 and waits for 35s to allow UE to perform cell reselection. | - | - | - | - |
| 13 | The SS transmits a Paging message including a matched ng-5G-S-TMSI on NR Cell 1. | <-- | NR RRC: Paging | - | - |
| 14 | Check: Does UE send RRCSetupRequest in 5 second? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 15 | Void | - | - | - | - |
| 16 | After expiry of connEstFailOffsetValidity, the SS waits 35s to allow UE to perform cell reselection. | - | - | - | - |
| 17 | Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 1? | - | - | 3 | - |
| 18 | The SS changes NR Cell 3 to high priority in SIB4. | - | - | - | - |
| 19 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 20 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 21 | Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3? | - | - | 4 | - |

8.1.1.2.1.3.3	Specific message contents

Table 8.1.1.2.1.3.3-1: RRCSetupRequest (steps 2 and 11, Table 8.1.1.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.1.3.3-2: SIB1 (step 7, Table 8.1.1.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| connEstFailureControl |  | ConnEstFailureControl | Table 8.1.1.2.1.3.3-3 |  |
| } |  |  |  |  |

Table 8.1.1.2.1.3.3-3: ConnEstFailureControl (Table 8.1.1.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConnEstFailureControl ::= SEQUENCE { |  |  |  |
| connEstFailOffsetValidity | s60 |  |  |
| connEstFailOffset | Not present | Value of infinity shall be used for Qoffsettemp |  |
| } |  |  |  |

Table 8.1.1.2.1.3.3-4: SIB4 (steps 18, Table 8.1.1.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 The entry corresponding to the carrier of NR Cell 3 |  |
| cellReselectionPriority | 7 | Higer priority than NR Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>