3GPP TSG-RAN5 Meeting #91-e	R5-213076r1

# Electronic Meeting, 17th May– 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0156 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR IMS 7.24-Forking-two responses one cancel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no testing procedures for IMS NR call forking with MO (Mobile Terminated) UE receiving two responses and one cancel in 34.229-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case for IMS NR call forking with MO UE receiving two responses and one cancel in 34.229-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of relevant test procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.24 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0283 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Editorial revisions. - Deleted 24.229 cl 5.1.3.1/6.1.4.2 from Conformance Requirements. - Added NG.114 Profile Requirements as cl 7.24.3. - Adjusted the sequence of the main test procedures (e.g. sequencies between 183 dialog 1/2, PRACK dialog 1/2, CANCEL dialog/1 etc.) |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.24	Forking / UE receives two responses and one CANCEL request / 5GS

7.24.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call with preconditions }

}

(2)

with { UE having sent INVITE }

ensure that {

when { UE receives two 100 Trying and two 183 Session Progress responses }

then { UE sends two PRACK requests for the two 183 Session Progress responses }

}

(3)

with { UE having sent two PRACK requests }

ensure that {

when { UE receives CANCEL for the first forked early dialog and 200 OK for PRACK for the second early dialog }

then { UE completes setup of voice call on the second early dialog }

}

7.24.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.4.1]:

If the UE sends a CANCEL request to cancel an initial INVITE request, the UE shall when applicable include in the CANCEL request a Reason header field with a protocol value set to "RELEASE_CAUSE" and a "cause" header field parameter as specified in subclause 7.2A.18.11.2. The UE may also include the "text" header field parameter with reason-text as specified in subclause 7.2A.18.11.2.

7.24.3	Profile Requirements (Informative)

[GSMA NG.114 V1.0, clause 2.3.7]:

The IMS core network can support sending and the UE must support receiving a SIP CANCEL request including a Reason header field with values of:

1. SIP; cause=200; text="Call completed elsewhere"

2. SIP; cause=603; text="Declined"

3. SIP; cause=600; text=” Busy Everywhere”

for forked calls as defined in 3GPP TS 24.229 [8].

7.24.4	Test description

7.24.4.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A and registered to IMS

7.24.4.2	Test procedure sequence

Table 7.24.4.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make UE initiate a voice call. |  |  |  |  |
| 2-9 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 10 | Check: Does the UE send INVITE with an SDP offer containing preconditions? (Step 1 of A.4.1) | -> | INVITE | 1 | P |
| 11 | SS responds with a 100 Trying in forked early dialog 1 (Step 2 of A.4.1) | <- | 100 Trying |  |  |
| 12 | SS responds with a 100 Trying in forked early dialog 2 (Step 2 of A.4.1) | <- | 100 Trying |  |  |
| 13 | SS responds with a 183 Session in Progress in forked dialog 1 (Step 3 of A.4.1) | <- | 183 Session in Progress |  |  |
| 14 | Check: Does the UE acknowledge the receipt of 183 response with PRACK in dialog 1? (Step 4 of A.4.1) | -> | PRACK | 2 | P |
| 15 | SS responds with a 183 Session in Progress in forked dialog 2 (Step 3 of A.4.1) | <- | 183 Session in Progress |  |  |
| 16 | Check: Does the UE acknowledge the receipt of 183 response with PRACK in dialog 2? (Step 4 of A.4.1) | -> | PRACK | 2 | P |
| 17 | SS sends CANCEL for the forked early dialog 1 | <- | CANCEL |  |  |
| 18 | UE sends 200 OK for the CANCEL | -> | 200 OK |  |  |
| 19 | SS sends 200 OK for the forked early dialog 2 (Step 5 of A.4.1) | <- | 200 OK |  |  |
|  | Note: resource reservation will be triggered. |  |  |  |  |
| 20 | Check: Does the UE send an UPDATE after having reserved the resources for the preconditions of the dialog 2? (Step 6 of A.4.1) | -> | UPDATE | 3 | P |
| 21 | The SS responds UPDATE with 200 OK and indicates having reserved the resources (Step 7 of A.4.1) | <- | 200 OK |  |  |
| 22 | SS sends 180 Ringing reliably (Step 8 of A.4.1) | <- | 180 Ringing |  |  |
| 23 | Check: Does the UE acknowledge reception of 180 Ringing? (Step 9 of A.4.1) | -> | PRACK | 3 | P |
| 24 | SS responds to PRACK. (Step 10 of A.4.1) | <- | 200 OK |  |  |
| 25 | The SS responds INVITE with 200 OK (Step 11 of A.4.1) | <- | 200 OK |  |  |
| 26 | Check: Does the UE acknowledge the receipt of 200 OK for INVITE? (Step 12 of A.4.1) | -> | ACK | 3 | P |
| 27 | Make the UE release the call. |  |  |  |  |
| 28-29 | Steps 1-2 of annex A.7 |  |  |  |  |

7.24.4.3	Specific message contents

Table 7.24.4.3-1: 100 Trying (step 12, table 7.24.4.2-1)

| Derivation Path: Step 2 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | Any value different from what is used in step 11 |  |  |

Table 7.24.4.3-2: 183 Session Progress (step 13, table 7.24.4.2-1)

| Derivation Path: Step 3 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 11 |  |  |

Table 7.24.4.3-4: PRACK (step 14, table 7.24.4.2-1)

| Derivation Path: Step 4 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 11 |  |  |

Table 7.24.4.3-3: 183 Session Progress (step 15, table 7.24.4.2-1)

| Derivation Path: Step 3 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-5: PRACK (step 16, table 7.24.4.2-1)

| Derivation Path: Step 4 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-6: CANCEL (step 17, table 7.24.4.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.15 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 11 |  |  |

Table 7.24.4.3-7: 200 OK for CANCEL (step 18, table 7.24.4.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, conditions A5 and A11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 11 |  |  |

Table 7.24.4.3-8: UPDATE (step 20, table 7.24.4.2-1)

| Derivation Path: Step 6 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-9: 200 OK for UPDATE (step 21, table 7.24.4.2-1)

| Derivation Path: Step 7 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-10: 180 Ringing (step 22, table 7.24.4.2-1)

| Derivation Path: Step 8 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-11: PRACK (step 23, table 7.24.4.2-1)

| Derivation Path: Step 9 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-12: 200 OK for PRACK (step 24, table 7.24.4.2-1)

| Derivation Path: Step 10 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-13: 200 OK for INVITE (step 25, table 7.24.4.2-1)

| Derivation Path: Step 11 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

Table 7.24.4.3-14: ACK (step 26, table 7.24.4.2-1)

| Derivation Path: Step 12 of A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To tag |  | The same value with what is used in step 12 |  |  |

<End of modified section 1>