3GPP TSG-RAN5 Meeting # 91-e	R5-213165r1

# Electronic Meeting,  17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1925 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mid range test frequencies update in case asymmetric bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current specifications does not include guidelines to update mid test frequencies in case BWDL(calculated as per clause 38.508-1 Annex C.2.3.2.2) does not fit within the DL frequency range for the band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added guidelines to center mid test frequencies for DL and UL in case adjustment required for any of the testing frequencies at the edges. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mid range testing frequencies won’t be centered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Year added in the coverpage date. |  |  |  |  |  |  |  |  |

<< Start of change >>

## C.2.3	Determination of test frequencies for bands supporting asymmetric channel bandwidth combinations

### C.2.3.1	General

The following procedure is used to calculate test frequencies for NR bands supporting asymmetric UL and DL channel bandwidths as described below, where CBWUL and CBWDL refer to the carrier’s UL and DL channel bandwidths; and BWUL and BWDL refer to the band’s total UL and DL bandwidths.

The procedure is also used to calculate test frequencies for symmetric UL and DL bandwidth combinations for asymmetric NR bands.

For FDD bands supporting asymmetric uplink and downlink bandwidth combinations a deviation of ΔFTX-RX (C.2.3.1-Eq1) is to be added to the default Tx-Rx carrier centre frequency separation, FTx-Rx_separation (TS 38.101-1 [7] clause 5.3.6).

| ΔFTX-RX = ¦(CBWDL – CBWUL)/2¦ | C.2.3.1-Eq1 |
| --- | --- |

For the case of asymmetric NR bands and symmetric UL and DL bandwidth combinations ΔFTX-RX = 0.To meet the Tx-Rx frequency separation requirement for asymmetric NR bands were the supported overall UL bandwidth is smaller than the supported overall DL bandwidth it may not be possible to cover the full DL frequency range for all UL and DL channel bandwidth combinations. For CA when such band is only used for DL CC the full range can be used for all DL channel bandwidths.

To maximize the tested frequency range for the non-CA case the UL frequency range, as being smaller than the DL frequency range, need to be used as the starting point to calculate the uplink and downlink test frequencies.

### C.2.3.2	Determination of Low-, Mid- and High-Range for bands supporting asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange, FDL_MidRange and FUL_MidRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq8a |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8b |

If FDL_HighRange is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange, FDL_MidRange and FUL_MidRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq11 |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq12 |

<< End of change>>