3GPP TSG-RAN5 Meeting #91-e	R5-213184r1

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1206 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extension of the RSRPB Rx Beak Peak Search method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM FR2 test cases have two different requirements to select the test direction: - Some tests need to be run in the Rx Beam Peak as defined in TS 38.521-2 Annex K.1.2 - Some tests need to be run in directions fulfilling the EIS spherical coverage criteria in TS 38.521-2 section 7.3.4.3 The current RSRPB based beam peak search method only provides a method for the first group of test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | An enhanced to the RSRPB beam peak search method to cover tests needing the EIS SC criteria has been proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RSRPB method will only cover a part of the RRM test cases and therefore the original Rx BPS method in TS 38.521-2 K.1.2 will still be needed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | I.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: reviewed based in offline feedback |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# I.4	Procedures to search test directions for RRM FR2

## I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search

This section provides RSRPB-based scan to find the Rx beam peak search of the UE for Setup 1, single AoA cases and one of the test directions for Setup 4. It also provides a method to find test directions fulfilling the EIS Spherical Coverage (TS 38.521-2 [18] clause 7.3.4.3) for Setup 2, 3 and 4. Other approaches are not precluded. RSRPB-based peak beam scan greatly reduces the time to find the peak beam position compared to traditional EIS scan using either constant step size or constant density. The procedure can be used with DFF as well as IFF methodology. The procedure is achieved implementing the following steps:

1.	Enable periodic RSRPB reporting from the UE

2.	Set of grid points for the UE scan can be user defined set or entire sphere.

3.	Set the DL SSB_RP (at the center of the QZ) to -95 dBm / SCS. For each grid point, record RSRPB first by connecting SS to the DUT through the measurement antenna with PolLink=  polarization to form the Rx beam towards the measurement antenna.

4.	Wait for BEAM_SELECT_WAIT_TIME before recording the RSRPB reports.

4a	For each of the grid points, calculate the RSRPB result for PolLink= as the linear sum of the 2 RSRPB reports (one per receiver branch).

4b	Repeat Steps 3, 4 and 4a with PolLink= .

5.	Once the grid points scan is completed, sort the grid points based on the linear sum of the RSRPB results for PolLink= and PolLink= .

6.	For the top 10 grid points, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2

7.	Select the grid point with the best REFSENS result among the 10 points from Step 6 as the Rx beam peak for RRM test cases using AoA Setup 1 or 4.

8.	Using the Rx beam peak selected in Step 7, record the RSRPB reports over a 10dB range of DL power levels for PolLink=  and PolLink=  and determine the linearity of the RSRPB reports with respect to the DL power changes. Use the result to linearize the RSRPB results from Step 4a and 4b..

9.	Use the EIS result for the RRM beam peak to offset the linearized RSRPB results to obtain an estimate of the EIS Spherical coverage (SC) map.

10.	From the EIS map in Step 9, select all the points meeting the EIS SC criteria (as defined in TS 38.521-2 [18] Section 7.3.4.3) as potential test directions for AoA Setup 2a, 2b, 3 and 4. In order to confirm that the grid point is suitable, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2 and verify that the EIS SC criteria is met. .

If no grid points are found using this method, fallback to using the method in Annex I.1, I.2, I.3 or I.3A as applicable.

< End of changes >