3GPP TSG-RAN5 Meeting # 91-e	R5-213211r1

# Electronic Meeting,  17th May - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0532 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 RX test cases ETC applicability update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Extreme testing conditions is only affecting Sensitivity measurements and should be extended to other Rx test cases according to R5-213210. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s note reviewed for following test cases: - 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) - 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) - 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) - 7.3A.2.4 Reference sensitivity power level for CA (5DL CA) - 7.3A.2.5 Reference sensitivity power level for CA (6DL CA) - 7.3A.2.6 Reference sensitivity power level for CA (7DL CA) - 7.3A.2.7 Reference sensitivity power level for CA (8DL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage under extreme environmental testing conditions will remain insufficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.1, 7.3A.2.2, 7.3A.2.3, 7.3A.2.4, 7.3A.2.5, 7.3A.2.6, 7.3A.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR approval will require endorsement of Proposals 2 & 3 in R5-213210. - Test Procedures for EIS beam peak Extreme Conditions are covered in R5-213251. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Year added in the coverpage date. |  |  |  |  |  |  |  |  |

<< Start of change 1>>

#### 7.3A.2.1	Reference sensitivity power level for CA (2DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.1.1	Test purpose

Same test purpose as in clause 7.3.2.1.

7.3A.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2DL CA.

7.3A.2.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.1.4	Test description

7.3A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 7.3A.2.1.4.1-1, Table 7.3A.2.1.4.1-2 and Table 7.3A.2.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low range, High range |  |
| Test CA Bandwidth combination as specified in subclause 5.5A |  |  | Maximum aggregated BW (contiguous CA) or Maximum cumulative aggregated BW (non-contiguous CA) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2, NOTE 3) |
| NOTE 1: Full RB allocation shall be used per each SCS and component carrier as specified in Table 7.3A.2.1.4.1-2. NOTE 2: REFSENS refers to Table 7.3A.2.1.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW. NOTE 3: Use single carrier UL when testing reference sensitivity power level for CA. |  |  |  |  |

Table 7.3A.2.1.4.1-2: Downlink Configuration of each RB allocation

| Component Carrier Bandwidth | SCS kHz | LCRBmax | RB allocation (LCRB@RBstart) |
| --- | --- | --- | --- |
| 50MHz | 120 | 32 | 32@0 |
| 100MHz | 120 | 66 | 66@0 |
| 200MHz | 120 | 132 | 132@0 |
| 400MHz | 120 | 264 | 264@0 |
| NOTE 1: CA Bandwidths are checked separately for each NR band, the applicable CA bandwidths are specified in Table 5.3A.4-1. |  |  |  |

Table 7.3A.2.1.4.1-3: Uplink configuration for reference sensitivity, LCRB@RBstart format

| Operating Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n258 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n260 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |
| n261 | 120 | 32@0 | 64@0 | 128@0 | 256@0 | TDD |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.1, and uplink signals according to Annex G.0, G.1 and G.3.1.

4.	The UL Reference Measurement channels are set according to Table 7.3A.2.1.4.1-1, Table 7.3A.2.1.4.1-2 and Table 7.3A.2.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 7.3A.2.1.4.3.

7.3A.2.1.4.2	Test Procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1. Message contents are defined in clause 7.3A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321[28], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.2).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3A.2.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information on PCC for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.3A.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

6.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

7.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2.. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

8.	For each component carrier, perform EIS procedure as stated in Annex K.1.4 to calculate “averaged EIS” by changing the power level of the wanted signal with a step size of 0.2dB. For each power step measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

9.	For each component carrier, compare the dB value of the “averaged EIS” value corresponding to the Rx beam peak direction (same as that found for single carrier in clause 7.3.2) identified in step 8 to the test requirement in Tables 7.3A.2.1.5-3 to Table 7.3A.2.1.5-7. If the EIS value is lower or equal to the value in Tables 7.3A.2.1.5-3 to Table 7.3A.2.1.5-7, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

7.3A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.1.

7.3A.2.1.5	Test requirement

For each component carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2 and A.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5) with peak reference sensitivity specified in Tables 7.3A.2.1.5-3 to 7.3A.2.1.5-7. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3A.2.1.5-1: ΔRIB EIS Relaxation per component carrier for intra-band contiguous CA

| Aggregated Channel BW ‘BWChannel_CA’ (MHz) | ΔRIB (dB) / CC |
| --- | --- |
| BWChannel_CA ≤ 800 | 0.0 |
| 800 < BWChannel_CA ≤ 1200 | 0.5 |

Table 7.3A.2.1.5-2: ΔRIB EIS Relaxation per component carrier for intra-band non-contiguous CA

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) / CC |
| --- | --- |
| ≤ 800 | 0.0 |
| [> 800 and ≤ 1400] | [0.5] |

Table 7.3A.2.1.5-3: Reference sensitivity per component carrier for power class 1

| Operating band | REFSENS (dBm) / CC |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |
| n258 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |
| n260 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n261 | -97.5+TT+ΔRIB | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB |

Table 7.3A.2.1.5-4: Reference sensitivity per component carrier for power class 2

| Operating band | REFSENS (dBm) / CC |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n258 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |
| n260 |  |  |  |  |
| n261 | -94.5+TT+ΔRIB | -91.5+TT+ΔRIB | -88.5+TT+ΔRIB | -85.5+TT+ΔRIB |

Table 7.3A.2.1.5-5: Reference sensitivity per component carrier for power class 3

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |
| n258 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |
| n260 | -85.7+TT+ΔRIB | -82.7+TT+ΔRIB | -79.7+TT+ΔRIB | -76.7+TT+ΔRIB |
| n261 | -88.3+TT+ΔRIB | -85.3+TT+ΔRIB | -82.3+TT+ΔRIB | -79.3+TT+ΔRIB |

Table 7.3A.2.1.5-6a: Test Tolerance per component carrier (Reference sensitivity for power class 3)

| Test Metric | f ≤ 40.8 GHz |
| --- | --- |
| IFF (Quiet Zone size ≤ 30 cm) | 3.37 dB |

Table 7.3A.2.1.5-7: Reference sensitivity per component carrier for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |
| n258 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |
| n260 | -95+TT+ΔRIB | -92+TT+ΔRIB | -89+TT+ΔRIB | -86+TT+ΔRIB |
| n261 | -97+TT+ΔRIB | -94+TT+ΔRIB | -91+TT+ΔRIB | -88+TT+ΔRIB |

<< End of change 1>>

<< Start of change 2>>

#### 7.3A.2.2	Reference sensitivity power level for CA (3DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.2.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3DL CA.

7.3A.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.2.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.2.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 2>>

<< Start of change 3>>

#### 7.3A.2.3	Reference sensitivity power level for CA (4DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.3.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4DL CA.

7.3A.2.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.3.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.3.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 3>>

<< Start of change 4>>

#### 7.3A.2.4	Reference sensitivity power level for CA (5DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.4.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 5DL CA.

7.3A.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.4.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.4.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 4>>

<< Start of change 5>>

#### 7.3A.2.5	Reference sensitivity power level for CA (6DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.5.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6DL CA.

7.3A.2.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.5.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.5.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 5>>

<< Start of change 6>>

#### 7.3A.2.6	Reference sensitivity power level for CA (7DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc.

-

7.3A.2.6.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7DL CA.

7.3A.2.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.6.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.6.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 6>>

<< Start of change 7>>

#### 7.3A.2.7	Reference sensitivity power level for CA (8DL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

7.3A.2.7.1	Test purpose

Same test purpose as in clause 7.3A.2.1.1.

7.3A.2.7.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3DL CA.

7.3A.2.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3A.2.0.

7.3A.2.7.4	Test description

Same test description as in clause 7.3A.2.1.4.

7.3A.2.7.5	Test requirement

For each component carrier, the test requirement is the same as in clause 7.3A.2.1.5.

<< End of change 7>>