3GPP TSG-RAN5 Meeting #91-e	R5-213262r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0150 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Functional Alias to MCPTT Config Documents 5.5.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MCPTT configuration documents needed the Rel-15 feature functional alias to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added functional alias-realted elements to Table 5.5.8.3-1: MCPTT User Profile Defaults and Table 5.5.8.4-1: MCPTT Service Configuration Defaults |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices will not have the proper configurations to support the testing of the functional alias feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Added 2 spaces before "allow-takeover-functional-alias-other-user" - Removed "functional-alias-entry" from Table 5.5.8.4-1 and adjusted the spacing from the elements below it - "px_MCPTT_User_A_ID" changed to "px_MCPTT_ID_User_A" |  |  |  |  |  |  |  |  |

### <START OF CHANGES>

#### 5.5.8.3	MCPTT User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 8.3, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 5.2.

The structure of the configuration document is based on the XML Schema in clause 8.3.2.3 of TS 24.484 [14] and XML "ruleset" schema according to IETF RFC 4745 [103]. To distinguish the schemas the prefix "cp" ("common policy") is used for the ruleset.

Table 5.5.8.3-1: MCPTT User Profile Defaults

| Derivation Path: TS 24.484 [14] clause 8.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| XUI-URI attribute | px_MCPTT_User_XUI_URI |  |  |  |
| user-profile-index attribute | "49" | value arbitrarily selected |  |  |
| Status | true | MCPTT user profile is enabled |  |  |
| ProfileName | px_MCPTT_User_A_Profile_Name | Profile name for the MCPTT user | TS 24.483 [13] clause 5.2.7B |  |
| Common |  |  |  |  |
| index attribute | "0" | Index for the particular MCPTT user profile |  |  |
| MCPTTUserID |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_ID_User_A | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user | TS 24.483 [13] clause 5.2.7 |  |
| UserAlias |  | Alphanumeric aliases of MCPTT user | TS 24.483 [13] clause 5.2.8 |  |
| alias-entry | px_MCPTT_User_A_Alias |  |  |  |
| ParticipantType | px_MCPTT_User_A_ParticipantType | Participant type of the MCPTT user | TS 24.483 [13] clause 5.2.10 |  |
| MissionCriticalOrganization | px_MCX_DomainName_Organization_A | Indicates the organization an MCPTT user belongs to | TS 24.483 [13] clause 5.2.11 |  |
| PrivateCall |  |  |  |  |
| PrivateCallList |  |  |  |  |
| PrivateCallURI[1] |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_ID_User_B | MCPTT user(s) who can be called in a MCPTT private call | TS 24.483 [13] clause 5.2.17 |  |
| display-name | "User B Name" | a human readable name for this User | TS 24.483 [13] clause 5.2.18 |  |
| PrivateCallURI[2] |  |  |  |  |
| index attribute | "1" |  |  |  |
| uri-entry | px_MCPTT_ID_User_C | MCPTT user(s) who can be called in a MCPTT private call | TS 24.483 [13] clause 5.2.17 |  |
| display-name | "User C Name" | a human readable name for this User | TS 24.483 [13] clause 5.2.18 |  |
| PrivateCallProSeUser[1] |  |  |  |  |
| index attribute | "0" |  |  |  |
| DiscoveryGroupID | "1234" | Discovery group ID in the ProSe discovery procedures | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |
| User-Info-ID | "5555" | Prose user Info ID in the ProSe discovery procedures | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |
| PrivateCallProSeUser[2] |  |  |  |  |
| index attribute | "1" |  |  |  |
| DiscoveryGroupID | "1234" | Discovery group ID in the ProSe discovery procedures | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |
| User-Info-ID | "6666" | Prose user Info ID in the ProSe discovery procedures | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |
| EmergencyCall |  |  |  |  |
| MCPTTPrivateRecipient |  |  |  |  |
| entry |  |  |  |  |
| entry-info attribute | "UsePreConfigured" | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. | TS 24.483 [13] clause 5.2.29F |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_ID_User_B | The MCPTT private recipient for an MCPTT emergency private call | TS 24.483 [13] clause 5.2.29B |  |
| display-name | "User B Name" | a human readable name for this User | TS 24.483 [13] clause 5.2.29E |  |
| ProSeUserID-entry |  |  |  |  |
| index attribute | "0" |  |  |  |
| DiscoveryGroupID | "1234" | Discovery group ID in the ProSe discovery procedures | TS 24.483 [13] clause 5.2.29C |  |
| User-Info-ID | "5555" | ProSe user Info ID in the ProSe discovery procedures | TS 24.483 [13] clause 5.2.29D |  |
| MCPTT-group-call |  |  |  |  |
| MaxSimultaneousCallsN6 | "3" | Indicates the maximum number of simultaneously received MCPTT group calls | TS 24.483 [13] clause 5.2.31 |  |
| EmergencyCall |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |
| entry |  |  |  |  |
| entry-info attribute | "UseCurrentlySelectedGroup" | Use currently selected MCPTT group for an on-network MCPTT emergency group call | TS 24.483 [13] clause 5.2.34D |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_Group_A_ID | The group used upon certain criteria on initiation of an MCPTT emergency group call | TS 24.483 [13] clause 5.2.34B |  |
| display-name | px_MCPTT_Group_A_Name | The display name for group used for emergency | TS 24.483 [13] clause 5.2.34C |  |
| ImminentPerilCall |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |
| entry |  |  |  |  |
| entry-info attribute | "UseCurrentlySelectedGroup" | Use currently selected MCPTT group for an on-network MCPTT imminent peril group call | TS 24.483 [13] clause 5.2.39D |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_Group_A_ID | the group used on initiation of an MCPTT imminent peril group call. | TS 24.483 [13] clause 5.2.39B |  |
| display-name | px_MCPTT_Group_A_Name | display name for group used for the imminent peril call | TS 24.483 [13] clause 5.2.39C |  |
| EmergencyAlert |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |
| entry |  |  |  |  |
| index attribute | "0" |  |  |  |
| entry-info attribute | "UseCurrentlySelectedGroup" | Use currently selected MCPTT group for emergency alert | TS 24.483 [13] clause 5.2.43E |  |
| uri-entry | px_MCPTT_Group_A_ID | Indicates the MCPTT group used upon certain criteria on initiation of an MCPTT emergency alert. | TS 24.483 [13] clause 5.2.43B |  |
| display-name | px_MCPTT_Group_A_Name | Optional; name of emergency alert group | TS 24.483 [13] clause 5.2.43D |  |
| Priority | "10" | Indicates the priority of the MCPTT group calls, 0-255 | TS 24.483 [13] clause 5.2.43F |  |
| OffNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCPTTGroupInfo |  |  |  |  |
| entry[1] |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_Group_A_ID | Indicates an off-network MCPTT group for use by an MCPTT user | TS 24.483 [13] clause 5.2.53 |  |
| display-name | px_MCPTT_Group_A_Name | The display name corresponding to off-network group id | TS 24.483 [13] clause 5.2.53A |  |
| User-Info-ID | "5555" | ProSe user info ID | TS 23.303 [68] TS 24.483 [13] clause 5.2.58 |  |
| OnNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCPTTGroupInfo |  |  |  |  |
| entry[1] |  | Group 1 the MCPTT user is allowed to affiliate to |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_Group_A_ID | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. | TS 24.483 [13] clause 5.2.48B4 |  |
| display-name | px_MCPTT_Group_A_Name | The display name for the group | TS 24.483 [13] clause 5.2.48B5 |  |
| MaxAffiliationsN2 | 20 |  |  |  |
| MaxSimultaneousTransmissionsN7 | 20 |  |  |  |
| ImplicitAffiliations |  | Group 1 the MCPTT user is implicitly affiliated to |  |  |
| entry |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_Group_A_ID | indicates a MCPTT group ID to which the MCPTT user is implicitly affiliated to | TS 24.483 [13] clause 5.2.48C4 |  |
| display-name | px_MCPTT_Group_A_Name | display name for implicitly affiliated group | TS 24.483 [13] clause 5.2.48C5 |  |
| PrivateEmergencyAlert |  |  |  |  |
| entry |  |  |  |  |
| entry-info attribute | "UsePreConfigured" | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. | TS 24.483 [13] clause 5.2.48O |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCPTT_ID_User_B | Indicates the default MCPTT user ID to be used upon certain criteria on initiation of an MCPTT private emergency alert for on-network | TS 24.483 [13] clause 5.2.48M |  |
| display-name | px_MCPTT_User_A_Alias | The display name corresponding to private emergency call id | TS 24.483 [13] clause 5.2.48N |  |
| anyExt |  |  |  |  |
| FunctionalAliasList |  |  | TS 24.483 [13] clause 5.2.48W6 |  |
| entry | px_MCPTT_ID_FA_A |  |  |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| allow-create-delete-user-alias | "true" | Indicates authorisation to create and delete aliases of other MCPTT users | TS 24.483 [13] clause 5.2.9 |  |
| allow-private-call | "true" | Indicates the authorisation to make a MCPTT private call | TS 24.483 [13] clause 5.2.13 |  |
| allow-private-call-to-any-user | "true" | indicates the authorisation to make a MCPTT private call to any MCPTT user | TS 24.483 [13] clause 5.2.14 |  |
| allow-manual-commencement | "true" | Indicates the authorisation to make a MCPTT private call with manual commencement | TS 24.483 [13] clause 5.2.20 |  |
| allow-automatic-commencement | "true" | Indicates the authorisation to make a MCPTT private call with automatic commencement | TS 24.483 [13] clause 5.2.21 |  |
| allow-force-auto-answer | "true" | Indicates the authorisation of MCPTT user to force automatic answer for a MCPTT private call | TS 24.483 [13] clause 5.2.22 |  |
| allow-failure-restriction | "false" | Indicates the authorisation to restrict the provision of a notification of call failure reason for a MCPTT private call | TS 24.483 [13] clause 5.2.23 |  |
| allow-private-call-media-protection | "true" | Indicates authorisation to protect confidentiality and integrity of media for MCPTT private calls | TS 24.483 [13] clause 5.2.24 |  |
| allow-private-call-floor-control-protection | "true" | Indicates authorisation to protect confidentiality and integrity of floor control signalling for MCPTT private calls. | TS 24.483 [13] clause 5.2.25 |  |
| allow-emergency-private-call | "true" | Indicates the authorisation to make an MCPTT emergency private call. | TS 24.483 [13] clause 5.2.27 |  |
| allow-cancel-private-emergency-call | "true" | Indicates the authorisation to cancel emergency priority in an MCPTT emergency private call by an authorised MCPTT user | TS 24.483 [13] clause 5.2.28 |  |
| allow-emergency-group-call | "true" | Indicates the authorisation to make an MCPTT emergency group call functionality enabled for MCPTT user | TS 24.483 [13] clause 5.2.33 |  |
| allow-cancel-group-emergency | "true" | Indicates the authorisation to cancel an in progress MCPTT emergency call associated with a group. | TS 24.483 [13] clause 5.2.35 |  |
| allow-imminent-peril-call | "true" | Indicates the authorisation to make an Imminent Peril group call | TS 24.483 [13] clause 5.2.37 |  |
| allow-cancel-imminent-peril | "true" | Indicates the authorisation for in-progress MCPTT imminent peril cancelation | TS 24.483 [13] clause 5.2.38 |  |
| allow-activate-emergency-alert | "true" | Indicates the authorisation to activate an MCPTT emergency alert | TS 24.483 [13] clause 5.2.41 |  |
| allow-cancel-emergency-alert | "true" | Indicates the authorisation to cancel an MCPTT emergency alert | TS 24.483 [13] clause 5.2.42 |  |
| allow-create-group-broadcast-group | "true" | Indicates the authorisation to create a group-broadcast group. | TS 24.483 [13] clause 5.2.46 |  |
| allow-create-user-broadcast-group | "true" | Indicates the authorisation to create a user-broadcast group | TS 24.483 [13] clause 5.2.48 |  |
| allow-offnetwork | "true" | Indicates the authorisation for off-network services | TS 24.483 [13] clause 5.2.50 |  |
| allow-listen-both-overriding-and-overridden | "false" | Indicates whether the MCPTT user is allowed to listen both overriding and override | TS 24.483 [13] clause 5.2.54 |  |
| allow-transmit-during-override | "false" | Indicates whether the MCPTT user is allowed to transmit in case of override (overriding and/or overridden) | TS 24.483 [13] clause 5.2.55 |  |
| allow-off-network-group-call-change-to-emergency | "true" | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT emergency group call | TS 24.483 [13] clause 5.2.56 |  |
| allow-imminent-peril-change | "true" | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT imminent peril group call | TS 24.483 [13] clause 5.2.57 |  |
| allow-regroup | "true" | Indicates whether the MCPTT user is authorised to perform dynamic regrouping operations | TS 24.483 [13] clause 5.2.48D |  |
| allow-presence-status | "true" | Indicates the presence status on the network of this MCPTT user is available | TS 24.483 [13] clause 5.2.48E |  |
| allow-request-presence | "true" | Indicates whether the MCPTT user is authorised to obtain whether a particular MCPTT User is present on the network | TS 24.483 [13] clause 5.2.48F |  |
| allow-private-call-participation | "true" | Indicates whether the MCPTT user is allowed to participate in MCPTT private calls that they are invited to | TS 24.483 [13] clause 5.2.48G |  |
| allow-override-of-transmission | "true" | Indicates whether the MCPTT user is authorised to override transmission in a MCPTT private call | TS 24.483 [13] clause 5.2.48H |  |
| allow-manual-off-network-switch | "true" | Indicates whether the MCPTT user is authorised to manually switch to off-network operation while in on-network operation | TS 24.483 [13] clause 5.2.48I |  |
| anyExt |  |  |  |  |
| allow-query-functional-alias-other-user | "true" | Indicates whether the MCPTT user is authorised to query the functional alias(es) activated by another MCPTT user | TS 24.483 [13] clause 5.2.48W8 |  |
| allow-takeover-functional-alias-other-user | "true" | Indicates whether he MCPTT user is authorised to take over the functional alias(es) previously activated by another MCPTT user | TS 24.483 [13] clause 5.2.48W9 |  |
| allow-location-info-when-talking | "false" | When set to "true" the MCPTT user is authorised to send its location information when it is requesting the floor. When set to "false" the MCPTT user is not authorised to send its location information when it is requesting the floor. | TS 24.483 [13] clause 5.2.48W10 |  |

#### 5.5.8.4	MCPTT Service Configuration

The structure of a user profile document is specified in TS 24.484 [14] clause 8.4, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 7.2.

Table 5.5.8.4-1: MCPTT Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 8.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| common |  |  |  |  |
| min-length-alias | "2" | Indicates minimum length of an alphanumeric identifier (i.e., alias) | TS 24.483 [13] clause 7.2.9 |  |
| broadcast-group |  |  |  |  |
| num-levels-group-hierarchy | "1" | Indicates the number of levels of group hierarchy for group-broadcast groups | TS 24.483 [13] clause 7.2.7 |  |
| num-levels-user-hierarchy | "1" | Indicates the number of levels of user hierarchy for user-broadcast groups | TS 24.483 [13] clause 7.2.8 |  |
| on-network |  |  |  |  |
| emergency-call |  |  |  |  |
| private-cancel-timeout | "PT30M" | 30 minutes |  |  |
| group-time-limit | "PT20M" | 20 minutes |  |  |
| private-call |  |  |  |  |
| hang-time | "PT30S" | 30 seconds |  |  |
| max-duration-with-floor-control | "PT30S" | 30 seconds |  |  |
| max-duration-without-floor-control | "PT20M" | 20 minutes |  |  |
| num-levels-priority-hierarchy | 10 |  |  |  |
| transmit-time |  |  |  |  |
| time-limit | "PT30S" | 30 seconds |  |  |
| time-warning | "PT20M" | 20 minutes |  |  |
| hang-time-warning | "PT20M" | 20 minutes |  |  |
| floor-control-queue |  |  |  |  |
| depth | 5 |  |  |  |
| max-user-request-time | "PT20M" | 20 minutes |  |  |
| fc-timers-counters |  |  |  |  |
| T1-end-of-rtp-media | "PT4S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T3-stop-talking-grace | "PT3S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T7-floor-idle | "PT2S" | Depends on the characteristic of the radio access network | TS 24.380 [10] clause 11 |  |
| T8-floor-revoke | "PT1S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T11-end-of-RTP-dual | "PT4S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T12-stop-talking-dual | "PT30S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T15-conversation | "PT30S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T16-map-group-to-bearer | "PT0.5S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T17-unmap-group-to-bearer | "PT0.2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T20-floor-granted | "PT1S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T55-connect | "PT2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| T56-disconnect | "PT2S" | Default value Value in seconds | TS 24.380 [10] clause 11 |  |
| C7-floor-idle | 10 | Default value | TS 24.380 [10] clause 11 |  |
| C17-unmap-group-to-bearer | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C20-floor-granted | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C55-connect | 3 | Default value | TS 24.380 [10] clause 11 |  |
| C56-disconnect | 3 | Default value | TS 24.380 [10] clause 11 |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | true |  |  |  |
| integrity-protection | true |  |  |  |
| protection-between-mcptt-servers |  |  |  |  |
| allow-signalling-protection | true |  |  |  |
| allow-floor-control-protection | true |  |  |  |
| emergency-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 |  |
| resource-priority-priority | "8" |  | RFC 8101 |  |
| imminent-peril-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 |  |
| resource-priority-priority | "5" |  | RFC 8101 |  |
| normal-resource-priority |  |  |  |  |
| resource-priority-namespace | "mcpttp" |  | RFC 8101 |  |
| resource-priority-priority | "1" |  | RFC 8101 |  |
| anyExt |  |  |  |  |
| functional-alias-list |  |  |  |  |
| functional-alias | px_MCPTT_ID_FA_A |  |  |  |
| max-simultaneous-activations | "1" |  |  |  |
| allow-takeover | "true" |  |  |  |
| mcptt-user-list |  |  |  |  |
| entry | px_MCPTT_ID_User_A |  |  |  |
| off-network |  |  |  |  |
| emergency-call |  |  |  |  |
| private-cancel-timeout | "PT5S" | 5 seconds; Indicates timeout value for the cancellation of an in progress emergency for an MCPTT private call. Values: : 0-65535 s | TS 24.483 [13] clause 7.2.14 |  |
| group-time-limit | "PT5S" | 5 seconds; Indicates time limit for an in progress MCPTT emergency call related to an MCPTT group. Values: 0-65535 s | TS 24.483 [13] clause 7.2.16 |  |
| private-call |  |  |  |  |
| hang-time | "PT5S" | 5 seconds; Indicates hang timer for private calls (with floor control). Values: 0-65535 s | TS 24.483 [13] clause 7.2.13 |  |
| max-duration-with-floor-control | "PT60S" | 60 seconds; Indicates max private call (with floor control) duration. Values: 0-65535 s | TS 24.483 [13] clause 7.2.12 |  |
| num-levels-priority-hierarchy | "4" | Indicates the number of levels of hierarchy for floor control override in off-network. Values: 4-256 | TS 24.483 [13] clause 7.2.17 |  |
| transmit-time |  |  |  |  |
| time-limit | "PT60S" | 60 seconds; Indicates transmit time limit from a single request to transmit in a group or private call. Values: 0-65535 s | TS 24.483 [13] clause 7.2.18 |  |
| time-warning | "PT50S" | 50 seconds; Indicates configuration of warning time before time limit of transmission is reached (off-network). Values: 0-255 s | TS 24.483 [13] clause 7.2.19 |  |
| hang-time-warning | "PT4S" | 4 seconds; Indicates configuration of warning time before hang time is reached (off-network). Values: Values: 0-255 s | TS 24.483 [13] clause 7.2.20 |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcptt-private-call-signalling | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.22 |  |
| mcptt-private-call-media | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.23 |  |
| mcptt-emergency-private-call-signalling | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.24 |  |
| mcptt-emergency-private-call-media | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value | TS 23.303 [68] TS 24.483 [13] clause 7.2.25 |  |
| allow-log-metadata | "true" | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group | TS 24.483 [13] clause 7.2.26 |  |

### <END OF CHANGES>