3GPP TSG-RAN5 Meeting #91-e	R5-213271r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1951 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR1 and E-UTRA OTA signal level testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Non-suitable "Off" cell power level can be defined even though calibration procedure for FR1 or E-UTRA OTA is not available. [TS38.133, 9.3.2] An inter-frequency cell shall be considered detectable when for each relevant SSB: - SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band, … As per above, irrespective of NR Operating band in FR1, -121dbm power level will be undetectable level for the UE. In case of uncalibrate link, uncertainity can be around 22dB. Therefore, -156dBm is selected to ensure as it is undetectable power level for Non-suitable "Off" cell in FR1 OTA. Similarly for, -156dBm is selected for E-UTRA OTA More than one E-UTRA or NR FR1 OTA Cells are allowed to be configured. At a given instance, more than one cell can be activated if test case does not require measurments in idle or connection mode. But, these conditions are not captured. Since (FR2 OTA ) or( EUTRA Cell with FR2 OTA ) or (FR1 OTA Cell with FR2 OTA) are clearly defined, the contents have rearranged to improve the readability. Since Editor’s note or TBD does not any value, these can be removed. If calibration procedure is defined, then, TBD can be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 6.2.2.2-4 and Table 6.2.2.2-7 Specified criteria to allow more than one EUTRA or NR OTA Cell to be configured or activated Rearragned contents of 6.2.2.2 into different section depending upon the configuration within the test case Removed Editor’s note and TBD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA Test environment for FR1 or E-UTRA does not include Non-suitable "Off" cell power level |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Added the restrictions as discussed in R5-213259 Editorial correction and rerranged sections for better readability |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 6.2.2	Signal levels

#### 6.2.2.1	Signal Levels for conducted testing

This section applies for the test cases which uses conducted testing for each configured cell

For NR FR1 cell, the downlink power settings in Table 6.2.2.1-1 and 6.2.2.1-2 are used unless otherwise specified in a test case.

Table 6.2.2.1-1: Default Downlink power levels for FR1 NR cell (5MHz – 25MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz |
| Channel BW Power | 15 | dBm | -63 | -60 | -58 | -57 | -56 |
|  | 30 | dBm | -67 | -63 | -61 | -60 | -59 |
|  | 60 | dBm | N/A | -67 | -65 | -63 | -62 |
| SS/PBCH SSS EPRE | All | dBm/SCS (Note 3) | -88 | -88 | -88 | -88 | -88 |
| Note 1: The channel bandwidth powers are informative, based on -88 dBm/ SCS(SubCarrier Spacing) SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of -88 dBm/SCS(SubCarrier Spacing). |  |  |  |  |  |  |  |

Table 6.2.2.1-2: Default Downlink power levels for FR1 NR cell (30MHz – 100MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30MHz | 40MHz | 50MHz | 60MHz | 80MHz | 90MHz | 100MHz |
| Channel BW Power | 15 | dBm | -55 | -54 | -53 | N/A | N/A | N/A | N/A |
|  | 30 | dBm | -58 | -57 | -56 | -55 | -54 | -53 | -53 |
|  | 60 | dBm | -61 | -60 | -59 | -58 | -57 | -56 | -56 |
| SS/PBCH SSS EPRE | All | dBm/SCS (Note 3) | -88 | -88 | -88 | -88 | -88 | -88 | -88 |
| Note 1: The channel bandwidth powers are informative, based on -88dBm/SCS(SubCarrier Spacing) SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with a power spectrum density of -88dBm/SCS(SubCarrier Spacing). |  |  |  |  |  |  |  |  |  |

With simultaneous transmission of 24 RBs, a maximum of -78dBm/SCS SS/PBCH SSS EPRE can be allocated as cell power level.

The default settings of suitable cells and non-suitable cells for NR are specified in table 6.2.2.1-3.

Cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.1-3.

Table 6.2.2.1-3: Default settings of suitable / non-suitable cells

| Power level type | NR (Note 1-3) |  | E-UTRAN |
| --- | --- | --- | --- |
|  | Unit | Power level |  |
| Serving cell | dBm/SCS | -88 | Table 6.2.2.1-1 [2] |
| Suitable neighbour intra-frequency cell | dBm/SCS | -94 | Table 6.2.2.1-1 [2] |
| Suitable neighbour inter-frequency cell | dBm/SCS | -99 | Table 6.2.2.1-1 [2] |
| Non-suitable cell | dBm/SCS | -115 | Table 6.2.2.1-1 [2] |
| Non-suitable "Off" cell | dBm/SCS | ≤ -145 | Table 6.2.2.1-1 [2] |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the Full RE allocation with no boost or deboost is assumed. SS. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of -88dBm/SCS. Note 4: The default settings assume that the UE is making relative measurements of neighbour cells compared to the serving cell. |  |  |  |

The default signal level uncertainty is specified in table 6.2.2.1-4 for any level specified, unless a tighter uncertainty is specified by a test case in TS 38.523-1 [12].

Table 6.2.2.1-4: SS signal level uncertainty

|  | Absolute signal level uncertainty for each cell | Relative signal level uncertainty between multiple cells |
| --- | --- | --- |
| Intra-frequency | +/-3 dB at each test port | +/-3 dB |
| Inter-frequency | +/-3 dB at each test port | See Note 1 |
| Note 1: For Inter-frequency cells the relative signal level uncertainty between multiple cells is determined by the absolute uncertainty of each cell, and does not have any additional constraint. |  |  |

SS/PBCH SSS EPRE setting should be equal to or higher than -115 dBm except for Non-suitable "Off" cell. The figure is chosen to ensure that for all bands the DL signal is within the RSRP measurement range specified in TS 38.133 [13], taking into account the SS default absolute signal level uncertainty.

NOTE:	(The power spectral density of a white noise source; specified in TS 38.133 [13]) can be assumed to be -Infinity [dBm/SCS] for all intra and inter frequency test cases. It is applicable to both idle mode and connected mode in TS 38.523-1 [12], unless otherwise specified in specific test cases.

##### 6.2.2.1.1	Measurement accuracy and side conditions

RSRP measurement accuracy in RRC_CONNECTED state is specified in table 6.2.2.1.1-1, derived from TS 38.133 [13] clauses 10.1.2 and 10.1.4 selecting Normal condition with maximum Io less than -50 dBm/BWChannel. The ranges and side conditions in TS 38.133 [13] clauses 10.1.2 and 10.1.4 apply. This measurement accuracy is applicable to connected mode test cases specified in TS 38.523-1 [12]. For the serving cell and suitable neighbour cells, the following side conditions shall be satisfied including the effect of signal level uncertainty.

- RSRP  -124 dBm

- RSRP Ês/Iot > -6 dB

- Io: 117.5 dBm/SCS for 15kHz SCS and -114.5 dBm/SCS for 15kHz SCS dBm/SCS … -50 dBm/BWChannel (for absolute and relative RSRP measurement accuracy).

RSRP measurement accuracy in RRC_CONNECTED state is specified in table 6.2.2.1.1-1, derived from TS 38.133 [13] clauses 10.1.2 and 10.1.4 selecting Normal condition.

Table 6.2.2.1.1-1: RSRP measurement accuracy in RRC_CONNECTED state

|  | Absolute RSRP measurement accuracy | Relative RSRP measurement accuracy |
| --- | --- | --- |
| Intra-frequency | +/-8 dB | +/-3 dB |
| Inter-frequency | +/-8 dB | +/-4.5 dB |

#### 6.2.2.2	Signal Levels for OTA testing

##### 6.2.2.2.1	General

This section applies for the test cases which requires at least one of configured Cell is FR2 OTA Cell.

The following assumption is made for OTA Testing:

#### -	AWGN is not configured in the test case

##### 6.2.2.2.2	Signal levels FR2 OTA Cell(s)

For NR FR2 cell, the downlink power settings in Table 6.2.2.2.2-1 are used unless otherwise specified in a test case.

Table 6.2.2.2.2-1: Default Downlink power levels for FR2 NR cell (50MHz - 400MHz)

|  | SCS(kHz) | Unit | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |
| Channel BW Power | 60 | dBm | FFS | FFS | FFS | FFS |
|  | 120 | dBm | -57 | -57 | -57 | -57 |
| SS/PBCH SSS EPRE | All | dBm/SCS | -82 | -82 | -82 | -82 |
| Note 1: The channel bandwidth powers are informative, based on -82 dBm/SCS SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. A maximumRE allocation of 24 simultaneously transmitted RBs with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |

The default settings of suitable cells and non-suitable cells for NR FR2 are specified in table 6.2.2.2.2-2.

NR FR2 cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2.2-2.

Table 6.2.2.2.2-2: Default settings of suitable / non-suitable FR2 NR cells

| Power level type | NR (Note 1-3) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -82 |
| Suitable neighbour intra-frequency cell | dBm/SCS | -91 |
| Suitable neighbour inter-frequency cell | dBm/SCS | -91 |
| Non-suitable cell | dBm/SCS | -100 |
| Non-suitable "Off" cell | dBm/SCS | ≤-139 |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration (µ) with the same power spectrum density in dBm/SCS (SubCarrier Spacing). |  |  |  |

The Test system default signal level uncertainty is specified in tables 6.2.2.2.2-3 and 6.2.2.2.2-4 for any level specified, unless a tighter uncertainty is specified by a test case in TS 38.523-1 [12].

Table 6.2.2.2.2-3: SS Absolute FR2 NR signal level uncertainty

|  | Absolute signal level uncertainty |
| --- | --- |
| At each frequency | +/-6 dB at centre of the quiet zone |

Table 6.2.2.2.2-4: SS Relative FR2 NR signal level uncertainty

|  | Relative signal level uncertainty between any two SS EPRE levels at the same frequency |
| --- | --- |
| At each frequency | +/-2.0 dB |

##### 6.2.2.2.3	NR FR1 OTA Cell(s) with FR2 OTA Cell(s)

For NR cell in FR1 with FR2 NR, since the NR FR1 OTA link is uncalibrated in the signalling test setup, -88dBm/SCS should be applied as defined in the table 6.2.2.1-1 and table 6.2.2.1-2. -88dBm/SCS is suggested value and it is left to the TE vendor to ensure that NR cell power level fulfils the cell selection criteria.

The default settings of suitable cells and non-suitable cells for NR in FR1 with FR2 NR are specified in table 6.2.2.2.3-1.

NR Cells in FR1 with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2.3-1.

Table 6.2.2.2.3-1: Default settings of suitable / non-suitable NR cells in FR1 with NR FR2

| Power level type | NR (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/SCS | -88 |
| Non-suitable "Off" cell | dBm/SCS | ≤-156 |
| Note 1: The power level is specified in terms of SS/PBCH SSS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

The following assumptions are considered for the test cases configures NR FR1 Cell

-	Multiple inter-frequency Cells can be configured.

-	Only two power levels are allowed:  Serving Cell power level or Non-suitable "Off" cell (as defined in Table 6.2.2.2.3-1)

-	If NR FR1 Cell does not require

-	Cell Reselection measurements

-	Connected Mode measurements

-	Cell Selection between two active cells

-	Multiple Signal Levels

-	Multiple NR FR1 cells can be activated;

-	Else

-	 No more than one NR FR1 cell is activated.

##### 6.2.2.2.4	EUTRA OTA Cell(s) with FR2 OTA Cell(s)

For E-UTRA cell with FR2 NR, since the LTE OTA link is uncalibrated in the signalling test setup, the table 6.2.2.2.4-1 provides only suggestive value. It is left to the TE vendor to ensure that LTE cell power level fulfils the cell selection criteria.

Table 6.2.2.2.4-1: Default Downlink power levels for E-UTRA cells with NR FR2

|  | Unit | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Number of RBs |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Channel BW Power | dBm | -77 | -73 | -71 | -68 | -66 | -65 |
| RS EPRE | dBm/15kHz | -96 | -96 | -96 | -96 | -96 | -96 |
| Note 1: The channel bandwidth powers are informative, based on -96 dBm/15kHz RS_EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |  |  |  |  |

The default setting of suitable cells for E-UTRA with FR2 NR are specified in table 6.2.2.2.4-2.

E-UTRA Cells with FR2 NR which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.2.4-2.

Table 6.2.2.2.4-2: Default settings of suitable / non-suitable E-UTRA cells with NR FR2

| Power level type | E-UTRAN (Note 1-2) |  |
| --- | --- | --- |
|  | Unit | Power level |
| Serving cell | dBm/15KHz | -96 |
| Non-suitable "Off" cell | dBm/15KHz | ≤-156 |
| Note 1: The power level is specified in terms ofcell-specific RS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: The power level is specified at the centre of quiet zone. |  |  |  |

The following assumptions are considered for the test cases configures E-UTRA Cell

-	Multiple inter-frequency Cells should be configured.

-	Only two power levels are allowed:  Serving Cell power level or Non-suitable "Off" cell (as defined in Table 6.2.2.2.4-2)

-	If NR FR1 Cell does not require

-	Cell Reselection measurements

-	Connected Mode measurements

-	Cell Selection between two active cells

-	Multiple Signal Levels,

-	Multiple E-UTRA cells can be activated;

-	Else

-	 No more than one E-UTRA cell is activated .

<End of Change>