3GPP TSG-RAN5 Meeting # 91-e	R5-213283r3

# Electronic Meeting,  17th May - 28th May 2021

Title:						On FR2 Carrier Aggregation Minimum Output power

Source:	Keysight Technologies UK Ltd

Agenda Item:		5.3.2.17

Document for:		Endorsement

# 1.	Introduction

# Current test specifications for FR2 NSA ([1]) include one editor note for test cases under section 6.3B.1.4_1	Minimum output power for inter-band EN-DC including FR2 (>2 CCs). Such editor’s note indicates that the corresponding test cases are incomplete because corresponding CA test cases in 38.521-2 ([2]) are incomplete.

# When checking 38.521-2 ([2]) test cases status under section 6.3A.1	Minimum output power for CA, the following editor’s notes can be found:

# This discussion paper proposes a way forward to progress on those test cases definition.

# 2.	Discussion

- Core requirements for standalone in case of Intra-band contiguous and non-contiguous are defined per component carrier ([3]):

- Test requirements for non-standalone mode ([1]) indicate that only one E-UTRA cell is required for EN-DC including FR2 as E-UTRA cell is only a functional link:

# 2.1 Testability, measurement uncertainties and test tolerances

# Taking above general statements into account, the testability of CA test cases is the same as the single carrier ones. Additionally, measurement uncertainties and test tolerances can be also leveraged from single CC test cases.

# Proposal 1: Remove editor’s note related to testability in test cases under section 6.3A.1 in 38.521-2.

# Proposal 2: Define measurement uncertainties and test tolerance same as single carrier test case and remove editor’s note related to them.

# These changes have been already included in [8].

# Same relaxations as in single CC test case should be used.

# Proposal 3: Leverage test requirements relaxations from single CC test requirements.

# 2.2 Tests configuration

# Regarding test configuration for these test cases, it is defined as same configuration for all CCs and equal to the single carrier test configuration.

# Considering that requirements are defined per component carrier such configuration makes sense and no additional revision is required.

# Proposal 4:  Remove editor’s note related to test configuration.

# 2.3 UE Beam lock for UL CA

# In RAN5#87, [4] was agreed. Hence editor’s note related to UBF can also be removed.

# Proposal 5:  Remove editor’s note related to UBF.

# 2.4 Beam Peak for UL CA

# In RAN5#87, [5] was discussed and the following proposal was endorsed:

# In RAN5#88, [6] and [7] were agreed:

# Hence the applicability of the beam peak of single UL has already been clarified.

# Proposal 6: Remove editor’s note related to Beam Peak of single UL.

# 2.5 Maximum UL CA aggregated bandwidths

# As test system complexity could increase for maximum aggregated bandwidths higher than 800 MHz (as UL CA  aggregated BW is lower than DL CA aggregated BW), all above proposals (proposal 1 to proposal 6) apply for a maximum aggregated bandwidth of up to 800 MHz.

# Proposal 7: Add editor’s note indicating that MU and TT for UL CA aggregated bandwidth higher than 800 MHz will require further study.

# 3.	Conclusion

# This document analyses open issues for FR2 CA Minimum output power test cases and suggests the following proposals to complete the test cases:

# Proposal 1: Remove editor’s note related to testability in test cases under section 6.3A.1 in 38.521-2.

# Proposal 2: Define measurement uncertainties and test tolerance same as single carrier test case and remove editor’s note related to them.

# Proposal 3: Leverage test requirements relaxations from single CC test requirements.

# Proposal 4:  Remove editor’s note related to test configuration.

# Proposal 5:  Remove editor’s note related to UBF.

# Proposal 6: Remove editor’s note related to Beam Peak of single UL.

# Proposal 7: Add editor’s note indicating that MU and TT for UL CA aggregated bandwidth higher than 800 MHz will require further study.

# 4.	References

[1] 38.521-3 v17.0.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[2] 38.521-2 v16.7.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[3] 38.101-2 v17.1.0 “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[4] R5-202884, “CR to 38.509 to clarify UBF for Intra-Band Carrier Aggregation “, Keysight Technologies UK Ltd, RAN5#87-e

[5] R5-202920, “On Beam Peak Searches for Intra-Band Contiguous CA “, Keysight Technologies UK Ltd, RAN5#87-e

[6] R5-204861, “CR to 38.521-2 to allow vendor declarations related to beam peak searches ",  Keysight Technologies UK Ltd, RAN5#88-e

[7] R5-204710, “CR to 38.508-2 to allow vendor declarations related to beam peak searches ",  Keysight Technologies UK Ltd, RAN5#88-e

[8] R5-213216, “Measurement Uncertainties updates for FR2 Extreme Testing Conditions",  Keysight Technologies UK Ltd, RAN5#91-e