3GPP TSG-RAN5 Meeting # 91-e	R5-213291r1

# Electronic Meeting,  17th May - 28th May

Title:						On FR2 EVM measurement uncertainties and test tolerances

Source:						Keysight Technologies

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

FR2 Error Vector Magnitude measurement uncertainty and test tolerances are still undefined.

This document provides some analysis and proposals so this topic can be progressed for power class 3 devices.

# 2.	Discussion

## 2.1.	EVM Test procedure analysis

## Test Procedure:

- EVM test procedure describes 3 algorithms for PUSCH, PUCCH and PRACH EVM measurements.

- UL transmission powers have been maximized to minimize testability issues due to high FR2 OTA losses.

- Lowest and highest channel bandwidths are tested.

## Initial Conditions:

- EVM PUSCH: Current FR2 EVM initial conditions for power class 3 devices include test points for all waveforms, modulations and RB llocations for Inner_Full and Outer_Full. Lowest UL signal power level will be related to maximum MPR.

- EVM PUCCH: UL power levels will be similar to those used in PUSCH although only lower modulation order applies.

- Wider channel bandwidths will imply higher Noise level integration.

## 2.2	Measurement Uncertainties and testability

## FR2 UL EVM measurement uncertainties will be mainly driven by the SNR available at TE receiver, which will produce a residual EVM.

## For PUSCH, considering Outer_Full RB allocations, the worst-case MU will be for a marginal UE using the whole MPR relaxation.

## Considering ACLR Noise Floor defined by TE vendors (-7.6dBm for FR2a and -5.5 dBm for FR2b):

- EVM for DFT-s-OFDM for Outer_Full RB allocations for a Power class 3 Device under FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | Residual EVM for CBW=50 MHz | Residual EVM for CBW=100 MHz | Residual EVM for CBW=200 MHz | Residual EVM for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | % | 30+TT | 30+TT | 1.8 % | 2.6 % | 3.7 % | 6.2 % |
| QPSK | % | 17.5+TT | 17.5+TT | 1.8 % | 2.6 % | 3.7 % | 6.2 % |
| 16 QAM | % | 12.5+TT | 12.5+TT | 2.6 % | 3.7 % | 5.2 % | 9.2% |
| 64 QAM | % | 8+TT | 8+TT | 4.1 % | 5.8 % | 8.2 % | 13% |

- EVM for DFT-s-OFDM for Inner_Full RB allocations Region 1 for a power class 3 device under FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | Residual EVM for CBW=50 MHz | Residual EVM for CBW=100 MHz | Residual EVM for CBW=200 MHz | Residual EVM for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | % | 30+TT | 30+TT | 1.3 % | 1.8 % | 2.5 % | 3.5 % |
| QPSK | % | 17.5+TT | 17.5+TT | 1.3 % | 1.8 % | 2.5 % | 3.5 % |
| 16 QAM | % | 12.5+TT | 12.5+TT | 2.2 % | 3.1 % | 4.4 % | 9.2 % |
| 64 QAM | % | 8+TT | 8+TT | 3.5 % | 4.9 % | 6.9 % | 13 % |

- EVM for CP-OFDM for Outer_Full RB allocations for a Power class 3 Device under FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | Residual EVM for CBW=50 MHz | Residual EVM for CBW=100 MHz | Residual EVM for CBW=200 MHz | Residual EVM for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | % | 17.5+TT | 17.5+TT | 2.8 % | 3.9 % | 5.5 % | 9.8 % |
| 16 QAM | % | 12.5+TT | 12.5+TT | 3.5 % | 4.9 % | 6.9 % | 13 % |
| 64 QAM | % | 8+TT | 8+TT | 5.2 % | 7.3 % | 10.3 % | 17.3 % |

- EVM for CP-OFDM for Inner_Full RB allocations Region 1 for a power class 3 device under FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | Residual EVM for CBW=50 MHz | Residual EVM for CBW=100 MHz | Residual EVM for CBW=200 MHz | Residual EVM for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | % | 17.5+TT | 17.5+TT | 2.6 % | 3.7 % | 5.2 % | 9.8 % |
| 16 QAM | % | 12.5+TT | 12.5+TT | 3.5 % | 4.9 % | 6.9 % | 13 % |
| 64 QAM | % | 8+TT | 8+TT | 5.2 % | 7.3 % | 10.3 % | 17.3 % |

Observation 1: Above residual EVM tables show that there might be still testability issues for some of the test points defined depending on real MPR in the device for FR2a.

Observation 2: Similar testability issues are expected for PUCCH although only for lower order modulations.

Proposal 1: Remove test points in red in tables above due to testability issues for FR2a.

Proposal 2: Add Relaxation equal to 1/5* Applicable Average EVM Level to test point in orange in tables above for FR2a.

Proposal 3: Keep relaxation equal to 0 for test points in green in tables above and define MU based on those residual EVM values for FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | EVM MU for CBW=50 MHz | EVM MU for CBW=100 MHz | EVM MU for CBW=200 MHz | EVM MU for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | % | 30+TT | 30+TT | 4 % | 4 % | 6% | 6.5 % DFT-s-OFDM |
| QPSK | % | 17.5+TT | 17.5+TT |  |  |  |  |
| 16 QAM | % | 12.5+TT | 12.5+TT |  | 5 % | 5.5 % DFT-s-OFDM | N/A |
| 64 QAM | % | 8+TT | 8+TT | 4 % DFT-s-OFDM, Inner full | N/A | N/A | N/A |

# 3.	Conclusions

This document reviews testability discussions on FR2 ON/OFF time mask test cases and provides the following observations and proposals to progress on this topic:

- Observation 1: Above residual EVM tables show that there might be still testability issues for some of the test points defined depending on real MPR in the device for FR2a.

- Observation 2: Similar testability issues are expected for PUCCH although only for lower order modulations.

- Proposal 1: Remove test points in red in tables above for FR2a.

- Proposal 2: Add Relaxation equal to 1/5* Applicable Average EVM Level to test point in orange in tables above for FR2a.

- Proposal 3: Keep relaxation equal to 0 for test points in green in tables above and define MU based on those residual EVM values for FR2a:

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level | EVM MU for CBW=50 MHz | EVM MU for CBW=100 MHz | EVM MU for CBW=200 MHz | EVM MU for CBW=400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | % | 30+TT | 30+TT | 4 % | 4 % | 6% | 6.5 % DFT-s-OFDM |
| QPSK | % | 17.5+TT | 17.5+TT |  |  |  |  |
| 16 QAM | % | 12.5+TT | 12.5+TT |  | 5 % | 5.5 % DFT-s-OFDM | N/A |
| 64 QAM | % | 8+TT | 8+TT | 4 % DFT-s-OFDM, Inner full | N/A | N/A | N/A |

# 4.	References

[1] 38.521-2 v16.7.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[2 R5-211104, “Correction of parameters of FR2 ACLR test cases”, Anritsu, RAN5#90e