3GPP TSG-RAN5 Meeting #91-e	R5-213427r1

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1341 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial update of PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some extra symbols were added to the PICS items. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some PICS items and fonts were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent PICS items will exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.4 and A.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-212114 with one revision. r1: uppercase in PICS A.4.4-1/158 corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-212114 -> R5-213427 |  |  |  |  |  |  |  |  |

[Start of change]

A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of USIM removal without power down |  | Rel-8 | pc_USIM_Removal |  |
| 2 | Support of Allowed CSG list | 36.331 Annex B.2 | Rel-8 | pc_Allowed_CSG_list | For Rel-8: CSG autonomous search is optional. For Rel-9 or later releases: CSG autonomous search is mandatory for UEs supporting CSG full functionality. |
| 3 | Support of Short Message Service (SMS) MT over SGs | 23.272, 8.2.4, 8.2.5 | Rel-8 | pc_SMS_SGs_MT |  |
| 4 | Support of Short Message Service (SMS) MO over SGs | 23.272, 8.2.2, 8.2.3 | Rel-8 | pc_SMS_SGs_MO |  |
| 5 | Support of ISR | 23.401, 4.3.5.6 | Rel-8 | pc_ISR |  |
| 6 | Support of Mobility management based on Dual-Stack Mobile IPv6 | 24.303 | Rel-8 | pc_DSMIPv6 |  |
| 7 | Support for being configured to discover the Home Agent address via DNS | 24.303 | Rel-8 | pc_HAAddress_via_DNS |  |
| 8 | Support of inter-RAT PS handover to E-UTRA (FDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eFDD |  |
| 9 | Support of EMM information message | 24.301, 5.4.5.3 | Rel-8 | pc_EMM_Information |  |
| 10 | Support for being configured to discover the Home Agent address via DHCPv6 | 24.303 | Rel-8 | pc_HAAddress_via_DHCPv6 |  |
| 11 | Void |  |  |  |  |
| 12 | Upon reception of ‘Full name for network’ information the UE stores/updates the network full name | 24.301, 8.2.13 | Rel-8 | pc_FullNameNetwork |  |
| 13 | Upon reception of ‘Short name for network’ information the UE stores/updates the network short name | 24.301, 8.2.13 | Rel-8 | pc_ShortNameNetwork |  |
| 14 | Upon reception of ‘Local time zone’ information the UE stores/updates the local time zone | 24.301, 8.2.13 | Rel-8 | pc_LocalTimeZone |  |
| 15 | Upon reception of ‘Universal time and local time zone’ information the UE stores/updates the universal time and local time zone | 24.301, 8.2.13 | Rel-8 | pc_UniversalAndLocalTimeZone |  |
| 16 | Void |  |  |  |  |
| 17 | Void |  |  |  |  |
| 18 | Support of ESM UE requested bearer resource allocation procedure | 24.301, 6.5.3 | Rel-8 | pc_ESM_MO_Bearer_Allocation |  |
| 19 | Support of ESM UE requested bearer resource modification procedure | 24.301, 6.5.4 | Rel-8 | pc_ESM_MO_Bearer_Modification |  |
| 20 | Support of ETWS message | 23.401, 5.12.2 | Rel-8 | pc_ETWS_message |  |
| 21 | Supports E-UTRAN Neighbour Cell measurements and MS autonomous cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_meas |  |
| 22 | Support for being configured to request the IPv6 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv6HAAddress_DuringAttach |  |
| 23 | Support for being configured to request the IPv4 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv4HAAddress_DuringAttach |  |
| 24 | Void |  |  |  |  |
| 25 | Support of IMS | 24.229 | Rel-8 | pc_IMS |  |
| 26 | Supports of disabling the EPS services | 24.301, 3.1, 5.5.2.1 | Rel-8 | pc_EPS_Services_Disable |  |
| 27 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach with detach type set to "re-attach required" | 24.301, 5.5.2.3.2 | Rel-8 | pc_Automatic_Re_Attach |  |
| 28 | Support of Compressed mode | 25.306 | Rel-8 | pc_UTRA_CompressedModeRequired |  |
| 29 | Support of GERAN to E-UTRAN PS Handover | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_PSHO |  |
| 30 | Support for multiple PDN connections | 23.401, 5.10 | Rel-8 | pc_Multiple_PDN |  |
| 31 | Support of use of the UTRA system information provided by RRCConnectionRelease upon redirection | 36.306 | Rel-9 | pc_eRedirectionUTRA |  |
| 32 | Support for SRVCC from E-UTRAN to GERAN/UTRAN | 24.301, 8.2.4 | Rel-8 | pc_SRVCC_GERAN_UTRAN |  |
| 33 | Support for VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" | 24.173, 24.229, 26.114, 5.2.1, GSMA PRD IR.92 | Rel-8 | pc_VoLTE | Multimedia telephony service participant initiating a speech session. UE supports sending DTMF events over RTP. |
| 34 | Support of detach for non-EPS services | 24.301, 5.5.2.1 | Rel-8 | pc_IMSI_Detach |  |
| 35 | Support for establishing the emergency call using the CS domain in UTRA after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_UTRA |  |
| 36 | Support for establishing the emergency call using the CS domain in GERAN after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_GERAN |  |
| 37 | Support for establishing the emergency call using the CS domain in 1xRTT after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_1xRTT |  |
| 38 | Support for EDTM | 44.060 8.9.1.2 | Rel-8 | pc_EDTM |  |
| 39 | Supports CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_measreporting_CCN |  |
| 40 | Support for ROHC profile0x0001 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0001 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 41 | Support for ROHC profile0x0002 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0002 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 42 | Support for ROHC profile0x0003 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0003 |  |
| 43 | Support for ROHC profile0x0004 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0004 |  |
| 44 | Support for ROHC profile0x0006 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0006 |  |
| 45 | Support for ROHC profile0x0101 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0101 |  |
| 46 | Support for ROHC profile0x0102 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0102 |  |
| 47 | Support for ROHC profile0x0103 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0103 |  |
| 48 | Support for ROHC profile0x0104 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0104 |  |
| 49 | Support of manual CSG selection | 36.331, Annex B2 | Rel-8 | pc_Manual_CSG_Selection | For Rel-8: manual CSG selection is optional. For Rel-9 or later releases: manual CSG selection is mandatory for UEs supporting CSG full functionality. |
| 50 | Support of semi-persistence scheduling | 36.331, Annex B1 | Rel-8 | pc_Semi_Persistence_Scheduling | For Rel-8: semi-persistence scheduling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases: semi-persistence scheduling is mandatory if pc_FeatrGrp_29 is set to true. |
| 51 | Support of TTI bundling | 36.331, Annex B1 | Rel-8 | pc_TTI_Bundling | For Rel-8: TTI bundling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases TDD: TTI bundling is mandatory if pc_FeatrGrp_28 is set to true. For Rel-9 or later releases FDD: TTI bundling is mandatory. |
| 52 | Support for inter-RAT PS handover from E-UTRAN to GERAN. | 36.306, 4.3.7.11 | Rel-8 | pc_E_UTRAN_2_GERAN_PSHO |  |
| 53 | Support of inter-RAT PS handover to E-UTRA (TDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eTDD |  |
| 54 | Support for UE requested modification of network allocated TFTs | 24.301, 6.5.4 | Rel-8 | pc_ESM_UE_Modification_NW_TFT |  |
| 55 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach even though UE has initiated a detach procedure with detach type set to "EPS detach" or "combined EPS/IMSI detach" | 24.301, 5.5.2.2.4 | Rel-8 | pc_Re_Attach_AfterDetachColl |  |
| 56 | Support of Squal based cell reselection to UTRAN from E-UTRAN | 25.304, 5.2.6.1.4a | Rel-9 | pc_Squal_based_CellReselection_to_UTRAN_from_E_UTRAN |  |
| 57 | Support of Squal based cell reselection to E-UTRAN from UTRAN | 36.304, 5.2.4.5 | Rel-9 | pc_Squal_based_CellReselection_to_E_UTRAN_from_UTRAN |  |
| 58 | Support of CMAS message | 36.331, 5.2.1.5 | Rel-9 | pc_CMAS_Message |  |
| 59 | Void |  |  |  |  |
| 60 | Void |  |  |  |  |
| 61 | Void |  |  |  |  |
| 62 | Support of logged measurements in RRC_IDLE | 36.306, 4.3.13.1 | Rel-10 | pc_LoggedMeasurementsIdle |  |
| 63 | Support of standalone GNSS receiver to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE | 36.306, 4.3.13.2 | Rel-10 | pc_standaloneGNSS_Location |  |
| 64 | Support of automatic re-activation of the EPS bearer(s) | 24.301 | Rel-8 | pc_Automatic_EPS_Re_Attach |  |
| 65 | Support of UTRAN ANR | 25.306, 4.15 | Rel-10 | pc_UTRAN_ANR |  |
| 66 | Void |  |  |  |  |
| 67 | Support of PWS upper layer | 23.041 clause 9.1.3.4.2 | Rel-9 | pc_PWS_UpperLayer |  |
| 68 | Support of automatic PDN connectivity in EUTRAN (i.e. UE upper layer provides PDN connectivity parameters) | 24.301, 6.5.1.1 | Rel-8 | pc_Auto_PDN_Connectivity |  |
| 69 | Support user initiated PLMN reselection in automatic mode | 23.122 | Rel-8 | pc_UserInitiatedPLMN_Reselection |  |
| 70 | Support of UL MIMO | 36.306, clause 4.3.4.6 | Rel-10 | pc_UL_MIMO |  |
| 71 | Support of ESM Notification procedure | 24.301, 6.6.2 | Rel-9 | pc_ESM_Notification |  |
| 72 | Support of sending concatenated multiple Short Message over SGs | 23.272, 8.2.3a | Rel-9 | pc_SMS_SGs_Multi_MO |  |
| 73 | Support TAU in connected mode | 23.221, 7.2a | Rel-8 | pc_TAU_connected_in_IMS | Applicable when configured to pc_voice_PS_1_CS_2 and pc_Attach |
| 74 | Support TAU in idle mode | 23.221, 7.2a | Rel-8 | pc_TAU_idle_in_IMS |  |
| 75 | Support of Intra Frequency Proximity Indication | 36.306, clause 4.3.10.1 | Rel-9 | pc_IntraFreq_ProximityIndication |  |
| 76 | Support of Inter Frequency Proximity Indication | 36.306, clause 4.3.10.2 | Rel-9 | pc_InterFreq_ProximityIndication |  |
| 77 | Support of UTRAN Proximity Indication | 36.306, clause 4.3.10.3 | Rel-9 | pc_UTRAN_ProximityIndication |  |
| 78 | Support of Access Technology Indication in available PLMNs list | 23.122, clause 4.4.3.1.2 | Rel-8 | pc_Available_PLMNs_AcT_Ind |  |
| 79 | Support of Squal based cell reselection between E-UTRAN and GERAN | 36.304, clause 5.2.4.5, 45.008, clause 6.6.6 | Rel-9 | pc_Squal_based_CellReselection_between_E_UTRAN_and_GERAN |  |
| 80 | Support of AttachWithIMSI | 24.368, 5.4 | Rel-10 | pc_eAttachWithIMSI |  |
| 81 | Support of T3412 extended value IE | 24.301, 8.2.1.12, 8.2.26.15 | Rel-10 | pc_T3412Extended |  |
| 82 | Void |  |  |  |  |
| 83 | Void |  |  |  |  |
| 84 | Support of MinimumPeriodicSearchTimer | 23.122, 4.4.3.3 | Rel-10 | pc_eMinimumPeriodicSearchTimer |  |
| 85 | Support of delivery of rachReport upon request from the network | 36.306, 4.3.12.1 | Rel-9 | pc_Rach_Report |  |
| 86 | Support of Power Preference Indication | 36.306 4.3.15.3, 36.331, 5.6.10 | Rel-11 | pc_PPI_Support |  |
| 87 | Support of ePDCCH | 36.306, 4.3.4.18 36.331, 6.3.6 | Rel-11 | pc_ePDCCH |  |
| 88 | Void |  |  |  |  |
| 89 | Void |  |  |  |  |
| 90 | Void |  |  |  |  |
| 91 | Support of Extended Access Barring Override | 24.368, 5.10, 31.102, 4.2.94 | Rel-11 | pc_EAB_override |  |
| 92 | Void |  |  |  |  |
| 93 | Upon reception of ‘Daylight saving time’ information the UE stores/updates the daylight saving time | 24.301, 8.2.13 | Rel-8 | pc_DaylightSavingTime |  |
| 94 | Support of Radio Link Failure Report for inter-RAT MRO | 36.306, clause 6.10.1 | Rel-11 | pc_RLF_ReportForInterRAT_MRO |  |
| 95 | Support of IPv4 | 23.221, 5.1 | Rel-5 | pc_IPv4 |  |
| 96 | Support of IPv6 | 23.221, 5.1 | Rel-5 | pc_IPv6 |  |
| 97 | Support of Automatic Mode EF_LRPLMSI PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_EF_LRPLMNSI_Automatic_Mode_Exception |  |
| 98 | Support of Manual Mode PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_Manual_Mode_Exception |  |
| 99 | Support of ZUC algorithm | 33.401,5.1.3.2 | Rel-11 | pc_ZUC |  |
| 100 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring UMTS cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.3 | Rel-9 | pc_SI_Neighbour_UMTS_Autonomous_Gaps |  |
| 101 | Support of reception of requestedFrequencyBands | 36.306, 4.3.5.6 | Rel-11 | pc_reqFreqBands |  |
| 102 | Support of more than 128 CA Band Combinations | 36.331, 5.6.3.3, 6.4 | Rel-11 | pc_More_Than_128_CAbandComb |  |
| 103 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring intra-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.1 | Rel-9 | pc_SI_Neighbour_intraFreq_Autonomous_Gaps |  |
| 104 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring inter-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.2 | Rel-9 | pc_SI_Neighbour_interFreq_Autonomous_Gaps |  |
| 105 | Support of Type B Half-duplex FDD operation | 36.211, 6.2.5 36.306, 4.2.6 | Rel-12 | pc_FDD_TypeB_HalfDuplex | Only applicable for UE supporting Category 0 and Category M1 and M2. When set transmission scheduling is performed in accordance to Half-Duplex operation Type B else in accordance to Full-Duplex operation. |
| 106 | Void |  |  |  |  |
| 107 | Support of enhanced HARQ pattern for TTI bundling operation for FDD | 36.306 4.3.4.27 | Rel-12 | pc_eHARQ_Pattern_for_TTI_bundling |  |
| 108 | Support of tdd-FDD-CA-PCellDuplex-r12 with the first bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_TDD_PCell |  |
| 109 | Support of tdd-FDD-CA-PCellDuplex-r12 with the second bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_FDD_PCell |  |
| 110 | Support of ProSe direct communication | 36.306, 4.3.21.1 | Rel-12 | pc_commSupportedBands | 36.306, 4.3.21.1: If a UE supports sidelink communication on at least one band, the UE shall support sidelink communication transmission based on UE autonomous resource selection and eNB scheduled resource allocation. |
| 111 | Support of ProSe direct discovery | 36.306, 4.3.21.3 | Rel-12 | pc_discSupportedBands |  |
| 112 | Support of ProSe EPC level discovery | 24.334, 7.2 | Rel-12 | pc_Prose_EPC_Discovery |  |
| 113 | Support of ProSe discovery SLSS transmission and reception | 36.306, 4.3.21.6 | Rel-12 | pc_discSLSS |  |
| 114 | Support of uplink 64QAM | 36.306, 4.3.4.39 | Rel-12 | pc_UL_64QAM |  |
| 115 | Support of Power Saving Mode | 24.301, 5.3.11 | Rel-12 | pc_ePSM |  |
| 116 | Support of downlink 256QAM | 36.306, 4.1, 4.1A | Rel-12 | pc_DL_256QAM | Applicable for UEs of category 11-12 and UEs of DL category 11 and onwards. It is mandatory for UEs of DL category 13-14. |
| 117 | Support for GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" | IEEE Std 802.11 GSMA PRD IR.51 | Rel-11 | pc_WLAN_voice | The IR.51 is based on 3GPP Rel-11. |
| 118 | Support of CSI-RS based discovery signals measurement | 36.306 4.3.6.10 | Rel-12 | pc_CSI_RS_DS_Meas |  |
| 119 | Support of simultaneous transmission of EUTRA and sidelink communication (on different carriers) in all bands for which the UE indicated simultaneous sidelink and EUTRA support in a band combination (using commSupportedBandsPerBC) | 36.306, 4.3.21.2 | Rel-12 | pc_commSimultaneousTx |  |
| 120 | ProSe Discovery for Public Safety supported | 24.334, 4.1 | Rel-12 | pc_disc_public_safety | If Support of ProSe direct discovery (entry 111) is indicated then if the present entry is set to FALSE this shall be understood as ProSe Discovery for non-Public Safety supported |
| 121 | Support of extended DRX | 24.301, 5.3.12 | Rel-13 | pc_edrx |  |
| 122 | Support of CE mode A | 36.306, 4.3.29.1 | Rel-13 | pc_CEmodeA | Mandatory for CAT M1 and M2 UEs |
| 123 | Support of CE mode B | 36.306, 4.3.29.2 | Rel-13 | pc_CEmodeB |  |
| 124 | Support of TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations | 36.306, 4.3.4.31 | Rel-12 | pc_eIMTA_TDD |  |
| 125 | Support of prioritization of the frequency bands in multiBandInfoList over the band in freqBandIndicator as defined by freqBandIndicatorPriority-r12 | 36.306, 4.3.5.11 | Rel-12 | pc_freqBandPriorityAdjustment |  |
| 126 | Support of MBMS reception via SC-PTM on configured SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_SCell |  |
| 127 | Support of MBMS reception via SC-PTM on a cell that may be additionally configured as an SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_NonServingCell |  |
| 128 | Support of extended Long DRX cycle | 36.306, 4.3.19.4 | Rel-13 | pc_extendedLongDRX |  |
| 129 | Supports downlink LAA operation | 36.306, 4.3.23.1 | Rel-13 | pc_downlink_LAA |  |
| 130 | Supports measurement and reporting for RSSI and channel occupancy | 36.306, 4.3.6.19 | Rel-13 | pc_rssiAndChannelOccupancyReporting |  |
| 131 | Support of QCI1 indication in Radio Link Failure Report | 36.306, 6.8.2 | Rel-13 | pc_qci1Indication_inRLF |  |
| 132 | Support of user plane CIoT optimisation in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_User_Plane_CIoT_Optimisation |  |
| 133 | Support of EMM-REGISTERED without PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithoutPDN |  |
| 134 | Support of EMM-REGISTERED with PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithPDN |  |
| 135 | Void |  |  |  |  |
| 136 | Void |  |  |  |  |
| 137 | Support of multiple DRBs in NB-IoT | 36.306, 4.3.8.5 | Rel-13 | pc_NB_MultiDRB |  |
| 138 | Support of Fast First Higher Priority PLMN search | 23.122, 4.4.3.3.1 | Rel-12 | pc_Fast_First_HPPLMN_Search |  |
| 139 | Support of TDD Bands38, 40, 41 and 42 Power class 2 operation | 36.101, 6.2.2 | Rel-14 | pc_TDD_band_UE_PC2 |  |
| 140 | Support for PDCP Packet Delay per QCI | TS 36.331 5.5.2 | Rel-13 | pc_PDCP_PktDelay |  |
| 141 | Void |  |  |  |  |
| 142 |  |  |  |  |  |
| 143 | Support of Control plane CIoT in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_Control_Plane_CIoT_Optimisation |  |
| 144 | Support of S1-U data transfer | 24.301, 5.3.15 | Rel-13 | pc_S1_U_DataTransfer | An UE supporting user plane CIoT optimization shall set this PICS to true. |
| 145 | Support for GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1" | GSMA PRD NG.108 | Rel-13 | pc_Category_M1_voice |  |
| 146 | Support of automatic PDN connection trigger on HRPD cell reselection | X.s0057, 6.4.1 | Rel-8 | pc_AutomaticHRPD_PDN_Connection |  |
| 147 | Support for Dual RM Coding | 36.331, 6.3.6 | Rel-10 | pc_DualRM_Coding |  |
| 148 | Support of V2X sidelink communication | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommSidelink |  |
| 149 | Support of V2X communication Via Uu | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommUu |  |
| 150 | Support of simultaneous transmission of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousTx |  |
| 151 | Support of simultaneous reception of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousRx |  |
| 152 | Support of transmitting PSCCH/PSSCH using dynamic scheduling | 36.306, 4.3.21.14 | Rel-14 | pc_v2xScheduling |  |
| 153 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing | 36.306, 4.3.21.15 | Rel-14 | pc_v2xFullSensing |  |
| 154 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing | 36.306, 4.3.21.16 | Rel-14 | pc_v2xPartialSensing |  |
| 155 | Support of SLSS transmission and reception for V2X sidelink communication | 36.306, 4.3.21.17 | Rel-14 | pc_v2xSLSS |  |
| 156 | Support of CBR measurement and reporting | 36.306, 4.3.21.18 | Rel-14 | pc_v2xCBRMeas |  |
| 157 | Support of zone based transmission resource pool selection for V2X sidelink communication | 36.306, 4.3.21.12 | Rel-14 | pc_v2xZoneBasedPoolSelection |  |
| 158 | Require intra-frequency measurement gaps for operating in CE Mode A or CE Mode B | 36.306, 4.3.5.1.2 | Rel-13 | pc_intraFreq-CE-NeedForGaps |  |
| 159 | Support of 4 layer spatial multiplexing with transmission mode 3 and transmission mode 4 | 36.306, 4.3.4.7 | Rel-10 | pc_4Layer_spatial_mux_tm3_tm4 |  |
| 160 | Support of delay budget reporting for MMTEL voice and video enhancements | 36.306, 4.3.32.1 | Rel-14 | pc_delayBudgetReporting |  |
| 161 | Support of PUSCH enhancement for MMTEL voice and video enhancements mode | 36.306, 4.3.32.2 | Rel-14 | pc_PUSCH_Ehn_MMTEL |  |
| 162 | Void |  |  |  |  |
| 163 | Support of PUCCH transmission on SCell in CA | 36.306, 4.3.4.47 | Rel-13 | pc_PUCCH_SCell |  |
| 164 | Support high speed enhancement for random access preambles generated from restricted set type B in high speed scenoario as specified in TS 36.211 | 36.306 | Rel-14 | pc_Highspeed_Enh_Prach |  |
| 165 | Support of RRC connection re-establishment | 36.306, 6.7.5 | Rel-14 | pc_RRC_re-establishment_CP_CIoT | An UE supporting S1-U data transfer shall set this PICS to true. |
| 166 | Support of SRS switching between a band pair | 36.306, 4.3.5.24, 4.3.5.25 | Rel-14 | pc_SRS_switching | Support of SRS switching between a band pair |
| 167 | Support of 2 HARQ processes in DL and UL in NB-IoT | 36.306, 4.3.4.62 | Rel-14 | pc_NB_TwoHARQ_Processes |  |
| 168 | Support of Release Assistance Indication (RAI) in NB-IoT | 36.306, 4.3.19.10 | Rel-14 | pc_NB_Rai_Support |  |
| 169 | Support of Announcing for ProSe Group Member Discovery | 24.334, 10A.2.6 | Rel-13 | pc_ProSeAnnForGroupMemberDiscovery |  |
| 170 | Support of SPS interval shorter than 10 subframes in FDD mode | 36.306, 4.3.19.5 | Rel-14 | pc_shortSPS_intervalFDD |  |
| 171 | Support of SPS interval shorter than 10 subframes in TDD mode | 36.306, 4.3.19.6 | Rel-14 | pc_shortSPS_intervalTDD |  |
| 172 | Support of skipping SPS UL transmissions if no data is available | 36.306, 4.3.19.8 | Rel-14 | pc_skipUplinkSPS | An UE supporting SPS interval shorter than 10 (pc_shortSPS_intervalFDD or pc_shortSPS_intervalTDD) shall set this PICS to true. |
| 173 | Support of skipping UL transmissions if no data is available | 36.306, 4.3.19.7 | Rel-14 | pc_skipUplinkDynamic |  |
| 174 | Supports uplink LAA operation | 36.306, 4.3.23.8 | Rel-14 | pc_uplink_LAA | Support of Enhanced LAA operations |
| 175 | Void |  |  |  |  |
| 176 | Supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B | 36.306, 4.3.23.10 | Rel-14 | pc_twoStepScheduling_uplink_LAA | UE supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B, applying to the UE supports uplink LAA operation |
| 177 | Supports multiple uplink SPS and reporting SPS assistance information | 36.306, 4.3.19.11 | Rel-14 | pc_multipleUplinkSPS | Support of multiple uplink SPS and reporting SPS assistance information |
| 178 | Support of V2X communication as Pedestrian UE | 36.300, 23.14.1.1 | Rel-14 | pc_P2X_UE |  |
| 179 | Support of the uplink data compression operation | 36.306, 4.3.1.7 | Rel-15 | pc_UDC |  |
| 180 | Support of UL data compression with SIP static dictionary | 36.306, 4.3.1.8 | Rel-15 | pc_UDC_SIP |  |
| 181 | Support of QoE Measurement Collection for Streaming Service | 36.306, 4.36.30 | Rel-15 | pc_qoe_MeasReport |  |
| 182 | Support of QoE Measurement Collection for MTSI Service | 36.306, 4.36.33 | Rel-15 | pc_qoe_MTSI_MeasReport |  |
| 183 | Support of 256QAM in UL | 36.306, 4.3.4.73 | Rel-14 | pc_UL_256QAM |  |
| 184 | Support of Bluetooth Measurement Collection in logged MDT | 36.306, 4.3.13.6 | Rel-15 | pc_BT_Meas_logged_MDT |  |
| 185 | Support of WLAN Measurement Collection in logged MDT | 36.306, 4.3.13.7 | Rel-15 | pc_WLAN_Meas_logged_MDT |  |
| 186 | Support of Bluetooth Measurement Collection in Immediate MDT | 36.306, 4.3.13.8 | Rel-15 | pc_BT_Meas_Imm_MDT |  |
| 187 | Support of WLAN Measurement Collection in Immediate MDT | 36.306, 4.3.13.9 | Rel-15 | pc_WLAN_Meas_Imm_MDT |  |
| 188 | Support of ce-PUSCH-NB-MaxTBS-r14 | 36.306, 4.3.4.63 | Rel-15 | pc_ce_PUSCH_NB_MaxTBS |  |
| 189 | Support of height-based measurement reporting | 36.306, 4.3.6.35 | Rel-15 | pc_heightMeas |  |
| 190 | Support of GNSS for height measurement |  | Rel-15 | pc_gnss_heightMeas |  |
| 191 | Support of measurement reporting triggered based on a number of cells | 36.306, 4.3.6.34 | Rel-15 | pc_Multiple_Cells_Meas_Ext |  |
| 192 | Support of flight path plan reporting | 36.306, 4.3.15.14 | Rel-15 | pc_FlightPathPlan |  |
| 193 | Void |  |  |  |  |
| 194 | Support of HARQ-ACK bundling | 36.213, 7.3.1 | Rel-14 | pc_ce_HARQ_AckBundling | Support of HARQ-ACK bundling |
| 195 | Support of eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode. | 36.306, 4.3.6.31 | Rel-15 | pc_idleModeMeasurement |  |
| 196 | Support of the dormant SCell state. | 36.306, 4.3.19.16 | Rel-15 | pc_dormantSCellState |  |
| 197 | Support of having SCell configured in dormant SCell state | 36.306, 4.3.19.18 | Rel-15 | pc_directSCellHibernation |  |
| 198 | Support of having SCell configured in activated SCell state | 36.306, 4.3.19.17 | Rel-15 | pc_directSCellActivation |  |
| 199 | Support of user plane CIoT optimisation in NB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_NB_User_Plane_CIoT_Optimisation |  |
| 200 | Support of Control Plane Early Data Transmission | 36.306, 6.8.4 | Rel-15 | pc_Control_Plane_CIoT_Optimisation_EDT |  |
| 201 | Support of User Plane Early Data Transmission | 36.306, 4.3.8.7 | Rel-15 | pc_User_Plane_CIoT_Optimisation_EDT |  |
| 202 | Support of RLC UM mode in NB-IoT | 36.306, 4.3.2.5 | Rel-15 | pc_NB_RLC_UM |  |
| 203 | Support of short TTI and/or short processing time | 36.306, 4.3.4.150 | Rel-15 | pc_sTTI_SPT |  |
| 204 | Support of short processing time for the corresponding frame structure types | 36.306, 4.3.4.100 | Rel-15 | pc_spt_Parameters |  |
| 205 | Support of sTTI in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_sTTI_Combinations |  |
| 206 | Support of {subslot, subslot} combinations in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_subslot_Combinations |  |
| 207 | Support of L1-based SPDCCH reuse | 36.306, 4.3.4.147 | Rel-15 | pc_SPDCCH_Reuse |  |
| 208 | Support of SRS trigerring via DCI format 7 for FS2 | 36.306, 4.3.4.181 | Rel-15 | pc_SRS_DCI7_Triggering |  |
| 209 | Support of UL asynchronous HARQ sharing between different TTI lengths for an UL serving cell. | 36.306, 4.3.4.156 | Rel-15 | pc_ul_AsyncHarqSharingDiffTTI |  |
| 210 | Support of Wake Up Signal | 36.306, 4.3.4.113 | Rel-15 | pc_wakeUpSignal |  |
| 211 | Support of physical layer SR with HARQ ACK | 36.306, 4.3.4.117 | Rel-15 | pc_SR_WithHARQ_ACK |  |
| 212 | Support of physical layer SR without HARQ ACK | 36.306, 4.3.4.118 | Rel-15 | pc_SR_WithoutHARQ_ACK |  |
| 213 | UE supports Ethernet header compression and decompression using EHC protocol | 36.306, 4.3.1.12 | Rel-16 | pc_EUTRAN_EHC |  |
| 214 | UE supports DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.40 | Rel-16 | pc_EUTRA_intraFreqDAPS |  |

Table A.4.4-1A: Additional UE radio access capabilities (Mandatory for Rel-11 and onward)

| Item | Additional capabilities | Ref. | Release | Status (Note 1) | Support Yes/No (Note 2) | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UL Coordinated Multi-Point operation | 36.306, 4.3.4.23 | Rel-11 | O.01 |  | pc_UL_CoMP | This is a Rel-11 Mandatory feature |
| 2 | Support of TDD special subframe | 36.306, 4.3.4.21 36.331, 6.3.6 | Rel-11 | O.01 |  | pc_TDD_SpecialSubframe | This is a Rel-11 Mandatory feature |
|  |  |  | Rel-9, Rel-10 | O |  |  | The Capability can optionally be implemented in UEs of the indicated Releases |
| 3 | Support of multiple timing advances for each band combination supported by the UE | 36.306, 4.3.5.3 | Rel-11 | O.01 |  | pc_multipleTimingAdvance | This is a Rel-11 Mandatory feature (Note 3) |
| 4 | Support of Extended Access Barring | 36.306, 7.3.1 | Rel-11 | O.01 |  | pc_EAB | This is a Rel-11 Mandatory feature (Note 4) |
| 5 | Support of transmission of discovery announcements based on network scheduled resource allocation. | 36.306, 4.3.21.4 | Rel-12 | O.01 |  | pc_discScheduledResourceAlloc | This is a Rel-12 Mandatory feature (Note 5) |
| 6 | Support of transmission of discovery announcements based on UE autonomous resource selection. | 36.306, 4.3.21.5 | Rel-12 | O.01 |  | pc_discUESelectedResourceAlloc | This is a Rel-12 Mandatory feature (Note 5) |
| 7 | Support of CRS interference handling | 36.306, 4.3.4.15 | Rel-11 | O.01 |  | pc_CRS_Interference_Handling | This is a Rel-11 Mandatory feature except UE Category 0 and Category M1 and M2 |
| 8 | Support of Synchronisation signal and common channel interference handling | 36.306, 4.3.4.20 | Rel-11 | O.01 |  | pc_ss_CCH_Interference_Handling | This is a Rel-11 Mandatory feature for TDD bands except UE Category 0 and Category M1 and M2 |
| 9 | Support of UL multi-tone transmissions on NPUSCH in NB-IoT | 36.306, 4.3.4.55 | Rel-13 | O.01 |  | pc_NB_MultiTone | This is a Rel-13 Mandatory feature for UEs of any ue-Category-NB |
| 10 | Support of multi-carrier operation in NB-IoT | 36.306, 4.3.4.56 | Rel-13 | O.01 |  | pc_NB_MultiCarrier | This is a Rel-13 Mandatory feature for UEs of any ue-Category-NB |
| 11 | Support of PRACH on non-anchor carrier in NB-IoT | 36.306, 4.3.4.75 | Rel-14 | O.01 |  | pc_NB_MultiCarrier_NPRACH | This is a Rel-14 Mandatory feature for UEs of any ue-Category-NB |
| 12 | Support of paging on non-anchor carriers in NB-IoT | 36.306, 4.3.4.76 | Rel-14 | O.01 |  | pc_NB_MultiCarrier_Paging | This is a Rel-14 Mandatory feature for UEs of any ue-Category-NB |
| 13 | Support of interference randomisation in connected mode in NB-IoT | 36.306, 4.3.4.80 | Rel-14 | O.01 |  | pc_NB_InterferenceRandomisation | This is a Rel-14 Mandatory feature for UEs of any ue-Category-NB |
| 14 | Support of eventA3 for intra-frequency neighbouring cells in normal coverage and CE Mode A | 36.306, 4.3.29.3 | Rel-13 | O.01 |  | pc_IntraFreqA3_CE_ModeA | This is a Rel-13 Mandatory feature for UEs supporting ce-ModeA-r13 |
| 15 | Support of intra-frequency handover to target cell in normal coverage and CE Mode A | 36.306, 4.3.29.5 | Rel-13 | O.01 |  | pc_IntraFreqHO_CE_ModeA | This is a Rel-13 Mandatory feature for UEs supporting ce-ModeA-r13 |
| 16 | Support of intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED | 36.306 4.3.6.23 | Rel-14 | O.01 |  | pc_CE_Measurements | This is a Rel-14 Mandatory feature for UEs supporting ce-ModeA-r13 (Note 6). |
| Note 1: From Rel-11 onwards 3GPP TSG RAN has discontinued the usage of FGI bits (see A.4.5). Instead it has introduced a different mechanism to accomplish the same purposes based on the following principles (TS 36.306 [1] clause 4): 'For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested.' Reflecting this situation, in the present table the status for Mandatory features would be indicated as conditional Optional (O.xx) until IOT testing availability is ensured. The decision when IOT testing availability can be considered ensured is made by 3GPP TSG RAN. After the 3GPP TSG RAN decision that IOT testing is available the status of the capability parameter will be changed to Mandatory (M) and the release from which this requirement apply will be explicitly stated. Note 2: If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release. Note 3: It is mandatory for UEs of this release of the specification to support this capability for band combinations having an UL on multiple FDD bands (see 36.306, 4.3.5.3). In the context of evaluating the status of the capability this would depend on the indication for UL support provided in Table A.4.3.3.3-3 i.e. if for at least one CA configurations for Inter-band CA the UE indicates A-A then the Support of multiple timing advances for this CA configuration is Mandatory. Note 4: It is mandatory for UEs which are supporting an access subject to Extended Access Barring (see 36.306, 7.1.3). Note 5: It is mandatory for UEs which are supporting ProSe direct discovery. Note 6: This UE capability is also used to identify general support of inter-frequency (e.g. including RRC_IDLE), which is mandatory for Rel-14 UEs supporting ce-ModeA-r13. |  |  |  |  |  |  |  |

Table A.4.4-1B: Additional UE radio access capabilities Conditions

O.01 IF The feature has been IOT-ed THEN Support shall be indicated ELSE Support shall not be indicated

Table A.4.4-2: Definition of UE implementation capabilities

| Item | Definition of UE implementation capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support EPS attach (with or without pre-configuration) | 24.301 (Note1) | Rel-8 | pc_Attach | UE supports to be configured to initiate EPS attach or will always initiate EPS attach. (pc_PS_voice_centric OR pc_PS_data_centric) shall set this PICS to true. |
| 2 | Support combined EPS/IMSI attach (with or without pre-configuration) | 24.301 | Rel-8 | pc_Combined_Attach | UE supports to be configured to initiate combined EPS/IMSI attach or will always initiate combined EPS/IMSI attach. Implication: ((pc_UTRA OR pc_GERAN) AND [8] pc_CS) OR pc_CS_Fallback OR pc_SMS_SGs OR pc_IMSI_detach OR pc_CS_Em_Call_in_UTRA OR pc_CS_Em_Call_in_GERAN OR pc_CS_PS_voice_centric OR pc_CS_PS_data_centric shall set this PICS to true. |
| 3 | Void |  |  |  |  |
| 4 | Support of CS/PS mode 1 | 24.301 | Rel-8 | pc_CS_PS_voice_centric | UE supports to be configured to consistently behave as a CS/PS Voice centric UE |
| 5 | Support of CS/PS mode 2 | 24.301 | Rel-8 | pc_CS_PS_data_centric | UE supports to be configured to consistently behave as a CS/PS Data centric UE. |
| 6 | Requiring UMI proceeding to paging response | 23.272 | Rel-8 | pc_UMI_ProcNeeded_DuringCSFB | UE requires UMI prior to paging response while CSFB to UTRA |
| 7 | Support of PS mode 1 | 24.301 | Rel-8 | pc_PS_voice_centric | UE supports to be configured to consistently behave as a PS Voice centric UE |
| 8 | Support of PS mode 2 | 24.301 | Rel-8 | pc_PS_data_centric | UE supports to be configured to consistently behave as a PS Data centric UE. |
| 9 | IMS PS voice preferred, CS Voice as secondary | 24.301 | Rel-8 | pc_voice_PS_1_CS_2 | Configured voice domain preference. |
| 10 | Keeps EPS Bearer Context parameters after completion of the normal DETACH procedure | 24.301 cl. 5.5.2.2.2 | Rel-8 | pc_KeepEpsBearerParametersAfterNormalDetach | If the UE supports this, then the next ATTACH after DETACH shall be done using AT command AT+CGATT=1. Otherwise it shall be done using AT+CGDCONT=1,"IP" followed by AT+CGACT=1 |
| 11 | IMS APN as default APN | 23.401 | Rel-8 | pc_IMS_APN_default | Configured with IMS APN as default APN. |
| 12 | XCAP only APN | 23.401 | Rel-8 | pc_XCAP_only_APN | Configured with an APN for XCAP only usage.(Note 2) |
| 13 | Provide IMS APN | 23.401 | Rel-8 | pc_Provide_IMS_APN | Configured to provide IMS APN during initial attach. |
| 14 | Provide IMS as second APN | 23.401 | Rel-8 | pc_Provide_IMS_as_second_APN | Configured to provide IMS APN as the second PDN connection. |
| 15 | Provide Internet as second APN | 23.401 | Rel-8 | pc_Provide_Internet_as_second_APN | Configured to provide Internet as the second PDN connection. |
| 16 | User initiated PDN disconnect | 24.301 | Rel-8 | pc_UE_supports_user_initiated_PDN_disconnect | UE supports user initiated PDN disconnect. |
| 17 | XCAP over Internet PDN | 23.401 | Rel-8 | pc_XCAP_over_Internet_APN | Configured to use internet PDN for XCAP signalling (Note 2) |
| 18 | Dynamically downgrades the GERAN release when the support of EPS is disabled | 24.301, 24.008 | Rel-8 | pc_Dynamic_GERAN_Rel_downgrade | UE may support e.g. from all GERAN Rel-8 features only those related to the interworking with EPS. When EPS is disabled then the Device may comply with a lower than Rel-8 GERAN release requirements. |
| 19 | Provide ProSe APN | 24.334 | Rel-12 | pc_Provide_ProSe_APN | Configured to provide ProSe APN and a PDN connection request. An UE supporting D2D ProSe shall set this PICS to true. |
| 20 | Provisioned FQDN ePDG | 24.302 | Rel-13 | pc_ePDG_FQDN_Provisioned | Configured with an ePDG FQDN provisioned by the home operator. |
| 21 | Operator Identifier FQDN format used for ePDG | 24.302 | Rel-13 | pc_ePDG_FQDN_constructed | Configured to construct the ePDG FQDN in the Operator Identifier FQDN format. |
| 22 | UE supports only NB-S1 mode (i.e. NB-IoT) | 24.301 | Rel-13 | pc_NB_S1_only |  |
| 23 | UE capable of requesting PDN of type "Non-IP" | 24.301 | Rel-13 | pc_NonIP_PDN |  |
| 24 | UE capable of requesting PDN of type "IP" | 24.301 | Rel-13 | pc_IP_PDN |  |
| 25 | The UE supports Non-IP Link MTU parameter | 24.301 | Rel-13 | pc_NonIP_Link_MTU_Parameter |  |
| 26 | The UE supports IPv4 Link MTU parameter | 24.301 | Rel-13 | pc_IPv4_Link_MTU_Parameter |  |
| 27 | The UE supports APN rate control | 24.301 | Rel-13 | pc_APN_RateControl |  |
| 28 | The UE supports Header compression for control plane CIoT EPS optimization | 24.301 | Rel-13 | pc_HCCPCIoT |  |
| 29 | The UE supports a mechanism to provide Daylight Saving Time | 24.301 | Rel-8 | pc_ProvideDST_inUse | Note 3 |
| 30 | The UE does not request IMS PDN connection when IMS VoPS set to ‘0’ | 24.301 | Rel-8 | pc_UE_NoReqIMS_IMSVoPS_0 | Configured not to request IMS PDN connection when IMS VoPS set to ‘0’ |
| 31 | The UE supports additional APN rate control for exception data reporting | 24.301 | Rel-14 | pc_Additional_APN_RateControl |  |
| 32 | The UE is configured to use SMS over IP | 24.167 | Rel-8 | pc_Use_SMS_over_IP | Configured to use SMS over IP |
| 33 | The UE supports a bearer with QCI 66 | 23.203 | Rel-14 | pc_Use_QCI_66 |  |
| Note 1: A UE supporting UTRAN and/or GERAN which is configured to initiate EPS attach considers UTRAN and GERAN cell as candidates for cell selection and cell reselection according to TS 36.304. A UE configured to initiate EPS attach which has selected a UTRAN or GERAN cell may perform registration procedures to the PS and CS domains, or to the PS domain only or to the CS domain only. Note 2: pc_XCAP_only_APN and pc_XCAP_over_Internet_APN are mutual exclusive i.e. shall not be set to true at the same time. Note 3: Shall be set to false when pc_DaylightSavingTime is false. |  |  |  |  |  |

A.4.5	Feature group indicators

For the purpose of conformance testing, the definition of each Feature Group Indicator (FGI) is duplicated from Rel-8 for each possible E-UTRA mode, i.e. FDD (Tables A.4.5-1a, A.4.5-1d and A.4.5-3a) and TDD (Tables A.4.5-1b, A.4.5-1e and A.4.5-3b). For each FGI (applicable to the Release supported by the UE):

-	If the UE supports E-UTRA FDD and TDD: both FDD and TDD support statuses shall be declared separately (see Note 2).

-	If the UE supports single E-UTRA xDD mode: only the xDD-specific support status needs to be declared.

Note 1:	From Rel-11 onwards 3GPP TSG RAN has discontinued the usage of FGI bits. Instead it has introduced a different mechanism to accomplish the same purposes based on the principles described in TS 36.306 [13] clause 4. These new principles where applicable should be catered for elsewhere in the present document e.g. in section A.4.4.

Note 2:	For Rel-8 UE, the separate declaration also applies to FGI 1-32.

Note 3:	'VoLTE' in the tables A.4.5-1a and A.4.5-1b corresponds to a UE which is IMS voice capable.

Table A.4.5-1: Void

Table A.4.5-1a: Feature group indicators 1-32 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of - Intra-subframe frequency hopping for PUSCH scheduled by UL grant - DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI | - set to 1 by category M1 and M2 UEs that have implemented and successfully tested “ZAperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PM" |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_1_F | Corresponding to the Index of Indicator, the leftmost binary bit 1. Set to true if supporting all functionalities in the feature group. |
| 2 | Support of - Simultaneous CQI and ACK/NACK on PUCCH, i.e. PUCCH format 2a and 2b - Absolute TPC command for PUSCH - Resource allocation type 1 for PDSCH - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_2_F | Corresponding to the Index of Indicator, the leftmost binary bit 2. Set to true if supporting all functionalities in the feature group. |
| 3 | Support of - Semi-persistent scheduling - TTI bundling - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_3_F | Corresponding to the Index of Indicator, the leftmost binary bit 3. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 3 in Table A.4.5-1b for TDD. |
|  | Support of - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 4 | Support of - Short DRX cycle | - can only be set to 1 if the UE has set bit number 5 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_4_F | Corresponding to the Index of Indicator, the leftmost binary bit 4. Set to true if supporting all functionalities in the feature group. |
| 5 | Support of - Long DRX cycle - DRX command MAC control element |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_5_F | Corresponding to the Index of Indicator, the leftmost binary bit 5. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 5 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 6 | Support of - Prioritized bit rate |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_6_F | Corresponding to the Index of Indicator, the leftmost binary bit 6. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 6 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 7 | Support of - RLC UM | - can only be set to 0 if the UE does not support voice |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_7_F | Corresponding to the Index of Indicator, the leftmost binary bit 7. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 7 in Table A.4.5-1b for TDD. |
|  |  |  | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 8 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH PS handover | - can only be set to 1 if the UE has set bit number 22 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_8_F | Corresponding to the Index of Indicator, the leftmost binary bit 8. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH PS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes (except for category M1 amd M2 UEs), if UE supports UTRA FDD | Rel-9 |  |  |  |
| 9 | Support of - EUTRA RRC_CONNECTED to GERAN GSM_Dedicated handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 23 to 1 |  | Rel-8 to Rel-10 | 36.331, Annex B.1 | pc_FeatrGrp_9_F | Corresponding to the Index of Indicator, the leftmost binary bit 9. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 and M2 UEs), if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 10 | Support of - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order with NACC (Network Assisted Cell Change) |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_10_F | Corresponding to the Index of Indicator, the leftmost binary bit 10. Set to true if supporting all functionalities in the feature group. |
| 11 | Support of - EUTRA RRC_CONNECTED to CDMA2000 1xRTT CS Active handover | - can only be set to 1 if the UE has sets bit number 24 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_11_F | Corresponding to the Index of Indicator, the leftmost binary bit 11.Set to true if supporting all functionalities in the feature group. |
| 12 | Support of - EUTRA RRC_CONNECTED to CDMA2000 HRPD Active handover | - can only be set to 1 if the UE has set bit number 26 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_12_F | Corresponding to the Index of Indicator, the leftmost binary bit 12. Set to true if supporting all functionalities in the feature group. |
| 13 | Support of - Inter-frequency handover (within FDD or TDD) | - can only be set to 1 if the UE has set bit number 25 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_13_F | Corresponding to the Index of Indicator, the leftmost binary bit 13. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 13 in Table A.4.5-1b for TDD. |
|  |  |  | Yes (except for category M1 and M2 UEs), unless UE only supports band 13 | Rel-9 |  |  |  |
| 14 | Support of - Measurement reporting event: Event A4 - Neighbour > threshold - Measurement reporting event: Event A5 - Serving < threshold1 & Neighbour > threshold2 |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_14_F | Corresponding to the Index of Indicator, the leftmost binary bit 14. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 14 in Table A.4.5-1b for TDD. |
|  |  |  | Yes (except for category M1 and M2 UEs) | Rel-9 |  |  |  |
| 15 | Support of - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Measurement reporting event: Event B1 - Neighbour > threshold for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set at least one of the bit number 22, 23, 24, 26 or 39 to 1. - even if the UE sets bits 41, it shall still set bit 15 to 1 if measurement reporting event B1 is tested for all RATs supported by UE - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_15_F | Corresponding to the Index of Indicator, the leftmost binary bit 15. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes for FDD, if UE supports only UTRAN FDD and does not support UTRAN TDD or GERAN or 1xRTT or HRPD | Rel-9 |  |  |  |
| 16 | Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells; - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1; and - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively NOTE: Event triggered periodical reporting (i.e. with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively NOTE: Event triggered periodical reporting (i.e., with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_16_F | Corresponding to the Index of Indicator, the leftmost binary bit 16.Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 16 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 17 | Support of Intra-frequency ANR features including: - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_17_F | Corresponding to the Index of Indicator, the leftmost binary bit 17. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 17 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 18 | Support of Inter-frequency ANR features including: - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_18_F | Corresponding to the Index of Indicator, the leftmost binary bit 18. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 18 in Table A.4.5-1b for TDD. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 19 | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN, 1xRTT or HRPD, if the UE has set bit number 22, 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 23, 24 or 26 to 1. - even if the UE sets bits 33 to 36, it shall still set bit 19 to 1 if inter-RAT ANR features are tested for all RATs for which inter-RAT measurement reporting is indicated as tested |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_19_F | Corresponding to the Index of Indicator, the leftmost binary bit 19. Set to true if supporting all functionalities in the feature group. |
|  | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for 1xRTT or HRPD, if the UE has set bit number 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRANTDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively |  |  | Rel-9 |  |  |  |
| 20 | If bit number 7 is set to ‘0’: - SRB1 and SRB2 for DCCH + 8x AM DRB If bit number 7 is set to ‘1’: - SRB1 and SRB2 for DCCH + 8x AM DRB - SRB1 and SRB2 for DCCH + 5x AM DRB + 3x UM DRB NOTE: UE which indicate support for a DRB combination also support all subsets of the DRB combination. Therefore, release of DRB(s) never results in an unsupported DRB combination. | - Regardless of what bit number 7 and bit number 20 is set to, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB - Regardless of what bit number 20 is set to, if bit number 7 is set to ‘1’, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB + 1x UM DRB |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_20_F | Corresponding to the Index of Indicator, the leftmost binary bit 20. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 20 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 21 | Support of - Predefined intra- and inter-subframe frequency hopping for PUSCH with N_sb > 1 - Predefined inter-subframe frequency hopping for PUSCH with N_sb > 1 | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_21_F | Corresponding to the Index of Indicator, the leftmost binary bit 21. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 21 in Table A.4.5-1b for TDD. |
| 22 | Support of - UTRAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_22_F | Corresponding to the Index of Indicator, the leftmost binary bit 22. Set to true if supporting all functionalities in the feature group. |
|  | Support of - UTRAN FDD or UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports either only UTRAN FDD or only UTRAN TDD - UTRAN FDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes, if UE supports UTRA | Rel-9 |  |  |  |
| 23 | Support of - GERAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_23_F | Corresponding to the Index of Indicator, the leftmost binary bit 23.Set to true if supporting all functionalities in the feature group. |
| 24 | Support of - 1xRTT measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_24_F | Corresponding to the Index of Indicator, the leftmost binary bit 24. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports enhanced 1xRTT CSFB | Rel-9 |  |  |  |
| 25 | Support of - Inter-frequency measurements and reporting in E-UTRA connected mode NOTE: The UE setting this bit to 1 and indicating support for FDD and TDD frequency bands in the UE capability signalling implements and is tested for FDD measurements while the UE is in TDD, and for TDD measurements while the UE is in FDD. | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_25_F | Corresponding to the Index of Indicator, the leftmost binary bit 25. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 25 in Table A.4.5-1b for TDD. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 26 | Support of - HRPD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_26_F | Corresponding to the Index of Indicator, the leftmost binary bit 26. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports HRPD | Rel-9 |  |  |  |
| 27 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH CS handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 8 to 1 and supports SR-VCC from EUTRA defined in TS 24.008. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_27_F | Corresponding to the Index of Indicator, the leftmost binary bit 27. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH CS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes, if UE supports VoLTE and UTRA FDD | Rel-9 |  |  |  |
| 28 | Support of - TTI bundling | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. | Yes | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_28_F | Corresponding to the Index of Indicator, the leftmost binary bit 28.Set to true if supporting all functionalities in the feature group. |
| 29 | Support of - Semi-Persistent Scheduling | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_29_F | Corresponding to the Index of Indicator, the leftmost binary bit 29.Set to true if supporting all functionalities in the feature group. |
| 30 | Support of - Handover between FDD and TDD | - can only be set to 1 if the UE has set bit number 13 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_30_F | Corresponding to the Index of Indicator, the leftmost binary bit 30. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 30 in Table A.4.5-1b for TDD. |
| 31 | Support of - Indicates whether the UE supports the mechanisms defined for cells broadcasting multi band information i.e. comprehending multiBandInfoList, disregarding in RRC_CONNECTED the related system information fields and understanding the EARFCN signalling for all bands, that overlap with the bands supported by the UE, and that are defined in the earliest version of TS 36.101 [42] that includes all UE supported bands. | - This FGI bit is concerns an optional release independent feature (as it was difficult to introduce this from REL-8 when using regular UE capability signalling) |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_31_F | Corresponding to the Index of Indicator, the leftmost binary bit 31. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 31 in Table A.4.5-1b for TDD. |
|  |  |  | Yes | Rel-10 |  |  |  |
| 32 | Undefined |  |  | Rel-8 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 32. |

Table A.4.5-1b: Feature group indicators 1-32 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of - Intra-subframe frequency hopping for PUSCH scheduled by UL grant - DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI | - set to 1 by category M1 and M2 UEs that have implemented and successfully tested "Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PM" |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_1_T | Corresponding to the Index of Indicator, the leftmost binary bit 1. Set to true if supporting all functionalities in the feature group. |
| 2 | Support of - Simultaneous CQI and ACK/NACK on PUCCH, i.e. PUCCH format 2a and 2b - Absolute TPC command for PUSCH - Resource allocation type 1 for PDSCH - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_2_T | Corresponding to the Index of Indicator, the leftmost binary bit 2. Set to true if supporting all functionalities in the feature group. |
| 3 | Support of - Semi-persistent scheduling - TTI bundling - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_3_T | Corresponding to the Index of Indicator, the leftmost binary bit 3. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 3 in Table A.4.5-1a for FDD. |
|  |  |  | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  | Support of - 5bit RLC UM SN - 7bit PDCP SN |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 4 | Support of - Short DRX cycle | - can only be set to 1 if the UE has set bit number 5 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_4_T | Corresponding to the Index of Indicator, the leftmost binary bit 4. Set to true if supporting all functionalities in the feature group. |
| 5 | Support of - Long DRX cycle - DRX command MAC control element |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_5_T | Corresponding to the Index of Indicator, the leftmost binary bit 5. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 5 in Table A.4.5-1a for FDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 6 | Support of - Prioritized bit rate |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_6_T | Corresponding to the Index of Indicator, the leftmost binary bit 6. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 6 in Table A.4.5-1a for FDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 7 | Support of - RLC UM | - can only be set to 0 if the UE does not support voice |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_7_T | Corresponding to the Index of Indicator, the leftmost binary bit 7. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 7 in Table A.4.5-1a for FDD. |
|  |  |  | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 8 | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH PS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - can only be set to 1 if the UE has set bit number 22 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_8_T | Corresponding to the Index of Indicator, the leftmost binary bit 8. Set to true if supporting all functionalities in the feature group. |
| 9 | Support of - EUTRA RRC_CONNECTED to GERAN GSM_Dedicated handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 23 to 1 |  | Rel-8 to Rel-10 | 36.331, Annex B.1 | pc_FeatrGrp_9_T | Corresponding to the Index of Indicator, the leftmost binary bit 9. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 and M2 UEs), if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 10 | Support of - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order with NACC (Network Assisted Cell Change) |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_10_T | Corresponding to the Index of Indicator, the leftmost binary bit 10. Set to true if supporting all functionalities in the feature group. |
| 11 | Support of - EUTRA RRC_CONNECTED to CDMA2000 1xRTT CS Active handover | - can only be set to 1 if the UE has sets bit number 24 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_11_T | Corresponding to the Index of Indicator, the leftmost binary bit 11. Set to true if supporting all functionalities in the feature group. |
| 12 | Support of - EUTRA RRC_CONNECTED to CDMA2000 HRPD Active handover | - can only be set to 1 if the UE has set bit number 26 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_12_T | Corresponding to the Index of Indicator, the leftmost binary bit 12. Set to true if supporting all functionalities in the feature group. |
| 13 | Support of - Inter-frequency handover (within FDD or TDD) | - can only be set to 1 if the UE has set bit number 25 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_13_T | Corresponding to the Index of Indicator, the leftmost binary bit 13. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 13 in Table A.4.5-1a for FDD. |
|  |  |  | Yes (except for category M1 and M2 UEs),, unless UE only supports band 13 | Rel-9 |  |  |  |
| 14 | Support of - Measurement reporting event: Event A4 - Neighbour > threshold - Measurement reporting event: Event A5 - Serving < threshold1 & Neighbour > threshold2 |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_14_T | Corresponding to the Index of Indicator, the leftmost binary bit 14. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 14 in Table A.4.5-1a for FDD. |
|  |  |  | Yes (except for category M1 and M2 UEs), | Rel-9 |  |  |  |
| 15 | Support of - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Measurement reporting event: Event B1 - Neighbour > threshold for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set at least one of the bit number 22, 23, 24, 26 or 39 to 1. - even if the UE sets bits 41, it shall still set bit 15 to 1 if measurement reporting event B1 is tested for all RATs supported by UE - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_15_T | Corresponding to the Index of Indicator, the leftmost binary bit 15. Set to true if supporting all functionalities in the feature group. |
| 16 | Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells; - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1; and - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively NOTE: Event triggered periodical reporting (i.e. with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_16_T | Corresponding to the Index of Indicator, the leftmost binary bit 16. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 16 in Table A.4.5-1a for FDD. |
|  | Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells; - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively NOTE: Event triggered periodical reporting (i.e. with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. |  | Yes | Rel-9 |  |  |  |
| 17 | Support of Intra-frequency ANR features including: - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_17_T | Corresponding to the Index of Indicator, the leftmost binary bit 17. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 17 in Table A.4.5-1a for FDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 18 | Support of Inter-frequency ANR features including: - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_18_T | Corresponding to the Index of Indicator, the leftmost binary bit 18. Set to true if supporting all functionalities in the feature groupIf UE supports FDD and TDD this item shall be set to same value as for item 18 in Table A.4.5-1a for FDD. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 19 | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN, 1xRTT or HRPD, if the UE has set bit number 22, 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 23, 24 or 26 to 1. - even if the UE sets bits 33 to 36, it shall still set bit 19 to 1 if inter-RAT ANR features are tested for all RATs for which inter-RAT measurement reporting is indicated as tested |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_19_T | Corresponding to the Index of Indicator, the leftmost binary bit 19.Set to true if supporting all functionalities in the feature group. |
|  | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for 1xRTT or HRPD, if the UE has set bit number 24 or 26 to 1, respectively. |  |  | Rel-9 |  |  |  |
| 20 | If bit number 7 is set to ‘0’: - SRB1 and SRB2 for DCCH + 8x AM DRB If bit number 7 is set to ‘1’: - SRB1 and SRB2 for DCCH + 8x AM DRB - SRB1 and SRB2 for DCCH + 5x AM DRB + 3x UM DRB NOTE: UE which indicate support for a DRB combination also support all subsets of the DRB combination. Therefore, release of DRB(s) never results in an unsupported DRB combination. | - Regardless of what bit number 7 and bit number 20 is set to, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB - Regardless of what bit number 20 is set to, if bit number 7 is set to ‘1’, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB + 1x UM DRB |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_20_T | Corresponding to the Index of Indicator, the leftmost binary bit 20. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 20 in Table A.4.5-1a for FDD. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 21 | Support of - Predefined intra- and inter-subframe frequency hopping for PUSCH with N_sb > 1 - Predefined inter-subframe frequency hopping for PUSCH with N_sb > 1 | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_21_T | Corresponding to the Index of Indicator, the leftmost binary bit 21. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 21 in Table A.4.5-1a for FDD. |
| 22 | Support of - UTRAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_22_T | Corresponding to the Index of Indicator, the leftmost binary bit 22. Set to true if supporting all functionalities in the feature group. |
|  | Support of - UTRAN FDD or UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports either only UTRAN FDD or only UTRAN TDD - UTRAN FDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD |  |  | Rel-9 |  |  |  |
| 23 | Support of - GERAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_23_T | Corresponding to the Index of Indicator, the leftmost binary bit 23. Set to true if supporting all functionalities in the feature group. |
| 24 | Support of - 1xRTT measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_24_T | Corresponding to the Index of Indicator, the leftmost binary bit 24. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports enhanced 1xRTT CSFB | Rel-9 |  |  |  |
| 25 | Support of - Inter-frequency measurements and reporting in E-UTRA connected mode NOTE: The UE setting this bit to 1 and indicating support for FDD and TDD frequency bands in the UE capability signalling implements and is tested for FDD measurements while the UE is in TDD, and for TDD measurements while the UE is in FDD. | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_25_T | Corresponding to the Index of Indicator, the leftmost binary bit 25. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 25 in Table A.4.5-1a for FDD. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 26 | Support of - HRPD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_26_T | Corresponding to the Index of Indicator, the leftmost binary bit 26. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports HRPD | Rel-9 |  |  |  |
| 27 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH CS handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 8 to 1 and supports SR-VCC from EUTRA defined in TS 24.008 - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_27_T | Corresponding to the Index of Indicator, the leftmost binary bit 27. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH CS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  |  | Rel-9 |  |  |  |
| 28 | Support of - TTI bundling | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_28_T | Corresponding to the Index of Indicator, the leftmost binary bit 28. Set to true if supporting all functionalities in the feature group. |
| 29 | Support of - Semi-Persistent Scheduling | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_29_T | Corresponding to the Index of Indicator, the leftmost binary bit 29. Set to true if supporting all functionalities in the feature group. |
| 30 | Support of - Handover between FDD and TDD | - can only be set to 1 if the UE has set bit number 13 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_30_T | Corresponding to the Index of Indicator, the leftmost binary bit 30. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 30 in Table A.4.5-1a for FDD. |
| 31 | Support of - Indicates whether the UE supports the mechanisms defined for cells broadcasting multi band information i.e. comprehending multiBandInfoList, disregarding in RRC_CONNECTED the related system information fields and understanding the EARFCN signalling for all bands, that overlap with the bands supported by the UE, and that are defined in the earliest version of TS 36.101[42] that includes all UE supported bands. | - This FGI bit is concerns an optional release independent feature (as it was difficult to introduce this from REL-8 when using regular UE capability signalling) |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_31_T | Corresponding to the Index of Indicator, the leftmost binary bit 31. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 31 in Table A.4.5-1a for FDD. |
|  |  |  | Yes | Rel-10 |  |  |  |
| 32 | Undefined |  |  | Rel-8 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 32. |

Table A.4.5-1c: Void

Table A.4.5-1d: Feature group indicators 33-64 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Inter-RAT ANR features for UTRAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 22 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_33_F | Corresponding to the Index of Indicator, the leftmost binary bit 33. Set to true if supporting all functionalities in the feature group. |
| 2 | Inter-RAT ANR features for GERAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 23 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_34_F | Corresponding to the Index of Indicator, the leftmost binary bit 34. Set to true if supporting all functionalities in the feature group. |
| 3 | Inter-RAT ANR features for 1xRTT including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 24 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_35_F | Corresponding to the Index of Indicator, the leftmost binary bit 35. Set to true if supporting all functionalities in the feature group. |
| 4 | Inter-RAT ANR features for HRPD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 26 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_36_F | Corresponding to the Index of Indicator, the leftmost binary bit 36. Set to true if supporting all functionalities in the feature group. |
| 5 | Inter-RAT ANR features for UTRAN TDD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and at least one of the bit number 22 (for UEs supporting only UTRA TDD) or the bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_37_F | Corresponding to the Index of Indicator, the leftmost binary bit 37. Set to true if supporting all functionalities in the feature group. |
| 6 | - EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - can only be set to 1 if the UE has set bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_38_F | Corresponding to the Index of Indicator, the leftmost binary bit 38. Set to true if supporting all functionalities in the feature group. |
| 7 | - UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_39_F | Corresponding to the Index of Indicator, the leftmost binary bit 39. Set to true if supporting all functionalities in the feature group. |
| 8 | - EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - related to SR-VCC - can only be set to 1 if the UE has set bit number 38 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_40_F | Corresponding to the Index of Indicator, the leftmost binary bit 40. Set to true if supporting all functionalities in the feature group. |
| 9 | Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD, if the UE supports UTRAN FDD and has set bit number 22 to 1 | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. | Yes for FDD, unless UE has set bit number 15 to 1 | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_41_F | Corresponding to the Index of Indicator, the leftmost binary bit 41. Set to true if supporting all functionalities in the feature group. |
| 10 | DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) |  |  | Rel-13 | 36.331, Annex B.1 | pc_FeatrGrp_42_F | Corresponding to the Index of Indicator, the leftmost binary bit 42. |
| 11 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 43. |
| 12 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 44. |
| 13 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 45. |
| 14 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 46. |
| 15 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 47. |
| 16 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 48. |
| 17 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 49. |
| 18 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 50. |
| 19 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 51. |
| 20 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 52. |
| 21 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 53. |
| 22 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 54. |
| 23 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 55. |
| 24 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 56. |
| 25 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 57. |
| 26 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 58. |
| 27 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 59. |
| 28 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 60. |
| 29 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 61. |
| 30 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 62. |
| 31 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 63. |
| 32 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 64. |

Table A.4.5-1e: Feature group indicators 33-64 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Inter-RAT ANR features for UTRAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 22 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_33_T | Corresponding to the Index of Indicator, the leftmost binary bit 33. Set to true if supporting all functionalities in the feature group. |
| 2 | Inter-RAT ANR features for GERAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 23 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_34_T | Corresponding to the Index of Indicator, the leftmost binary bit 34. Set to true if supporting all functionalities in the feature group. |
| 3 | Inter-RAT ANR features for 1xRTT including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 24 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_35_T | Corresponding to the Index of Indicator, the leftmost binary bit 35. Set to true if supporting all functionalities in the feature group. |
| 4 | Inter-RAT ANR features for HRPD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 26 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_36_T | Corresponding to the Index of Indicator, the leftmost binary bit 36. Set to true if supporting all functionalities in the feature group. |
| 5 | Inter-RAT ANR features for UTRAN TDD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and at least one of the bit number 22 (for UEs supporting only UTRA TDD) or the bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_37_T | Corresponding to the Index of Indicator, the leftmost binary bit 37. Set to true if supporting all functionalities in the feature group. |
| 6 | - EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - can only be set to 1 if the UE has set bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_38_T | Corresponding to the Index of Indicator, the leftmost binary bit 38. Set to true if supporting all functionalities in the feature group. |
| 7 | - UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_39_T | Corresponding to the Index of Indicator, the leftmost binary bit 39. Set to true if supporting all functionalities in the feature group. |
| 8 | - EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - related to SR-VCC - can only be set to 1 if the UE has set bit number 38 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_40_T | Corresponding to the Index of Indicator, the leftmost binary bit 40. Set to true if supporting all functionalities in the feature group. |
| 9 | Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD, if the UE supports UTRAN FDD and has set bit number 22 to 1 | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_41_T | Corresponding to the Index of Indicator, the leftmost binary bit 41. Set to true if supporting all functionalities in the feature group. |
| 10 | DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) |  |  | Rel-13 | 36.331, Annex B.1 | pc_FeatrGrp_42_T | Corresponding to the Index of Indicator, the leftmost binary bit 42. |
| 11 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 43. |
| 12 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 44. |
| 13 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 45. |
| 14 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 46. |
| 15 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 47. |
| 16 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 48. |
| 17 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 49. |
| 18 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 50. |
| 19 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 51. |
| 20 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 52. |
| 21 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 53. |
| 22 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 54. |
| 23 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 55. |
| 24 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 56. |
| 25 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 57. |
| 26 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 58. |
| 27 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 59. |
| 28 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 60. |
| 29 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 61. |
| 30 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 62. |
| 31 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 63. |
| 32 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 64. |

Table A.4.5-2: EUTRA Feature group indicators

| Item | Additional information | Notes | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of - UTRA CELL_PCH to EUTRA RRC_IDLE cell reselection - UTRA URA_PCH to EUTRA RRC_IDLE cell reselection |  | 25.331, Annex E | Rel-8 | pc_UTRA_FeatrGrp_1 | Corresponding to the Index of Indicator, the leftmost binary bit 1 For Rel-8: Set to true if supporting all functionalities in the feature group For Rel-9 or later releases: this FGI bit is set to TRUE s |
| 2 | Support of - EUTRAN measurements and reporting in connected mode |  | 25.331, Annex E | Rel-8 | pc_UTRA_FeatrGrp_2 | Corresponding to the Index of Indicator, the leftmost binary bit 2 Set to true if supporting all functionalities in the feature group |
| 3 | Support of - UTRA CELL_FACH absolute priority cell reselection for high priority layers |  | 25.331, Annex E | Rel-8 to Rel-10 | pc_UTRA_FeatrGrp_3 | Corresponding to the Index of Indicator, the leftmost binary bit 3 Set to true if supporting all functionalities in the feature group |
|  |  | UE supporting E-UTRAN shall set this bit to ‘TRUE’ in this version of specification. |  | Rel-11 |  |  |
| 4 | Support of - UTRA CELL_FACH absolute priority cell reselection for all layers |  | 25.331, Annex E | Rel-8 to Rel-10 | pc_UTRA_FeatrGrp_4 | Corresponding to the Index of Indicator, the leftmost binary bit 4 Set to true if supporting all functionalities in the feature group |
|  |  | UE supporting E-UTRAN shall set this bit to ‘TRUE’ in this version of specification. |  | Rel-11 |  |  |

Table A.4.5-3: Void

Table A.4.5-3a: Release 10 AS feature group indicators 101-132 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - DMRS with OCC (orthogonal cover code) and SGH (sequence group hopping) disabling | - if the UE supports two or more layers for spatial multiplexing in UL, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_101_F | Corresponding to the Index of Indicator, the leftmost binary bit 101. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 1 in Table A.4.5-3b for TDD. |
|  |  | - If a category 0 UE does not support this feature, this bit shall be set to 0. |  | Rel-12 |  |  |  |
| 2 | - Trigger type 1 SRS (aperiodic SRS) transmission (Up to X ports) NOTE: X = number of supported layers on given band |  |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_102_F | Corresponding to the Index of Indicator, the leftmost binary bit 102. Set to true if supporting all functionalities in the feature group. |
| 3 | - PDSCH transmission mode 9 when up to 4 CSI reference signal ports are configured | - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_103_F | Corresponding to the Index of Indicator, the leftmost binary bit 103. Set to true if supporting all functionalities in the feature group. |
|  |  | - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 4 | - PDSCH transmission mode 9 for TDD when 8 CSI reference signal ports are configured | - if the UE does not support TDD, this bit is irrelevant (capability signalling exists for FDD for this feature), and this bit shall be set to 0. - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_104_F | Corresponding to the Index of Indicator, the leftmost binary bit 104. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 4 in Table A.4.5-3b for TDD. |
|  |  | - if the UE does not support TDD, this bit is irrelevant, and this bit shall be set to 0. - this bit is not applicable to FDD (capability signalling exists for FDD for this feature). - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for TDD, for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 5 | - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 2 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_105_F | Corresponding to the Index of Indicator, the leftmost binary bit 105. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if index 2 is set to 1 for both FDD and TDD, and index 103 is set to 1 either for FDD and TDD. |  | Rel-12 |  |  |  |
| 6 | - Periodic CQI/PMI/RI/PTI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’) and if index 2 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_106_F | Corresponding to the Index of Indicator, the leftmost binary bit 106. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’, and if index 2 is set to 1 for both FDD and TDD. |  | Rel-12 |  |  |  |
| 7 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 1 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_107_F | Corresponding to the Index of Indicator, the leftmost binary bit 107. Set to true if supporting all functionalities in the feature group. |
| 8 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’) and if index 1 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_108_F | Corresponding to the Index of Indicator, the leftmost binary bit 108. Set to true if supporting all functionalities in the feature group. |
| 9 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 1 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_109_F | Corresponding to the Index of Indicator, the leftmost binary bit 109. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 10 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 2 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_110_F | Corresponding to the Index of Indicator, the leftmost binary bit 110. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 11 | - Measurement reporting trigger Event A6 | - this bit can be set to 1 only if the UE supports carrier aggregation. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_111_F | Corresponding to the Index of Indicator, the leftmost binary bit 111. Set to true if supporting all functionalities in the feature group. |
| 12 | - SCell addition within the Handover to EUTRA procedure | - this bit can be set to 1 only if the UE supports carrier aggregation and the Handover to EUTRA procedure. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_112_F | Corresponding to the Index of Indicator, the leftmost binary bit 112. Set to true if supporting all functionalities in the feature group. |
| 13 | - Trigger type 0 SRS (periodic SRS) transmission on X Serving Cells NOTE: X = number of supported component carriers in a given band combination | - this bit can be set to 1 only if the UE supports carrier aggregation in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_113_F | Corresponding to the Index of Indicator, the leftmost binary bit 113. Set to true if supporting all functionalities in the feature group. |
| 14 | - Reporting of both UTRA CPICH RSCP and Ec/N0 in a Measurement Report | - this bit can be set to 1 only if index 22 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_114_F | Corresponding to the Index of Indicator, the leftmost binary bit 114. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 14 in Table A.4.5-3b for TDD. |
| 15 | - time domain ICIC RLM/RRM measurement subframe restriction for the serving cell - time domain ICIC RRM measurement subframe restriction for neighbour cells - time domain ICIC CSI measurement subframe restriction | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_115_F | Corresponding to the Index of Indicator, the leftmost binary bit 115. Set to true if supporting all functionalities in the feature group. |
| 16 | - Relative transmit phase continuity for spatial multiplexing in UL | - this bit can be set to 1 only if the UE supports two or more layers for spatial multiplexing in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_116_F | Corresponding to the Index of Indicator, the leftmost binary bit 116. Set to true if supporting all functionalities in the feature group. |
| 17 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 117. |
| 18 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 118. |
| 19 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 119. |
| 20 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 120. |
| 21 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 121. |
| 22 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 122. |
| 23 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 123. |
| 24 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 124. |
| 25 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 125. |
| 26 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 126. |
| 27 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 127. |
| 28 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 128. |
| 29 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 129. |
| 30 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 130. |
| 31 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 131. |
| 32 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 132. |

Table A.4.5-3b: Release 10 AS feature group indicators 101-132 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - DMRS with OCC (orthogonal cover code) and SGH (sequence group hopping) disabling | - if the UE supports two or more layers for spatial multiplexing in UL, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_101_T | Corresponding to the Index of Indicator, the leftmost binary bit 101. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 1 in Table A.4.5-3a for FDD. |
|  |  | - If a category 0 UE does not support this feature, this bit shall be set to 0. |  | Rel-12 |  |  |  |
| 2 | - Trigger type 1 SRS (aperiodic SRS) transmission (Up to X ports) NOTE: X = number of supported layers on given band |  |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_102_T | Corresponding to the Index of Indicator, the leftmost binary bit 102. Set to true if supporting all functionalities in the feature group. |
| 3 | - PDSCH transmission mode 9 when up to 4 CSI reference signal ports are configured | - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_103_T | Corresponding to the Index of Indicator, the leftmost binary bit 103. Set to true if supporting all functionalities in the feature group. |
|  |  | - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 4 | - PDSCH transmission mode 9 for TDD when 8 CSI reference signal ports are configured | - if the UE does not support TDD, this bit is irrelevant (capability signalling exists for FDD for this feature), and this bit shall be set to 0. - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_104_T | Corresponding to the Index of Indicator, the leftmost binary bit 104. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 4 in Table A.4.5-3a for FDD. |
|  |  | - if the UE does not support TDD, this bit is irrelevant, and this bit shall be set to 0. - this bit is not applicable to FDD (capability signalling exists for FDD for this feature). - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for TDD, for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 5 | - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 2 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_105_T | Corresponding to the Index of Indicator, the leftmost binary bit 105. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if index 2 is set to 1 for both FDD and TDD, and index 103 is set to 1 either for FDD and TDD. |  | Rel-12 |  |  |  |
| 6 | - Periodic CQI/PMI/RI/PTI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’) and if index 2 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_106_T | Corresponding to the Index of Indicator, the leftmost binary bit 106. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’, and if index 2 is set to 1 for both FDD and TDD. |  | Rel-12 |  |  |  |
| 7 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 1 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_107_T | Corresponding to the Index of Indicator, the leftmost binary bit 107. Set to true if supporting all functionalities in the feature group. |
| 8 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’) and if index 1 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_108_T | Corresponding to the Index of Indicator, the leftmost binary bit 108. Set to true if supporting all functionalities in the feature group. |
| 9 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 1 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_109_T | Corresponding to the Index of Indicator, the leftmost binary bit 109. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 10 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 2 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ‘supported’). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_110_T | Corresponding to the Index of Indicator, the leftmost binary bit 110. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 11 | - Measurement reporting trigger Event A6 | - this bit can be set to 1 only if the UE supports carrier aggregation. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_111_T | Corresponding to the Index of Indicator, the leftmost binary bit 111. Set to true if supporting all functionalities in the feature group. |
| 12 | - SCell addition within the Handover to EUTRA procedure | - this bit can be set to 1 only if the UE supports carrier aggregation and the Handover to EUTRA procedure. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_112_T | Corresponding to the Index of Indicator, the leftmost binary bit 112. Set to true if supporting all functionalities in the feature group. |
| 13 | - Trigger type 0 SRS (periodic SRS) transmission on X Serving Cells NOTE: X = number of supported component carriers in a given band combination | - this bit can be set to 1 only if the UE supports carrier aggregation in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_113_T | Corresponding to the Index of Indicator, the leftmost binary bit 113. Set to true if supporting all functionalities in the feature group. |
| 14 | - Reporting of both UTRA CPICH RSCP and Ec/N0 in a Measurement Report | - this bit can be set to 1 only if index 22 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_114_T | Corresponding to the Index of Indicator, the leftmost binary bit 114. Set to true if supporting all functionalities in the feature group. If UE supports FDD and TDD this item shall be set to same value as for item 14 in Table A.4.5-3a for FDD. |
| 15 | - time domain ICIC RLM/RRM measurement subframe restriction for the serving cell - time domain ICIC RRM measurement subframe restriction for neighbour cells - time domain ICIC CSI measurement subframe restriction | - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_115_T | Corresponding to the Index of Indicator, the leftmost binary bit 115. Set to true if supporting all functionalities in the feature group. |
| 16 | - Relative transmit phase continuity for spatial multiplexing in UL | - this bit can be set to 1 only if the UE supports two or more layers for spatial multiplexing in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_116_T | Corresponding to the Index of Indicator, the leftmost binary bit 116. Set to true if supporting all functionalities in the feature group. |
| 17 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 117. |
| 18 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 118. |
| 19 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 119. |
| 20 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 120. |
| 21 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 121. |
| 22 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 122. |
| 23 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 123. |
| 24 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 124. |
| 25 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 125. |
| 26 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 126. |
| 27 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 127. |
| 28 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 128. |
| 29 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 129. |
| 30 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 130. |
| 31 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 131. |
| 32 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 132. |

[End of change]