3GPP TSG-RAN5 Meeting #93-e	R5-216429

# Electronic meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5353 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of A-MPR with PUSCH frequency hopping test requirements for NS_56 (B24) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing A-MPR with PUSCH frequency hopping test requirements for NS_56 (B24) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove FFS from clause 6.2.4_3 - Add A-MPR with PUSCH frequency hopping test requirements into Tables 6.2.4_3.5-1 and 6.2.4_3.5-2 for NS_56 (B24) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR with PUSCH frequency hopping test requirements for NS_56 not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4_3, 6.2.4_3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.4_3	Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping

Following aspects missing or not yet determined:

- Emissions test cases covering this test is missing

- HARQ configuration for re-transmissions is missing

-------- Skipped unchanged clauses --------

##### 6.2.4_3.5	Test requirements

The maximum output power, derived in step 3 and 4 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table 6.2.4_3.5-1 and 6.2.4_3.5-2. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

Table 6.2.4_3.5-1: UE Power Class 3 test requirements (for TTI where CURRENT_TX_NB mod2=0 hence without mirroring)

| Configuration ID | NS-value | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NS_07 | 0 | 12 | 0 | 11 | 6 | 25.7 | 4.3 |
| 2 | NS_10 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | NS_11 | 0 | 10 | 0 | 13 | 5 | 25.7 | 7.3 |
| 4 | NS_12 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 5 | NS_13 | 0 | 3 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| 6 | NS_14 | 0 | 3 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| 7 | NS_15 | 0 | 4 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 8 | NS_16 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 9 | NS_19 | 0 | 2 | 0 | 21 | 2 | 25.7 | 18.3 |
| 10 | NS_20 | 0 | 16 | 0 | 7 | 7 | 25.7 | -0.7 |
| 11 | NS_56 | 0 | 7 | 0 | 16 | 5 | 25.7 | 10.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

Table 6.2.4_3.5-2: UE Power Class 3 test requirements (for TTI where CURRENT_TX_NB mod2=1 hence with mirroring)

| Configuration ID | NS-value | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NS_07 | 0 | 3 | 0 | 20 | 6 | 25.7 | 13.3 |
| 2 | NS_10 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 3 | NS_11 | 0 | 6.5 | 0 | 16.5 | 5 | 25.7 | 10.8 |
| 4 | NS_12 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 5 | NS_13 | 0 | 0 | 0 | 23 | 2.5 | 25.7 | 19.8 |
| 6 | NS_14 | 0 | 0 | 0 | 23 | 2.5 | 25.7 | 19.8 |
| 7 | NS_15 | 0 | 9 | 0 | 14 | 5 | 25.7 | 8.3 |
| 8 | NS_16 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 9 | NS_19 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 10 | NS_20 | 0 | 6 | 0 | 17 | 7 | 25.7 | 9.3 |
| 11 | NS_56 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

-------- End of changes --------