3GPP TSG-RAN5 Meeting #92-e	R5-216527

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5361 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UL-MIMO MOP and MPR test cases for LTE band 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The tolerance for maximum output power for UL-MIMO for LTE band 24 has been changed in TS 36.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to conformance and test requirements of UL-MIMO MOP and MPR test cases for LTE band 24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance and test requirements will not reflect the recent updates for LTE band 24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2B, 6.2.3B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.2B	UE Maximum Output Power for UL-MIMO

#### 6.2.2B.1	Test purpose

To verify that the error of UE maximum output power in UL MIMO does not exceed the range prescribed by the specified UL MIMO Power Class and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.2B.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 10 and forward that support UL MIMO.

#### 6.2.2B.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2.2B.3-1 with the UL-MIMO configurations specified in Table 6.2.2B.3-2. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2B.3-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2/-3 |  |  |
| 2 |  |  |  |  | 23 | +2/-32 |  |  |
| 3 |  |  |  |  | 23 | +2/-32 |  |  |
| 4 |  |  |  |  | 23 | +2/-3 |  |  |
| 5 |  |  |  |  | 23 | +2/-3 |  |  |
| 6 |  |  |  |  | 23 | +2/-3 |  |  |
| 7 |  |  |  |  | 23 | +2/-32 |  |  |
| 8 |  |  |  |  | 23 | +2/-32 |  |  |
| 9 |  |  |  |  | 23 | +2/-3 |  |  |
| 10 |  |  |  |  | 23 | +2/-3 |  |  |
| 11 |  |  |  |  | 23 | +2/-3 |  |  |
| 12 |  |  |  |  | 23 | +2/-32 |  |  |
| 13 |  |  |  |  | 23 | +2/-3 |  |  |
| 14 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2/-3 |  |  |
| 18 |  |  |  |  | 23 | +2/-3 |  |  |
| 19 |  |  |  |  | 23 | +2/-3 |  |  |
| 20 |  |  |  |  | 23 | +2/-32 |  |  |
| 21 |  |  |  |  | 23 | +2/-3 |  |  |
| 22 |  |  |  |  | 23 | +2/-4.52 |  |  |
| 23 |  |  |  |  | 23 | +2/-3 |  |  |
| 24 |  |  |  |  | 23 | +2/-42 |  |  |
| 25 |  |  |  |  | 23 | +2/-32 |  |  |
| 26 |  |  |  |  | 23 | +2/-32 |  |  |
| 27 |  |  |  |  | 23 | +2/-3 |  |  |
| 28 |  |  |  |  | 23 | +2/[-3] |  |  |
| 30 |  |  |  |  | 23 | +2/-3 |  |  |
| 31 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2/-3 |  |  |
| 34 |  |  |  |  | 23 | +2/-3 |  |  |
| 35 |  |  |  |  | 23 | +2/-3 |  |  |
| 36 |  |  |  |  | 23 | +2/-3 |  |  |
| 37 |  |  |  |  | 23 | +2/-3 |  |  |
| 38 |  |  |  |  | 23 | +2/-3 |  |  |
| 39 |  |  |  |  | 23 | +2/-3 |  |  |
| 40 |  |  |  |  | 23 | +2/-3 |  |  |
| 41 |  |  |  |  | 23 | +2/-32 |  |  |
| 42 |  |  |  |  | 23 | +2/-4 |  |  |
| 43 |  |  |  |  | 23 | +2/-4 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2/-3 |  |  |
| 66 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2/-3 |  |  |
| 72 |  |  |  |  | 23 | +2/-3 |  |  |
| 73 |  |  |  |  | 23 | +2/-3 |  |  |
| 74 |  |  |  |  | 23 | +2/-3 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

Table 6.2.2B.3-2: UL-MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission mode | DCI format | Codebook Index |
| --- | --- | --- |
| Mode 2 | DCI format 4 | Codebook index 0 |

The normative reference for this requirement is TS 36.101 clause 6.2.2B.

#### 6.2.2B.4	Test description

##### 6.2.2B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.2B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 1 | 1 |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 3MHz |  |  | QPSK | 1 | 1 |
| 3MHz |  |  | QPSK | 4 | 4 |
| 5MHz |  |  | QPSK | 1 | 1 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 10MHz |  |  | QPSK | 1 | 1 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 15MHz |  |  | QPSK | 1 | 1 |
| 15MHz |  |  | QPSK | 16 | 16 |
| 20MHz |  |  | QPSK | 1 | 1 |
| 20MHz |  |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For E-UTRA bands not applied with Note 2 in Table 6.2.2B.3-1: - The 1 RB allocation shall be tested at RB#0 for low and mid range, RB #max for high range test frequency. - The starting resource block of non-1RB allocation shall be RB #0 for low and mid range, RB# (max +1 - RB allocation) for high range test frequency. Note 3: For E-UTRA bands applied with Note 2 in Table 6.2.2B.3-1: - If the test channel bandwidth is larger than 4MHz, then the 1 RB allocation shall be tested at both RB #0 and RB #max. - If the test channel bandwidth is smaller or equal to 4MHz, then the 1 RB allocation shall be tested at RB #0. - If the test channel bandwidth = (FUL_high - FUL_low) specified by the operating band, then only one frequency range shall be tested and the 1 RB allocation shall be tested at RB #0, RB # and RB #max. - For non-1RB allocation, test frequency is middle range, and the starting resource block shall be RB #0. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.2B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RFaccording to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.2B.4.3.

##### 6.2.2B.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.2.2B.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean sum power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

##### 6.2.2B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6.

#### 6.2.2B.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2B.5-1.

Table 6.2.2B.5-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |  | Class 4 (dBm) |  | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 2 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 3 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 4 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 5 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 6 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 7 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 8 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 9 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 10 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 11 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 12 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 13 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 14 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 18 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 19 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 20 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 21 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 22 |  |  |  |  | 23 | +3/-5.52 |  |  |  |  |
| 23 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 24 |  |  |  |  | 23 | +2.7/-4.72 |  |  |  |  |
| 25 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 26 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 27 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 28 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |  |  |
| 30 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 31 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 34 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 35 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 36 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 37 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 38 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 39 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 40 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 41 |  |  |  |  | 23 | +2.7/-3.72 |  |  |  |  |
| 42 |  |  |  |  | 23 | +3/-5 |  |  |  |  |
| 43 |  |  |  |  | 23 | +3/-5 |  |  |  |  |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |  |  |
| 45 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| .. |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 66 |  |  |  |  | 23 | +2/-3 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 72 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 73 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7/-3.7 |  |  |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 4: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Tables 6.2.5.3-2, 6.2.5.3-3 and 6.2.5.3-4 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.3B	Maximum Power Reduction (MPR) for UL-MIMO

#### 6.2.3B.1	Test purpose

The number of RB identified in Table 6.2.3B.3-1 is based on meeting the requirements for adjacent channel leakage ratio for UL-MIMO and the maximum power reduction (MPR) for UL-MIMO due to Cubic Metric (CM).

#### 6.2.3B.2	Test applicability

The requirements of this test apply in test cases 6.6.2.3B Adjacent Channel Leakage power Ratio to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

NOTE:	As a result TC 6.2.3B has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.3B.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. For UE Power Class 3, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2B.3-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3B.3-1 with UL-MIMO configurations defined in Table 6.2.2B.3-2.

Table 6.2.3B.3-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | Channel bandwidth / Transmission bandwidth configuration [RB] |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2.5B.3 apply.

The normative reference for this requirement is TS 36.101 clause 6.2.3B.

#### 6.2.3B.4	Test description

##### 6.2.3B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.4.2B.1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.3B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause 4.3.1 |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] clause 4.3.1 |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  | 16QAM | 5 | 5 |
| 1.4MHz |  |  | 16QAM | 6 | 6 |
| 3.0MHz |  |  | QPSK | 4 | 4 |
| 3.0MHz |  |  | QPSK | 15 | 15 |
| 3.0MHz |  |  | 16QAM | 4 | 4 |
| 3.0MHz |  |  | 16QAM | 15 | 15 |
| 5MHz |  |  | QPSK | 8 | 8 |
| 5MHz |  |  | QPSK | 25 | 25 |
| 5MHz |  |  | 16QAM | 8 | 8 |
| 5MHz |  |  | 16QAM | 25 | 25 |
| 10MHz |  |  | QPSK | 12 | 12 |
| 10MHz |  |  | QPSK | 50 | 50 |
| 10MHz |  |  | 16QAM | 12 | 12 |
| 10MHz |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 15MHz |  |  | QPSK | 16 | 16 |
| 15MHz |  |  | QPSK | 75 | 75 |
| 15MHz |  |  | 16QAM | 16 | 16 |
| 15MHz |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 20MHz |  |  | QPSK | 18 | 18 |
| 20MHz |  |  | QPSK | 100 | 100 |
| 20MHz |  |  | 16QAM | 18 | 18 |
| 20MHz |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. |  |  |  |  |  |

1.	Connect the SS and interfering sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2.3B.4.1-1.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.3B.4.3.

##### 6.2.3B.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.2.3B.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

##### 6.2.3B.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

#### 6.2.3B.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3B.5-1.

Table 6.2.3B.5-1: UE Power Class test requirements

| E-UTRA Band | Class 1 (dBm) | Tol. (Db) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | QPSK partial RB allocation Tol. (dB)) | QPSK full RB allocation Tol. (dB)) | 16QAM partial RB allocation Tol. (dB) | 16QAM full RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 2 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 3 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 4 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 5 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 6 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 7 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 8 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 9 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 10 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 11 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 12 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 13 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 14 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 18 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 19 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 20 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.71,2 | +2.7/-4.71,2 | +2.7/-5.71,2 |
| 21 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 22 |  |  |  |  | 23 | +3.0/-5.5 | +3.0/-6.5 | +3.0/-6.5 | +3.0/-7.5 |
| … |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 24 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-5.71,2 | +2.7/-5.71,2 | +2.7/-6.71,2 |
| 25 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-[4.7]1,2 | +2.7/-[4.7]1,2 | +2.7/-[5.7]1,2 |
| 26 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 27 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 28 |  |  |  |  | 23 | +2.7/[-3.7] | +2.7/[-4.7] | +2.7/[-4.7] | +2.7/[-5.7] |
| 30 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 31 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| ... |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 34 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 35 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 36 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 37 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 38 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 39 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 40 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 41 |  |  |  |  | 23 | +2.7/-3.71 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 42 |  |  |  |  | 23 | +3.0/-5.0 | +3.0/-6.01,2 | +3.0/-6.01,2 | +3.0/-7.01,2 |
| 43 |  |  |  |  | 23 | +3.0/-5.0 | +3.0/-6.0 | +3.0/-6.0 | +3.0/-7.01,2 |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] | +2.7/[-4.7] | +2.7/[-4.7] | +2.7/[-5.7] |
| 45 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 66 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| 72 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| … |  |  |  |  |  |  |  |  |  |
| 74 |  |  |  |  | 23 | +2.7/-3.7 | +2.7/-4.7 | +2.7/-4.7 | +2.7/-5.7 |
| Note 1: For transmission configurations (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: For the UE maximum output power modified by MPR, the power limits specified in Table 6.2.5B.3-1 apply |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------