3GPP TSG-RAN5 Meeting #93-e	R5-216531

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1397 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-MPR test case for n24 and n99 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adds configurations and requirements for A-MPR test cases for NS_56 for n24 and n99; updates were made to 38.101 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to align with 38.101; Addition of tables related to A-MPR test configurations for NS_56; Addition of table related to A-MPR test requirements for NS_56 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR test cases for n24 and n99 will be not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2.3.3.28, 6.2.3.4, 6.2.3.5, 6.2C.5, 6.2C.5.4, 6.2C.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- PC1 requirements for NR operating bands other than Band n14 are not fully defined in RAN4 Rel-15 and Rel-16 specifications due to the lack of MPR requirements.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.2.3.3.28	A-MPR for NS_56

Table 6.2.3.3.28: A-MPR for NS_56

For 5 MHz channel centred on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centred on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with centre frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)}], and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 5 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 3 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 4 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 2 dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

<<Unchanged Text Skipped>>

Table 6.2.3.4.1-30: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1630.0 | 5 | 15 | N/A for A-MPR testing | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | 1630.0 | 5 | 30 |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | 1630.0 | 5 | Default |  | 6 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | 1630.0 | 5 | Default |  | 4 |  | PI/2 BPSK |  | 20@4 | 8@3 | N/A |
| 5 | 1630.0 | 5 | Default |  | 4 |  | PI/2 BPSK |  | 1@4 | 1@3 | N/A |
| 6 | 1630.0 | 5 | 15 |  | 14 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 7 | 1630.0 | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 8 | 1630.0 | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 9 | 1630.0 | 5 | Default |  | 4 |  | QPSK |  | 20@4 | 8@3 | N/A |
| 10 | 1630.0 | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 11 | 1630.0 | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | 1630.0 | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | 1630.0 | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | 1630.0 | 5 | Default |  | 4 |  | 16 QAM |  | 20@4 | 8@3 | N/A |
| 15 | 1630.0 | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 16 | 1630.0 | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | 1630.0 | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | 1630.0 | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | 1630.0 | 5 | Default |  | 4 |  | 64 QAM |  | 20@4 | 8@3 | N/A |
| 20 | 1630.0 | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 21 | 1630.0 | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 22 | 1630.0 | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 23 | 1630.0 | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 24 | 1630.0 | 5 | Default |  | 4 |  | 256 QAM |  | 20@4 | 8@3 | N/A |
| 25 | 1630.0 | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 26 | 1632.5 | 10 | 15 |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 27 | 1632.5 | 10 | Highest |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 28 | 1632.5 | 10 | Default |  | 8 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 29 | 1632.5 | 10 | Default |  | 6 |  | PI/2 BPSK |  | 1@3 | 1@2 | 1@1 |
| 30 | 1632.5 | 10 | Default |  | 6 |  | PI/2 BPSK |  | 40@9 | 18@5 | 8@2 |
| 31 | 1632.5 | 10 | Default |  | 4 |  | PI/2 BPSK |  | 1@35 | 1@17 | 1@8 |
| 32 | 1632.5 | 10 | Default |  | 2 |  | PI/2 BPSK |  | 16@35 | 5@19 | 3@8 |
| 33 | 1632.5 | 10 | Default |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 34 | 1632.5 | 10 | 15 |  | 5 |  | PI/2 BPSK |  | 1@40 | N/A | N/A |
| 35 | 1632.5 | 10 | 30 |  | 5 |  | PI/2 BPSK |  | N/A | 1@20 | N/A |
| 36 | 1632.5 | 10 | 15 |  | 3 |  | PI/2 BPSK |  | 8@44 | N/A | N/A |
| 37 | 1632.5 | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 38 | 1632.5 | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 39 | 1632.5 | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 40 | 1632.5 | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 41 | 1632.5 | 10 | Default |  | 6 |  | QPSK |  | 40@9 | 18@5 | 8@2 |
| 42 | 1632.5 | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 43 | 1632.5 | 10 | Default |  | 2 |  | QPSK |  | 16@35 | 5@19 | 3@8 |
| 44 | 1632.5 | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 45 | 1632.5 | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |
| 46 | 1632.5 | 10 | 30 |  | 5 |  | QPSK |  | N/A | 1@20 | N/A |
| 47 | 1632.5 | 10 | 15 |  | 3 |  | QPSK |  | 8@44 | N/A | N/A |
| 48 | 1632.5 | 10 | 15 |  | 12 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 49 | 1632.5 | 10 | Highest |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 50 | 1632.5 | 10 | Default |  | 8 |  | 16 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 51 | 1632.5 | 10 | Default |  | 6 |  | 16 QAM |  | 1@3 | 1@2 | 1@1 |
| 52 | 1632.5 | 10 | Default |  | 6 |  | 16 QAM |  | 40@9 | 18@5 | 8@2 |
| 53 | 1632.5 | 10 | Default |  | 4 |  | 16 QAM |  | 1@35 | 1@17 | 1@8 |
| 54 | 1632.5 | 10 | Default |  | 2 |  | 16 QAM |  | 16@35 | 5@19 | 3@8 |
| 55 | 1632.5 | 10 | Default |  | 5 |  | 16 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 56 | 1632.5 | 10 | 15 |  | 5 |  | 16 QAM |  | 1@40 | N/A | N/A |
| 57 | 1632.5 | 10 | 30 |  | 5 |  | 16 QAM |  | N/A | 1@20 | N/A |
| 58 | 1632.5 | 10 | 15 |  | 3 |  | 16 QAM |  | 8@44 | N/A | N/A |
| 59 | 1632.5 | 10 | 15 |  | 12 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 60 | 1632.5 | 10 | Highest |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 61 | 1632.5 | 10 | Default |  | 8 |  | 64 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 62 | 1632.5 | 10 | Default |  | 6 |  | 64 QAM |  | 1@3 | 1@2 | 1@1 |
| 63 | 1632.5 | 10 | Default |  | 6 |  | 64 QAM |  | 40@9 | 18@5 | 8@2 |
| 64 | 1632.5 | 10 | Default |  | 4 |  | 64 QAM |  | 1@35 | 1@17 | 1@8 |
| 65 | 1632.5 | 10 | Default |  | 2 |  | 64 QAM |  | 16@35 | 5@19 | 3@8 |
| 66 | 1632.5 | 10 | Default |  | 5 |  | 64 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 67 | 1632.5 | 10 | 15 |  | 5 |  | 64 QAM |  | 1@40 | N/A | N/A |
| 68 | 1632.5 | 10 | 30 |  | 5 |  | 64 QAM |  | N/A | 1@20 | N/A |
| 69 | 1632.5 | 10 | 15 |  | 3 |  | 64 QAM |  | 8@44 | N/A | N/A |
| 70 | 1632.5 | 10 | 15 |  | 12 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 71 | 1632.5 | 10 | Highest |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 72 | 1632.5 | 10 | Default |  | 8 |  | 256 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 73 | 1632.5 | 10 | Default |  | 6 |  | 256 QAM |  | 1@3 | 1@2 | 1@1 |
| 74 | 1632.5 | 10 | Default |  | 6 |  | 256 QAM |  | 40@9 | 18@5 | 8@2 |
| 75 | 1632.5 | 10 | Default |  | 4 |  | 256 QAM |  | 1@35 | 1@17 | 1@8 |
| 76 | 1632.5 | 10 | Default |  | 2 |  | 256 QAM |  | 16@35 | 5@19 | 3@8 |
| 77 | 1632.5 | 10 | Default |  | 5 |  | 256 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 78 | 1632.5 | 10 | 15 |  | 5 |  | 256 QAM |  | 1@40 | N/A | N/A |
| 79 | 1632.5 | 10 | 30 |  | 5 |  | 256 QAM |  | N/A | 1@20 | N/A |
| 80 | 1632.5 | 10 | 15 |  | 3 |  | 256 QAM |  | 8@44 | N/A | N/A |
| 81 | 1630.0 | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 82 | 1630.0 | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 83 | 1630.0 | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 84 | 1630.0 | 5 | Default |  | 4 |  | QPSK |  | 21@4 | 8@3 | N/A |
| 85 | 1630.0 | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 86 | 1630.0 | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 87 | 1630.0 | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 88 | 1630.0 | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 89 | 1630.0 | 5 | Default |  | 4 |  | 16 QAM |  | 21@4 | 8@3 | N/A |
| 90 | 1630.0 | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 91 | 1630.0 | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 92 | 1630.0 | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 93 | 1630.0 | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 94 | 1630.0 | 5 | Default |  | 4 |  | 64 QAM |  | 21@4 | 8@3 | N/A |
| 95 | 1630.0 | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 96 | 1630.0 | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 97 | 1630.0 | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 98 | 1630.0 | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 99 | 1630.0 | 5 | Default |  | 4 |  | 256 QAM |  | 21@4 | 8@3 | N/A |
| 100 | 1630.0 | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 101 | 1632.5 | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 102 | 1632.5 | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 103 | 1632.5 | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 104 | 1632.5 | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 105 | 1632.5 | 10 | Default |  | 6 |  | QPSK |  | 43@9 | 19@5 | 8@2 |
| 106 | 1632.5 | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 107 | 1632.5 | 10 | Default |  | 2 |  | QPSK |  | 17@35 | 5@19 | 3@8 |
| 108 | 1632.5 | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 109 | 1632.5 | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |
| 110 | 1632.5 | 10 | 30 |  | 5 |  | QPSK |  | N/A | 1@20 | N/A |
| 111 | 1632.5 | 10 | 15 |  | 3 |  | QPSK |  | 8@44 | N/A | N/A |
| 112 | 1632.5 | 10 | 15 |  | 12 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 113 | 1632.5 | 10 | Highest |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 114 | 1632.5 | 10 | Default |  | 8 |  | 16 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 115 | 1632.5 | 10 | Default |  | 6 |  | 16 QAM |  | 1@3 | 1@2 | 1@1 |
| 116 | 1632.5 | 10 | Default |  | 6 |  | 16 QAM |  | 43@9 | 19@5 | 8@2 |
| 117 | 1632.5 | 10 | Default |  | 4 |  | 16 QAM |  | 1@35 | 1@17 | 1@8 |
| 118 | 1632.5 | 10 | Default |  | 2 |  | 16 QAM |  | 17@35 | 5@19 | 3@8 |
| 119 | 1632.5 | 10 | Default |  | 5 |  | 16 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 120 | 1632.5 | 10 | 15 |  | 5 |  | 16 QAM |  | 1@40 | N/A | N/A |
| 121 | 1632.5 | 10 | 30 |  | 5 |  | 16 QAM |  | N/A | 1@20 | N/A |
| 122 | 1632.5 | 10 | 15 |  | 3 |  | 16 QAM |  | 8@44 | N/A | N/A |
| 123 | 1632.5 | 10 | 15 |  | 12 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 124 | 1632.5 | 10 | Highest |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 125 | 1632.5 | 10 | Default |  | 8 |  | 64 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 126 | 1632.5 | 10 | Default |  | 6 |  | 64 QAM |  | 1@3 | 1@2 | 1@1 |
| 127 | 1632.5 | 10 | Default |  | 6 |  | 64 QAM |  | 43@9 | 19@5 | 8@2 |
| 128 | 1632.5 | 10 | Default |  | 4 |  | 64 QAM |  | 1@35 | 1@17 | 1@8 |
| 129 | 1632.5 | 10 | Default |  | 2 |  | 64 QAM |  | 17@35 | 5@19 | 3@8 |
| 130 | 1632.5 | 10 | Default |  | 5 |  | 64 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 131 | 1632.5 | 10 | 15 |  | 5 |  | 64 QAM |  | 1@40 | N/A | N/A |
| 132 | 1632.5 | 10 | 30 |  | 5 |  | 64 QAM |  | N/A | 1@20 | N/A |
| 133 | 1632.5 | 10 | 15 |  | 3 |  | 64 QAM |  | 8@44 | N/A | N/A |
| 134 | 1632.5 | 10 | 15 |  | 12 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 135 | 1632.5 | 10 | Highest |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 136 | 1632.5 | 10 | Default |  | 8 |  | 256 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 137 | 1632.5 | 10 | Default |  | 6 |  | 256 QAM |  | 1@3 | 1@2 | 1@1 |
| 138 | 1632.5 | 10 | Default |  | 6 |  | 256 QAM |  | 43@9 | 19@5 | 8@2 |
| 139 | 1632.5 | 10 | Default |  | 4 |  | 256 QAM |  | 1@35 | 1@17 | 1@8 |
| 140 | 1632.5 | 10 | Default |  | 2 |  | 256 QAM |  | 17@35 | 5@19 | 3@8 |
| 141 | 1632.5 | 10 | Default |  | 5 |  | 256 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 142 | 1632.5 | 10 | 15 |  | 5 |  | 256 QAM |  | 1@40 | N/A | N/A |
| 143 | 1632.5 | 10 | 30 |  | 5 |  | 256 QAM |  | N/A | 1@20 | N/A |
| 144 | 1632.5 | 10 | 15 |  | 3 |  | 256 QAM |  | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

<<Unchanged Text Skipped>>

Table 6.2.3.5-36: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 7 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 8 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 12 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 13 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 17 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 18 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 22 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 25 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 26 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 27 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 29 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 33 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 38 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 39 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 40 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 44 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 51 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 53 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 55 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 57 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 58 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 59 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 60 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 61 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 62 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 63 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 64 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 65 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 66 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 67 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 68 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 69 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 70 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 71 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 73 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 74 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 75 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 77 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 78 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 79 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 80 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 82 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 83 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 84 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 85 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 87 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 88 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 89 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 91 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 92 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 95 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 97 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 98 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 3 | 12 | 0 | 9 | 6 | 3 | 25+TT | 5-TT |
| 102 | 23 | 3 | 8 | 0 | 11 | 5 | 3 | 25+TT | 10-TT |
| 103 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 104 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 105 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 106 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 108 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 109 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 110 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 111 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 112 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 113 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 114 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 115 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 116 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 117 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 119 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 120 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 121 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 122 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 123 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 124 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 125 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 126 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 127 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 128 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 129 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 130 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 131 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 133 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 134 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 135 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 136 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 137 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 138 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 139 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 140 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 141 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 142 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 143 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 144 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2C.5	UE additional maximum output power reduction for SUL

Editor’s notes:

- Tests for network signalling values NS_17, NS_43 not complete.

6.2C.5.1	Test purpose

Same test purpose as in clause 6.2.3.1

6.2C.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.2.3.

6.2C.5.4	Test description

Same test description as specified in clause 6.2.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

For NS_03: Instead of table 6.2.3.4.1-3  use Table 6.2C.5.4-1

For NS_18: Instead of table 6.2.3.4.1-11  use Table 6.2C.5.4-2

For NS_48: Instead of table 6.2.3.4.1-19  use Table 6.2C.5.4-3

For NS_49: Instead of table 6.2.3.4.1-29  use Table 6.2C.5.4-4

For NS_05: Instead of table 6.2.3.4.1-4  use Table 6.2C.5.4-5

For NS_05U: Instead of table 6.2.3.4.1-5  use Table 6.2C.5.4-6

For NS_100: Instead of table 6.2.3.5-12  use Table 6.2C.5.4-7

For NS_56: Instead of table 6.2.3.4.1-30  use Table 6.2C.5.4-8

<<Unchanged Text Skipped>>

Table 6.2C.5.4-8: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | 15 kHz for all carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Modulation (NOTE 2) |  |  |  |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 |  | 1630.0 |  | 5 |  | N/A for A-MPR testing |  | N/A |  | 14 |  | DFT-s-OFDM |  | PI/2 BPSK |  |  | Edge_1RB_Left |
| 2 |  | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | PI/2 BPSK |  |  | Outer_Full |
| 3 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | PI/2 BPSK |  |  | 20@4 |
| 4 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | PI/2 BPSK |  |  | 1@4 |
| 5 |  | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | QPSK |  |  | Edge_1RB_Left |
| 6 |  | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | Outer_Full |
| 7 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 20@4 |
| 8 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 1@4 |
| 9 |  | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 16 QAM |  |  | Edge_1RB_Left |
| 10 |  | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | Outer_Full |
| 11 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 20@4 |
| 12 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 1@4 |
| 13 |  | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 64 QAM |  |  | Edge_1RB_Left |
| 14 |  | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  | Outer_Full |
| 15 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 20@4 |
| 16 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 1@4 |
| 17 |  | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 256 QAM |  |  | Edge_1RB_Left |
| 18 |  | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | Outer_Full |
| 19 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 20@4 |
| 20 |  | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 1@4 |
| 21 |  | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | PI/2 BPSK |  |  | Edge_1RB_Left |
| 22 |  | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | PI/2 BPSK |  |  | Outer_Full |
| 23 |  | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | PI/2 BPSK |  |  | 1@3 |
| 24 |  | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | PI/2 BPSK |  |  | 40@9 |
| 25 |  | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | PI/2 BPSK |  |  | 1@35 |
| 26 |  | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | PI/2 BPSK |  |  | 16@35 |
| 27 |  | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | PI/2 BPSK |  |  | Edge_1RB_Right |
| 28 |  | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | PI/2 BPSK |  |  | 1@40 |
| 29 |  | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | PI/2 BPSK |  |  | 8@44 |
| 30 |  | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | QPSK |  |  | Edge_1RB_Left |
| 31 |  | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | QPSK |  |  | Outer_Full |
| 32 |  | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | 1@3 |
| 33 |  | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | 40@9 |
| 34 |  | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 1@35 |
| 35 |  | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | QPSK |  |  | 16@35 |
| 36 |  | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | QPSK |  |  | Edge_1RB_Right |
| 37 |  | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | QPSK |  |  | 1@40 |
| 38 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | QPSK |  |  | 8@44 |  |
| 39 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 16 QAM |  |  | Edge_1RB_Left |  |
| 40 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 16 QAM |  |  | Outer_Full |  |
| 41 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | 1@3 |  |
| 42 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | 40@9 |  |
| 43 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 1@35 |  |
| 44 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 16 QAM |  |  | 16@35 |  |
| 45 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 16 QAM |  |  | Edge_1RB_Right |  |
| 46 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 16 QAM |  |  | 1@40 |  |
| 47 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 16 QAM |  |  | 8@44 |  |
| 48 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 64 QAM |  |  | Edge_1RB_Left Outer_Full |  |
| 49 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 64 QAM |  |  |  |  |
| 50 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  | 1@3 40@9 |  |
| 51 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  |  |  |
| 52 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 1@35 |  |
| 53 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 64 QAM |  |  | 16@35 |  |
| 54 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 64 QAM |  |  | Edge_1RB_Right |  |
| 55 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 64 QAM |  |  | 1@40 |  |
| 56 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 64 QAM |  |  | 8@44 |  |
| 57 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 256 QAM |  |  | Edge_1RB_Left |  |
| 58 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 256 QAM |  |  | Outer_Full |  |
| 59 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | 1@3 |  |
| 60 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | 40@9 |  |
| 61 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 1@35 |  |
| 62 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 256 QAM |  |  | 16@35 |  |
| 63 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 256 QAM |  |  | Edge_1RB_Right |  |
| 64 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 256 QAM |  |  | 1@40 |  |
| 65 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 256 QAM |  |  | 8@44 |  |
| 66 | 1630.0 |  | 5 |  | N/A for A-MPR testing |  | N/A |  | 14 |  | CP-OFDM |  | QPSK |  |  | Edge_1RB_Left |  |
| 67 | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | Outer_Full |  |
| 68 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 21@4 |  |
| 69 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 1@4 |  |
| 70 | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 16 QAM |  |  | Edge_1RB_Left |  |
| 71 | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | Outer_Full |  |
| 72 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 21@4 |  |
| 73 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 1@4 |  |
| 74 | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 64 QAM |  |  | Edge_1RB_Left |  |
| 75 | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  | Outer_Full |  |
| 76 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 21@4 |  |
| 77 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 1@4 |  |
| 78 | 1630.0 |  | 5 |  |  |  |  |  | 14 |  |  |  | 256 QAM |  |  | Edge_1RB_Left |  |
| 79 | 1630.0 |  | 5 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | Outer_Full |  |
| 80 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 21@4 |  |
| 81 | 1630.0 |  | 5 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 1@4 |  |
| 82 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | QPSK |  |  | Edge_1RB_Left |  |
| 83 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | QPSK |  |  | Outer_Full |  |
| 84 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | 1@3 |  |
| 85 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | QPSK |  |  | 43@9 |  |
| 86 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | QPSK |  |  | 1@35 |  |
| 87 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | QPSK |  |  | 17@35 |  |
| 88 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | QPSK |  |  | Edge_1RB_Right |  |
| 89 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | QPSK |  |  | 1@40 |  |
| 90 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | QPSK |  |  | 8@44 |  |
| 91 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 16 QAM |  |  | Edge_1RB_Left |  |
| 92 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 16 QAM |  |  | Outer_Full |  |
| 93 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | 1@3 |  |
| 94 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 16 QAM |  |  | 43@9 |  |
| 95 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 16 QAM |  |  | 1@35 |  |
| 96 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 16 QAM |  |  | 17@35 |  |
| 97 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 16 QAM |  |  | Edge_1RB_Right |  |
| 98 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 16 QAM |  |  | 1@40 |  |
| 99 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 16 QAM |  |  | 8@44 |  |
| 100 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 64 QAM |  |  | Edge_1RB_Left |  |
| 101 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 64 QAM |  |  | Outer_Full |  |
| 102 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  | 1@3 |  |
| 103 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 64 QAM |  |  | 43@9 |  |
| 104 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 64 QAM |  |  | 1@35 |  |
| 105 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 64 QAM |  |  | 17@35 |  |
| 106 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 64 QAM |  |  | Edge_1RB_Right |  |
| 107 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 64 QAM |  |  | 1@40 |  |
| 108 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 64 QAM |  |  | 8@44 |  |
| 109 | 1632.5 |  | 10 |  |  |  |  |  | 12 |  |  |  | 256 QAM |  |  | Edge_1RB_Left |  |
| 110 | 1632.5 |  | 10 |  |  |  |  |  | 8 |  |  |  | 256 QAM |  |  | Outer_Full |  |
| 111 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | 1@3 |  |
| 112 | 1632.5 |  | 10 |  |  |  |  |  | 6 |  |  |  | 256 QAM |  |  | 43@9 |  |
| 113 | 1632.5 |  | 10 |  |  |  |  |  | 4 |  |  |  | 256 QAM |  |  | 1@35 |  |
| 114 | 1632.5 |  | 10 |  |  |  |  |  | 2 |  |  |  | 256 QAM |  |  | 17@35 |  |
| 115 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 256 QAM |  |  | Edge_1RB_Right |  |
| 116 | 1632.5 |  | 10 |  |  |  |  |  | 5 |  |  |  | 256 QAM |  |  | 1@40 |  |
| 117 | 1632.5 |  | 10 |  |  |  |  |  | 3 |  |  |  | 256 QAM |  |  | 8@44 |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

6.2C.5.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2C.5.5-1 to table 6.2C.5.5-8. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

<<Unchanged Text Skipped>>

Table 6.2C.5.5-8: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 6 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 10 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 14 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 17 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 18 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 20 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 21 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 22 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 25 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 27 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 29 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 30 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 31 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 32 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 35 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 36 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 37 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 38 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 39 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 40 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 43 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 45 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 51 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 53 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 54 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 55 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 58 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 59 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 62 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 64 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 65 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 66 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 67 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 69 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 70 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 71 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 72 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 73 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 74 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 75 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 76 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 77 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 78 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 79 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 80 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 81 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 82 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 83 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 84 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 85 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 86 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 87 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 88 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 89 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 90 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 91 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 92 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 95 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 97 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 98 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 99 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 100 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 101 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 102 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 103 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 104 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 105 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 106 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 107 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 108 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 109 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 110 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 111 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 113 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 114 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 115 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 116 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 117 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES -----------------------------------------------------