3GPP TSG-RAN5 Meeting # 93-e	R5-216603

# Electronic Meeting, 8th - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2051 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR band n95 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intertek, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR band n95 is missed in Table 4.3.1.0A-1, 4.3.1.0B-1 and 4.3.1.0C-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add band n95 in Table 4.3.1.0A-1, 4.3.1.0B-1 and 4.3.1.0C-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test spec. will be incompleted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of the change>>

#### 4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 156, 257 |
| n2 | 15 |
| n3 | 20 |
| n5 | 15 |
| n7 | 156, 257 |
| n8 | 15 |
| n12 | 10 |
| n14 | 10 |
| n20 | 15 |
| n24 | 10 |
| n25 | 15 |
| n26 | 108 |
| n28 | 15 |
| n29 | 102 |
| n30 | 10 |
| n34 | 10 |
| n38 | 15 |
| n39 | 20 |
| n40 | 30 |
| n41 | 60 |
| n48 | 204, 405 |
| n50 | 20 |
| n51 | 5 |
| n53 | 10 |
| n65 | 15 |
| n66 | 20 |
| n70 | 15 |
| n71 | 10 |
| n74 | 15 |
| n75 | 152 |
| n76 | 52 |
| n77 | 50 |
| n78 | 50 |
| n79 | 60 |
| n80 | 203 |
| n81 | 153 |
| n82 | 153 |
| n83 | 153 |
| n84 | 153, 6, 257 |
| n86 | 203 |
| n95 | 103 |
| n97 | 30 |
| n99 | 103 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 Ues. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, Pcell in CA or Pcell in DC configuration. Note 5: Applicable for use as Scell in CA or Scell in DC configuration. Note 6: This Mid test channel bandwidth is applicable to Ues supporting maximum channel bandwidth 20MHz. Note 7: This Mid test channel bandwidth is applicable to Ues supporting maximum channel bandwidth 50MHz. Note 8: This Mid test channel bandwidth is chosen since it is more commonly used. |  |

Table 4.3.0A.1-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: For Ues with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 Ues. |  |

#### 4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1 and Table 4.3.1.0B-2 for FR1 and FR2 respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 5 |
| n2 | 5 |
| n3 | 5 |
| n5 | 5 |
| n7 | 5 |
| n8 | 5 |
| n12 | 5 |
| n14 | 5 |
| n20 | 5 |
| n24 | 5 |
| n25 | 5 |
| n26 | 5 |
| n28 | 5 |
| n29 | 52 |
| n30 | 5 |
| n34 | 5 |
| n38 | 5 |
| n39 | 5 |
| n40 | 54,105 |
| n41 | 10 |
| n48 | 54, 105 |
| n50 | 54,105 |
| n51 | 5 |
| n53 | 5 |
| n65 | 5 |
| n66 | 5 |
| n70 | 5 |
| n71 | 5 |
| n74 | 5 |
| n75 | 52 |
| n76 | 52 |
| n77 | 10 |
| n78 | 10 |
| n79 | 40 |
| n80 | 53 |
| n81 | 53 |
| n82 | 53 |
| n83 | 53 |
| n84 | 53 |
| n86 | 53 |
| n95 | 53 |
| n97 | 5 |
| n99 | 53 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 Ues. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as Scell in CA or Scell in DC configuration. Note 5: Applicable for use as single carrier, Pcell in CA or Pcell in DC configuration. |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 50 |
| n258 | 50 |
| n260 | 50 |
| n261 | 50 |
| Note 1: For Ues with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 Ues.. |  |

#### 4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1 and Table 4.3.1.0C-2 for FR1 and FR2 respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 206, 507 |
| n2 | 20 |
| n3 | 308, 409 |
| n5 | 20 |
| n7 | 206, 507 |
| n8 | 20 |
| n12 | 15 |
| n14 | 10 |
| n20 | 20 |
| n24 | 10 |
| n25 | 206, 409 |
| n26 | 20 |
| n28 | 206, 308 |
| n29 | 102 |
| n30 | 10 |
| n34 | 15 |
| n38 | 20 |
| n39 | 40 |
| n40 | 80 |
| n41 | 100 |
| n48 | 403, 1004 |
| n50 | 80 |
| n51 | 5 |
| n53 | 10 |
| n65 | 20 |
| n66 | 40 |
| n70 | 151/252 |
| n71 | 20 |
| n74 | 20 |
| n75 | 202 |
| n76 | 52 |
| n77 | 100 |
| n78 | 100 |
| n79 | 100 |
| n80 | 301 |
| n81 | 201 |
| n82 | 201 |
| n83 | 201, 308 |
| n84 | 201, 507 |
| n86 | 401 |
| n95 | 151 |
| n97 | 80 |
| n99 | 101 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. Note 6: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 30MHz. Note 9: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 40MHz. |  |

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 400 |
| n258 | 400 |
| n260 | 400 |
| n261 | 400 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. |  |

<< end of the change>>