3GPP TSG-RAN5 Meeting #93-e	R5-216604

# Online, , 8th Nov - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1480 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections regarding the To header in usages of MO REFER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Most usages of an MO REFER request are inside a dialog (apart from test case 15.18 which uses an MO REFER request outside a dialog – this test case has its own wording for the To header in MO REFER). The To header for such in dialog requests had been fixed by R5-182192. This, apart from minor wording improvements, should be used by generic procedures C.19 and C.37 – note that for C.19 the proposed change is just an improvement while it is real fix for C.37. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Improved the wording in A.2.10. Let C.19 refer to the MO REFER default message rather than preceding INVITE. Let C.37 refer to the MO REFER default message rather than using faulty conf factory address. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Faulty resp inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.10, C.19, C.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## A.2.10	MO REFER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | REFER |  |  |
| Request-URI |  | same URI value as the SS has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2 | IP address or FQDN and unprotected server port of the UE |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| Route |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] |
| route-param | A1 | URIs of the Record-Route header of 183 response in reverse order |  |  |
|  | A2 | URIs of the Record-Route header of 183 response in reverse order |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | local SIP URI of the UE which must be the same URI as used for the UE in the earlier requests within the dialog |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS which must be the same URI as used by the UE in earlier requests within the dialog |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as in the first INVITE during the call setup |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | REFER |  |  |
| Require | A1 |  |  | RFC 3261 [15] RFC 3312 [31] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1 |  |  | RFC 3261 [15] |
| option-tag |  | sec-agree |  | RFC 3329 [21] |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2 | not present |  | RFC 3329 [21] |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A1 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A2 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A3 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| Refer-To |  |  |  | RFC 3515 [72] |
| addr-spec |  | SIP or Tel URI of the transfer target (Note 1) |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A4 | access network information and, if applicable, the cell ID |  |  |
|  | A5 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A3 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A4 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A5 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |

NOTE 1:	The SIP URI may contain a "Replaces" header referring to the dialog ID which has been established before.

# *** Next Change ***

# C.19	Generic test procedure for Inviting user to conference by sending a REFER request to the conference focus - EPS

The generic test procedure for Inviting user to conference by sending a REFER request to the conference focus may be performed after successful IMS or early IMS registration.

Test procedure

1)	UE invites a user to the conference created. SS waits the UE to send to the conference focus a REFER request, which refers to the user to be invited to the conference.

2)	SS responds to the REFER request with a valid 202 Accepted response.

3)	SS sends an initial NOTIFY to tell that the invited user is trying to join the conference.

4)	UE responds to the NOTIFY request with valid 200 OK response.

5)	SS sends the final NOTIFY to tell that the invited user has successfully joined the conference.

6)	UE responds to the NOTIFY request with a valid 200 OK response.

7)	Optional: If UE subscribed the conference event package during the generic test procedure of Annex C.10, SS sends a NOTIFY for the conference event package to the UE to notify that the user joined the conference.

8) If SS sent a NOTIFY, SS waits the UE to respond the NOTIFY with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REFER | UE sends REFER to SS referring to the conference |
| 2 |  |  | 202 Accepted | The SS responds with a 202 final response |
| 3 |  |  | NOTIFY | The SS sends initial NOTIFY for the implicit subscription created by the REFER request |
| 4 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 5 |  |  | NOTIFY | The SS sends a NOTIFY related to REFER request to confirm that the invited user was able to join the conference |
| 6 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 7 |  |  | NOTIFY | Optional: If the UE has subscribed the conference event package, the SS sends a NOTIFY for conference event package to inform that the invited user was able to join the conference |
| 8 |  |  | 200 OK | Optional: The UE responds the NOTIFY with 200 OK |

Specific Message Contents

REFER (Step 1)

Use the default message “MO REFER” in annex A.2.10 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| Refer-To |  |
| addr-spec | SIP URI or tel URI of the user invited to the conference. If an active session exists, the Replaces header in the header portion of the SIP URI shall be included (mandatory inclusion is stated in IR.92 [133]) and set to the dialog ID of the active session according to RFC 3891. In this case, if the user has been invited with a tel URI, the UE shall convert the tel URI to a SIP URI according to RFC 3261 [15] clause 19.1.6. (NOTE: the dialog ID is percent encoded according to RFC 3986). |
| Route route-param | URIs of the Record-Route header of 183 response sent in step 4 of C.10 in reverse order |

NOTIFY (Step 3)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SIP/2.0 100 Trying |

NOTIFY (Step 5)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Subscription-State |  |
| substate-value | terminated |
| expires | omitted from the request |
| reason | noresource |
| Message-body | SIP/2.0 200 OK |

NOTIFY (Step 7)

Use the default message “NOTIFY for conference event package” in annex A.5.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity="sip:final@conf-factory. appended with px_IMS_HomeDomainName" state="partial" version=" value as in previous notification for conference event package but incremented by one" <users> <user entity=" SIP URI or tel URI of the invited user"> <endpoint entity=" Contact URI of the invited user"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label> unique identifier for the media stream between the focus and the endpoint of the invited user (e.g. 11223)</label> <src-id>random SSRC value</src-id> <status>sendrecv</status> </media> </endpoint> </users> </conference-info> |

# *** Next Change ***

# C.37	Generic test procedure for Inviting user to Video conference by sending a REFER request to the conference focus - EPS

Test procedure

1)	UE invites a user to the conference created. SS waits the UE to send to the conference focus a REFER request, which refers to the user to be invited to the conference.

2)	SS responds to the REFER request with a valid 202 Accepted response.

3)	SS sends an initial NOTIFY to tell that the invited user is trying to join the conference.

4)	UE responds to the NOTIFY request with valid 200 OK response.

5)	SS sends the final NOTIFY to tell that the invited user has successfully joined the conference.

6)	UE responds to the NOTIFY request with a valid 200 OK response.

7)	Optional: If UE subscribed the conference event package during the generic test procedure of Annex C.10, SS sends a NOTIFY for the conference event package to the UE to notify that the user joined the conference.

8)	If SS sent a NOTIFY, SS waits the UE to respond the NOTIFY with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REFER | UE sends REFER to SS referring to the conference |
| 2 |  |  | 202 Accepted | The SS responds with a 202 final response |
| 3 |  |  | NOTIFY | The SS sends initial NOTIFY for the implicit subscription created by the REFER request |
| 4 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 5 |  |  | NOTIFY | The SS sends a NOTIFY related to REFER request to confirm that the invited user was able to join the conference |
| 6 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 7 |  |  | NOTIFY | Optional: If the UE has subscribed the conference event package, the SS sends a NOTIFY for conference event package to inform that the invited user was able to join the conference |
| 8 |  |  | 200 OK | Optional: The UE responds the NOTIFY with 200 OK |

Specific Message Contents

REFER (Step 1)

Use the default message “MO REFER” in annex A.2.10 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| Refer-To |  |
| addr-spec | SIP URI or tel URI of the user invited to the conference. If an active session exists, the Replaces header in the header portion of the SIP URI shall be included (mandatory inclusion is stated in IR.92 [133]) and set to the dialog ID of the active session according to RFC 3891. In this case, if the user has been invited with a tel URI, the UE shall convert the tel URI to a SIP URI according to RFC 3261 [15] clause 19.1.6. (NOTE: the dialog ID is percent encoded according to RFC 3986). |
| Route route-param | URIs of the Record-Route header of 183 response sent in step 4 of C.38 in reverse order |

NOTIFY (Step 3)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SIP/2.0 100 Trying |

NOTIFY (Step 5)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Subscription-State |  |
| substate-value | terminated |
| expires | omitted from the request |
| reason | noresource |
| Message-body | SIP/2.0 200 OK |

NOTIFY (Step 7)

Use the default message “NOTIFY for conference event package” in annex A.5.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity="sip:final@conf-factory. appended with px_IMS_HomeDomainName" state="partial" version="1" <users> <user entity=" SIP URI or tel URI of the invited user"> <endpoint entity=" Contact URI of the invited user"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>11223</label> <src-id>random SSRC value</src-id> <status>sendrecv</status> </media> <media id="2"> <type>video</type> <label>11224</label> <src-id>random SSRC value</src-id> <status>sendrecv</status> </media> </endpoint> </users> </conference-info> |