3GPP TSG-RAN WG5 Meeting #93-e	R5-216605

Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0047 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR positioning methods information in the RESET UE POSITIONING STORED INFORMATION message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NR positioning test cases in TS 37.571-1 and TS 37.571-2 need to use the RESET UE POSITIONING STORED INFORMATION message to command the UE to reset the already acquired UE positioning information. Now the UE Positioning Technology values for new NR positioning methods are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the TS 37.355 as the reference file. - Clarified the UE behaviours when receiving the RESET UE POSITIONING STORED INFORMATION message. - Added the UE Positioning Technology values for DL-TDOA, DL-AoD and Multi-RTT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to NR release 16 NR positioning methods conformance testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.6, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 37.571-2 CR 0152 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment ".

[3]	3GPP TS 38.523-1: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol ".

[4]	3GPP TS 38.523-3: "5GS; User Equipment (UE) conformance specification; Part 3: Protocol Test Suites "

[5]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

[6]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[8]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[9]	3GPP TS 44.014: "Individual equipment type requirements and interworking; Special conformance testing functions".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (EUTRA) Radio Resource Control (RRC) Protocol Specification".

[11]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[12]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[13]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[14]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[15]	3GPP TS 38.521-4: "NR; User Equipment conformance specification; Radio transmission and reception; Part 4: Performance".

[16]	3GPP TS 38.533: NR; "User Equipment (UE) conformance specification; Radio resource management".

[17]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[18]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[19]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[20]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[21]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[22]	3GPP TS 38.215: "NR; Physical Layer Measurements".

[23]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[X1]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

# < Unchanged sections omitted >

5.6	UE Positioning test mode procedures

5.6.1	Reset UE Positioning Stored Information

5.6.1.1	General

Same as TS 36.509 [6], subclause 5.5.1.1.

5.6.1.2	Initiation

Same as TS 36.509 [6], subclause 5.5.1.2.

5.6.1.3	Reception of RESET UE POSITIONING STORED INFORMATION message by UE

When UE receives RESET UE POSITIONING STORED INFORMATION message then the UE shall:

1>	if the IE "UE Positioning Technology" has the value ‘AGNSS’:

2>	discard any stored GNSS reference time, reference position, and any other aiding data obtained or derived during the previous test instance (e.g. expected ranges and Doppler);

2>	temporarily disable any sensor data (if applicable) that can aid the positioning for the duration of the subsequent positioning procedure.

1>	if the IE "UE Positioning Technology" has the value ‘OTDOA’:

2>	discard any stored OTDOA reference and neighbour cell info assistance data;

2>	use the reference cell in the provided OTDOA assistance data as reference cell for reporting RSTD measurements in the subsequent positioning procedure.

NOTE 1:	Where OTDOA is mentioned this applies to OTDOA using LTE cells.

1>	if the IE "UE Positioning Technology" has the value ‘MBS’:

2>	discard any stored MBS assistance data and information obtained or derived during the previous test instance (e.g. Beacon PN code and slot information and code phase measurements);

2>	temporarily disable any sensor data (if applicable) that can aid the positioning for the duration of the subsequent positioning procedure.

1>	if the IE "UE Positioning Technology" has the value ‘WLAN’:

2>	discard any stored WLAN assistance data and information obtained or derived during the previous test instance;

1>	if the IE "UE Positioning Technology" has the value ‘Bluetooth’:

2>	discard any stored Bluetooth data obtained or derived during the previous test instance;

1>	if the IE "UE Positioning Technology" has the value ‘Sensor’:

2>	discard any stored Sensor assistance data and information obtained or derived during the previous test instance;

1>	if the IE "UE Positioning Technology" has the value ‘DL-TDOA’:

2>	discard any stored DL-PRS assistance data and information obtained or derived during the previous test instance;

1>	if the IE "UE Positioning Technology" has the value ‘DL-AoD’:

2>	discard any stored DL-PRS assistance data and information obtained or derived during the previous test instance;

1>	if the IE "UE Positioning Technology" has the value ‘Multi-RTT’:

2>	discard any stored UL-SRS configuration and DL-PRS assistance data obtained or derived during the previous test instance;

1>	otherwise:

2>	ignore the message.

# < Unchanged sections omitted >

6.6	UE Positioning messages

6.6.1	RESET UE POSITIONING STORED INFORMATION

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [7], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [7], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| UE Positioning Technology |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | octet 1 |

where UE Positioning Technology is a single octet IE:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Positioning Technology |  |  |  |  |  |  |  | octet 1 |

UE Positioning Technology value

Bits

8 7 6 5 4 3 2 1

0 0 0 0 0 0 0 0	AGNSS

0 0 0 0 0 0 0 1	OTDOA

0 0 0 0 0 0 1 0	MBS

0 0 0 0 0 0 1 1	WLAN

0 0 0 0 0 1 0 0	Bluetooth

0 0 0 0 0 1 0 1	Sensor

0 0 0 0 0 1 1 0	Multi-RTT

0 0 0 0 0 1 1 1	DL-TDOA

0 0 0 0 1 0 0 0	DL-AoD

All other cause values are reserved for future use.