3GPP TSG-RAN WG5 Meeting #93-e	R5-216668

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0192 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.17.1 - SIP 200 (OK) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For MCPTT px_MCPTT_PublicServiceId_A has been replaced by tsc_MCPTT_PublicServiceId_A; similar changes are requiredfor MCVideo and MCData - For MCData the Contact header field's media feature tags need to distinguish SDS and FD - Invalid spec reference for P-Associated-URI in Table 5.5.2.17.1.1-1 - MCPTT condition missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - px_MCVideo_PublicServiceId_A replaced by tsc_MCVideo_PublicServiceId_A; px_MCData_PublicServiceId_A replaced by tsc_MCData_PublicServiceId_A - New conditions MCDATA_SDS and MCDATA_FD used when INVITE-RSP applies; in case of REGISTER-RSP for MCData the SS shall send the media feature tags for SDS and FD. - Spec reference corrected - MCPTT condition added - Editorial correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.5.2.17.1	SIP 200 (OK)

5.5.2.17.1.1	SIP 200 (OK) from the UE

Table 5.5.2.17.1.1-1: SIP 200 (OK) from the UE

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| rec-route | same as received in the request |  |  |  |
| From |  |  |  |  |
| addr-spec | Same value as received in the request |  |  |  |
| tag | same value as received in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as received in the request |  |  |  |
| tag | same value as received in the request or any value if missing in the request. |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| user-info and host | IP address or FQDN |  |  |  |
| port | protected server port of UE | as assigned during registration |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "+g.3gpp.icsi-ref= urn:urn- 7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  | TS 24.282 [87] clause 9.2.3.2.4 | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  | TS 24.282 [87] clause 10.2.5.2.4 | MCDATA_FD |
| feature-param | "audio" |  |  | MCPTT OR MCVideo |
| feature-param | "video" |  |  | MCVIDEO |
| feature-param | "text" |  |  | MCDATA |
| feature-param | "isfocus" |  |  |  |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| delta-seconds | Same value as session expires header in SIP INVITE |  | RFC 4028 [30] TS 24.229 [16] cl.5.1.4.1 |  |
| refresher | "uas" |  |  |  |
| Content-Type |  |  | RFC 5621 [58] | INVITE-RSP |
| value | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| P-Answer-State | If present |  | RFC 4964 [118] TS 24.379 [9] clause 6.2.3.1.2 | INVITE-RSP AND GROUP-CALL |
| answer-type | “confirmed” |  |  |  |
| Message-body | not present |  | RFC 3261 [22] |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| MIME body part |  | SDP message |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/sdp" |  | RFC 4566 [27] |  |
| MIME-part-body | SDP message as described in Table 5.5.3.1.1-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.1-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.1-3 | FFS |  | MCDATA |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-header |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| INVITE-RSP | 200 OK is the response to the SIP INVITE |
| MCDATA_SDS | INVITE for SDS communication |
| MCDATA_FD | INVITE for FD communication |

5.5.2.17.1.2	SIP 200 (OK) from the SS

Table 5.5.2.17.1.2-1: SIP 200 (OK) from the SS

| Derivation Path: RFC 3261 [22] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Status-Code | "200" |  |  |  |
| Reason-Phrase | "OK" |  |  |  |
| Via | same as received in the request |  | RFC 3261 [22] RFC 3581 [55] |  |
| Record-Route |  |  | RFC 3261 [22] | INVITE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | pcscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | scscf.other.com |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[3] | SIP URI |  |  |  |
| user-info and host | orig@scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[4] | SIP URI |  |  |  |
| user-info and host | same address as sent by the UE in the first entry of the Route header of the INVITE | P-CSCF address |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| Record-Route |  |  | RFC 3261 [22] | SUBSCRIBE-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| From |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in the request |  |  |  |
| tag | same value as in the request or To-tag assigned by the SS if missing in the request |  |  |  |
| Expires |  |  | RFC 3261 [22] RFC 3903 [43] | SUBSCRIBE-RSP, PUBLISH-RSP |
| value | same value as in the request |  |  |  |
| Contact |  |  |  | REGISTER-RSP |
| addr-spec | same value as received in the REGISTER |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
| feature-param | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| feature-param | "+g.3gpp.mcdata.fd" |  |  | MCDATA |
| expires | "600000" |  |  |  |
| Contact |  |  |  | SUBSCRIBE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
|  | "sip:" & tsc_MCX_CMS_Hostname |  |  | CONFIG |
|  | "sip:" & tsc_MCX_GMS_Hostname |  |  | GROUPCONFIG |
| port | not present |  |  |  |
| Contact |  |  |  | INVITE-RSP |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  | MCPTT |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| port | not present |  |  |  |
| feature-param | “audio” |  |  | MCPTT OR MCVIDEO |
| feature-param | “video” |  |  | MCVIDEO |
| feature-param | “text” |  |  | MCDATA |
| Call-ID |  |  |  |  |
| callid | same value as received in the request |  |  |  |
| CSeq |  |  |  |  |
| value | same value as received in the request |  |  |  |
| Require |  |  |  | INVITE-RSP |
| option-tag | "timer" |  |  |  |
| Session-Expires |  |  |  | INVITE-RSP |
| generic-param | "3600" |  |  |  |
| refresher | "uac" |  |  |  |
| Supported |  |  |  | INVITE-RSP |
| option-tag | "tdialog" |  |  |  |
| option-tag | "norefersub" |  |  |  |
| option-tag | "explicitsub" |  |  |  |
| option-tag | "nosub" |  |  |  |
| Refer-Sub |  |  | RFC 4488 [36] | REFER-RSP |
| refer-sub-value | “false” |  |  |  |
| P-Associated-URI |  |  | RFC 7315 [52] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | px_MCX_SIP_PublicUserId_A_1 |  |  |  |
| port | not present |  |  |  |
| Service-Route |  |  | RFC 3261 [22] | REGISTER-RSP |
| addr-spec[1] | SIP URI |  |  |  |
| host | scscf.3gpp.org |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| SIP-ETag |  |  | RFC 3903 [43] | PUBLISH-RSP |
| entity-tag | unique value arbitrarily selected by the SS |  |  |  |
| Content-Type |  |  | RFC 4566 [27] | INVITE-RSP |
| media-type | "application/sdp" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] | INVITE-RSP |
| SDP message | SDP message as described in Table 5.5.3.1.2-1 |  |  | MCPTT |
|  | SDP message as described in Table 5.5.3.1.2-2 |  |  | MCVIDEO |
|  | SDP message as described in Table 5.5.3.1.2-3 | FFS |  | MCDATA |

| Condition | Explanation |
| --- | --- |
| REGISTER-RSP | 200 OK is the response to a SIP REGISTER |
| INVITE-RSP | 200 OK is the response to a SIP INVITE |
| SUBSCRIBE-RSP | 200 OK is the response to a SIP SUBSCRIBE |
| PUBLISH-RSP | 200 OK is the response to a SIP PUBLISH |
| REFER-RSP | 200 OK is the response to a SIP REFER |