3GPP TSG-RAN WG5 Meeting #93-e	R5-216671

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0195 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.2.7 - SIP MESSAGE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The conditions for Table 5.5.2.7.1-1 and Table 5.5.2.7.2-1 need to be more generic to allow the tables to be used for different use cases. - For MCData SDS and FD need to be distiguished. - According to 24.379 clause 13.2.2 the participating function "shall include the public service identity of the participating MCPTT function in the P-Asserted-Identity header field" and according to clause 13.3.4.2 the client shall use this URI as request URI in a stand-alone SIP MESSAGE for location reporting. - In most cases according to 24.379 a DL SIP MESSAGE's Accept-Contact header contains only the Service Reference media feature-tag "g.3gpp.icsi-ref", but e.g. for privat call call-back and remotely initiated private call it contains a media feature tag ("+g.3gpp.mcptt", "+g.3gpp.mcvideo", "+g.3gpp.mcdata.sds") too. - In 24.379 there are cases where it is explicitly specified that the UE may include a P-Preferred-Identity header field in the SIP MESSAGE (e.g. clause 11.1.5.2.1) and there are cases where such statement is missing for the SIP MESSAGE (e.g. clause 9.2.1.4). Nevertheless according to 24.229 clause 5.1.2A.1.1 "In accordance with RFC 3325 [34] the UE may insert a P-Preferred-Identity header field in any initial request for a dialog or request for a standalone transaction as a hint for creation of an asserted identity". - According to 24.379 clause 9.2.1.5 the client shall check the P-Asserted-Service in the SIP MESSAGE for receiving affiliation status change request; the same applies for MCVideo (24.281 clause 8.2.1.5) and MCData (24.282 clause 8.2.5) - For MCPTT px_MCPTT_PublicServiceId_A has been replaced by tsc_MCPTT_PublicServiceId_A; similar changes are required for MCVideo and MCData - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New conditions to cope with different requirements of MCData - Several header fields are distinguished for SDS and FD - P-Asserted-Identity added to Table 5.5.2.7.2-1 for (new) condition LOCATION-CONFIG; Request-URI with (new) condition LOCATION-REPORT added to Table 5.5.2.7.1-1 - Condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG added to 2nd entry of Accept-Contact in Table 5.5.2.7.2-1 - Optional P-Preferred-Identity header field added to Table 5.5.2.7.1-1. - SS needs to include the P-Asserted-Service in the SIP MESSAGE for affiliation status change as otherwise the client may not respond with 200 OK - px_MCVideo_PublicServiceId_A replaced by tsc_MCVideo_PublicServiceId_A; px_MCData_PublicServiceId_A replaced by tsc_MCData_PublicServiceId_A - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification; non-conformant UE may pass |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.7	SIP MESSAGE

##### 5.5.2.7.1	SIP MESSAGE from the UE

Table 5.5.2.7.1-1: SIP MESSAGE from the UE

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7a, A.2.2.4.7a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "MESSAGE" |  |  |  |
| Request-URI | tsc_MCPTT_PublicServiceId_A | The public service identity identifying the originating participating MCPTT function serving the MCPTT user |  |  |
|  | tsc_MCVideo_PublicServiceId_A | The public service identity identifying the originating participating MCVideo function serving the MCVideo user |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The public service identity identifying the originating participating MCData function serving the MCData user |  | MCDATA |
|  | same URI as provided in the Asserted-Identity header field of the SIP MESSAGE for location reporting configuration |  |  | LOCATION-REPORT |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE | as assigned during registration |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) | The URI of the UE |  |  |
| port | not present |  |  |  |
| tag | any allowed value |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A | The URI of the SS |  |  |
|  | tsc_MCVideo_PublicServiceId_A | The URI of the SS |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A | The URI of the SS |  | MCDATA |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| method | "MESSAGE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-spec | Access network technology and, if applicable, the cell ID |  |  |  |
| Route | same as specified for INVITE sent by the UE in Table 5.5.2.5.1-1 |  | RFC 3261 [22] |  |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | MCDATA_SDS, MCDATA_FD |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Preferred-Identity | if present |  | RFC 3325 [32] |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of message-body |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 |  |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  |  | MCDATA |
| MIME body part |  | Affiliation-Command |  | AFFILIATION |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |  |
|  | "application/vnd.3gpp. mcvideo-affiliation-command+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the affiliation-command XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Affiliation-Command as described in Table 5.5.3.7-1 |  | TS 24.379 [9] clause F.4 |  |
|  | MCVideo-Affiliation-Command as described in Table 5.5.3.7-2 |  | TS 24.281 [86] clause F.4 | MCVIDEO |
|  | MCData-Affiliation-Command as described in Table 5.5.3.7-3 |  | TS 24.282 [87] clause D.3 | MCDATA |
| MIME body part |  | Resource lists | RFC 5366 [35] | RESOURCE_LISTS |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 |  |  |  |
|  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | Location info | TS 24.379 [9] clause F.3 | LOCATION-INFO, LOCATION_REPORT |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" | This MIME part shall be included if the MCPTT-Info 'alert-ind' element sent in the MCPTT-Info is set to true. |  |  |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  |  |  |
| MIME body part |  | MIKEY message |  | MIKEY |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2A | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | MCData Data signalling message |  | MCDATA_SIGNALLING |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | SIGNALLING PAYLOAD as described in Table 5.5.3.8.1-1 |  | TS 24.282 [87] |  |
| MIME body part |  | MCData Data message |  | MCDATA_PAYLOAD |
| MIME-part-headers |  |  |  |  |
| Content-Type | application/vnd.3gpp.mcdata-payload |  |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 5.5.3.9.1-1 |  | TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| RESOURCE_LISTS | Message-body contains Resource lists |
| LOCATION_REPORT | Message-body contains location information report according to TS 24.379 [2] clause 13.3.4.2 |
| MIKEY | Message-body contains MIKEY message (e.g. for MCData 1-to-1 communication) |
| MCDATA_SIGNALLING | Message-body contains MCData Data signalling message |
| MCDATA_PAYLOAD | Message-body contains MCData Data message (DATA PAYLOAD) |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| For further conditions see table 5.5.1-1 |  |

##### 5.5.2.7.2	SIP MESSAGE from the SS

Table 5.5.2.7.2-1: SIP MESSAGE from the SS

| Derivation Path: TS 24.229 [16], clause A.2.1.4.7a, A.2.2.4.7a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "MESSAGE" |  |  |  |
| Request-URI | Public user id associated to the MC service id | px_MCX_SIP_PublicUserId_A_1 (in general) |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol[1] | "SIP/2.0/TCP" |  |  |  |
| sent-by[1] |  | Address of the P-CSCF that communicates with the called party |  |  |
| ….host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| via-branch[1] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[2] | "SIP/2.0/UDP" |  |  |  |
| sent-by[2] |  |  |  |  |
| ….host | “scscf.3gpp.org“ |  |  |  |
| port | Value assigned by the SS | Caller’s port number |  |  |
| via-branch[2] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| sent-protocol[3] | "SIP/2.0/UDP" |  |  |  |
| sent-by[3] |  |  |  |  |
| host | host name of the MC server |  |  |  |
| port | not present |  |  |  |
| via-branch[3] | Value assigned by the SS starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCPTT_PublicServiceId_A |  |  |  |
|  | tsc_MCVideo_PublicServiceId_A |  |  | MCVIDEO |
|  | tsc_MCData_PublicServiceId_A |  |  | MCDATA |
| port | not present |  |  |  |
| tag | Value assigned by the SS |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | same URI as used as Request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | Value assigned by the SS |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | Value assigned by the SS |  |  |  |
| method | "MESSAGE" |  |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | "67" | The recommended initial value is 70 in RFC 3261. Assuming 3 hops as according to the Via header this results in a value of 67 in the message sent to the UE |  |  |
| P-Asserted-Service |  |  | RFC 6050 [31] | MCDATA_SDS, MCDATA_FD |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| P-Asserted-Service |  |  | RFC 6050 [31] | AFFILIATION |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |
| feature-param | "+g.3gpp.mcptt" |  |  |  |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  | MCDATA_SDS, MCDATA_FD |
| feature-param | "+g.3gpp.mcdata.sds" |  |  | MCDATA_SDS |
|  | "+g.3gpp.mcdata.fd" |  |  | MCDATA_FD |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] | MCDATA_SDS, MCDATA_FD |
| name-addr | px_MCData_ID_User_B | The public user identity of the originating MCData User |  |  |
| P-Asserted-Identity |  |  | RFC 3325 [32] | LOCATION-CONFIG |
| name-addr | tsc_MCPTT_PublicServiceId_PF_A | URI of the participating MCPTT function which configures the location reporting at the UE |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length |  |  | RFC 3261 [22] |  |
| value | length of message-body |  |  |  |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.2-1 |  | TS 24.379 [9] clause F.1 |  |
|  | MCVideo-Info as described in Table 5.5.3.2.2-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.2-3 |  | TS 24.282 [87] clause D.1.2 | MCDATA |
| MIME body part |  | Affiliation-Command |  | AFFILIATION |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-affiliation-command+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-affiliation-command+xml" |  |  | MCVIDEO |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the affiliation-command XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Affiliation-Command as described in Table 5.5.3.7-1 |  | TS 24.379 [9] clause F.4 |  |
|  | MCVideo-Affiliation-Command as described in Table 5.5.3.7-2 |  | TS 24.281 [86] clause F.4 | MCVIDEO |
|  | MCData-Affiliation-Command as described in Table 5.5.3.7-3 |  | TS 24.282 [87] clause D.3 | MCDATA |
| MIME body part |  | Resource lists | RFC 5366 [35] | RESOURCE_LISTS |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.2-1 |  |  |  |
|  | Resource-lists as described in Table 5.5.3.3.2-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.2-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | LOCATION-INFO, LOCATION_CONFIG |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.2-1 |  | TS 24.379 [9] clause F.3 |  |
|  | Location-info as described in Table 5.5.3.4.2-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | MIKEY message |  | MIKEY |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  |  |  |
| MIME-part-body | As described in Table 5.5.9.1-2 | MIKEY message, containing the PSK | TS 33.180 [30] TS 24.282 [87] |  |
| MIME body part |  | MCData Data signalling message |  | MCDATA_SIGNALLING |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | SIGNALLING PAYLOAD as described in Table 5.5.3.8.2-1 |  | TS 24.282 [87] |  |
| MIME body part |  | MCData Data message |  | MCDATA_PAYLOAD |
| MIME-part-headers |  |  |  |  |
| Content-Type | application/vnd.3gpp.mcdata-payload |  |  |  |
| MIME-part-body | DATA PAYLOAD as described in Table 5.5.3.9.1-2 |  | TS 24.282 [87] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-2 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| RESOURCE_LISTS | Message-body contains Resource lists |
| LOCATION_CONFIG | Message-body contains location reporting configuration according to TS 24.379 [2] clause 13.2.2 |
| MIKEY | Message-body contains MIKEY message (e.g. for MCData 1-to-1 communication) |
| MCDATA_SIGNALLING | Message-body contains MCData Data signalling message |
| MCDATA_PAYLOAD | Message-body contains MCData Data message (DATA PAYLOAD) |
| MCDATA_SDS | SDS message or SDS disposition notification |
| MCDATA_FD | FD message or FD disposition notification |
| ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG | Accept-Contact header field contains media feature tag ("+g.3gpp.mcptt", "+g.3gpp.mcvideo" or "+g.3gpp.mcdata") |
| For further conditions see table 5.5.1-1 |  |