3GPP TSG-RAN WG5 Meeting #93-e	R5-216680

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0204 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.8 - SDS Signaling Payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - 36.579-1 does not specify SDS NOTIFICATION messages yet - In general MCData Data signalling messages shall be protected - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SDS NOTIFICATION messages added; title of clause 5.5.3.8 changed to allow other messages than SDS Signalling Payload to be contained - Clarification added at the beginning of clause 5.5.3.8 that the MCData Data signalling messages are protected - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.8	MCData Data signalling messages

The MCData Data signalling messages specified in this clause are protected according to TS 33.180 clause 8.5.4, i.e. a MCData Data signalling message is contained in the protected payload of a MCData Protected Payload Message according to clause 5.5.3.10 with condition PROTECTED_MESSAGE and CSK.

The following conditions apply throughout clause 5.5.3.8:

Table 5.5.3.8-1: Conditions

| Condition | Explanation |
| --- | --- |
| DELIVERED | Disposition request/notification type DELIVERED |
| READ | Disposition request/notification type READ |
| DELIVERED_READ | Disposition request/notification type DELIVERED AND READ |

##### 5.5.3.8.1	SDS SIGNALLING PAYLOAD message from the UE

Table 5.5.3.8.1-1: SDS SIGNALLING PAYLOAD message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS signalling payload message identity | '00000001'B | SDS SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Any allowed value | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Any allowed value | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| SDS disposition request type | '0001'B |  | TS 24.282 [87] clause 15.2.3 | DELIVERED |
|  | '0010'B |  |  | READ |
|  | '0011'B |  |  | DELIVERED_READ |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| User location | Any allowed value if present |  | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.2	SDS SIGNALLING PAYLOAD message from the SS

Table 5.5.3.8.2-1: SDS SIGNALLING PAYLOAD message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS signalling payload message identity | '00000001'B | SDS SIGNALLING PAYLOAD | TS 24.282 [87] clause 15.2.2 |  |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | '01010101010101010101010101010101'O | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | '01010101010101010101010101010101'O | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| InReplyTo message ID | Not present |  | TS 24.282 [87] clause 15.2.11 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| SDS disposition request type | '0001'B |  | TS 24.282 [87] clause 15.2.3 | DELIVERED |
|  | '0010'B |  |  | READ |
|  | '0011'B |  |  | DELIVERED_READ |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| User location | Not present |  | TS 24.282 [87] clause 15.2.25 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.3	SDS NOTIFICATION message from the UE

Table 5.5.3.8.3-1: SDS NOTIFICATION message from the UE

| Derivation Path: TS 24.282 [87] clause 15.1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS notification message identity | '00000101'B | SDS NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| SDS disposition notification type | '00000010'B |  | TS 24.282 [87] clause 15.2.5 | DELIVERED |
|  | '00000011'B |  |  | READ |
|  | '00000100'B |  |  | DELIVERED_READ |
| Date and time | Any allowed value | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD sent to the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD sent to the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |

##### 5.5.3.8.4	SDS NOTIFICATION message from the SS

Table 5.5.3.8.4-1: SDS NOTIFICATION message from the SS

| Derivation Path: TS 24.282 [87] clause 15.1.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SDS notification message identity | '00000101'B | SDS NOTIFICATION | TS 24.282 [87] clause 15.2.2 |  |
| SDS disposition notification type | '00000010'B |  | TS 24.282 [87] clause 15.2.5 | DELIVERED |
|  | '00000011'B |  |  | READ |
|  | '00000100'B |  |  | DELIVERED_READ |
| Date and time | The current date and time | The Date and time value is an unsigned integer containing UTC time of the time when a message was sent, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). | TS 24.282 [87] clause 15.2.8 |  |
| Conversation ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD received from the UE | The Conversation ID contains a number uniquely identifying the conversation. The value is a universally unique identifier. | TS 24.282 [87] clause 15.2.9 |  |
| Message ID | Same value as in the corresponding SDS SIGNALLING PAYLOAD received from the UE | The Message ID contains a number uniquely identifying a message. The value is a universally unique identifier | TS 24.282 [87] clause 15.2.10 |  |
| Application ID | Not present |  | TS 24.282 [87] clause 15.2.7 |  |
| Extended application ID | Not present |  | TS 24.282 [87] clause 15.2.24 |  |
| Sender MCData user ID | Not present |  | TS 24.282 [87] clause 15.2.15 |  |