3GPP TSG-RAN WG5 Meeting #93-e	R5-216687

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0211 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Generic Test Procedure for MCPTT CO call establishment using a pre-established session 5.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Incorrect procedure called in EXCEPTION |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Procedure called in EXCEPTION corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and names may result in incorrect MCX test case implementation, test case can not be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.9	Generic Test Procedure for MCPTT CO call establishment using a pre-established session

5.3.9.1	Initial conditions

As specified in the test case which calls the procedure.

5.3.9.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.9.3	Procedure

Table 5.3.9.3-1: MCPTT CO call establishment using a pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1-1a7 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.13 'Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session' starting with step 2 take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP REFER message to request the establishment of an MCPTT call using a pre-established session? | --> | SIP REFER | - | P |
| 3 | The SS (MCPTT Server) responds with a SIP 200 (OK) message indicating that the MCPTT call has been established | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a Connect message | <-- | Connect | - | - |
| 5 | Check: Does the UE (MCPTT Client) send an Acknowledgement in response to the Connect message? | --> | Acknowledge | - | P |

5.3.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none