3GPP TSG-RAN WG5 Meeting #93-e	R5-216704

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0228 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCX generic test procedures for SIP MESSAGE message flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - No SIP MESSAGE generic test procedures for MC Services CO/CT - No SIP PUBLISH / NOTIFY generic test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SIP MESSAGE generic test procedures for MC Services defined - SIP PUBLISH / NOTIFY generic test procedure defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and names may result in incorrect MCX test case implementation, test case can not be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New: 5.3.30 to 5.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.30	Generic Test Procedure for MCX SIP MESSAGE Request - Accept CO

5.3.30.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT or MCVideo

5.3.30.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.30.3	Procedure

Table 5.3.30.3-1: MCX SIP MESSAGE Request - Accept CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment take place. Dependent on the MC service calling the procedure the Generic test procedure 5.4.3 for MCPTT, 5.4.3A for MCVideo or 5.4.3B for MCData is executed. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX server) sends SIP MESSAGE accepting the request. | <-- | SIP MESSAGE | - | - |
| 5 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 6 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.30.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.31	Generic Test Procedure for MCX SIP MESSAGE Request - Accept CT

5.3.31.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT or MCVideo

5.3.31.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.31.3	Procedure

Table 5.3.31.3-1: MCX SIP MESSAGE Request - Accept CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment take place. Dependent on the MC service calling the procedure the Generic test procedure 5.4.3 for MCPTT, 5.4.3A for MCVideo or 5.4.3B for MCData is executed. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 5 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 6 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.31.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.32	Generic Test Procedure for MCX SIP MESSAGE CO

5.3.32.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.32.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.32.3	Procedure

Table 5.3.32.3-1: MCX SIP MESSAGE CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment take place. Dependent on the MC service calling the procedure the Generic test procedure 5.4.3 for MCPTT, 5.4.3A for MCVideo or 5.4.3B for MCData is executed. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.32.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.33	Generic Test Procedure for MCX SIP MESSAGE CT

5.3.33.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.33.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.33.3	Procedure

Table 5.3.33.3-1: MCX SIP MESSAGE CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment take place. Dependent on the MC service calling the procedure the Generic test procedure 5.4.4 for MCPTT, 5.4.4A for MCVideo or 5.4.4B for MCData is executed. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | EXCEPTION: The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.33.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.34	Generic Test Procedure for MCX Group Affiliation Status Change

5.3.34.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.34.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.34.3	Procedure

Table 5.3.34.3-1: MCX Group Affiliation Status Change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment take place. Dependent on the MC service calling the procedure the Generic test procedure 5.4.4 for MCPTT, 5.4.4A for MCVideo or 5.4.4B for MCData is executed. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH message? | --> | SIP PUBLISH | - | P |
| 3 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a SIP NOTIFY message informing about the status change progress. | <-- | SIP NOTIFY | - | - |
| 5 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 6 | The SS sends a SIP NOTIFY informing about the affiliation status of the user. | <-- | SIP NOTIFY | - | - |
| 7 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 8 | EXCEPTION: The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.34.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None