3GPP TSG-RAN WG5 Meeting #93-e	R5-216729

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0248 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT Test Case 6.1.2.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Generic procedures 5.3.10 and 5.3.12 were renamed as generic procedure was generalised for all services - Further Conditions for media control messages were introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Call of generic procedure renamed - Conditions applied in Floor control messages - Reference and further editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0207, 0219, 0222 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.2.12	On-network / Chat Group Call / Join Chat Group Session / Upgrade to Emergency / Cancel Emergency / Upgrade to Imminent Peril / Cancel Imminent Peril / Client Originated (CT)

6.1.2.12.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { the MCPTT User requests to initiate an MCPTT On-demand Chat Group Call with implicit floor control }

then { UE (MCPTT Client) sends a SIP INVITE message and respects the floor control imposed by the MCPTT Server and provides floor granted notification to the MCPTT User }

}

(2)

with { UE (MCPTT Client) having an ongoing On-demand Chat Group Call }

ensure that {

when { UE (MCPTT Client) receives a SIP re-INVITE message to upgrade the MCPTT group session to an emergency condition }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message and respects the floor control imposed by the MCPTT Server }

}

(3)

with { UE (MCPTT Client) participating in an ongoing Emergency Chat Group Call }

ensure that {

when { UE (MCPTT Client) receives a SIP re-INVITE message to cancel the in-progress emergency condition }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message and respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) participating in an ongoing On-demand Chat Group Call }

ensure that {

when { UE (MCPTT Client) receives a SIP re-INVITE message to upgrade the MCPTT group session to an imminent peril condition }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message and respects the floor control imposed by the MCPTT Server }

}

(5)

with { UE (MCPTT Client) participating in an ongoing Emergency Chat Group Call }

ensure that {

when { UE (MCPTT Client) receives a SIP re-INVITE message to cancel the in-progress imminent peril condition }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message and respects the floor control imposed by the MCPTT Server }

}

(6)

with { UE (MCPTT Client) participating in an ongoing On-demand Pre-arranged Group Call with Manual  Commencement Mode }

ensure that {

when { UE (MCPTT Client) receives a SIP BYE message to end the Chat Group Call }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }

}

6.1.2.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 24.379, clauses 10.1.2.2.1.1, 10.1.2.2.1.2, 6.2.6, TS 24.380, clauses 6.2.4.2.3, 6.2.4.3.3, 6.2.4.3.2. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379, clause 10.1.2.2.1.1]

Upon receiving a request from an MCPTT user to initiate or join an MCPTT group session using an MCPTT group identity, identifying a chat MCPTT group, the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

…

3)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

7)	should include the "timer" option tag in the Supported header field;

8)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the refresher parameter is omitted. If included, the refresher parameter shall be set to "uac";

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCPTT function serving the MCPTT user;

NOTE 1:	The MCPTT client is configured with public service identity identifying the participating MCPTT function serving the MCPTT user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

…

13)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "chat";

b)	the <mcptt-request-uri> element set to the group identity; and

c)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client;

NOTE 2:	The MCPTT ID of the originating MCPTT user is not included in the body, as this will be inserted into the body of the SIP INVITE request that is sent by the originating participating MCPTT function.

14)	shall include in the SIP INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

15)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

16)	shall send the SIP INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5]; and

2)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" or the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted", the MCPTT client shall perform the actions specified in subclause 6.2.8.1.4.

[TS 24.379, clause 10.1.2.2.1.2]

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request the MCPTT client:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT emergency group call and an indication that this is an MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

c)	shall set the MCPTT emergency group state to "MEG 2: in-progress";

d)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";and

e)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress";

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "false":

i)	should display to the MCPTT user an indication of the MCPTT emergency alert cancellation and the MCPTT ID of the MCPTT user cancelling the MCPTT emergency alert; and

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body including an <originated-by> element:

A)	should display to the MCPTT user the MCPTT ID contained in the <originated-by> element of the MCPTT user that originated the MCPTT emergency alert; and

B)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user, shall set the MCPTT emergency alert state to "MEA 1: no-alert";

c)	shall set the MCPTT emergency group state to "MEG 1: no-emergency"; and

d)	if the MCPTT emergency group call state of the group is set to "MEGC 3: emergency-call-granted", shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable";

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call;

b)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

c)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

5)	may check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

7)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

9)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

10)	if the SIP re-INVITE request was received within a pre-established session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

NOTE:	The SIP re-INVITE request can be received within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is received within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

11)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

[TS 24.379, clause 6.2.6]

Upon receiving a SIP BYE request, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

2)	shall send SIP 200 (OK) response towards MCPTT server according to 3GPP TS 24.229 [4].

[TS 24.380, clause 6.2.4.2.3]

When an MCPTT call is established, the terminating floor participant:

1.	shall create an instance of a 'Floor participant state transition diagram for basic operation'; and

2.	shall enter the 'U: has no permission' state.

NOTE:	From a floor participant perspective the MCPTT call is established when the application and signalling plane sends the SIP 200 (OK) response.

[TS 24.380, clause 6.2.4.3.3]

Upon receiving the Floor Taken message, the floor participant:

1.	if the first bit in the subtype of the Floor Taken message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '2' (Floor Taken); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	may provide a floor taken notification to the user;

3.	if the Floor Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	should start the optional timer T103 (End of RTP media); and

5.	shall remain in the 'U: has no permission' state.

[TS 24.380, clause 6.2.4.3.2]

Upon receiving a Floor Idle message, the participant:

1.	if the first bit in the subtype of the Floor Idle message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '5' (Floor Idle); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	may provide floor idle notification to the user, if it has not already done so;

3.	shall stop the optional timer T103 (End of RTP media), if it is running; and

4.	shall remain in the 'U: has no permission' state.

6.1.2.12.3	Test description

6.1.2.12.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.1.2.12.3.2	Test procedure sequence

Table 6.1.2.12.3.2-1: Main Behaviour

| St |  | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 |  | Make the MCPTT User request the establishment of an MCPTT On-demand chat group call with implicit floor control. (NOTE 1) | - | - | - | - |
| 2 |  | Check: Does the UE (MCPTT client) perform the Generic Test Procedure for MCPTT CO establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.7.3-1 for the establishment of an MCPTT On-demand chat group call? Option b.ii in TS 36.579-1 [2] Table 5.3.7.3-1 is used. | - | - | 1 | P |
| 3-6 |  | Void | - |  | - | - |
| 7 |  | Check: Does the UE (MCPTT client) provide floor granted notification to the MCPTT User? (NOTE 1) | - | - | 1 | P |
| 7A |  | Make the MCPTT User release the floor. (NOTE 1) | - | - | - | - |
| 7B |  | The UE (MCPTT client) performs Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1. | - | - | - | - |
| 8 |  | Check: Is the Generic Test Procedure for MCX CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to upgrade the call to an emergency chat group call correctly performed? | - | - | 2 | P |
| 9-9A |  | Void |  | - | - | - |
| 10 |  | The SS (MCPTT server) sends a Floor Taken messages with an acknowledgement required. | <-- | Floor Taken | - | - |
| 11 |  | Check: Does the UE (MCPTT client) send a Floor Ack message in response to the Floor Taken message? | --> | Floor Ack | 2 | P |
| 12 |  | The SS sends a Floor Idle message with no acknowledgement required. | <-- | Floor Idle | - | - |
| 13 |  | Check: Is the Generic Test Procedure for MCX CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to cancel the emergency condition correctly performed? |  |  | 3 | P |
| 14-14A |  | Void |  | - | - | - |
| 15 |  | The SS (MCPTT server) sends a Floor Taken messages with an acknowledgement required. | <-- | Floor Taken | - | - |
| 16 |  | Check: Does the UE (MCPTT client) send a Floor Ack message in response to the Floor Taken message? | --> | Floor Ack | 3 | P |
| 17 |  | The SS sends a Floor Idle message with no acknowledgement required. | <-- | Floor Idle | - | - |
| 18 |  | Check: Is the Generic Test Procedure for MCX CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to upgrade the call to an imminent peril chat group call correctly performed? | - | - | 4 | P |
| 19-19A |  | Void | - | - | - | - |
| 20 |  | The SS (MCPTT server) sends a Floor Taken messages with an acknowledgement required. | <-- | Floor Taken | - | - |
| 21 |  | Check: Does the UE (MCPTT client) send a Floor Ack message in response to the Floor Taken message? | --> | Floor Ack | 4 | P |
| 22 |  | The SS sends a Floor Idle message with no acknowledgement required. | <-- | Floor Idle | - | - |
| 23 |  | Check: Is the Generic Test Procedure for MCX CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to cancel the imminent peril condition correctly performed? | - | - | 5 | P |
| 24-24A |  | Void | - | - | - | - |
| 25 |  | The SS (MCPTT server) sends a Floor Taken messages with an acknowledgement required. | <-- | Floor Taken | - | - |
| 26 |  | Check: Does the UE (MCPTT client) send a Floor Ack message in response to the Floor Taken message? | --> | Floor Ack | 5 | P |
| 27 |  | The SS sends a Floor Idle message with no acknowledgement required. | <-- | Floor Idle | - | - |
| 28 |  | Check: Is the Generic Test Procedure for MCX CT call release as described in TS 36.579-1 [2] Table 5.3.12.3-1 to end the On-demand Pre-arranged Group Call correctly performed? | - | - | 6 | P |
| 29 |  | Void | -- | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |  |

6.1.2.12.3.3	Specific message contents

Table 6.1.2.12.3.3-1: SIP INVITE (step 2, Table 6.1.2.12.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.1.2.12.3.3-1A |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.2.12.3.3-2 |  |  |  |

Table 6.1.2.12.3.3-1A: SDP in SIP INVITE (Tables 6.1.2.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, conditions INITIAL_SDP_OFFER and IMPLICIT_GRANT_REQUESTED

Table 6.1.2.12.3.3-1B: SIP 200 (OK) (step 2, Table 6.1.2.12.3.3-1; step 4, TS 36.579-1 [2] Table 5.3.7.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.17.1.2-1 condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.1.2.12.3.3-1C |  |  |  |

Table 6.1.2.12.3.3-1C: SDP in SIP 200 (OK) (Table 6.1.2.12.3.3-1B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1 conditions SDP_ANSWER, IMPLICIT_GRANT_REQUESTED

Table 6.1.2.12.3.3-2: MCPTT-Info in SIP INVITE (Table 6.1.2.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2-1, conditions INVITE_REFER, CHAT-GROUP-CALL

Table 6.1.2.12.3.3-3: Floor Granted (step 2, Table 6.1.2.12.3.3-1; step 6a1, TS 36.579-1 [2] Table 5.3.7.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-1, condition ON-NETWORK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-4: SIP INVITE (step 8, Table 6.1.2.12.3.3-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 conditions re_INVITE, EMERGENCY-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.2.12.3.3-5 |  |  |  |

Table 6.1.2.12.3.3-5: MCPTT-Info in SIP INVITE (Table 6.1.2.12.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 condition CHAT-GROUP-CALL, EMERGENCY-CALL

Table 6.1.2.12.3.3-5A: SIP 200 (OK) (step 8, Table 6.1.2.12.3.3-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: Table 5.5.2.17.1.1-1 condition INVITE-RSP

Table 6.1.2.12.3.3-6: Floor Taken (step 10, Table 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK, EMERGENCY-CALL, ACK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-7: Floor Ack (steps 11, 16, 21, 26, Table 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.11-1 condition UPLINK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| Message Type | "00010010" | Floor Ack message for Floor Taken message which requested acknowledgment |  |

Table 6.1.2.12.3.3-8: Floor Idle (step 12, Table 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.6-1 condition EMERGENCY-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-9: SIP INVITE (step 13, Table 6.1.2.12.3.3-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  | MCPTT Info |  |  |
| MIME body part |  |  |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.2.12.3.3-10 |  |  |  |

Table 6.1.2.12.3.3-10: MCPTT-Info in SIP INVITE (Table 6.1.2.12.3.3-9)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 condition CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| emergency-ind | "false" |  |  |  |
| alert-ind | "false" |  |  |  |

Table 6.1.2.12.3.3-10A: SIP 200 (OK) (step 13, Table 6.1.2.12.3.3-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: Table 5.5.2.17.1.1-1 condition INVITE-RSP

Table 6.1.2.12.3.3-11: Floor Taken (steps 15, 25, 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK, ACK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-12: Floor Idle (steps 17, 27, Table 6.1.2.12.3.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.6-1

Table 6.1.2.12.3.3-13: SIP INVITE (step 18, Table 6.1.2.12.3.3-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 conditions re_INVITE, IMMPERIL-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  | MCPTT Info |  |  |
| MIME body part |  |  |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.2.12.3.3-14 |  |  |  |

Table 6.1.2.12.3.3-14: MCPTT-Info in SIP INVITE (Table 6.1.2.12.3.3-13)

Derivation Path: TS 36.579-1 [2], Table5.5.3.2.2-1 condition CHAT-GROUP-CALL, IMMPERIL-CALL

Table 6.1.2.12.3.3-14A: SIP 200 (OK) (step 18, Table 6.1.2.12.3.3-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: Table 5.5.2.17.1.1-1 condition INVITE-RSP

Table 6.1.2.12.3.3-15: Floor Taken (step 20, Table 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK, IMMPERIL-CALL, ACK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-16: Floor Idle (step 22, Table 6.1.2.12.3.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.6-1 condition IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.2.12.3.3-17: SIP INVITE (step 23, Table 6.1.2.12.3.3-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition re_INVITE, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.2.12.3.3-18 |  |  |  |

Table 6.1.2.12.3.3-18: MCPTT-Info in SIP INVITE (Table 6.1.2.12.3.3-17)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 condition CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| imminentperil-ind | "false" |  |  |  |

Table 6.1.2.12.3.3-18A: SIP 200 (OK) (step 23, Table 6.1.2.12.3.3-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: Table 5.5.2.17.1.1-1 condition INVITE-RSP

Table 6.1.2.12.3.3-19: Void